3GPP TSG- WG4 Meeting # 114	R4-2500140

# Athens, Greece, February 17 – 22, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on HPUE MSD

Agenda Item:	7.1.1.1.1

Document for:	Approval

Introduction

In RAN4#113, the WF[1] for MSD rules was approved with simplifications to replace the existing HPUE MSD tables.

Agreement: - For new Rel-19 inter-band CA combinations within FR1, and for MSD due to harmonic, receiver harmonic mixing and cross-band isolation interference, the 1UL HPUE MSD requirements are specified using a delta MSD relative to the default power class 3 MSD as: PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5, - Companies are invited to evaluate two options, - option 1: RAN4 keeps on specifying two sets PC2 requirements [4,5]: - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB, - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB, - option 2: RAN4 specifies a single set PC2 requirement [3]: - one PC2 LUT/equation PC2 MSD = PC31Tx MSD + PC2, e.g. plateau at ~6dB. - To ensure that 1UL PC1.5 MSD is greater than 1UL PC2 2Tx MSD, companies are invited to derive the HPUE delta MSD LUT/Equations values assuming: - a [9dB] 1Tx Main/Div interference imbalance. Imbalance assumptions less than 9dB are not precluded, - [0.5]dB RevIM (Rtx) and 3dB interferer increase for harmonic/harmonic mixing and for crossband isolation. . - Handling of band combination without MSD for PC3 but with small or moderate PC2/PC1.5 MSD: - option 1: FFS LUT rule and FFS on test point configuration - option 2: Legacy HPUE MSD tables may still be used to capture an exception MSD test point - option 3: no HPUE MSD requirement for those cases. - For proponents of LUT simplification approach, FFS the LUT granularity (e.g. 1dB steps) Agreement: - Companies are invited to - Provide 2UL HPUE MSD simplification rules to cover the Rel-19 supported 2UL configurations using either equations/LUT or single value de-ratings relative to the default power class 3 MSD, - Study means to minimize the number of LUTs / set of equations that cover all cases. - Check if all Rel-19 cases have been correctly captured and to look at the initial proposals in [4] and [7]. - FFS if only IMD orders are sufficient to design LUT/ set of equations or if both the IMD order and the IMD indexes should be accounted for. - FFS to account for other IMD contributors than just the power amplifier.

This paper is to provide further thinking concerning the HPUE MSD rules in Rel-19.

Simplification for HPUE IMD

In [2][3][4][5], plenty of impressive observations and various ways to simplifying the HPUE IMD are presented, which is quite helpful to analyse the HPUE MSD simplification issues.

When calculating the IMD MSD value, the parameters listed in Table 1 are unchanged for different power class in CA_nA-nB.

Table 1 Parameters for MSD analysis (CA_nA-nB)

| Parameter | value |
| --- | --- |
| IP2/3/4/5: Antenna switch (dBm) | 115/68/55/53 |
| IP2/3/4/5: Duplexer (dBm) | 100/74/55/53 |
| IP2/3/4/5: PA forward (dBm) | 28/30/28/27 |
| IP2/3/4/5: PA reverse (dBm) | 38/28/32/32 |
| IP2/3/4/5: LNA (dBm) | 6/-6/-6/-10 |
| Antenna isolation (dB) | 10 |
| PCB isolation (dB) | 65 |
| Diplexer isolation (dB) | 15 |
| L-H diplexer IL (dB) | 0.5 |
| M-H diplexer IL (dB) | 1 |
| Antenna switch IL (dB) | 1 |
| Duplexer IL (dB) | 4 |
| Band nA Tx-Rx duplexer rejection (dB) | xx |
| Band nB Tx-Rx duplexer rejection (dB) | xx |
| PA gain | 25 |
| nA Channel BW (MHz) | 5 |

The coefficient related to IMD order for each uplink component applies to the value of diplexer, antenna switch, duplexer for both main path and diversity path, and band A/B PA forward /reverse mixing, resulting in MSD value for the band combination. Compared to PC3 IMD calculation, the main difference lies in output power for the two uplink components with PC2/PC1.5 in one band combination. It is noted that for a band combination with two bands, it could be implemented either with 2Tx or 3Tx.

Table 7.3A.5-1: IMD for PC3 CA NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 Table 7.3A.5-1a: IMD for PC2 CA NOTE 1: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 Table 7.3A.5-1b: IMD for PC1.5 CA NOTE 1: This band combination is specified for inter-band UL CA with UL MIMO or Tx diversity capabilites, and the transmitter shall be set at min (+23 dBm, PCMAX_L,f,c) for the band with single Tx antenna connector as defined in clause 6.2A.4, and set at min (+27.8 dBm, PCMAX_L,f,c) for the band with two Tx antenna connectors as defined in clause 6.2H.3 or 6.2L.3.4.

For PC2 IMD MSD, it was agreed in Rel-18[7] that the requirements are same for 3Tx and 2Tx.

- 3Tx MSD for PC2 IMD: The requirements specified based on 1Tx-1Tx UL configuration are applicable for 1Tx-2Tx UL configuration. - It is allowed for companies to check the requirements for particular cases. - 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band A is 23dBm and band B is 24.8dBm+24.8dBm with 2Tx

For PC 1.5 IMD with 3Tx, according to the RF front end architecture in [6], compared to 2Tx PC2 MSD analysis, there is one more forward IMD and reverse IMD from the 2nd PA of band n78 in the other branch for 3Tx PC1.5 CA_n3-n78. The isolation of antenna, PCB and diplexers, etc, between the two branch is quite large, and there is barely a little contribution from the 2nd PA to the PC1.5 MSD value. Therefore, only 24.8dBm output power from the band with 2Tx is considered.

Figure 1 IMD MSD for CA_n3-n78 with 1Tx or 2Tx @ n78

Observation 1: For 3Tx IMD analysis, there is barely a little contribution from the 2nd PA to the PC1.5 MSD value. Therefore, only 24.8dBm output power from the band with 2Tx is considered.

In the existing specification, not PA configuration but the power of each uplink component is illustrated. With 3dB power increasement for transmitter of PC2 than that of PC3, and (24.8dBm-20dBm) improvement for each Tx antenna connector of PC1.5 than that of PC3. The delta_MSD between different power classes are listed in Table 2 below.

Table 2 Delta_MSD for HPUE and the coefficient for IMD order

| IMD order | Sum of the coefficient for the two uplink components | Proposed Delta_MSD btw PC2 and PC3 | Proposed Delta_MSD btw PC1.5 and PC2 |
| --- | --- | --- | --- |
| IMD2 | 2 | 6dB, i.e., 2*(23dBm-20dBm) | 1*(24.8dBm -23dBm) |
| IMD3 | 3 | 9dB, i.e., 3*(23dBm-20dBm) | 1*(24.8dBm -23dBm), or 2*(24.8dBm -23dBm) |
| IMD4 | 4 | 12dB, i.e., 4*(23dBm-20dBm) | Min:1*(24.8dBm -23dBm), or Max:3*(24.8dBm -23dBm) |
| IMD5 | 5 | 15dB, i.e., 5*(23dBm-20dBm) | Min:1*(24.8dBm -23dBm), or Max:4*(24.8dBm -23dBm) |

For delta_MSD of PC2 and PC3, (6, 9, 12, 15) corresponded to IMD2/3/4/5 should be considered.

Margin could also be considered based on the Delta_MSD btw PC1.5/PC2 and the default power class for testing. 1.5dB can be considered.

Proposal 1: For delta_MSD of PC2, [(6, 9, 12, 15) +margin] dBm corresponded to IMD2/3/4/5 should be considered.

- Margin = 1.5 can be considered.

The delta_MSD between PC1.5 and PC2 does not exhibit a clear or consistent pattern based on the existing requirements. Unlike PC2, the MSD for PC1.5 may vary depending on whether 3Tx or 2Tx is used, primarily due to differences in PA configurations for the constituent bands. Additionally, particularly for high-order intermodulation distortion (IMD), such as IMD4, which can result from components like 2f2-2f1 or 3f2-f1, the positions of the IMD products and the PA output power are key factors that contribute to the significant differences in delta_MSD between PC1.5 and PC2.

Proposal 2: For delta_MSD between PC1.5 and PC3, on top of the value between PC2 and PC3, additional value with worst case could be considered, i.e. (N-1)*1.8, where N is the order of IMD.

Moreover, for higher order IMD, the MSD value with PC3 is 0, such as CA_n8-n77 with IMD4, and CA_n7-n20 with IMD3, and the delta_MSD of PC2/PC1.5 may not be fully applied. Case by case analysis may still be required for higher order IMD.

Proposal 2: For higher order IMD, the MSD value with PC3 is 0, and the delta_MSD of PC2/PC1.5 may not be fully applied. Case by case analysis may still be required for higher order IMD.

Simplification for HPUE Harmonic

In RAN4#113 WF[1], it was discussed that whether the two sets PC2 requirement considering both PC21Tx and PC22Tx should be specified for harmonic. Given that the MSD difference for PC21Tx and PC22Tx is up to 5dB, two sets PC2 requirements for harmonic should be defined.

Proposal 3: Two sets of PC2 harmonic requirements for PC21Tx and PC22Tx should be defined. PC21Tx = 3dB, PC22Tx= 8dB for harmonic MSD.

Simplification for HPUE Harmonic mixing and cross band isolation

For harmonic mixing with the order of UL1/DLx, the uplink is fundamental harmonic, and DLx is up to the harmonic rejection of local oscillator. Only the different output power should be considered. For the harmonic mixing order of UL1/DL3 and UL1/UL5, it is quite evident that delta_MSD btw PC2 and PC3 is 3dB, and delta_MSD btw PC1.5 and PC3 is 6dB. For UL1/DL2 and UL1/UL4, delta_MSD btw PC2 and PC3 is 2.5dB, and delta_MSD btw PC1.5 and PC3 is 5dB.

Observation 2: For the harmonic mixing order of UL1/DLx (x=2,3,4,5), the delta_MSD btw PC2 and PC3 is up to 3dB, and delta_MSD btw PC1.5 and PC3 is up to 6dB.

However, for ULx/DLy, including UL3/DL4, UL4/DL3 and UL2/DL3 respectively, although the delta_MSD roughly follows the values in Observation 2, the MSD values for some of the band combinations are weird and may need to be revisited. CA_n39-n41, CA_n25-n41 and CA_n40-n78.

At least for the band combinations with same order harmonic mixing, such as CA_n40-n78 with UL2/DL3, the delta_MSD should be similar to others, if no specific reasons. Otherwise, it would be difficult to find HPUE MSD rules with some inaccurate MSD values.

Proposal 4: For ULx/DLy (x>1, y>1, x≠y), the HPUE calculation for some of the band combinations should be revisited. PC2 = 3dB, PC1.5= 6dB for harmonic mixing with UL1/DLx (x=2,3,4,5)

For cross band isolation, one uplink transmission is assumed. The MSD is up to filter capability for different bands. Case by case analysis is required.

Proposal 5: The MSD of cross band isolation is up to filter capability for different bands. Case by case analysis is required.

Based on the above analysis and observations, it is evident that theoretical rules can be established for certain cases, although some of these rules may rely on simplified assumptions. Nevertheless, the derived HPUE MSD requirements should remain theoretically sound. Simply fitting formulas to existing requirements in some instances may lead to incorrect conclusions, as certain MSD requirements in the spec themselves could be inaccurate or flawed.

Proposal 6: The derivation of HPUE MSD from the baseline power class should be grounded in theoretical soundness rather than merely relying on fitting formulas to existing requirements.

Conclusion

In this contribution we discussed on HPUE MSD rules, according to the analysis, we have the following observations and proposals:

Observation 1: For 3Tx IMD analysis, there is barely a little contribution from the 2nd PA to the PC1.5 MSD value. Therefore, only 24.8dBm output power from the band with 2Tx is considered.

Observation 2: For the harmonic mixing order of UL1/DLx (x=2,3,4,5), the delta_MSD btw PC2 and PC3 is up to 3dB, and delta_MSD btw PC1.5 and PC3 is up to 6dB.

Proposal 1: For delta_MSD of PC2, [(6, 9, 12, 15) +margin] dBm corresponded to IMD2/3/4/5 should be considered.

- Margin = 1.5 can be considered.

Proposal 2: For higher order IMD, the MSD value with PC3 is 0, and the delta_MSD of PC2/PC1.5 may not be fully applied. Case by case analysis may still be required for higher order IMD.

Proposal 3: Two sets of PC2 harmonic requirements for PC21Tx and PC22Tx should be defined. PC21Tx = 3dB, PC22Tx= 8dB for harmonic MSD.

Proposal 4: For ULx/DLy (x>1, y>1, x≠y), the HPUE calculation for some of the band combinations should be revisited. PC2 = 3dB, PC1.5= 6dB for harmonic mixing with UL1/DLx (x=2,3,4,5).

Proposal 5: The MSD of cross band isolation is up to filter capability for different bands. Case by case analysis is required.

Proposal 6: The derivation of HPUE MSD from the baseline power class should be grounded in theoretical soundness rather than merely relying on fitting formulas to existing requirements.

- References

[1] R4-2420380, WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#113, November, 2024

[2] R4-2417784, Further discussion on HPUE MSD rules, Murata, RAN4#113, November, 2024

[3] R4-2417794, Simplification of HPUE 2UL IMD MSDs, Skyworks Solutions, Inc., RAN4#113, November, 2024

[4] R4-2417999, HPUE MSD requirements, Nokia, RAN4#113, November, 2024

[5] R4-2419082, MSD for HPUE, Qualcomm France, RAN4#113, November, 2024

[6] R4-2307146, On UE MSD requirements for FWA UE supporting 3T for two bands, Huawei, RAN4#107, May, 2023

[7] RP-232937 SR of 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, OPPO, RAN#102, December, 2023

- Annex

A.1 IMD delta_MSD comparisons

Reference Table 1: Delta_MSD comparison for IMD2

|  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n1-n77 | n1 | 1950 | 5 | 25 | 2140 | 29.8 | 35.8 | 6 |  |  | FDD | IMD2 |
|  | n77 | 4090 | 10 | 50 | 4090 |  | N/A |  |  |  | TDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | 32.10 | 6.1 | 35.2 | 9.2 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | 32.1 | 6.1 | 37.5 | 11.5 | FDD | IMD24 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | 34.33 | 3.33 | 40 | 9 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 |  | N/A |  |  |  | TDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | 37 | 6 | 41.8 | 10.8 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | 31.9 | 5.9 |  |  | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 | 3575 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | 31.9 | 5.9 |  |  | FDD | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | 32.10 | 6.1 |  |  | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |

Reference Table 2: Delta_MSD comparison for IMD3

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | Duplex mode |  |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 15.6 | 24.6 | 9 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 |  | N/A |  | TDD | N/A |

Reference Table 3: Delta_MSD comparison for IMD4

|  |  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) |  | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n1-n774 | n1 | 1950 |  | 5 | 25 | 2140 | 8.0 | 17.8 | 9.8 |  |  | FDD | IMD4 |
|  | n77 | 3710 |  | 10 | 50 | 3710 |  | N/A |  |  |  | TDD | N/A |
| CA_n1-n78 | n1 | 1950 |  | 5 | 25 | 2140 | 8 | 17.8 | 9.8 |  |  | FDD | IMD4 |
|  | n78 | 3710 |  | 10 | 50 | 3710 |  | N/A |  |  |  | TDD | N/A |
| CA_n2-n77 | n2 | 1900 |  | 5 | 25 | 1980 | 5 | 19.10 | 14.1 | 26.4 | 21.4 | FDD | IMD4 |
|  | n77 | 3720 |  | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n77 | n5 | 844 |  | 5 | 25 | 889 | 8.3 | 18.6 | 10.3 | 25.5 | 17.2 | FDD | IMD44,13 |
|  | n77 | 3421 |  | 10 | 50 | 3421 |  | N/A |  |  |  | TDD | N/A |
| CA_n7-n77 | n7 | 2540 |  | 5 | 25 | 2660 | 7.1 | [15.8] | 8.7 | 29.7 | 22.6 | FDD | IMD4 |
|  | n77 | 3870 |  | 10 | 50 | 3870 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n77 | n25 | 1900 |  | 5 | 25 | 1980 | 8.0 | 19.1 | 11.1 | 25.8 | 17.8 | FDD | IMD4 |
|  | n77 | 3720 |  | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n41-n71 | n41 | 2614 |  | 5 | 25 | 2614 |  | N/A |  |  |  | TDD | N/A |
|  | n71 | 665 |  | 5 | 25 | 619 | 11 | 16.3 | 5.3 | 25.4 | 14.4 | FDD | IMD4 |
| CA_n3-n41 | n3 | 1740 |  | 5 | 25 | 1835 | 8.2 | 18.4 | 10.2 |  |  | FDD | IMD4 |
|  | n41 | 2657.5 |  | 10 | 50 | 2657.5 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n77 | n3 | 1765 |  | 5 | 25 | 1860 | 8 | 18.5 | 10.5 |  |  | FDD | IMD4 |
|  | n77 | 3435 |  | 10 | 50 | 3435 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n78 | n3 | 1765 |  | 5 | 25 | 1860 | 8 | 18.5 | 10.5 |  |  | FDD | IMD4 |
|  | n78 | 3435 |  | 10 | 50 | 3435 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n78 | n5 | 844 |  | 5 | 25 | 889 | 8.3 | 18.6 | 10.3 |  |  | FDD | IMD4 |
|  | n78 | 3421 |  | 10 | 50 | 3421 |  | N/A |  |  |  | TDD | N/A |
| CA_n8-n77 | n8 | 897.5 |  | 5 | 25 | 942.5 | void | 15.5 |  |  |  | FDD | IMD4 |
|  | n77 | 3635 |  | 10 | 50 | 3635 |  | N/A |  |  |  | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 |  | 5 | 25 | 942.5 | 8.3 | 15.5 | 7.2 |  |  | FDD | IMD4 |
|  | n78 | 3635 |  | 10 | 50 | 3635 |  | N/A |  |  |  | TDD | N/A |
| CA_n18-n77 | n18 | 827.5 |  | 5 | 25 | 872.5 | 8.4 | 17.5 | 9.1 |  |  | FDD | IMD416 |
|  | n77 | 3355 |  | 10 | 50 | 3355 |  | N/A |  |  |  | TDD | N/A |
| CA_n26-n78 | n26 | 836.5 |  | 5 | 25 | 881.5 | 11.1 | 23.8 | 12.7 |  |  | FDD | IMD4 |
|  | n78 | 3391 |  | 10 | 50 | 3391 |  | N/A |  |  |  | TDD | N/A |
| CA_n30-n77 | n30 | 2310 |  | 5 | 25 | 2355 | 8.0 | 17.6 | 9.6 |  |  | FDD | IMD4 |
|  | n77 | 3487.5 |  | 10 | 50 | 3487.5 |  | N/A |  |  |  | TDD | N/A |

Reference Table 4: Delta_MSD comparison for IMD5

|  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | DeltaMSD | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n2-n774 | n2 | 1885 | 5 | 25 | 1965 | 8 | 20.0 | 12 | 28 | 20 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n772 | n5 | 826.5 | 5 | 25 | 871.5 | 5.5 | Void | Void | 24.3 | 18.8 | FDD | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 |  |  |  | N/A |  | TDD | N/A |
| CA_n25-n77 | n25 | 1885 | 5 | 25 | 1965 | 5 | Void | Void | 29.8 | 24.8 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 |  |  |  |  |  | TDD | N/A |
| CA_n66-n77 | n66 | 1760 | 5 | 25 | 2160 | 5.0 | 11.27 | 6.27 | 26 | 21 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n70-n77 | n70 | 1697.5 | 5 | 25 | 1997.5 | 5.0 | 18.4 | 13.4 | 27.9 | 22.9 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 |  | N/A |  |  |  | TDD | N/A |
| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | 11.4 | 5.9 | 16 | 10.5 | FDD | IMD513 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 |  | N/A |  |  |  | TDD | N/A |

A.2 Harmonic delta_MSD comparisons (only the 1st test points are illustrated)

Reference Table 5: Delta_MSD comparison for Harmonic

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | PC2 MSD_1Tx | DeltaMSD | PC2 MSD_2Tx | DeltaMSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) | (dB) | (dB) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 12 | 10 | 23.9 | 27.1 | 3.2 | 32.3 | 8.4 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 | 10 | 23.9 | 26.9 | 3 | 31.9 | 8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 | 10 | 23.9 | 26.9 | 3 | 31.9 | 8 | NOTE 2 | UL2/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 | 10 | 13 | 15.9 | 2.9 | 20 | 7 | NOTE 2 | UL3/DL1 direct-hit |
| n71 | n25 | 5 | 15 | 8 | 5 | 6.9 | 9.4 | 2.5 | 12.9 | 6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 10 | 10.8 | 13.6 | 2.8 | 17.8 | 7 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 | 10 | 12 | 14.9 | 2.9 | 19 | 7 | NOTE 2 | UL5/DL1 direct-hit |
| n71 | n77 | 5 | 15 | 5 | 10 | 10.4 | 13.2 | 2.8 | 17.4 | 7 | NOTE 5 | UL5/DL1 direct-hit |

A.3 Harmonic mixing delta_MSD comparisons (only the 1st test points are illustrated)

Reference Table 6: Delta_MSD comparison for Harmonic mixing UL1/DL2

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n2 | 10 | 15 | 25 | 5 | 6.7 | 9.2 | 2.5 | 11.8 | 5.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 5 | 5.6 | 7.9 | 2.3 | 10.6 | 5 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 4 | UL1/DL2 |

Reference Table 7: Delta_MSD comparison for Harmonic mixing UL1/DL3

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_2Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |  |  |
| n25 | n713 | 5 | 15 | 25 | 5 | 26.8 | 29.5 | 3 | 34.5 | 7.7 |  |  | NOTE 4 | UL1/DL3 |
| n40 | n28 | 5 | 15 | 6 | 5 | 37.8 | 40.8 | 3 |  |  | 43.8 | 6 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 5 | 26.3 | 29.3 | 3 |  |  |  |  | NOTE 7 | UL1/DL3 |

Reference Table 8: Delta_MSD comparison for Harmonic mixing UL1/DL4

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n5 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 |  |  | NOTE 5 | UL1/DL4 |
| n78 | n5 | 10 | 15 | 25 | 5 | 5.7 | [8.1] | 2.4 |  |  | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.6 | 4.9 | NOTE 5 | UL1/DL4 |
| n78 | n26 | 10 | 15 | 25 | 5 | 5.4 |  |  | 10.1 | 4.7 |  | UL1/DL4 |

Reference Table 9: Delta_MSD comparison for Harmonic mixing UL1/DL5

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n12 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 | 5 | 25 | 28.0 | 3 | 31 | 6 | NOTE 1 | UL1/DL5 |

Reference Table 10: Delta_MSD comparison for Harmonic mixing ULx/DLy

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_2Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |  |  |
| n41 | n39 | 10 | 15 | 8 | 5 | 4.3 | 6.1 | 1.8 |  |  | 10.5 | 6.2 | NOTE 9 | UL3/DL4 |
| n25 | n41 | 5 | 15 | 25 | 10 | 1.3 | 2.2 | 0.9 | 3.2 | 1.9 |  |  | NOTE 11 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 5 | 9.3 | 10.6 | 1.3 |  |  |  |  | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 10 | 8.9 | 11.6 | 2.7 |  |  | 14.5 | 5.6 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 9.3 | 12.1 | 2.8 |  |  | 14.9 | 5.6 | NOTE 8 | UL4/DL3 |
| n77 | n7 | 10 | 15 | 12 | 5 | 14.7 | 17.7 | 3 |  |  | 17.5 | 2.8 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 5 | 10.4 | 13.2 | 2.8 |  |  | 16.1 | 5.7 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 11.7 | 14.5 | 2.8 |  |  | 16.1 | 4.4 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 11.7 | 14.5 | 2.8 |  |  | 17.5 | 5.8 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 10 | 14.7 | 13.2 | -1.5 |  |  | 16.1 | 1.4 | NOTE 3 | UL2/DL3 |

Reference Table 11: Delta_MSD comparison for Cross band isolation

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | PC3 MSD | PC2-1Tx MSD | Delta_PC2-PC3 | PC2- 2Tx MSD | Delta_PC2-2Tx-PC3 | PC1.5 MSD | Delta_PC1.5-PC3 | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) |  | (dB) |  |  |  |  |  |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 23 | 26 | 3 | 32.3 | 9.3 |  |  | ACLR1 |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | 31.1 | 3 |  |  | 37.9 | 9.8 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | 34.4 | 3 |  |  | 41.2 | 9.8 | ACLR2 |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2 | 10.9 | 2.7 | 15.9 | 7.7 |  |  | ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0 | 0.7 | 0.7 | 0.9 | 0.9 |  |  | >ACLR2 |
| n3 | n1 | 1760 | 50 | 15 | 50 (RBstart=220) | 2112.5 | 5 |  | 0.8 |  | 1.1 |  |  |  | >ACLR2 |
| n3 | n7 | 1760 | 50 | 15 | 50 (RBstart=220) | 2622.5 | 5 |  | 0.5 |  | 0.7 |  |  |  | >ACLR2 |
| n7 | n1 | 2525 | 50 | 15 | 45 (RBstart=0) | 2167.5 | 5 |  | 0.8 |  | 1.1 |  |  |  | >ACLR2 |
| n7 | n3 | 2525 | 50 | 15 | 45 (RBstart=0) | 1877.5 | 5 | 0.6 | 1.1 | 0.5 | 1.5 | 0.9 |  |  | >ACLR2 |
| n25 | n41 | 1760 | 40 | 15 | 40 (RBstart=176) | 2501 | 10 |  | 0.8 |  | 1 |  |  |  | >ACLR2 |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 |  | 0.7 |  | 0.9 |  |  |  | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | 4.7 | 1.4 |  |  |  |  | >ACLR2 |
| n40 | n3 | 2350 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 |  | 0.6 |  |  |  | 1.2 |  | >ACLR2 |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 18.1 | 20.8 | 2.7 |  |  | 23.5 | 5.4 | >ACLR2 |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 0.6 | 2.3 | 1.7 |  |  | 3.9 | 3.3 | >ACLR2 |
| n41 | n25 | 2546 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | 0.6 | 1.6 | 1 |  |  | 2.8 | 2.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | 2.7 | 1.1 |  |  | 6.7 | 5.1 | >ACLR2 |
| n41 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 10.5 | 13.1 | 2.6 |  |  | 7.7 | -2.8 | >ACLR2 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | 10.5 | 2.2 |  |  | 13.3 | 5 | >ACLR2 |
| n41 | n79 | 2640 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 |  | 3.1 |  |  |  |  |  | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.2 | 1.9 | 0.7 | 3.3 | 2.1 |  |  | >ACLR2 |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.4 | 2.2 | 0.8 | 3.8 | 2.4 |  |  | >ACLR2 |
| n66 | n41 | 1760 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 0.4 | 0.8 | 0.4 | 1 | 0.6 |  |  | >ACLR2 |
| n66 | n70 | 1760 | 40 | 15 | 216 (RBstart=0) | 1997.5 | 5 |  | 1.9 |  | 3.3 |  |  |  | >ACLR2 |
| n70 | n66 | 1702.5 | 15 | 15 | 75 (RBstart=4) | 2112.5 | 5 |  | 0.4 |  | 0.5 |  |  |  | >ACLR2 |
| n77 | n2 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n25 | 3350 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n30 | 3350 | 100 | 30 | 270 (RBstart=0) | 2357.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | 6.5 | 2 |  |  | 2.2 | -2.3 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n78 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | 1.2 | -3.3 |  |  | 2.2 | -2.3 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 2 | 5 | 3 |  |  | 8 | 6 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 2 | 5 | 3 |  |  | 8 | 6 | >ACLR2 |
| n79 | n41 | 4450 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 |  | 3.5 |  |  |  |  |  | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 2.6 | 5.6 | 3 |  |  | 8.6 | 6 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 2.6 | 5.6 | 3 |  |  | 8.6 | 6 | >ACLR2 |