| 3GPP TR 38.749 V0.1.0 (2025-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Protection of Earth Exploration Satellite Service (EESS) (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Work item objective	7

5	Study outcome in previous releases for EESS	7

6	Regulation background	7

7	A-MPR evaluation for EESS protection in the frequency bands 23.6-24 GHz	7

7.1	A-MPR simulation scenarios and conditions	7

7.1.1	Common to single carrier and carrier aggregation	7

7.1.2	Single carrier	7

7.1.3	Carrier aggregation	8

7.2	A-MPR simulation results for single carrier	8

7.2.1      Scenario A	8

7.2.2      Scenario B	8

7.3	A-MPR simulation results for carrier aggregation	8

7.3.1       A-MPR simulation results	8

7.4      Summary of A-MPR simulation results	8

8	UE RF requirements	8

9	BS RF requirements	8

Annex: Change history	8

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for the protection of Earth Exploration Satellite Service (EESS) in Rel-19 according to WRC-23 conclusion. It covers the A-MPR/MPR simulation evaluations for protection of the EESS -5dBm/200MHz in the frequency bands 23.6-24 GHz, analysis on A-MPR needed to comply with the regulatory requirements and NS values aligned with the WRC-23 outcome.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] ITU-R Radio Regulations, 2020 Edition

[3] EU Decision 2019/784, Commission Implementing Decision (EU) 2019/784 of 14 May 2019 on harmonisation of the 24,25-27,5 GHz frequency band for terrestrial systems capable of providing wireless broadband electronic communications services in the Union.

[4] EU Decision 2020/590, Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band.

[5] ECC(20) 055 Annex 18, CEPT letter dated 6 March 2020 ‘CEPT response on additional input regarding the impact of the WRC-19 outcome on the harmonised technical conditions for the 26 GHz band’

[6] ECC Decision(18)06, Harmonised technical conditions for Mobile/Fixed Communications Networks (MFCN) in the band 24.25- 27.5 GHz, Approved 06 July 2019, last amended 20 November 2020.

[7] R4-2417982, Discussion on Rel-19 EESS, Huawei, Hisilicon, RAN4#113

[8] R4-2415726, 24 GHz EESS band phase 2 requirements and UE compliance, Qualcomm Incorporated, RAN4#112bis

[9] R4-2415329, Initial considerations on EESS protection requirements, Apple, RAN4#112bis

[10] R4-2416065, mmWave UE spurious emission UE RF, Nokia, RAN4#112bis

It is preferred that the reference to TR 21.905 be the first in the list.

3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

EESS		Earth Exploration-Satellite Service

IMT		International Mobile Telecommunications

ITU-R		Radiocommunication Sector of the International Telecommunication Union

WRC		World Radiocommunication Conference

4	Work item objective

For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258, where

- Introduce new corresponding Network Signalling(s) and associated A-MPR(s) if needed by taking into the consideration of the relationship with the existing NS_200, NS_202, NS_203, CA_NS_200, CA_NS_202 and CA_NS_203

- All the power classes (FR2 PC1, PC2, PC3, PC4, PC5, PC6, PC7) are considered in the above work

For BS/IAB/Repeater, reflect the latest regulatory requirements on protection of Earth Exploration Satellite Service requirements if any

5	Study outcome in previous releases for EESS

In Rel-15, protection of EESS frequency range 23.6-24GHz was discussed with the following study outcomes:

- In RAN4#88 meeting, CR(R4-1811499) was agreed to introduce NS_201and CA_NS_201 for NR band n258, compliance with the additional spurious emission requirement of -8dBm/200MHz based on ECC decision [6].

-  In RAN4#92 meeting, Draft CR(R4-1910261) was endorsed to introduce NS_202 and CA_NS_202 for NR bands n257 and n258 with updates NS_201 and CA_NS_201 A-MPR requirements, compliance with -10dBm/100MHz additional spurious emission requirement to keep the consistence with ERC Rec. 74-01[3].

- In RAN4#95e meeting, CR(R4-2006785) was agreed to update the NS_202 requirements to stay consistent with EU emissions requirement changes (+1 dBm/200MHz).

In Rel-16, protection of EESS frequency range 23.6-24GHz was also discussed based on the outcome of WRC-19, the study outcome can be summarized in the following:

- In RAN4#97e meeting, CR (R4-2016785) was agreed to introduce NS_203 and CA_NS_203 for NR band n258 from Rel-15 TS38.101-2 specification, compliance with +1dBm/200MHz additional spurious emission requirement. ModifiedMPRbehaviour indication for NS_203 and CA_NS_203 is also captured in the specification.

- NS_201 and CA_NS_201 are obsolete.

- In addition, RAN4 agreed to apply -5dBm/200MHz after Sep, 2027. But it is not reflected in RAN4 specification that revision is needed before the requirement taking effect, given that companies didn’t come to consensus. More details can be referred to the clause 7.5.4 in TR38.817-01.

6	Regulation background

6.1	ITU

The ITU-R World Radiocommunication Conference in 2019 (WRC-19) identified on a global basis the 24.25-27.5 GHz (’26 GHz’) frequency band for IMT, with focus on 5G.  The ITU-R Radio Regulations [2], as amended by WRC-19 Resolution 750, introduced the technical conditions for use of the 26 GHz frequency band, as captured in Figure 6.1-1.

Figure 6.1-1: Extract from Radio Regulations [2] - WRC-19 Resolution 750

To protect EESS services, ITU-R has specified two sets of unwanted emissions limits for IMT base stations and IMT mobile:

- The first set with less stringent limits (respectively -33dBW/200MHz and -29dBW/200MHz) is applicable to BS and mobile brought into use before the 1 September 2027.

- The second set with more stringent limits (respectively -39dBW/200MHz and -35dBW/200MHz) is applicable to BS and mobile brought into use after the 1 September 2027.

6.2	Europe and CEPT

The ECC Decision(18)06, amended on 20 November 2020 [6], on harmonised technical conditions for mobile/fixed communications networks (MFCN) in the 26 GHz (24.25-27.5 GHz) band reflects the objective of CEPT to harmonise the 24.25-27.5 GHz band for Europe for 5G. Like ITU-R, WRC-19 Resolution 750, this ECC Decision specifies as well 2 sets of unwanted emissions in the 23.6-24.0 GHz frequency range but with different application date, the transition date being 1 January 2024 in this ECC Decision.

In April 2020, the EU Decision 2019/784 [3] was amended with the Commission Implementing Decision 2020/590 [4]. Based on CEPT recommendation [5], the EU decided to set the transition to the second stage limits to an earlier date (1 January 2024) , this to better protect EESS in the 23.6-24.0 GHz frequency range while early enabling 26GHz deployment. Such decision is binding to all EU countries.

Figure 6.2-1: Annex of EU Decision 2020/590 [4] amending EU Decision 2019/784

7	A-MPR evaluation and analysis for EESS protection in the frequency bands 23.6-24 GHz

7.1	A-MPR simulation scenarios and conditions

Editor note: All the scenarios for single carrier and CA are summarized.

7.1.1	Common to single carrier and carrier aggregation

Editor note: Simulations conditions are captured in a Figure with frequency range. Conditions are captured l the scenarios for single carrier and CA are summarized. Here also some explanation is made, e.g., not only conventional study to place UE channel bandwidth/cumulative channel bandwidth adjacent to EESS protection range, but also study frequency offsets required for A-MPR to be 0 dB.

7.1.2	Single carrier

Editor note: This may be combined with CA if most of the content is similar.

7.1.3	Carrier aggregation

7.2	A-MPR simulation results for single carrier

In [7], PA calibration point should follow current definition in Spec 38.101-2:

  - The waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

  - Calculate MPR as total backoff needed for 256QAM from this calibration point.

For band n258, to satisfy the RF requirements of EESS with -5dBm/200MHz additional spurious emission, SEM, ACLR, etc, the maximum power back-off is approximately 4.5dB for PC1, which is not higher than MPR or A-MPR corresponded to NS_202, even though the margin is taken into consideration.

Figure 1 Power back-off for band n258 to meet RF requirements including EESS

In [8], for NS_205 introduced in correspondence to NS_203,

|  | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| PC1 | 9-1x3=6 | < 5-1/3*3, i.e. < 4 | 8.0 (reducing to 6.0 with enough frequency offset) |
| PC2/3/4 | 9-1x3=6 | < 9-1/3*3, i.e. < 8 | 9.0 (reducing to 6.0 with enough frequency offset) |

In [9], the power back-off is simulated as 1dB for 400MHz channel placed at the lower band edge of n258. The carrier frequency is at 24.45GHz. This leaves a 250MHz gap between the lower edge of the channel and the upper edge of the protected EESS range.

Figure 1: Power back-off of QPSK modulated 400MHz carrier with fc at 24.45GHz

In [10], it is evaluated that the A-MPR for power class 3 is 0dB.

Figure 1: 400 MHz full allocation (n257, n258)

Figure 2: 100 MHz full allocation (n257, n258)

Figure 3: 1RB allocation (n258)

7.3	A-MPR simulation results for carrier aggregation

7.3.1       A-MPR simulation results

7.4      Summary of A-MPR simulation results

8	UE RF requirements

9	BS RF requirements

The Earth Exploration Satellite Service (EESS) protection requirements in the frequency range 23.6-24.0 GHz (as per WRC-19 Radio Regulations Resolution 750 [7] and EU Decision 2020/590 [8]) were specified via additional OTA OBUE requirements and additional OTA spurious emission requirements in the BS specifications.

The only modification in legacy specifications was that ITU-R applicability date (i.e. 1 September 2027) was considered in those specifications. Within this WI, the BS additional OTA OBUE limits and additional OTA transmitter spurious emissions requirements for protection of EESS were defined as:

-The emission level of -3dBm/200MHz applies to BS brought into use on, or before 1 September 2027 in countries not adopting EU Decision 2020/590 [], while

- The emission level of -9dBm/200MHz applies to BS brought into use after 1 September 2027, or to BS in countries adopting EU Decision 2020/590 [].

Annex:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting |  | TDoc |  | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | RAN4-112bis |  | R4-2417199 |  |  |  |  | TR Skeleton | 0.0.1 |
| 2023-11 | RAN4-113 | R4-2418920 |  |  |  | EESS protection - TP to TR 38.749 on regulatory aspects |  |
| 2023-11 | RAN4-113 |  | R4-2420355 |  |  |  |  | TP for TR38.749 BS requirements (Clause 9) |  |
| 2025-02 | RAN4-114 |  | R4-2500144 |  |  |  |  | TP for TR38.749: EESS in Rel-19 |  |
| 2025-02 | RAN4-114 |  | R4-2500945 |  |  |  |  | TP for TR38.749 Study outcome in previous releases (Clause 5) |  |
| 2025-02 | RAN4-114 |  |  | R4-2502128 |  |  |  | TP to TR 38.749: BS RF requirements section correction (clause 9) |  |
| 2025-02 | RAN4-114 | R4-2500145 |  |  |  | TR 38.749 v0.1.0 Protection of Earth Exploration Satellite Service (EESS) | 0.1.0 |