3GPP TSG-RAN WG4 Meeting #114                                           R4-2500157

Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on general aspects of LP-WUS

Agenda Item:	7.26.1

Document for:	Approval

Introduction

The RF requirements for UE and BS in LP-WUS have been extensively discussed in several meetings. However, we feel that the basic principles for supporting LP-WUS in RAN4 among companies are not well-aligned. This misalignment not only hinders the progress of standardization but, more critically, could compromise the utility of this feature, which is likely to be a foundational aspect of 6G for UE implementation from the very start.

The definitions of LP-WUR types were presented in the previous meeting, which we believe is a critical foundational issue that should be resolved prior to addressing the detailed requirements. We resubmit the contribution and look forward to more discussions on the proposed LP-WUR types.

Discussion

During the SI phase, a question lingered among companies in RAN4 regarding the power consumption target for LP-WUR. RAN4 subsequently forwarded this inquiry to RAN1 [1]. Although RAN1 didn't provide a direct response, we can glean valuable insights from the study outcomes and conclusions presented in the technical report [2]. Below are some key information cited from the TR 38.869:

Table 6.3-2 in TR 38.869 and Table 18 in TR 38.840 listed the power consumption model for LR and MR respectively.

Table 6.3-2: Power model for LP-WUR [2]

| Power State | Relative Power (unit) | Additional transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| Off[1] | 0.001/ 0.02/ 0.1, For 0.1, [oscillator option 3/4] are not used for envelope detection based receiver | TLR, ramp-up *(PON-POFF)/2 Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840 | company to report TLR, ramp-up |
| On[2] | 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 |  |  |
| [1] Relative power unit for LP-WUR 'off' state, i.e., the LP-WUR does not perform monitoring [2] Relative power unit for LP-WUR 'on' state, i.e., the LP-WUR performs monitoring |  |  |  |

-	For evaluation, 10/20/30 for LP-WUR ON power state are not used for envelope detection-based receiver for LP-WUS monitoring.

-	For evaluation, 10/20/30 for LP-WUR ON power state are used for OFDM receiver when noise figure is less than [MR noise figure + 2.5dB], [0.2/0.5/1/2/4] for LP-WUS can be assumed for other NF values larger than [MR noise figure + 2.5dB]

Table 18: UE power consumption model for FR1 [3]

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

It is noted that the models for both LR and MR are using relative values. To gain a clearer impression, we can substitute these relative values with some actual numbers. For MR, the relative value for PDCCH-only or SSB/CSI-RS proc. is 100, which in normal case could be considered as the power consumption of single Rx. In comparison, for OOK-1/2/4 with homodyne/zero-IF architecture with baseband envelope detection, most companies consider the relative power consumption value for LR is around 1, which means the typical power consumption for LR is 1/100 of the MR. Typically, the power consumption for single Rx chain including ADC is around 100mW, in other words, the primary design goal for LR power consumption is around 1 mW, which is also a widely accepted target for Wi-Fi WUR.

Observation 1: For OOK based LP-WUR, the evaluation in RAN1 by most companies in SI is targeting to the maximum relative power consumption of 1, in other words, around 1mW.

Observation 2: For OFDM based LP-WUR, the upper bound considered by companies in SI for the relative power consumption could be in the comparable level as MR.

During the SI study phase, RAN1 made the following conclusion for OOK based and OFDMA-based signals for the LP-WUR architecture,

- For OOK waveform, 3 types of receiver architectures have been considered: RF envelope detection, heterodyne architecture with IF envelope detection, and homodyne/zero-IF architecture with baseband envelope detection. - For homodyne/zero-IF architecture, the reported relative power consumption for ON state is in the range of 0.05~4, and the reported noise figure is in the range of 10~16 dB. - For OFDMA-based signals, sequence-based signals have been considered, where the LP WUR performs either time-domain correlation without FFT or frequency-domain correlation after FFT. - For time-domain correlation, the reported relative power consumption for ON state is in the range of 0.15~10/30, and the reported noise figure is in the range of 7~25. - For frequency-domain correlation, the reported relative power consumption for ON state is in the range of 1~30, and the reported noise figure is in the range of 7~12.

It has been observed that power consumption is also closely linked to NF. From an implementation perspective, the front-end LNA alone may consume around 10mW, which is far above the 1mW design target for OOK-based LP-WUR. However, power consumption in the tens of mW range could be a possible choice for OFDM-based LP-WUR, as suggested by the SI conclusions in the TR.

In RAN4, the group is exploring the possibility of defining separate requirements for OOK-based and OFDM-based LP-WUR, which could lead to two distinct implementation paths. One would prioritize ultra-low power consumption with a target of around 1mW, while the other could aim for enhanced performance and coverage, with power consumption around tens of mW but still below the MR threshold. It’s essential to recognize that these two approaches target entirely different design objectives and would entail fundamentally different implementations.

In addition, zero-IF receiver is selected as baseline architecture for both OOK based and OFDM based LP-WUR.

Issue 2-2-2: Baseline architecture for OOK-based LP-WUS

Agreement in RAN4#112:

  - use zero-IF receiver as a baseline architecture for envelop based LP_WUR

Issue 2-2-1: Baseline architecture for FR1 OFDM-based LP-WUS

Agreements in RAN4#112bis:

  - use zero-IF receiver as a baseline RF architecture for OFDM based LP-WUR.

With above assumption, there is possibility that supporting OOK and OFDM based LP-WUS signals share the same receiver.

Observation 3: OOK-based and OFDM-based LP-WUS signals may share the same LP-WUR architecture.

From the power consumption model in the TR as follows, it is noted that OFDM receiver with ultra-low power consumption also is combined with larger noise figure.

- For evaluation, 10/20/30 for LP-WUR ON power state are not used for envelope detection based receiver for LP-WUS monitoring. - For evaluation, 10/20/30 for LP-WUR ON power state are used for OFDM receiver when noise figure is less than [MR noise figure + 2.5dB], [0.2/0.5/1/2/4] for LP-WUS can be assumed for other NF values larger than [MR noise figure + 2.5dB]

Clearly, power consumption is a threshold to distinguish two different LP-WUR implementations and consequently two different set of requirements. To facilitate the following discussion in RAN4, we may consider the following options:

- Option 1: UE indicates different LP-WUR types like those for NTN UE, with requirements tied to each specific UE type

  - LP-WUR UE type 1: targets ultra-low power consumption

    - Larger NF: e.g. MR NF + 8dB

    - LO with worse phase noise: profiles like those in Ref R4-2415201, R4-2415780, and R4-2309204

    - Relaxed Rx requirements due to implementation constraints imposed by lower power consumption

    - Supports OOK and/or OFDM LP-WUS based on UE capability

  - LP-WUR UE type 2: targets moderate power consumption, but still lower than MR

    - Smaller NF: e.g. MR NF + 2dB

    - LO with phase noise profile comparable to MR

    - Slightly relaxed Rx requirements or similar to MR Rx requirements

    - Supports OOK and/or OFDM LP-WUS based on UE capability

- Option 2: Requirements tailored specifically for OOK-based and OFDM-based receivers

Way forward in RAN4#110bis:

  - Discussions could be separated for OOK-based receiver and OFDM-based receiver. And latter stage to discuss whether a generic requirement could be defined, or not.

    - Further discuss the possibility of a dedicated set of requirements for OOK LP-WUR

    - FFS whether there is any other factors should be considered for LP-WUR requirements discussion

  - SNR and NF may be different for these two types.

- Option 3: Requirements are set based on UE indicated support for specific waveforms, similar to the intraBandENDC-Support approach, allowing several enumerated values. For LP-WUS, this could be a UE capability lpWUS-Support with {OOK, OFDM, both} values, where "both" indicates support for both OOK and OFDM signals.

For above possible options to specify the LP-WUR requirements, if the group agrees to define separate LP-WUR requirements for UEs supporting LP-WUS, we can first eliminate option 3. The aim of option 3 seems to link the LP-WUR implementation to the supported LP-WUS waveforms, with the consideration that OOK-based signals would require low power consumption and OFDM might consume more. However, a UE designed for OFDM LP-WUR with higher power consumption could also support OOK signals if needed, making it difficult for option 3 to distinguish between supported WUS signals and match them directly to specific LP-WUR implementations.

Similarly, option 2 presents the same issue: an OFDM-based receiver can also support OOK signals, and vice versa. Furthermore, OFDM does not necessarily mean increased power consumption. Therefore, the primary consideration should be the UE types, each designed for different power consumption targets but capable of supporting both OOK and OFDM signals based on the UE capability indication.

This leads us to favor option 1 for specifying LP-WUR RF requirements, as it better addresses the differing perspectives among companies in defining the requirements. Option 1 allows a unified specification approach that takes into account the power consumption constraints and performance requirements without compromising adaptability across scenarios. While option 3 alone is not sufficient to specify all the requirements, when combined with option 1 regarding UE types and the indicated LP-WUS signals, it can effectively deliver the complete set of requirements.

Specifically, while the targeted power consumption for different LP-WUR types isn’t explicitly stated in the specification, the underlying assumptions for each type serve as valuable guidance for implementation considerations.

Proposal 1: RAN4 could consider defining two LP-WUR types aimed at distinct power consumption levels to facilitate requirements specification. The choice of supported WUS signals (OOK-based and/or OFDM-based) would then depend on the associated UE capabilities for each LP-WUR type.

Proposal 2: LP-WUR types could be defined with distinct noise figure assumptions.

Different LP-WUR types are defined based on different power consumption assumptions, each with distinct noise figure (NF) values compared to the MR. The requirements are specified for different LP-WUR types. The LP-WUR types are specified in Table 1 below.

Table 1: The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure (NF) [8] dB higher than the MR. |
| 2 | The LP-WUR is designed for moderate power consumption, assuming a noise figure (NF) [2] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

Conclusion

This contribution shares our thoughts on a general approach to specifying LP-WUR requirements. The SI study indicates that OOK-based LP-WUS can be supported by ultra-low power LP-WUR, while OFDM-based LP-WUS fits within a wider power consumption range. This suggests two distinct UE types, each suited to different LP-WUR implementations. Consequently, the requirements should be specified around the two different UE types.

Proposal 1: RAN4 could consider defining two LP-WUR types aimed at distinct power consumption levels to facilitate requirements specification. The choice of supported WUS signals (OOK-based and/or OFDM-based) would then depend on the associated UE capabilities for each LP-WUR type.

Proposal 2: LP-WUR types could be defined with distinct noise figure assumptions.

Different LP-WUR types are defined based on different power consumption assumptions, each with distinct noise figure (NF) values compared to the MR. The requirements are specified for different LP-WUR types. The LP-WUR types are specified in Table 1 below.

Table 1: The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure (NF) [8] dB higher than the MR. |
| 2 | The LP-WUR is designed for moderate power consumption, assuming a noise figure (NF) [2] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

- References

[1] R4-2303712, Reply LS to RAN1 on low-power wake-up receiver architectures, vivo

[2] TR 38.869, Study on low-power wake-up signal and receiver for NR

[3] TR 38.840, Study on User Equipment (UE) power saving in NR

[4] R4-2406619, WF on NR LP-WUS UE requirements, RAN4#110bis

[5] R4-2410569, WF on NR LP-WUS UE RF, RAN4#111

[6] R4-2414309, WF on LP-WUS UE RF requirements, RAN4#112

[7] R4-2417112, WF on UE RF requirements for LP-WUS, RAN4#112bis