3GPP TSG-RAN WG4 Meeting # 114	R4- 2500171

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM impacts of SBFD operation

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on the remain open issues on RRM impacts of SBFD operation described in WF [1].

Discussion

Issue 1-2-5: Requirements for RACH requirements - Proposal 1 (LGE): - RAN4 need to update the correct behavior for the RACH occasion switching operation. And other RRM impact by RACH occasion switching operation also need to check. - Proposal 2 (vivo, E///, MTK): - No impact on RACH requirements due to SBFD operation.

During last RAN2#127bis meeting, RAN2 made an agreement regarding RACH resource selection as below:

RACH resource selection - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

According to the agreement, the UE is allowed to switch from SBFD RACH occasions to legacy RACH occasions after certain (configured) number of times of RACH attempt. And about the case when UE select legacy ROs first also can be discussed further from RAN2. RAN4 need to check whether the description of random access should be update for RO switch or not.

6.2.2.2 Requirements for 4-step RA type The UE shall select the type of random access at initiation of the random access procedure based on network configuration, as specified in clause 5.1.1 in TS 38.321 [7]. The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula defined in clause 7.4 of TS 38.213 [3] and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as specified in Table 6.3.4.2-1 of TS 38.101-1 [18] for FR1 and in Table 6.3.4.2-1 of TS 38.101-2 [19] for FR2. The relative power applied to additional preambles shall have an accuracy as specified in Table 6.3.4.3-1 of TS 38.101-1 [18] for FR1 and clause 6.3.4.3 of TS38.101-2 [19] for FR2. The UE shall indicate a random access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in clause 5.1.4 in TS 38.321 [7]. The SBFD-aware UE is allowed to switch to legacy RACH occasions after certain (configured) number of times of RACH attempt in SBFD RACH occasions. The requirements in this clause apply for UE in SA operation mode or any MR-DC operation mode.

Proposal 1: RAN4 need to consider to add following sentence in 6.2.2.2 for clear SBFD-aware UE’s RACH procedure.

The detail wording can be decided after further discussion of RAN2.

- The SBFD-aware UE is allowed to switch to legacy RACH occasions after certain (configured) number of times of RACH attempt in SBFD RACH occasions.

Issue 1-2-7: Requirements for MG and BWP switch - Proposal 1 (vivo, HW): - Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification, e.g. in MG and BWP switch requirements. - Proposal 2 (MTK): - There is no need to update the term UL slot for MG and BWP switch.

Regarding the requirements for MG and BWP switch, we think the SBFD slot should be included in the applicable description for clarification. Current UL slot only description is not enough to include the SBFD operation.

Proposal 2: Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification, e.g. in MG and BWP switch requirements.

Issue 1-2-8: Requirements for UL resource muting Recommended WF - Interested companies are encouraged to bring proposals on exact RRM impacts, if any

Based on the feature of UL resource muting for PUSCH, the UE can measure CLI at muted RE. Currently RAN1 assuming comb-2 muting and up to 2 OFDM symbols in time domain. For keeping the time domain power level, the transmit power of PUSCH REs in the same OFDM symbols are considered 3dB power boosting.

RF part need to check the impact on transmit signal quality/MPR requirement. But RRM perspective, there is no spec. impact.

Proposal 3: RAN4 RRM does not need to consider the impact by UL resource muting for PUSCH feature.

Summary

Proposal 1: RAN4 need to consider to add following sentence in 6.2.2.2 for clear SBFD-aware UE’s RACH procedure.

The detail wording can be decided after further discussion of RAN2.

- The SBFD-aware UE is allowed to switch to legacy RACH occasions after certain (configured) number of times of RACH attempt in SBFD RACH occasions.

Proposal 2: Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification, e.g. in MG and BWP switch requirements.

Proposal 3: RAN4 RRM does not need to consider the impact by UL resource muting for PUSCH feature.

Reference

[1] R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei