3GPP TSG-RAN WG4 Meeting #114                                                          R4-2500225

# Athens, GR, 17th – 21st February, 2025

Source:	Apple

Title:	Discussion on Beam Management: RRM Core Requirements

Agenda Item:	7.19.3

Release:            Rel-19

Document for:	Discussion

Introduction

Rel-18 Study Item was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface with the target to study the 3GPP framework for AI/ML for air-interface corresponding to each targeted use cases (i.e., CSI feedback enhancement, beam management, and positioning accuracy enhancements for different scenarios) regarding aspects such as performance, complexity, and potential specification impact [1] [2].

According to latest WID in [2], RAN4 is required to study the interoperability and testability aspects for each use case: Specifically, RAN4 is expected to study the requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [3]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for beam management.

Discussion

## 2.1 Framework for Defining Requirements in Tests for AI/ML

From RAN1 evaluation results there are many implementation factors that could influence the AI/ML model performance. Diverse company-specific assumptions and implementations regarding model structure and parameters often lead to significant performance variations. To ensure consistency across companies' results, incorporating a reference AI/ML decoder model becomes critical in defining performance benchmarks for the UE side encoder.

From a UE processing capability limitation perspective, RAN4 will need to find a measure to access and evaluate complexity. There should be some limitations and complexity considerations when RAN4 decides on the requirements.

As an example, from RAN1 performance evaluations for the single sided BM use case, the complexity of AI/ML models reported across the companies can vary drastically as it is shown in the figure below from TR:

Fig 1: Complexity of AI/ML models from evaluation results 
in terms of FLOPs and number of parameters for BM cases

Accordingly, the performance variations across companies as outlined in Section 6.3.2.1.1 from the TR can vary from 60% accuracy to 90% accuracy where most of the performance variation could be attributed to model complexity and other differences in simulation assumptions (training procedures, training data, etc)

One question that arises is: What should be the granularity for defining the parameters to specify the reference AI/ML model for BM use case? In addition to model parameters, other factors that could affect performance include the model output type and the associated types of training data. For example, significant performance variations across companies are reported for the BM use case in section 6.3.2.3 of the TR. These variations depend on the output type, such as labels (classifier network) or L1-RSRP (CNN or MLP). For training input types for BM we could also differentiate between wide beams (SSB based, wide Tx beams) vs narrow beams (CSI-RS based, narrow Tx beams) types of set B of beams.

Similarly to the list of parameters specified for the two-sided model in CSI use case, RAN4 should investigate the set of parameters and granularity for the other use cases. (BM and positioning)

For example, for the BM case we could start with the following parameters:

- General Complexity

  - Number of parameters

  - Memory Size

  - FLOPS

- Training output type (labels vs L1-RSRP, Classifier vs MLP)

- Training Input type (narrow beams (CSI-RS) vs wide beams (SSB) Tx beams)

We provide a table with the proposed parameters for specifying reference model for BM use case. (table 1)

Proposal 1: RAN4 should investigate the specification of reference AI/ML models for BM, the associated training procedures, and the training datasets to ensure alignment of simulation results. This will help pave the way for defining performance requirements for the BM use case, with careful consideration of constraints on model complexity. To initiate the discussion, we have provided a reference table as a starting point.

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Model architecture parameters | Model type | CNN(case 1), MLP (case 1_, RNN (Case 2), LSTM (Case 2) |
|  | Fixed point representation | Int8, int16, floating point etc |
| Model Training/Inference related parameters | Training procedure | FFS (e.g Initialization method, training duration, training completion criteria, etc) |
|  | Loss function | NMSE, Cross Entropy etc. |
|  | Training datasets | Channel model (CDL) number of set -B/ set-A beams, specify TX codebooks. Best Rx beam selection (best UE beam) SSB beams (setA and setB) And /or CSI RS beams |
|  | Impairments (can be used for inference too) | RSRP measurements with impairments: Impairment types affecting set B measurements and set A measurements. |
|  | Hyperparameters | Learning rate, batch size, regularization techniques and strength, optimization algorithm, max epochs,etc. |
|  | Cross-validation details | Dataset splits for training/testing/validation |
| Complexity | Model Parameter Size |  |
|  | Model Memory Size |  |
|  | FLOPS |  |
| Scalability | Supported set A | 64 beams |
|  | Supported set B pattern | 1/4, => 16 beams |
|  | Time Window configuration (BM case 2) |  |
|  | Number of future predicted instances (BM case 2) |  |
| Performance Metric | Beam ID, RSRP accuracy | Top-K beams, values of K? |

Table 1: Parameters for Specifying Reference Model for BM use cases

## 2.2  KPIs/Test Metrics for BM Use case

In TR 38.843 the following agreements are captured  for KPIs/Test metrices for BM use case.

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered.

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

-	Option 1: RSRP accuracy

-	Option 2: Beam prediction accuracy

-	Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-	Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-	Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

-	Related measurement accuracy can be considered to determine x

-	Option 4: combinations of above options

The overhead/latency reduction should be considered for the requirements as the side condition.

For option 1, RSRP prediction accuracy focus is about the RSRP difference for the same beam. RSRP prediction accuracy is more similar as legacy RSRP measurement accuracy definition:

Absolute L1-RSRP accuracy = measured L1-RSRP of beam index n - ideal absolute L1-RSRP of beam index n

RSRP prediction accuracy metric is defined in clause 6.3.1 in TR 38.843:

For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

From RAN4#110-bis the following agreement has been reached:

Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy. Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity. Following this perspective and in accordance with legacy schemes, a minimum RSRP accuracy test should be applied to all reported/predicted beams. Additionally, in legacy systems, there is no testing to confirm if the reported beam powers correspond to the best genie beam indices. Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams.

Observation 1: From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity

Observation 2: Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams

In line with the legacy approach, we aim to ensure RSRP accuracy for all predicted beams. When using neural networks for regression, we can rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Observation 3: When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Proposal 2: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR.

When considering the necessity of testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

Proposal 3: When considering the necessity of additionally testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially considering our definition of RSRP accuracy.

During RAN4#112bis the following agreements were reached:

Issue 2-3: RSRP Absolute accuracy

Agreement:

- If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Issue 2-5: Requirements for beam predictions

Agreement:

  - Requirements to be defined:

    - Measurement period for prediction (how long will set B be measured before making a prediction)

      - FFS on whether the legacy measurement period can be reused or not.

    - Prediction accuracy

      - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc)

    - TCI state known/unknown

  - FFS on whether the other requirements are needed

Measurement Period for prediction

The legacy L1-RSRP measurement period and accuracy requirement is based on 1 shot measurement. The measurement period has resource sharing factor ‘P’, number of sample ‘M’ configured by measurement configuration, RX beam sweeping factor ‘N’ in FR2. The measurement period for Set-B would highly depend on the accuracy needed for prediction. We propose to use the legacy measurement period as a starting point and further evaluate if it is sufficient to achieve good prediction accuracy.

Observation 4: The measurement period for Set-B would depend on the accuracy needed for prediction.

Proposal 4: Use legacy measurement period as a starting point for Set-B for evaluation if sufficient to achieve good prediction accuracy.

For the FFS part in bracket [in top-K beams based on ground-truth L1-RSRP], we would like to put several possibilities on the table for further analysis.

Option 1: The index i may be any beam index in top-K beams based on ground-truth L1-RSRP

Option 2:  The index i may be any beam index in top-K beams based on predicted L1-RSRP

For option 1, if the predicted beams selected to meet accuracy requirement are based on ground-truth L1-RSRP, there may be a case to be noticeable as follows.

Case: The Top-K beam based on predicted L1-RSRP (e.g., beam X) is not any of the Top-K beams based on ground-truth L1-RSRP

To explore the probability of this case occurring in actual predictions, we have some simulations, and the results provided below.  We can see that due to mis-selection of top-K predicted beams the genie RSRP (at the predicted indices) is less than the Genie RSRP of the genie beam indices, indicating that there is certain probability that the Top-K predicted beam is not any of the Top-K beams based on ground-truth RSRP

Fig 1: Top-K predicted beam is not any of the top-K beams

Observation 5: There is a probability that the Top-K predicted beams may not align with the Top-K beams determined by ground-truth RSRP measurements

Under Option 1, the predicted L1-RSRP of beam X is not required to meet the accuracy requirement because beam X is not among the Top-K beams based on ground-truth L1-RSRP. However, the predicted L1-RSRP of beam X must still be reported if necessary. For DL Tx beam prediction, the UE is responsible for predicting and reporting the strongest beams along with their corresponding predicted RSRP values. This is because any reported beam could potentially be selected by the network as the Tx beam for transmission, meaning the predicted RSRP for each reported beam must maintain a certain level of accuracy. Consequently, if Option 1 is adopted, the accuracy of the predicted RSRP cannot be guaranteed in cases where the Top-K beams based on predicted L1-RSRP do not match the Top-K beams based on ground-truth L1-RSRP.

Observation 6 :For RSRP absolute accuracy, if the index i can represent any beam within the Top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for beams that are part of the Top-K predicted beams but not included in the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Based on the above analysis, we propose Option 2, where the index i refers to any beam within the Top-K beams based on predicted L1-RSRP. This definition ensures that the accuracy of all predicted RSRP values for the reported beams is maintained

Proposal 5: The absolute RSRP accuracy for AI/ML-based beam prediction is defined as the difference between the predicted L1-RSRP of beam index i and the ground-truth L1-RSRP of beam index i. Here, the index i can represent any beam within the Top-K beams based on predicted L1-RSRP.

RAN1 has already agreed that the beam index must be included as a mandatory element in BM reporting. From a key performance indicators (KPIs) standpoint, the accuracy of predicting the beam index is a critical factor. RAN1 is also actively defining the details for performance monitoring, where beam prediction accuracy has been established as a primary metric. Taking these aspects into account, beam prediction accuracy serves as one of the core metrics/KPIs for establishing BM requirements

Proposal 6: RAN4 should further evaluate whether beam prediction accuracy should be established as a metric/KPI for defining beam management requirements

Proposal 7: If RAN4 agrees that beam prediction accuracy should be established for defining requirements a beam rank metric should be used that considers the cumulative beak ranking other. Potential options are:

- Kendall/Spearman correlation

- Normalized Discounted Cumulative Gain (nDCG)

- Rank-biased overlap

Known condition for TCI State

The known condition for TCI state is defined based on the period between TCI state switch and the last transmission of RS or RS QCLed with target TCI state. If the target TCI state is a predicted beam, and the RS was not transmitted to, measured and reported by UE, then we need to introduce a new definition for known condition. We also need to differentiate between UE side prediction and NW side prediction. For the case when target TCI state is a RS based on beam prediction -

UE side model: The UE predicts and reports the top K beams based on measurements on set B. Since the UE is predicting the beam(s) reported, it is known to the UE if a TCI state is received within a certain period. The known condition can be determined based on last transmission of set-B.

NW side model: The UE reports the measurements for Set B and target TCI state is based on NW side prediction, the TCI state is unknown to the UE. In case the RS of target TCI state is transmitted to the UE, measured and reported prior to the TCI state switch, then it is known to the UE.

Figure 2: Known condition for UE side prediction

Observation 7: The legacy definition for known TCI state cannot be used when target TCI state is based on predicted beam and not transmitted to and measured & reported by UE.

Observation 8: RAN4 needs to discuss known TCI for UE side and NW side prediction.

Proposal 8: For UE side model known TCI state is determined based on Set B transmission and UE report of top-K.

Proposal 9: For NW side model - 
(1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch
(2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable

For UE side Rx beam assumption, there are also different assumptions:

Alt 1: UE always uses the best Rx beam to measure set B beams and set A beams and the corresponding L1-RSRP

This imply beam sweeping in the data collaboration procedure at the UE side for set A and set B measurement.

In training data collection for NW side model based on MDT framework, the UE report the gNB beam with corresponding L1-RSRP with best UE Rx beam.

During inference, UE report the L1-RSRP and corresponding NW beam index(es) (CRI/SSBRI) assuming the best UE Rx beam.

How best UE Rx beam is determined requires advanced UE side Rx beam management implementation. For example,

UE record the best Rx beam in global coordinates during training,

During inference, based on sensor information and UE orientation, to figure out the best Rx beam to use for the set A.

During performance monitoring, best Rx beam for set A and set B respectively is used.

Alt 2: UE always use the best Rx beam for set B measurement, and UE used the best Rx beam of set B for set A measurement during training and inferencing.

This imply that UE beam sweeping for set B measurement during training and inference.

In addition, using data collection for training procedure, the RS configuration in gNB needs to indicate the root RS of set A configuration with respect to set B RS.

There can be proprietary information disclosures concern at the NW vendor. There is no agreement in RAN1 (no related proposals either in this case).

In training data collection for NW side model based on MDT framework, the UE report the gNB beam with corresponding L1-RSRP with best UE Rx beam for set B, and set A L1-RSRP with best UE Rx beam QCLed to set B beam.

During inference, UE report the set B L1-RSRP and corresponding NW beam index (CRI/SSBRI, can be more than 4) assuming the best UE Rx beam for set B.

The same QCL type D root RS to set B should be signaled by the NW, as the training phase.

Proposal 10: NW side data collection, QCL type D is signaled between set A and set B RS.

Proposal 11: For NW side model, TCI state is known if QCL source is signaled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown.

Proposal 12: (for Alt 1):  Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied.

Proposal 13: (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B),

The TCI state is known when the following is met:

Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

X = 1280 ms (legacy)

UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

TCI state and SSB of TCI state remain detectable during TCI state switching period

Otherwise, it is unknown.

## 2.3  Datasets for training/testing

For training the AI/ML models for beam management we need to agree on what the source of the training data is.  In RAN4#111 the following agreement was reached:

Issue 2-6:	Datasets for training/testing

- Proposals

  - Option 1: Training Data set to be specified in RAN4(directly or through some algorithm )

  - Option 2: Training data set to be left to implementation (companies can generate it based on knowledge of the test environment)

  - Option 3: others

- Recommended WF

  - To be discussed

Agreement:

- Vendors may take into account the test environment/conditions defined by RAN4 when training the UE

  - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training

- FFS on proposals to augment training data to avoid overfitting of UE models to the test environment

- RAN4 to strive to make the test conditions similar to field deployment conditions

We need to agree if the goal of the training data is to train the model for testing purposes only or for training the model for real deployment as well.

If the training data will be used to train one model (used in testing and deployment) it would be hard to gurantee similar performance (testing environment and in the field) since Tx codebooks could be different from vendor to vendor.

Proposal 14: RAN4 should agree that the training dataset used in testing is strictly for testing purposes. In real-world applications, it will be the responsibility of UE implementations to determine the appropriate training dataset. This could involve using a vendor-specific dataset or creating a mixed dataset sourced from multiple vendors for training their models.

RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

Observation 9: RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

From RAN4 #112 the following agreements were reached:

Issue 1-2: Channel modeling

Agreement:

- Use synthetic channels as the default assumption for conformance requirements/test.

  - RAN4 may conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data.

    - The field data can be considered for the analysis.

Issue 1-3: Testing goals

Agreement:

- The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

  - FFS on whether and how many different test configurations/parameters are used for tests.

  - LCM would be tested

For better clarity and understanding, it is beneficial to distinguish between three specific sets of scenarios/conditions:

- All possible Deployment Scenarios/Conditions: These include the entirety of scenarios in which the AI functionality is intended for deployment. These conditions encapsulate the comprehensive real-world scenarios in which the AI system will operate, with the aim of meeting the specified RAN4 requirements within this context.  The requirements could in general be met with either AI/ML or legacy procedures.

- RAN4 Specified Test Conditions: These conditions encompass the specific scenarios for which RAN4 tests will be defined, ensuring that the requirements for these tests are met. These conditions should include the identified scenario as well as the other scenarios for generalization tests.

- Non-Tested Conditions that are part of possible Deployment scenarios/conditions: This category includes additional conditions that are part of the possible deployment conditions but are not explicitly addressed during RAN4 testing. These additional conditions could be met with either AI/ML or legacy operation through LCM procedures and post deployment  (Set 1 = Set 2 union Set 3)

For RAN4 specified test conditions, the testing data could be generated across a variety of different conditions/congiguations:

- Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

- Different paterns of set B/set A

- Tx codebooks, Tx antenna architecture layout , antenna spacing

- SNR conditions

- Carrier Frequency

- Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

We have the following proposals:

Proposal 15: To define the testing data for BM and provide sufficient test environments/conditions for vendors to generate the necessary training data, RAN4 should select a set of standardized test conditions. UEs will be tested across these identified conditions, and it will be up to the UE implementation to either switch between models or employ a supermodel capable of accommodating all these conditions during. Example of test conditions/configurations are:

- Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

- Different paterns of set B/ set A

- Tx codebooks,Tx antenna architecture layout, antenna spacing

- SNR conditions

- Carrier Frequency

- Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

Proposal 16: For all potential non-tested conditions that are part of possible deployment scenarios, including additional conditions not explicitly addressed during RAN4 testing, we propose to employ Post-Deployment Procedures. These procedures will augment RAN4 conformance testing to effectively manage and assess performance across all possible deployment conditions

## 2.4 Measurement error impact and the ground truth

During RAN4#112 and RAN4112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements are copied below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

In the following section, we simulated the impact of measurement errors for an AI/ML model in inference operation. We employed spatial domain prediction with 32 predicted beams in Set A, and 8 measured beams in Set B which is subset of Set A. We trained a regression AI/ML MLP model to predict the RSRPs in set A from set B.

## 2.4.1 Methodology

We followed the procedure described in R4-2417211, where we trained an AI/ML model with a subset of samples from an ideal L1-RSRP dataset generated from the system level simulation (SLS) following the assumptions defined in TR 38.843 Table 6.3.1-1. We considered Approach A, and we used the LLS simulation assumptions defined in Table 6 of Annex I to generate the Baseband (BB) errors. We also implemented the RF errors, and we added both BB and RF errors to the subset of ideal L1-RSRP data to derive the dataset with measurement errors as follows

The results are compared to the performance metrics without considering measurement errors, for which we used another subset of samples from the ideal L1-RSRP dataset generated from the SLS and without measurement errors.

For RF error modeling, we consider following options:

•	No error: This is the baseline case where RF errors are not considered as input of the AI/ML model.

•	Gaussian (Independent for each TX beam): 95% of users are distributed within ±4.5 dB error margin with mean zero. The RF error is generated independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

•	Uniform distribution (Independent for each TX beam): All users are distributed within ±4.5 dB error margin with mean zero. The RF error is added independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

For evaluating the performance of the AI/ML prediction we employed the following metrics:

- Top-K/1 and Top-1/K prediction accuracy, where K = 1, 2, 3, 4, 5

- 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams

The set A and set B of angles are shown in Fig 3.

Figure 3: set A and set B of angles

## 2.4.2 Evaluation Results with no Errors

In this section we present results for inference with no measurement errors.  In Fig 4 we show the TopK/1 and Top1/K beam prediction with no measurement errors.

Figure 4:  TopK/1 and Top1/K beam prediction

In Fig 5 we present results for the metric:  90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam.

Figure 5: 90%-tile L1-RSRP difference of genie and top predicted beams

- In Fig 6 we present results for the metric:  90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams

Figure 6: 90%-tile L1-RSRP difference of genie and top predicted beams with same beam index

## 2.4.3 Evaluation Results with RF Gaussian errors

In this section we present results with independent (per Tx beam) Gaussian RF errors.

In Fig 7 we show results for TopK/1 and Top1/K beam prediction accuracy and degradation with RF errors. We can see that with RF Gaussian errors the TopK/1 prediction accuracy will be reduced by 11.7% for Top1 and 8.3 % for Top2.

Figure 7:  TopK/1 and Top1/K beam prediction accuracy and degradation with RF errors

In Fig 8 we present results for the metric:  90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam with RF Gaussian errors.

Figure 8:  90%-tile L1-RSRP difference of genie and top predicted beams with RF Gaussian Errors

In Fig 9 we present results for the metric:  90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams with RF Gaussian errors.

Figure 9:  90%-tile L1-RSRP difference of genie and top predicted beams with same beam index with RF errors (Degradation)

## 2.4.4 Evaluation Results with RF Gaussian errors + LLS errors

In  figure 10, we present the CDF of our simulated BB errors generated at SNR=-3dB. In figure 11, we present results for topK/1 accuracy in the presence of BB and RF errors (Gaussian). In figure 12, we present results for RSRP prediction accuracy with BB and RF (Gaussian) errors for both the absolute RSRP prediction and relative degradation due to measurement errors. In figure 13, we present results for topK/1 accuracy in the presence of BB and RF errors (Uniform). In figure 14, we present results for RSRP prediction accuracy with BB and RF (Uniform) errors for both the absolute RSRP prediction and relative degradation due to measurement errors.

Figure 10:  CDF of LLS BB errors

Figure 11:  TopK/1 and Top1/K beam prediction accuracy and degradation with RF plus LLS errors

Figure 12: 90%-tile L1-RSRP difference of genie and top predicted beams with same beam index with RF plus LLS errors

Figure 13: 90%-tile L1-RSRP difference of genie and top predicted beams with same beam index with RF plus LLS errors

Figure 14: 90%-tile L1-RSRP difference of genie and top predicted beams with same beam index with RF plus LLS errors

Proposal 17: RAN4 should consider adopting a realistic approach to model RF errors, as the prediction accuracy difference of the AI/ML model varies significantly between the worst-case scenario (independent RF errors) and the best-case scenario (same RF error for all Tx beams). To study the impact of measurement errors, the method of RF error modeling should be included in RAN4 discussions. Potential approaches include:

Using a truncated uniform distribution;

Using a truncated Gaussian distribution.

## 2.5 Generalization Issues/Associated ID

For the UE-side beam prediction models (BM-Case 1 and 2), an AI/ML-based prediction model is used to identify the optimal beam from the base station, leveraging a codebook that was included in the training data for model development.

However, if the codebook used by the base station differs from the one (or ones) used during training, the model may not be effective in predicting the best beam.

To address this challenge, RAN1 has proposed a solution involving the introduction of an associated ID approach, as outlined in the following agreement:

With the associated ID approach, the UE must store or download multiple models when codebooks vary across different vendors or environments.

This requirement could expose proprietary algorithms used by network vendors for beam management, as the indication of codebook switches may inadvertently reveal such vendor-specific details

To avoid the extra complexity of managing associated IDs across infra-vendors, a NW can equalize the differences between its own proprietary codebook and a RAN4 specified codebook for training the model. The network employs an AI/ML-based RSRP Equalizer to align UE-based predictions with those that account for the network's specific codebook (CB) implementations. This approach utilizes a one-sided AI/ML model implemented at the UE, which sends additional RSRP measurements to the network. The network then uses these measurements/predictions to equalize and adjust the UE's predictions, ensuring compatibility with its own CB design.

Proposal 18: To avoid the extra complexity of managing associated IDs across infra-vendors, develop a framework where the NW can equalize the differences between its own proprietary codebook and a RAN4 specified codebook for training the model. The network could employ a proprietary RSRP Equalizer to align UE-based predictions with those that account for the network's specific codebook (CB) implementations.

# Conclusion

In this section we summarize our discussion by listing the observations and proposals discussed in this contribution:

Observation 1: From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity

Observation 2: Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams

Observation 3: When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Observation 4: The measurement period for Set-B would depend on the accuracy needed for prediction.

Observation 5: There is a probability that the Top-K predicted beams may not align with the Top-K beams determined by ground-truth RSRP measurements

Observation 6: For RSRP absolute accuracy, if the index i can represent any beam within the Top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for beams that are part of the Top-K predicted beams but not included in the Top-K beams based on ground-truth RSRP cannot be guaranteed

Observation 7: The legacy definition for known TCI state cannot be used when target TCI state is based on predicted beam and not transmitted to and measured & reported by UE.

Observation 8: RAN4 needs to discuss known TCI for UE side and NW side prediction.

Observation 9: RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

Proposal 1: RAN4 should investigate the specification of reference AI/ML models for BM, the associated training procedures, and the training datasets to ensure alignment of simulation results. This will help pave the way for defining performance requirements for the BM use case, with careful consideration of constraints on model complexity. To initiate the discussion, we have provided a reference table as a starting point.

Proposal 2: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR.

Proposal 3: When considering the necessity of additionally testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

Proposal 4: Use legacy measurement period as a starting point for Set-B for evaluation if sufficient to achieve good prediction accuracy.

Proposal 5: The absolute RSRP accuracy for AI/ML-based beam prediction is defined as the difference between the predicted L1-RSRP of beam index i and the ground-truth L1-RSRP of beam index i. Here, the index i can represent any beam within the Top-K beams based on predicted L1-RSRP.

Proposal 6: RAN4 should further evaluate whether beam prediction accuracy should be established as a metric/KPI for defining beam management requirements

Proposal 7: If RAN4 agrees that beam prediction accuracy should be established for defining requirements a beam rank metric should be used that considers the cumulative beak ranking other. Potential options are:

- Kendall/Spearman correlation

- Normalized Discounted Cumulative Gain (nDCG)

- Rank-biased overlap

Proposal 8: For UE side model known TCI state is determined based on Set B transmission and UE report of top-K.

Proposal 9: For NW side model - 
(1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch
(2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable

Proposal 10: NW side data collection, QCL type D is signaled between set A and set B RS.

Proposal 11: For NW side model, TCI state is known if QCL source is signaled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown.

Proposal 12: (for Alt 1):  Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied.

Proposal 13: (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B),

The TCI state is known when the following is met:

Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

X = 1280 ms (legacy)

UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

TCI state and SSB of TCI state remain detectable during TCI state switching period

Otherwise, it is unknown.

Proposal 14: RAN4 should agree that the training dataset used in testing is strictly for testing purposes. In real-world applications, it will be the responsibility of UE implementations to determine the appropriate training dataset. This could involve using a vendor-specific dataset or creating a mixed dataset sourced from multiple vendors for training their models.

Proposal 15: To define the testing data for BM and provide sufficient test environments/conditions for vendors to generate the necessary training data, RAN4 should select a set of standardized test conditions. UEs will be tested across these identified conditions, and it will be up to the UE implementation to either switch between models or employ a supermodel capable of accommodating all these conditions during. Example of test conditions/configurations are:

- Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

- Different paterns of set B/ set A

- Tx codebooks,Tx antenna architecture layout, antenna spacing

- SNR conditions

- Carrier Frequency

- Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

Proposal 16: For all potential non-tested conditions that are part of possible deployment scenarios, including additional conditions not explicitly addressed during RAN4 testing, we propose to employ Post-Deployment Procedures. These procedures will augment RAN4 conformance testing to effectively manage and assess performance across all possible deployment conditions

Proposal 17: RAN4 should consider adopting a realistic approach to model RF errors, as the prediction accuracy difference of the AI/ML model varies significantly between the worst-case scenario (independent RF errors) and the best-case scenario (same RF error for all Tx beams). To study the impact of measurement errors, the method of RF error modeling should be included in RAN4 discussions. Potential approaches include:

Using a truncated uniform distribution;

Using a truncated Gaussian distribution.

Proposal 18: To avoid the extra complexity of managing associated IDs across infra-vendors, develop a framework where the NW can equalize the differences between its own proprietary codebook and a RAN4 specified codebook for training the model. The network could employ a proprietary RSRP Equalizer to align UE-based predictions with those that account for the network's specific codebook (CB) implementations.

- References

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94e.

[2] RP-242399 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,

[3] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[4] TR 38.810, v16.7.0, “Study on test methods”.