# 3GPP TSG-RAN WG4 Meeting #114	R4-2500237

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.8.3

Source:	Apple

Title:	On RRM requirement for R19 LB CA

Document for:	Discussion

# Introduction

A new WI has been approved in RAN#106 meeting, and the WID[1] is as following,

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

It mentioned in the WID that the new mechanism of carrier switching will need possible RRM requirements on RAN4. In this contribution, we discuss the scope of corresponding RRM requirement for this R19 LB CA scenario.

# RRM working scopes for discussion

As agreed in the WID, there are two cases to consider:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

The example band combination for the analysis is CA_n5A-n29A in this WI, and the case switching can be illustrated in the following figure.

Figure 1. n5A-n29A case switching example

The key point for this LB CA with SDL carrier is that UE cannot support simultaneous reception on PCC and SDL CC, i.e., in the above example, when UE switches to n29 SDL CC, UE is not able to keep connection with n5 PCC.

With such implementation limitation, the following categories of RRM requirements might be discussed further:

- Interruption requirement for carrier switching for LB CA

  - The carrier switching between the PCC and SDL CC could cause interruption to the UE

- SDL SCC related RRM requirement

  - SDL SCell measurement, e.g., which measurement cycle to follow on SDL SCell,

  - Deactivated SDL SCell measurement related interruption., e.g., interruption length and interruption applicability (whether or not to be relevant to the measCycleSCell),

  - SDL SCell activation delay and related interruption

- Potential impacts to the PCell related requirement

  - Impact to PCell L3 and L1 measurement, e.g., can UE switch back to PCC to follow SMTC for RRM measurement?

  - Impact to PCell time tracking, e.g., can switching pattern guarantee UE to have SSB for timing tracking in 160ms prior to the UL transmission?

Proposal 1: RAN4 scopes for RRM requirements for R19 LB CA can be:

- Interruption requirement for carrier switching

- SDL SCC related RRM requirement

  - SDL SCell measurement

  - Deactivated SDL SCell measurement related interruption

  - SDL SCell activation delay and related interruption

- Potential impacts to the PCell related requirement

  - Impact to PCell L3 and L1 measurement

  - Impact to PCell time tracking

# Detailed RRM requirements discussion

Based on proposal 1, we discuss the corresponding RRM requirement in this section:

- Interruption requirement for carrier switching

As shown in the following figure 2, even though the LB CA in this WI is co-located and synchronized network deployment for both DL carriers, the TA between DL and UL can cause the switching gap colliding with two slots on time domain.

Figure 2. carrier switching interruption example for LB CA

RF session from this meeting will also discuss the switching time from PCC to SDL SCC and the way back, and we think the total switching time including switching and transition can be up to 140us. Thus, in RRM we propose to have slot level interruption requirements, and in order to make the requirement forward compatible to the scenario with additional active DL non-SDL SCC, the interruption requirement shall be applied as generally as possible to both UL and DL CCs.

As illustrated in the figure 2, the slot level interruption requirement due to carrier switching for R19 LB CA can be defined as 	TswitchingSlot-length-victim+1, where Tswitching is the switching time concluded in RF session.

Proposal 2: RAN4 to define slot level interruption requirement for carrier switching in R19 LB CA.

Proposal 3: The interruption requirement for carrier switching in R19 LB CA is TswitchingSlot-length-victim+1, where Tswitching is the switching time concluded in RF session.

- SDL SCC related RRM requirement

The current deactivated SCell measurement consider the interruption for RF tuning/retuning was specified below,

If measCycleSCell is smaller than 640ms, currently UE is required to stay on deactivated SCC after SCell addition(no interruption is allowed). However, in R19 new LB CA scenario, since UE cannot keep the connection with PCell when it stays on the SDL SCC for measurement, it would be necessary to allow UE switch back to PCC for traffic/control reception/transmission after deactivated SCell measurement regardless of the measCycleSCell.

Proposal 4: For deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell.

In current requirement, the deactivated SCell measurement follows the measCycleSCell as shown in the following table,

The carrier switching pattern may or may not match the SCell measurement cycle configured by network, and therefore UE may not be able to perform SDL SCell measurement if it’s not allowed to switch to the SDL SCC. In our view, the measurement cycle in LB CA shall be decided by both measCycleSCell and carrier switching pattern.

Proposal 5: UE only performs deactivated SCell measurement following measCycleSCell and carrier switching pattern, i.e., no SDL SCell measurement is required outside switching pattern to SDL SCell.

Regarding SDL SCell activation, there are also some issues to discuss, e.g.,

- Legacy SCell activation delay is defined based on SMTC and SSB periodicity, but will such periodicity be replaced by carrier switch pattern periodicity?

- Legacy interruption for SCell activation is defined in an interruption window, like following. But do we allow UE to stay on the SDL SCell during activation period? If yes, then UE will cause interruption to PCell as the whole activation time.

- Whether or not we can reuse SSB-less SCell activation requirement for SDL SCell activation, based on the condition below.

  - Co-located and synchronized network deployment for both carriers

  - Both carriers are in a single TAG

If the legacy SSB based inter-band SCell activation procedure is assumed for LB CA, then SCell activation delay shall be defined based on switching pattern periodicity, or if UE is allowed to stay on the SDL SCell during activation time, the interruption can be as large as the whole SCell activation delay after n+1+THARQ.

Proposal 6: If the legacy SSB based inter-band SCell activation procedure is assumed for LB CA, then SCell activation delay shall be defined based on switching pattern periodicity, or if UE is allowed to stay on the SDL SCell during activation time, the interruption can be as large as the whole SCell activation delay after n+1+THARQ.

- Potential impacts to the PCell related requirement

Since carrier switching pattern will be introduced for SDL SCC in R19 LB CA, it may also impact the PCell related RRM requirement, e.g., PCell measurement and time tracking. Currently PCell L3 and L1 measurement follows SMTC periodicity and RS periodicity respectively, like following tables. However, the DL switching pattern may have larger periodicity and the SMTC or SSB periodicity, as shown in figure 3.

Figure 3. SMTC periodicity vs. switching pattern periodicity

Thus, the carrier switching pattern shall also be considered to decide the PCell L3 or L1 measurement. In our view, the more reasonable design is to make sure the switching pattern can guarantee UE has at least one RLM/BFD/L3 occasion for evaluation/measurement in every max(DRX cycle, RS periodicity (L1) or SMTC periodicity (L3)).

Regarding the UE timing, the current required UE to meet Te requirement only if at least one SSB is available in the last 160ms before UL transmission, as following. In our view, the design of switching pattern shall also make sure UE to not stay on SDL SCell for longer than 160ms, i.e., UE will have SSB reception available in last 160ms before the UL transmission on PCell. Or alternatively, since PCell and SDL SCell belongs to the same TAG and these cells are collocated and synchronized, we may allow reference cell switching between PCell and SDL SCell in same TAG for R19 DL CA, i.e., UE tracks timing on SDL SCell when it’s on SDL SCell following the switching pattern, and then such timing information can be used for UL transmission on PCell. These issues are related with the switching pattern design, and therefore we propose to consider them later.

Proposal 7: RAN4 to discuss potential impacts to the PCell related RRM requirement once the switching pattern is concluded.

# Conclusion

In this contribution, we discuss the scope of corresponding RRM requirement for this R19 LB CA scenario.

Proposal 1: RAN4 scopes for RRM requirements for R19 LB CA can be:

- Interruption requirement for carrier switching

- SDL SCC related RRM requirement

  - SDL SCell measurement

  - Deactivated SDL SCell measurement related interruption

  - SDL SCell activation delay and related interruption

- Potential impacts to the PCell related requirement

  - Impact to PCell L3 and L1 measurement

  - Impact to PCell time tracking

Proposal 2: RAN4 to define slot level interruption requirement for carrier switching in R19 LB CA.

Proposal 3: The interruption requirement for carrier switching in R19 LB CA is TswitchingSlot-length-victim+1, where Tswitching is the switching time concluded in RF session.

Proposal 4: For deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell.

Proposal 5: UE only performs deactivated SCell measurement following measCycleSCell and carrier switching pattern, i.e., no SDL SCell measurement is required outside switching pattern to SDL SCell.

Proposal 6: If the legacy SSB based inter-band SCell activation procedure is assumed for LB CA, then SCell activation delay shall be defined based on switching pattern periodicity, or if UE is allowed to stay on the SDL SCell during activation time, the interruption can be as large as the whole SCell activation delay after n+1+THARQ.

Proposal 7: RAN4 to discuss potential impacts to the PCell related RRM requirement once the switching pattern is concluded.

# References

[1] RP-243317, New WID on low band carrier aggregation via switching, Apple, RAN#106