3GPP TSG-RAN WG4 Meeting #114		R4-2500265

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.18.2

Source:	Apple

Title:	On UE demodulation performance requirements with 8RX

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#113 the UE demodulation enhancements in Rel-19 were discussed in detail and way forward [1] was agreed. In this contribution we share our views on UE demodulation requirements with 8RX.

# 2. Discussion

## General

On open issues related to general aspects, we provide our view below –

Definition of MMSE-IRC receiver capability for 8Rx UE

- Proposals

  - Option 1:
Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver.

  - Option 2:
Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI.

  - Option 3: 
Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability.

  - Option 4:
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

In Rel-18 when requirements with simplified receiver for 8RX were introduced UE capability (without signal) for testing purposes was introduced.

In R19 we are discussing the possibility of requirements with simplified receiver for MU-MIMO with 8RX. Firstly, we have not yet concluded if we will define different requirements with simplified and baseline receiver. There is no difference in processing for SU-MIMO and MU-MIMO that warrants a new capability definition with MMSE-IRC receiver. If we land up with different requirements with different UE implementations, then the UE capability from R18 can be updated to include MU-MIMO.

- RAN4 has not concluded if we would have different requirements for differ UE implementations.

- There is no difference in processing with SU-MIMO and MU-MIMO with MMSE-IRC receiver.

- If we define different requirements to cater to different UE implementations for MU-MIMO requirements in R19, update the R18 UE capability definition.

8Rx UE MMSE-IRC receiver coverage

- Proposals

  - Option 1:
RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver).

  - Option 2: 
Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition.

  - Option 3: 
Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

It is premature to assume that we need 2 sets of requirements for different UE implementations without results from companies.

We propose the following methodology:

- If the average SNR delta between simplified and baseline receiver is ≤ 2.5 dB, then define same requirements for both UE implementations

- If the average SNR delta between simplified and baseline receiver is > 2.5 dB and ≤ 3.5 dB, define the same requirements by adding additional margin to account for span

- If the average SNR delta between simplified and baseline receiver is > 3.5 dB, define different requirements

The average SNR shall be calculated after outlier results are removed, with reasonable number of results left.

- It is premature to assume that 2 sets of requirements are needed without looking at the results.

- Criteria for deciding if separate or same requirements for different UE implementation.
Calculate the average SNR for baseline and simplified receiver after removing outlier results, with a reasonable number of results left
- If the average SNR delta between simplified and baseline receiver is ≤ 2.5 dB, then define same requirements for both UE implementations
- If the average SNR delta between simplified and baseline receiver is > 2.5 dB and ≤ 3.5 dB, define the same requirements by adding additional margin to account for span
- If the average SNR delta between simplified and baseline receiver is > 3.5 dB, define different requirements

## PDSCH requirements with inter cell interference

For PDSCH requirements for inter-cell interference the following agreements were made:

Rank and MCS Agreement: - Use the following for evaluation. TBD on requirements: - Rank1: MCS17 - Rank2: MCS17 - Rank4: MCS13 - 2nd priority simulations can be removed for next meeting for both baseline and simplified receiver. Propagation conditions Agreement: - TDLC300-100 for 2 interfering cells with rank 1 serving cell (HomNet) - TDLA30-10 for all other configurations. Other parameters Agreement: - For PDSCH demodulation performance of 8Rx inter-cell interference scenario, for not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation.

As a starting point for evaluation initial simulation parameters were agreed. Our simulation results based on agreed simulation parameters for evaluation are presented in [2]. Based on simulation results for PDSCH demodulation in inter-cell interference, our observations for baseline receiver are:

- For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

- For all the cases the SINR is feasible – not very low

- The operating SNR/SINR with rank 1 MCS 17 is around the same as R17 requirement with 4RX

- The SNR with 4 layers is high and not practical in intercell interference scenarios

Based on the observations, we propose to introduce requirements with 8RX for inter-cell interference with MCS 17 rank 1.

- For PDSCH demod requirements in inter-cell interference configure target UE MCS 17 with rank 1.

## PDSCH requirements with intra cell inter user interference

For PDSCH requirements with intra-cell inter-user interference with 8 RX the following agreements were made:

Agreement: - Define requirements for: - Rank 2+2 - FFS: MCS17/19 - TBD: Rank 4+4: MCS13 Antenna configuration and MIMO correlation Agreement: - Rank 2+2 for 4T8R ULA Low - Rank 4+4 for 8T8R ULA Low Precoding method for the co-scheduled UE Agreement: - Rank 2+2: orthogonal - Rank 4+4 (if introduced): orthogonal DMRS Ports Agreement: - For 4+4, use different CDM group. Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007} Other parameters Agreement: - For PDSCH demod requirements in intra-cell inter-user interference reuse the parameters from Rel-17 2RX/4RX for those not explicitly discussed/ captured.

Simulation parameters for initial evaluation were agreed in [1] and our results are summarized in [2]. Based on the results for baseline receiver we have the following observations:

- For 2+2 rank combination MCS 17/19 show performance gain over MMSE.

For rank 2+2, in R17 we introduce requirements with MCS13, in R18 with advanced receiver we introduced requirements with MCS13/17 depending on the scenario. MCS 17 would be suitable to define requirements with 8RX in R19.

Rank 4+4 is feasible in simulation and shows large gain over MMSE, but we don’t think 4+4 rank combination is very practical in real deployments. Hence, we propose to only define requirements with rank combination 2+2 with 8RX.

- Configuration with rank 4+4 although feasible with 8RX and shows gain over MMSE is not a practical scenario in deployment.

In Rel-17 for 2RX/4RX requirements for MU-MIMO were defined with MCS13 for both 1+1 and 2+2. For 8RX, we propose to define requirements with MCS 17 for both 2+2 and 1+1.

- For rank combination 2+2 define requirements with MCS 17.

- Do not define requirements with rank combination 4+4.

## Applicability rule and release independence

On applicability rule and release independence the following agreements were made in [1]:

Test applicability for 8Rx UEs Agreement: - New requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. - TBD: additional test applicability rule.

On the open issues –

Test applicability to skip 2/4Rx requirements

- Proposals

  - Option 1: 
Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed.

    - Option 1a: 
For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

    - Option 1b: 
For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

We support option 1b above to reduce test burden.

- For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Test applicability for FDD/TDD

- Proposals

  - Option 1: 
For 8Rx inter-cell interference scenario, for demodulation performance tests, introduce similar applicability rules defined in section 5.1.1.10 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx, i.e., if UE is supporting both TDD and FDD with 8Rx, UE will pass test case under HomNet scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

  - Option 2: 
Do not reduce UE effort to only comply to either 1 or 2 interference cells.

From UE processing there is no difference between 1 and 2 interference cells, but in R17 it was agreed to define requirements for both 1 and 2 interference cells based on request from operators. To reduce test burden a applicability rule between TDD and FDD requirement were introduced. We support option 1 above.

- For PDSCH demod requirements in ICI with 8RX introduce applicability rule as: If UE supports FDD and TDD modes, requirement with 2 interference cells is applicable in FDD, requirement with 1 interference cell is applicable in TDD.

Release independent

- Proposals

  - Option 1: 
Discuss release independence requirements at a later stage in the WI.

    - Option 1a: 
RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113.

  - Option 2: 
The new requirements for 8Rx should be release independent from Rel-18.

    - Option 2a: 
The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced.

  - Option 3: 
The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

Regarding release independence it is very early to discuss this as this is the first few meetings to discuss this WID.

- Discuss release independence requirements at a later stage in the WI.

# 3. Conclusion

In this paper, we provide our views on UE demodulation requirements with 8RX. Our observations and proposals are captured below:

General

RAN4 has not concluded if we would have different requirements for differ UE implementations.

There is no difference in processing with SU-MIMO and MU-MIMO with MMSE-IRC receiver.

- If we define different requirements to cater to different UE implementations for MU-MIMO requirements in R19, update the R18 UE capability definition.

It is premature to assume that 2 sets of requirements are needed without looking at the results.

- Criteria for deciding if separate or same requirements for different UE implementation.
Calculate the average SNR for baseline and simplified receiver after removing outlier results, with a reasonable number of results left
- If the average SNR delta between simplified and baseline receiver is ≤ 2.5 dB, then define same requirements for both UE implementations
- If the average SNR delta between simplified and baseline receiver is > 2.5 dB and ≤ 3.5 dB, define the same requirements by adding additional margin to account for span
- If the average SNR delta between simplified and baseline receiver is > 3.5 dB, define different requirements

PDSCH requirements with inter cell interference

For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

For all the cases the SINR is feasible – not very low

The operating SNR/SINR with rank 1 MCS 17 is around the same as R17 requirement with 4RX

The SNR with 4 layers is high and not practical in intercell interference scenarios

- For PDSCH demod requirements in inter-cell interference configure target UE MCS 17 with rank 1.

PDSCH requirements with intra cell inter user interference

- For 2+2 rank combination MCS 17/19 show performance gain over MMSE.

- Configuration with rank 4+4 although feasible with 8RX and shows gain over MMSE is not a practical scenario in deployment.

- For rank combination 2+2 define requirements with MCS 17.

- Do not define requirements with rank combination 4+4.

Applicability rule and release independence

- For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

- For PDSCH demod requirements in ICI with 8RX introduce applicability rule as: If UE supports FDD and TDD modes, requirement with 2 interference cells is applicable in FDD, requirement with 1 interference cell is applicable in TDD.

- Discuss release independence requirements at a later stage in the WI.

# Reference

- R4-2419784, “Way Forward for [113][322] NR_demod_Ph5_Part2_UE”, Nokia.

- R4-2500266, “Simulation results for ICI and MU-MIMO with 8RX”, Apple.