3GPP TSG-RAN WG4 Meeting #114		R4-2500266

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.18.2

Source:	Apple

Title:	Simulation results for ICI and MU-MIMO with 8RX

Release:	Rel-19

Document for:	Information

# 1. Introduction

In RAN4#113 UE demodulation enhancements in Rel-19 was discussed and way forward [1] was agreed. The simulation parameters for evaluation of performance in ICI and MU-MIMO with 8RX were agreed. In this contribution present simulation results for initial evaluation to aide with finalizing the test parameters for requirements.

# 2. Simulation Results

## PDSCH requirements with inter cell interference

For evaluation of performance in inter-cell interference scenario, the same interference scenarios and interference model was used.

Homogeneous network

2 Interference cells

INR1: 7.77 dB

INR2: 2.29 dB

Heterogeneous network

1 Interference cell

INR1: 7.58 dB

Table 1 presents results with baseline receiver for FDD mode.

Table 1: Simulation Results for PDSCH in ICI with baseline receiver

| TC # | Scenario | MIMO | Rank (target UE) | MCS (target UE) | Channel Model | SNR @ 70% Max TP (dB) |  |  | SINR (dB) with MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC | MMSE | Gain over MMSE |  |
| 1 | HomNet (2 Inf cells) | 2T8R | 1 | 17 | TDLC300-100 | 6.5 | 10.5 | 4.1 | -2.9 |
| 2 |  | 2T8R | 2 | 17 | TDLA30-10 | 8.4 | 12.5 | 4.1 | -1.0 |
| 3 |  | 4T8R | 4 | 13 |  | 8.9 | 13.7 | 4.8 | -0.5 |
| 4 | HetNet (1 inf cell) | 2T8R | 1 | 17 | TDLA30-10 | 3.5 | 7.0 | 3.5 | -4.8 |
| 5 |  | 2T8R | 2 | 17 |  | 7.1 | 10.9 | 3.8 | -1.2 |
| 6 |  | 4T8R | 4 | 13 |  | 7.2 | 12.4 | 5.2 | -1.1 |

Based on the results in table 1, we have the following observations for baseline receiver:

- For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

- For all the cases the SINR is feasible – not very low

- The operating SNR/SINR with rank 1 MCS 17 is around the same as R17 requirement with 4RX

- The SNR with 4 layers is high and not practical in intercell interference scenarios

- For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

- For all the cases the SINR is feasible – not very low

- The operating SNR/SINR with rank 1 MCS 17 is around the same as R17 requirement with 4RX

- The SNR with 4 layers is high and not practical in intercell interference scenarios

## PDSCH requirements with intra cell inter user interference

For the simulation assumptions agreed in [1] Table 2 below captures the simulation results from initial evaluation with baseline receiver.

Table 2: Simulation results for PDSCH in intra-cell inter user interference with baseline receiver- FDD

| TC # | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Modulation order for the co-scheduled UE | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC | MMSE | Gain over MMSE |
| 1 | 4T8R | 2+2 | MCS 17 | Orthogonal | 16QAM | 9.5 | NaN | Inf |
| 2 |  |  | MCS 19 |  |  | 11.1 | NaN | Inf |
| 3 | 8T8R | 4+4 | MCS 13 | Orthogonal |  | 13.2 | NaN | Inf |

Table 3: Simulation results for PDSCH in intra-cell inter user interference with baseline receiver- TDD

| TC # | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Modulation order for the co-scheduled UE | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC | MMSE | Gain over MMSE |
| 7 | 4T8R | 2+2 | MCS 17 | Orthogonal | 16QAM | 9.4 | NaN | Inf |
| 8 |  |  | MCS 19 |  |  | 11.1 | NaN | Inf |
| 9 | 8T8R | 4+4 | MCS 13 | Orthogonal |  | 13.3 | NaN | Inf |

Based on the results in Table 2, we have the following observations:

- For 2+2 rank combination MCS 17/19 show performance gain over MMSE

- Configuration with rank 4+4 although feasible with 8RX and shows gain over MMSE is not a practical scenario in deployment

- For 2+2 rank combination MCS 17/19 show performance gain over MMSE.

- Configuration with rank 4+4 although feasible with 8RX and shows gain over MMSE is not a practical scenario in deployment.

# 3. Conclusion

In this paper, we provide our simulation results for initial evaluation for UE demodulation requirements with 8RX. Our observations are captured below:

PDSCH demod in inter-cell interference

- For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

- For all the cases the SINR is feasible – not very low

- The operating SNR/SINR with rank 1 MCS 17 is around the same as R17 requirement with 4RX

- The SNR with 4 layers is high and not practical in intercell interference scenarios

PDSCH demod in intra-cell inter-user interference

- For 2+2 rank combination MCS 17/19 show performance gain over MMSE.

- Configuration with rank 4+4 although feasible with 8RX and shows gain over MMSE is not a practical scenario in deployment.

# Reference

- R4-2419784, “Way Forward for [113][322] NR_demod_Ph5_Part2_UE”, Nokia.