# 3GPP TSG-RAN WG4 Meeting #114                               	R4-2500276

# Athens, Greece, February 17 – 21, 2025

Agenda item:	7.26.4.1

Source:	Apple

Title:	Simulation Assumptions and Results for LP-WUR Core Requirements

Document for:	Discussion

# Introduction

Relevant objectives of the work item to RRM are the following [1]:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In last RAN4#112bis meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [2], and open issues have been summarized in [3] and [4]. On the other hand, during RAN4#113 there was no discussion on this sub-agenda. Hence, in this contribution we continue the discussion on simulation assumptions, provide preliminary results, and propose, again, a framework for RAN4 RRM to manage the consolidation of companies’ simulation results moving forward.

# Methodology

RAN4 RRM has been actively engaging in discussions regarding simulation assumptions and parameters, with the aim of establishing the necessary RRM requirements for LP-WUS/WUR once the RAN1 design is completed. At the time of these discussions, consensus on the LP-SS sequence design, duration, and modulation had not yet been reached. However, RAN4 has decided to proceed with the RRM requirements discussion, allowing companies to align on the aspects that can be addressed in advance. More detailed and specific discussions, including the precise numerical requirements, will be deferred until the RAN1/RAN2 design is fully finalized.

This approach implies that various simulation scenarios should be tested ahead of time to ensure the selection of the right scenario or scenarios. We do not propose RAN4 to be fully dependent on RAN1's progress, but rather that RAN4 should adopt a systematic approach to move forward, minimizing excessive reliance on RAN1's timeline.

During the last RAN1 meeting, several relevant agreements have been made. For example:

RAN1 Agreement

For collecting LP-SS binary sequence results

- Endorse the template for collecting LP-SS binary sequence results in R1-2410925

- Email discussion for collecting LP-SS binary sequences by 10th Jan.

- Please include your cross-checking results (e.g., a separate excel sheet) in the t-doc for #120.

Observation 1: RAN1 has started an email discussion for collecting LP-SS binary sequences in preparation for this meeting. RAN4 can start a parallel but complementary discussion.

Observation 2: RAN1 simulation results collection prioritizes LP-SS binary sequence selection based on sync accuracy subject to a minimum RRM measurement accuracy of 3.5dB. RAN4 focus rather on RSRP/RSRQ measurement accuracy and the minimum number of required samples, subject to the LP-SS binary sequence by RAN1.

We believe that a critical factor in maintaining progress is achieving alignment on all LP-SS simulation parameters to ensure that results from different companies can be directly compared. We believe this alignment is crucial for identifying outliers, potential misunderstandings in simulation assumptions or signal processing, and determining whether discrepancies in the reported results are attributable to legitimate differences in LR/UE implementation or stem from other factors, or even errors.

Observation 3: During RAN4, even though companies have started submitting LP-WUS/WUR simulation results, these results are not quite comparable since the assumed LP-SS sequences, duration and modulation are vastly different.

Therefore, we believe that implementing a systematic approach to align and consolidate simulation results would greatly benefit the ongoing discussions and contribute to the final definition of requirements.

Proposal 1: RAN4 to start a simulation results campaign with simulation assumptions agreed by the end of RAN4#114.

Proposal 2: RAN4 to consider spreadsheet format in R4-2500277 as a starting point, potentially subject to offline discussions during RAN4#114.

Given the current understanding of the scenarios that need to be simulated, we believe that the specific simulation assumptions can be quickly agreed upon. As a result, interested companies will be able to align clearly on the reported accuracy, delay requirements, and other implications for these RRM requirements.

# Simulation Assumptions

As previously discussed during RAN4#112bis, we believe that the simulations conducted by RAN4 should closely align with the simulation assumptions used in RAN1’s evaluation. In general, RAN4 may introduce modifications and improvements as needed, tailored to our specific requirement design work. Our proposal is aimed at ensuring that RAN4’s assumptions for requirement design are closely aligned with the performance expectations of the RAN1 WG’s work, as well as with the ongoing RAN4 RF agenda, which continues to progress in parallel.

Observation 4: For LP-WUR based RRM requirements design, RAN1 link level simulation assumptions and RAN4 RF session’s simulation assumptions should be continuously monitored as the baseline for RAN4 RRM simulation work.

During the RAN1 meeting in October, an update was provided regarding the SNR values to be considered in their evaluations, as outlined in the agreement below. Additionally, a value of -6dB is currently being used in RAN1’s simulation results collection.

RAN1 Agreement

To determine the binary sequences for LP-SS, consider the following for performance comparison:

- Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority))

  - Assume the sliding window size is at least [-25us, 25us].

- RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s

  - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison

- Cross correlation: maximum cross-cor value/ maximum auto-cor value

  - FFS how to use cross-correlation for comparison

- Note: refer to RAN4 RRM assumption for the inter-cell interference

Hence, RAN4 RRM assumptions should be updated accordingly.

Proposal 4: Update SINR values {-6dB; -3dB; -0.5dB; 2dB} but consider -0.5dB and 2dB as low priority cases. Keep SINR side condition open, and decide it based on simulation results.

Furthermore, RAN1 has made progress in refining the set of M and L parameters for LP-SS. Specifically, the following parameters have been determined:

RAN1 Agreement

For the length L of LP-SS binary sequence, limit the selection to the following:

- M=1, L= {4, 6, 8}

- M=2, L= {8, 12, 16}

- M=4, L= {16, 32}

With this agreement in place, it becomes easier to narrow down a set of scenarios based on the number of OFDM symbols that the LP-SS may occupy, considering the modulation factor M and the binary sequence length L.

Top of Form

Bottom of Form

Observation 5: The number of occupied OFDM symbols by LP-SS has been narrowed down to 4, 6 and 8 symbols.

Hence, at the moment RAN4 can make a decision on how many scenarios to simulate and collect. For this, specific binary sequences need to be defined, and our proposal is based on initial sync accuracy results collected by RAN1.

Proposal 5: Define {M,L} pairs {2, 8}, {2, 16}, {4, 16} and {4, 32} for alignment purposes. For each of them, define specific LP-SS binary sequences taken from RAN1 reported simulation results.

Moving forward, another aspect that needs to be clarified is the source of interference in the current requirements scenario. In one case, we are defining an OOK interferer that collides with the LP-SS of the serving cells. In another scenario, we define an OFDM interferer. At least in concept, it is evident that each interference source has a distinct impact. Refer to the figure below.

At the moment, we cannot really distinguish whether the accuracy reported by companies is primarily based on the choice of sequences, or based on the assumed implementation.

Observation 6: Current simulation assumptions leave up to company report the binary sequence to be used for serving cell and the binary sequence use for interference cell.

Proposal 6: Define the following LP-SS binary sequences for simulation alignment purposes:

| M value | Sequence Length | Serving Cell / Interfering Cell Binary Sequence |
| --- | --- | --- |
| M=2 | 8 | [0 1 0 1 0 1 1 0] [0 1 0 1 1 0 1 0] |
| M=2 | 16 | [1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1] [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1] |
| M=4 | 16 | [0 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1] [0 1 1 0 0 1 1 0 0 1 0 1 0 1 0 1] |
| M=4 | 32 | [1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 0 1 0 1 0 1 0 1 0 1] [1 0 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1] [1 0 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1] |

Moving forward, we believe there is another topic that requires clarification. For instance, when estimating RSRP as the difference between ON-duration energy and OFF-duration energy, no significant issues are observed, since OFDM interference does not follow a specific time-domain pattern and can be modeled as random QPSK noise. However, in the case of a colliding OOK interferer, the interference during the ON duration may differ from that during the OFF duration. It is unclear how to address the implications for LR design or what the specific reception assumptions should be. Clarifying this would help in better understanding the key implementation aspects that are common across all companies.

Top of Form

Bottom of Form

Finally, another aspect that needs to be discussed is the use of OFDM overlaid sequences. From past RAN1 agreements, it has been decided that overlaid sequences will be transmitted.

RAN1 Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

RAN1 agreement

In case of overlaid OFDM sequence not carrying information:

- Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol.

- Note 1: multiple overlaid OFDM sequences are specified.

- Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement

For overlaid OFDM sequence at least for LP-WUS,

- For OOK-4 with M>1, support ZC sequence for overlaid sequence.

  - FFS details

Observation 7: It is still unclear how LP-SS will be deployed across multiple cells in terms of time and frequency resources. Therefore, RAN4 should consider both the case of OFDM interference into LP-SS and the case of colliding LP-SS interference into the serving cell LP-SS.

Proposal 7: For LP-SS overlaid sequence configuration, use L=127 ZC sequences with different root indexes: q=1 for the serving cell, and q=63 for the interfering cell.

Finally, upon reviewing the current set of simulation assumptions, there are three parameters that still require confirmation: 1) the receiver filter, specifically whether a 3rd or 5th order Butterworth filter with a 3.96 MHz bandwidth should be used, 2) the receiver ADC bit width, whether a 4-bit or 8-bit ADC is preferred, and 3) the receiver sampling rate for LP-SS, whether 3.84 MHz or 7.68 MHz should be used.

Observation 8: Receiver-specific parameters remain to be confirmed in the set of simulation assumptions.

Proposal 8: Discuss and confirm the assumptions regarding 1) receiver filter bandwidth, 2) ADC bit width, and 3) receiver sampling rate. Alternatively, leaving these options to be determined by UE implementation is also an acceptable outcome, which would require these parameters to be excluded from the set of simulation assumptions.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: RAN1 has started an email discussion for collecting LP-SS binary sequences in preparation for this meeting. RAN4 can start a parallel but complementary discussion.

Observation 2: RAN1 simulation results collection prioritizes LP-SS binary sequence selection based on sync accuracy subject to a minimum RRM measurement accuracy of 3.5dB. RAN4 focus rather on RSRP/RSRQ measurement accuracy and the minimum number of required samples, subject to the LP-SS binary sequence by RAN1.

Observation 3: During RAN4, even though companies have started submitting LP-WUS/WUR simulation results, these results are not quite comparable since the assumed LP-SS sequences, duration and modulation are vastly different.

Proposal 1: RAN4 to start a simulation results campaign with simulation assumptions agreed by the end of RAN4#114.

Proposal 2: RAN4 to consider spreadsheet format in R4-2500277 as a starting point, potentially subject to offline discussions during RAN4#114.

Observation 4: For LP-WUR based RRM requirements design, RAN1 link level simulation assumptions and RAN4 RF session’s simulation assumptions should be continuously monitored as the baseline for RAN4 RRM simulation work.
Proposal 4: Update SINR values {-6dB; -3dB; -0.5dB; 2dB} but consider -0.5dB and 2dB as low priority cases. Keep SINR side condition open, and decide it based on simulation results.

Observation 5: The number of occupied OFDM symbols by LP-SS has been narrowed down to 4, 6 and 8 symbols.

Proposal 5: Define {M,L} pairs {2, 8}, {2, 16}, {4, 16} and {4, 32} for alignment purposes. For each of them, define specific LP-SS binary sequences taken from RAN1 reported simulation results.

Observation 6: Current simulation assumptions leave up to company report the binary sequence to be used for serving cell and the binary sequence use for interference cell.

Proposal 6: Define the following LP-SS binary sequences for simulation alignment purposes:

| M value | Sequence Length | Serving Cell / Interfering Cell Binary Sequence |
| --- | --- | --- |
| M=2 | 8 | [0 1 0 1 0 1 1 0] [0 1 0 1 1 0 1 0] |
| M=2 | 16 | [1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1] [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1] |
| M=4 | 16 | [0 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1] [0 1 1 0 0 1 1 0 0 1 0 1 0 1 0 1] |
| M=4 | 32 | [1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 0 1 0 1 0 1 0 1 0 1] [1 0 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1] [1 0 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 1] |

Observation 7: It is still unclear how LP-SS will be deployed across multiple cells in terms of time and frequency resources. Therefore, RAN4 should consider both the case of OFDM interference into LP-SS and the case of colliding LP-SS interference into the serving cell LP-SS.

Proposal 7: For LP-SS overlaid sequence configuration, use L=127 ZC sequences with different root indexes: q=1 for the serving cell, and q=63 for the interfering cell.

Observation 8: Receiver-specific parameters remain to be confirmed in the set of simulation assumptions.

Proposal 8: Discuss and confirm the assumptions regarding 1) receiver filter bandwidth, 2) ADC bit width, and 3) receiver sampling rate. Alternatively, leaving these options to be determined by UE implementation is also an acceptable outcome, which would require these parameters to be excluded from the set of simulation assumptions.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024

[2] R4-2416861, WF for RRM core requirements for LP-WUS/WUR, Moderator (vivo), RAN4 #112bis

[3] R4-2415664, Topic summary for [112b][229] NR_LPWUS, vivo, RAN4 #112bis

[4] R4-2416883, Coffee break discussion minutes for [112b][223] NR_LPWUS, vivo, RAN4 #112