3GPP TSG-RAN WG4 Meeting #114	R4-2500290

# Athens, Greece,  17th February - 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.922 | CR | draft | rev | - | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections for the UE IMT parameters for the 15GHz range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IMT_4400_7125_14800MHz |  |  |  |  |  | Date: |  |  | 2024-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 6.4.1.3, the following sentence is added “The actual ACLR for this frequency range or parts there-of should be further studied in the WI phase” to align the wording with other sub-clauses. In 6.5.2.1 and 6.5.2.2, Figure numbers are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.3, 6.5.2.1, 6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.4	UE parameters

### 6.4.1	Transmitter characteristics

#### 6.4.1.1	Power dynamic range

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. Based on the study in sub-clause 6.5.2, the UE antenna type for 14800 – 15350 MHz can reuse existing FR1 antenna characteristics so that both nominal maximum and minimum output power refer to existing FR1 bands, i.e., power dynamic range is 56 dB.

#### 6.4.1.2	Spectral mask

The UE spectral mask is described in TS 38.101-1 [12] clause 6.5.2.2 Table 6.5.2.2-1. Emission mask for 200 MHz BW is based on the CA mask as defined in TS 38.101-1 [12] clause 6.5A.2.2. The SEM requirements in this frequency range may be relaxed than FR1 intra-band CA with 200MHz aggregated bandwidth in the WI phase.

#### 6.4.1.3	ACLR

According to the previous studies and considering that the UE needs to meet the Occupied bandwidth (OBW) requirements and is higher than the ACLR needed for co-existence as noted in sub-clause 6.1.3, it was concluded that 24 dB ACLR would be sufficient for 14800 – 15350 MHz. Thus, ACLR of 24 dB can be considered for the frequency range 14800 – 15350 MHz.

The actual ACLR for this frequency range or parts there-of should be further studied in the WI phase.

#### 6.4.1.4	Spurious emissions

The general spurious emissions defined in TS 38.101-1 [12] clause 6.5.3 Table 6.5.3.1-2 with OOB boundary given in Table 6.5A.3.1-1 can apply to the frequency range 14800 -15350 MHz.

#### 6.4.1.5	Maximum output power

Based on the study in sub-clause 6.5.2, the UE maximum output power for the considered frequency ranges could be 23 dBm. Other UE power classes, e.g., 20dBm, 26dBm and 29dBm, are not precluded (corresponding ACLR limit will be adapted accordingly to avoid additional interference).

#### 6.4.1.6	Average output power

NOTE:	This parameter was not mentioned in the previous response to ITU-R WP5D.

********** NEXT CHANGED SECTION **********

### 6.5.2	UE antenna characteristics

#### 6.5.2.1	General considerations

One of the key issues for this range is the UE RF architecture. While existing FR1 system parameters are defined for frequencies up to 7.1GHz, same FR1 principles are already assumed for e.g. 7.1-8.4GHz, which is however below the considered range of 14.8-15.3GHz. At the same time, the lower bound for the FR2 starts at 24.25GHz, which is notably higher than 15.3GHz. A choice for a particular UE RF architecture at these frequency ranges should consider implementation feasibility, e.g. physical constraints in case of handheld devices, and anticipated performance, such as coverage (as highlighted in Figure 6.5.2-1 in TR 38.820). The capabilities of PA technology in the FR2 frequency range motivated 3GPP to consider a UE architecture which relies on panels performing analog beam-forming, which was also considered as a feasible option from the handheld device implementation perspective. In contrast, panels and analog beam-forming are not feasible for lower FR1 frequencies.

In the context of the UE RF architecture, better UL coverage at FR2 frequencies was one of the key driving factors to consider the UE RF design with panels performing analog beam-forming. An FR2-like array antenna system incorporates analog beamforming into the array antenna to increase the coherent EIRP gain, thus helping to reduce the path loss between the UE and the base station. Furthermore, the FR2 array antenna, LNAs, PAs, beamformers, and PMIC are integrated into a single package, small form factor of which was also considered as a feasible option from the handheld device implementation perspective. As for the frequencies around 10-15GHz, previous work in TR 38.820 studied feasibility of FR2 principles in hand-held devices operating in that frequency range. One of the main challenges is the size of potential FR2 panel and, as noted in TR 38.820, "… we can see that the antenna module doubles in size going from 28 GHz to 20 GHz and further doubles at 14 GHz". Figure 6.5.2.1-1 presents an example of the patch width and ground plane extension based on the microstrip patch array design assuming four elements with half-wavelength spacing.

Figure 6.5.2.1-1: Patch array dimensions vs. frequency

For more detailed information on the potential array antenna size operating at the considered frequency of 15GHz, Figure 6.5.2.1-2 below presents several example cases with different antenna arrangements accounting for the typical spacing between elements of the array antenna of half wavelength. As can be seen from the figure for a typical inter-element antenna spacing, the total antenna array size becomes much larger comparing to the FR2 sizes.

Figure 6.5.2.1-2: Estimated antenna size at 15GHz

Figure 6.5.2.1-3 below illustrates an example of a survey of the internal area used by four different commercially available 5G devices for FR1 (based on individual company contributions in RAN4).  The FR2 area is calculated based a single 4x1 reference design, the configuration used as the basis for current FR2 spherical coverage requirements.  Additionally, an estimate of the required 12-15GHz antenna array area is provided for comparison, similarly based on a 4x1 microstrip patch array with half-wavelength spacing. So, assuming dedicated apertures, an estimated 4x1 array at 12-15 GHz would represent an additional area growth on the order of 30-70% relative to antenna area usage currently needed to support FR1.

Figure 6.5.2.1-3: A survey of internal area usage for FR1, FR2 and estimated 13-15GHz antennas

In addition, support of analog beamforming in 15GHz might impose a substantial growth and volume requirements dedicated for cellular antennas.  Furthermore, the nature of such an array might constrain the dimensions leading to implementation challenges in densely integrated consumer devices that can be of different form factors. For instance, Figure 6.5.2.1-4 shows an example of the PCB of a foldable phone, from which one can see that it might be challenging to mount single patch antennas on four edges or the backside of the smartphone considering the larger array size and the already-packed smartphone space. In other words, it can be difficult for a hand-held to assume that there will be extra space in all required physical dimensions making it challenging, at least from the packing perspective, to put FR2-like panels supporting the 15GHz range.

Figure 6.5.2.1-4: Main PCB of a foldable mobile phone.

#### 6.5.2.2	Design with patch antennas

A possible design approach is the adoption of patch antennas (i.e., FR1-like design) which gives more flexibility to the OEM vendors because even if a device implements more than one Tx, there is an additional flexibility on where Tx antennas can be placed. Furthermore, more than one Tx antenna can attain coherent EIRP gain through the corresponding placement of co-directional and co-located antennas when coherent Tx is implemented. Each antenna in such FR1-like system is linked to its own digital chain, allowing for easy support of coherent Tx. It can also reduce the PA power requirement for each antenna element, thus enhancing the overall power consumption of the UE at 15GHz, and it can help increase the coverage distance between the base station and the UE. For instance, Figure 6.5.2.2-1 presents an example of a general mobile device form factor with two patch antennas placed within the device as presented at the distance corresponding to approximately three wavelengths.

Figure 6.5.2.2-1: Exemplary placement of two patch antennas within a mobile device.

In this example, each patch antenna gain is around 2dB with the Tx power 23dBm. Additionally, Figure 6.5.2.3-2 below presents separately distribution of antenna and EIRP gains for the considered example antennas, where the baseline curve is only one antenna. Gain values are obtained by probing all points in 3D space around considered antennas, which allows us to find the best beam at each point of space independent of the base station location. In other words, these figures illustrate possible gain values, including the best one, regardless of the phone orientation with respect to the base station. As can be seen from this example, from the antenna gain perspective, the average gain comparing to one Tx is around 2.5dB. From the EIRP perspective, the average gain comparing to one Tx is around 5.5dB, which comprises 2.5dB antenna gain and 3dB coming from the fact that there are two Tx antennas each transmitting at 23dBm.

Figure 6.5.2.2-2: Antenna and EIRP gain for the considered two Tx antennas.

An additional aspect for the FR1-like antenna design is the resulting radiation pattern. It is worth noting that unlike at the antenna and EIRP gain, which are not very sensitive for the actual placement of Tx antennas, the radiation pattern may vary a lot. For instance, Figure 6.5.2.2-3 below presents an example resulting beam pattern from two Tx antennas physical location of which is the same as in Figure 6.5.2.2-1. For the sake of better clarity, we present the resulting radiation pattern in 2D and 3D spaces. Since two Tx antennas are placed at the back side of the phone, the resulting radiation pattern has a big negative gain in the opposite direction.

Figure 6.5.2.2-3: Radiation pattern for two Tx antennas presented in Figure 6.5.2.2-2.

On the Rx side, more than one discrete/ FR1-like antenna with digital beamforming can maximize the received signal power to improve the cell edge performance. For a cell-center user, instead of maximizing the received signal power, all antennas can be used jointly to achieve spatial multiplexing gain.

#### 6.5.2.3	Design with metal frame antennas

A simulation with two general metal frame Tx antennas was conducted to estimate the antenna gain and performance for cases of single-Tx and multi-Tx without beam steering assumptions. The antenna placement for the evaluation is captured in 6.5.2.3-1.

Figure 6.5.2.3-1: Exemplary placement of metal frame antennas within a mobile device.

With the assumption above, simulation results of the above example related to antenna performance are summarized in Figure 6.5.2.3-2 and Table 6.5.2.3-1, which also capture radiation efficiency and directivity in terms of peak antenna gains of each scenario.

Figure 6.5.2.3-2: CDF of peak antenna gains for metal antennas

Table 6.5.2.3-1: Peak antenna gains for one-Tx and two-Tx

|  | Ant1 | Ant2 | Ant3 |
| --- | --- | --- | --- |
| Gain (dB) | 3.9 | 4.8 | 2.7 |
| Rad. Efficiency (dB) | -6.3 | -4.8 | -6.3 |
| Directivity (dB) | 10.2 | 9.6 | 9.0 |
|  | Ant1 + Ant2 | Ant1 + Ant 3 | Ant2 + Ant3 |
| Combined gain (dB) | 3.8 | 2.6 | 3.9 |
| Gain drop (dB) | -1.0 | -1.3 | -0.9 |

As seen from Figure 6.5.2.3-2 and Table 6.5.2.3-1, in terms of the peak antenna gain with metal frame antennas, it is shown that for the considered example, the single-Tx antenna gain can reach up to 4.8 dB with its directivity and efficiency characteristics at the frequency range around 15 GHz. However, for the case of two-Tx, it is shown that the peak gain of combined two Tx antennas does not show better performance, but rather drops compared to the single-Tx case for the antenna placement in Figure 6.5.2.3-1. This would be coming from two different radiation patterns as depicted in Figure 6.5.2.3-3.

Figure 6.5.2.3-3: Simulated radiation patterns for one-Tx and two-Tx cases

The radiation pattern of antennas in a practical device has different peak direction compared with the boresight direction since the UE ground has much longer electric strength at this frequency range as shown in Figure 6.5.2.3-3. For example, Ant2 would form its peak direction toward Ant3, and similarly, Ant1 has the peak gain at the opposite direction toward the bottom side of UE at this case. On the other hand, if two Tx antennas are adjacent with the same orientation, it may have increased peak antenna gain thanks to the same radiation pattern between two Tx antennas although it would face a different issue such as antenna isolation to keep the fundamental performance.

In this regard, for FR1-like design with metal frame antennas, similar to the design approach with FR1-like patch antenna case, it should be noted that multi-Tx antennas may not help improving peak gain performance unless their placements are not designed carefully in terms of both radiation patterns and isolation.