3GPP RAN WG4 Meeting #114																	R4-2500291

Athens, Greece, 17th – 21st February 2025

Agenda item:	6.18.2

Source:	Apple

Title:	Further input on UE power back-off while operating in 2000-2020 MHz UL range of the NTN band n252

WI/SI:	NR_NTN_Sband-Core

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. For instance, after the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2]. So-called extended L-band was also completed in Rel-18.

And after the RAN#104 meeting another spectrum WI was agreed for a satellite band operating in the S-band frequency range [3]. The UL block of a new NTN band covers the frequency range of 2000-2020MHz, which is close to the DL of another terrestrial band n25. And as discussed during the previous RAN4#112 meeting, there are some concerns on the potential impact from UL transmission on a new NTN band n252 to the DL receivers operating in the terrestrial band n25. Consequently, there were also concerns on the potential power back-off that band n252 UE may apply to meet existing requirements to protect band n25 receivers. RAN4#112bis concluded [5] to consider an option to "Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz ". And during the RAN4#113 meeting it was decided to consider further -40dBm/MHz and -30dBm/MHz protection limits. Thus, to facilitate further technical discussions on potential trade-offs between the relaxed protection requirements and potential power back-off, we conduct a set of additional simulations.

# 2	UE power back-off values

Here we present power back-off results for the NTN band n252 with the following common assumptions:

-	Tx power: PC3 (+23dBm)

-	LO placement: always in the centre of the carrier

-	Additional requirements: -40dBm/MHz and -30dBm/MHz protection criterion for band n25

## 2.1	Power back-off simulations for -40dBm/MHz

Figure 2.1-1 and Figure 2.1-2 below present simulation results for the 5MHz channel, which is placed in the following locations within the band: left edge of the band (Fc=2002.5MHz) and when the center frequency shifted further away from the left edge (Fc=2007.5MHz).  For the sake of brevity, we present only DFT-s-OFDM and CP-OFDM results (DFT-s-OFDM PI/2 BPSK results are usually close to DFT-s-OFDM QPSK and sometimes even outperform the latter). So, referring to Figure 2.1-1 and Figure 2.1-2 the following general observations could be made for the 5MHz channel:

-	If the 5MHz channel is right at the left edge of the band, then large UL allocations may need large power back-off of up to 5dB even for DFT-s-OFDM. On the other hand, if we consider small UL allocations, which are likely to be the case for the NTN channels, then no power back-off is needed. In fact even medium size UL allocations of up to 10RBs are not likely to have any power back-off.

-	If the 5MHz channel is shifted farther away from the band lower edge, then required power back-off reduces dramatically. Only large UL allocations may need power back-off.

Figure 2.1-1: Power back-off values for the 5MHz channel (Fc=2002.5MHz)

Figure 2.1-2: Power back-off values for the 5MHz channel (Fc=2007.5MHz)

Figure 2.1-3 and Figure 2.1-4 present power back-off values for the 10MHz channel considering the following locations of the channel: right at the left edge of the band (Fc=2005) and shifted to the right so that the offset from the band edge is 5MHz (Fc=2010). The following general observations can be made:

-	For the first case, i.e. when the 10MHz channel resides at the left edge of the band, large power back-off is needed even for small UL allocations.

-	If the 10MHz channel is offset by at least the 5MHz gap from the left edge, then small UL allocations will require no power back-off backoff.

Figure 2.1-3: Power back-off values for the 10MHz channel (Fc=2005MHz)

Figure 2.1-4: Power back-off values for the 10MHz channel (Fc=2010MHz)

Next, Figure 2.1-5 and 2.1-6 present power back-off values for the 15MHz channel at the following locations: right at the left edge of the band (Fc=2007.5) and shifted to the right so that the offset from the band edge is 5MHz (Fc=2012.5). As can be seen from the figures, irrespective of the15MHz channel location, edge allocations, even small ones, will require large power back-off.

Figure 2.1-5: Power back-off values for the 15MHz channel (Fc=2007.5MHz)

Figure 2.1-6: Power back-off values for the 15MHz channel (Fc=2012.5MHz)

## 2.2	Power back-off simulations for -30dBm/MHz

In this section we present UE power back-off values for the same channel sizes and center frequencies as in the previous sub-section, but a more relaxed -30dBm/MHz protection criterion is used.

Figure 2.2-1 below presents power back-off values for the 5MHz channel, which is right at the left edge of the band. As can be seen, only large allocations might need power back-off, which should be even within the existing MPR margins.

Figure 2.2-1: Power back-off values for the 5MHz channel (Fc=2002.5MHz)

Figure 2.2-2 and Figure 2.2-3 present power back-off values for the 10MHz channel. General observations are the same as for the -40dBm/MHz protection limit, with the only noticeable difference that for the edge 10MHz channel bandwidth only the leftmost edge allocations will need high power back-off values.

Figure 2.2-2: Power back-off values for the 10MHz channel (Fc=2005MHz)

Figure 2.2-3: Power back-off values for the 10MHz channel (Fc=2010MHz)

Figure 2.2-4 and Figure 2.2-5 present power back-off values for the 15MHz channel. Like in the previous simulation case of the -40dBm/MHz protection limit, the edge 15MHz channel will require large back-off values for a wide range of UL allocations. Once shifted to the right, large power back-off values are mostly constrained to the leftmost edge RBs.

Figure 2.2-4: Power back-off values for the 15MHz channel (Fc=2007.5MHz)

Figure 2.2-5: Power back-off values for the 15MHz channel (Fc=2012.5MHz)

## 2.3	Summary

For the -40dBm/MHz protection requirement the following general observations can be made on how potential channel configurations may look like:

-	5+5+10MHz. In this case the leftmost channel is configured as 5MHz because at least small UL allocations can be scheduled without requiring power back-off. The remaining channels can be either 5 or 10MHz, whereupon even large UL allocations can be used.

-	5+15MHz. In this option the leftmost channel is also 5MHz followed by the 15MHz channel. While the edge allocations of the 15MHz channel are not likely to be scheduled in UL because of the required power back-off, there is a contiguous block of around 60RBs, which provides more scheduling flexibility for the network to balance resources between UEs. In DL, however, all RBs can be used for scheduling. If an operator deploys not only NR, but also NB-IOT and/or LTE CatM, then dynamic spectrum sharing can be used in edge RBs of the 15MHz channel.

-	10+10MHz. In this option the leftmost channel is 10MHz edge allocations of which can be shared with another technology, e.g. LTE CatM or NB-IOT. Two 10MHz channels will provide more scheduling flexibilities for the network when compared to 5+5+10MHz.

Figure 2.3-1: Exemplary allocation and usage of the NR UL channels to avoid unnecessary power back-off (-40dBm/MHz)

For the -30dBm/MHz protection requirement same considerations for the potential channel arrangements may apply with the only major difference is that 10MHz and 15MHz channels may need power back-off only for the left-most edge RBs.  So, possible channel arrangements may look like presented below in Figure 2.3-2. If there are other NTN technologies, such as LTE CatM and/or NB-IOT, then DSS can be used in the left-most edge RBs of the corresponding 10 and 15MHz channels.

Figure 2.3-2: Exemplary allocation and usage of the NR UL channels to avoid unnecessary power back-off (-30dBm/MHz)

# 3	Conclusions

In this discussion paper we presented power back-off simulation results for a UE operating in the UL range of 2000-2020 MHz, i.e. NTN band n252 UL, in presence of the existing requirements to protect band n25 DL considering two protection limits: -40dBm/MHz and -30dBm/MHz. These results can be used for further technical discussions on finding a suitable trade-off between the relaxed protection requirement, smaller UL channel bandwidths/allocations and where the UL channels are configured.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

RP-241661, "New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)", Dish Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

- R4-2417206, "WF on NTN and IoT-NTN bands", Moderator (Inmarsat)

Annex A: Results for -50dBm/MHz requirement

Figure A-1: Power back-off values for the 5MHz channel with DFT-s-OFDM

Figure A-2: Power back-off values for the 10MHz channel with DFT-s-OFDM

Figure A-3: Power back-off values for the 20MHz channel with DFT-s-OFDM