3GPP RAN WG4 Meeting #114																	R4-2500293

Athens, Greece, 17th – 21st February 2025

Agenda item:	7.10.2.2.1

Source:	Apple

Title:	Further considerations on the high-power UE Tx requirements

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. That time the only available power class for NTN was PC3 offering the maximum nominal output power of 23dBm. During the RAN#104 meeting a new WI was agreed [2] with one of the objectives to enable higher power classes for NTN.

During the previous meetings RAN WG4 discussed which power classes and UE architectures should be considered further for the normative work. Thus, in this discussion paper we present our further views on NTN power classes and associated power domain enhancements.

# 2	High-power Tx requirements

## 2.1	Power classes and UE architectures

During the previous meetings companies discussed which power classes should be considered further for the NTN core specifications and under which UE RF architecture assumptions. It is worth noting that the existing NTN power class 3 is based on 1TX architecture that is common for both terrestrial and satellite bands FDD bands. Similarly, the existing power class 2, which is also based on 1TX architecture, can be easily extended from the existing terrestrial FDD bands to the satellite FDD bands. The PC1 power class is also based on 1TX architecture and is defined for the terrestrial FDD bands, from which it can be extended to the satellite operation.

Observation 1a:	Both PC2 and PC1 are based on the 1TX architecture and are defined for the existing terrestrial FDD bands, which makes them ideal candidates for efficient operation in the satellite FDD bands.

There was also a discussion on whether and how to enable the PC1.5 power class, for which several major concerns were raised. Firstly, this power class is defined only for the terrestrial TDD bands and is not applicable to the FDD bands. Secondly, since this power class assumes 2TX architecture, the resulting performance could be quite sub-optimal comparing to the 29dBm transmission with 1TX if it is enabled for the FDD satellite links. The matter is that 2TX architecture will require configuring UL MIMO, performance of which is questionable for the satellite channels. So it could be the case that 23+23dBm configuration will perform even worse than a single 23dBm chain in case of the satellite channel. Another important aspect to consider is the fact that 2Tx architecture and UL MIMO will require CP-OFDM, power back-off values for which are larger than DFT-s-OFDM.

Observation 1b:	 The existing PC1.5 power class is defined only for the TDD terrestrial bands and is based on the 2TX architecture.

Observation 1c:	 2TX architecture will require UL MIMO, resulting performance of which is not clear for the satellite channels.

Observation 1d:	 2TX architecture and UL MIMO will also require CP-OFDM, MPR margins and A-MPR of which is much larger compared to DFT-s-OFDM (see the corresponding A-MPR values for e.g. band n254 and n256).

Observation 1c:	There is no clear evidence that PC1.5 based on 2TX with UL MIMO and CP-OFDM will perform better than PC2 1TX with DFT-s-OFDM.

Nevertheless, since there is an interest from the industry to enable 29dBm transmission power for the NTN devices, we suggest studying it further in the follow-up work once RAN WG4 completes the work on PC2 and PC1 classes.

Proposal 1a:	In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices and 31dBm 1TX (PC1) for non-handheld devices.

Proposal 1b:	29dBm for the FDD bands and handheld devices can be tackled in the follow-up normative work.

## 2.2	Potential power domain enhancements

During the previous RAN4#112bis meeting several companies expressed the view that some of the existing parameters, such as ACLR or SEM, can be relaxed to increase the maximum output power. Generally speaking, it is indeed true that unless even more stringent regulatory requirements are used, the ACLR and SEM are the major limiting factors. And since the ACLR is the requirement defined by 3GPP, it looks quite appealing to boost the UE power for NTN by relaxing the corresponding ACLR values. However, it is worth noting that ACLR becomes the limiting factor only for large UL allocations. As can be seen from the Figure 2.2-1 below, the ACLR usually limits the maximum output power only when a UE is scheduled with a large UL allocation.  On the contrary to it, small UL allocations are limited by SEM, not by the ACLR. And since a UE will be allocated small UL grants to maintain link budget of the satellite link, potential relaxations of ACLR will not provide any gains in the maximum output power. Of course, 3GPP can contemplate further relaxations in the spectral emission mask. On the other hand, SEM is defined by regulations, not by 3GPP, and thus it remains somewhat questionable on whether regulations will allow this type of relaxation.

Figure 2.2-1: Illustration of limiting factors with ACLR 30dBc.

Observation 2a:	ACLR becomes the limiting factor only for large allocations, whereas small UL allocations are limited by SEM.

Observation 2b:	Relaxing ACLR will not boost the maximum output power for the satellite links because a UE will be assigned small UL allocations.

Observation 2c:	It is not clear whether relaxations of the existing SEM is possible because it is defined by regulations, not by 3GPP.

As summary, it is important to ensure that an NTN UE uses the highest possible maximum output power. And to achieve that 3GPP can consider further which relaxations of the existing requirements can indeed benefit the satellite links without violating the regulatory requirements.  Meanwhile, existing 3GPP ACLR and SEM requirements can be adopted for the high-power NTN UEs as a baseline assumption.

Proposal 2a:	As a baseline approach, existing Tx requirements from the TN specifications are re-used for the high-power NTN UE.

Proposal 2b:	RAN WG4 can study further which requirements, SEM or ACLR, can be relaxed for the best power gains for NTN.

## 2.3	Tx requirements for PC2

Based on our considerations in the previous sections, we suggest re-using existing PC2 Tx requirements including SEM, ACLR and MPR. In addition to that, A-MPR will be (re-)evaluated, when needed, because a higher Tx might also need higher A-MPR values.

Proposal 3a:	For NTN PC2 power class, existing SEM applies.

Proposal 3b:	For NTN PC2 power class, existing MPR requirements for PC2 in TS 38.101-1 apply.

Proposal 3c:	For NTN PC2 power class, existing ACLR requirement of 31dB for PC2 in TS 38.101-1 apply.

Proposal 3d:	For NTN PC2 power class, A-MPR will be re-evaluated (accounting for the SEM, ACLR and band specific requirements).

# 3	Conclusions

In this discussion paper we have presented our further views on several open issues discussed during the previous RAN WG4 meetings. More specifically, we suggest focusing on the PC2 and PC1 power classes for NTN because these power classes are based on the 1TX architecture and are already defined for the FDD bands making them ideal candidates for the further NTN enhancements. Even higher power classes for the handheld devices, e.g. 29dBm, can be tackled in the follow-up normative work.  As for SEM and ACLR, our preference is to take existing requirements as a baseline for further definition of the PC2 power class for NTN. At the same time, RAN WG4 can study further which relaxations can result in the best gains for the maximum output power.

Observation 1a:	Both PC2 and PC1 are based on the 1TX architecture and are defined for the existing terrestrial FDD bands, which makes them ideal candidates for efficient operation in the satellite FDD bands

Observation 1b:	 The existing PC1.5 power class is defined only for the TDD terrestrial bands and is based on the 2TX architecture.

Observation 1c:	 2TX architecture will require UL MIMO, resulting performance of which is not clear for the satellite channels.

Observation 1d:	 2TX architecture and UL MIMO will also require CP-OFDM, MPR margins and A-MPR of which is much larger compared to DFT-s-OFDM (see the corresponding A-MPR values for e.g. band n254 and n256).

Observation 1e:	There is no clear evidence that PC1.5 based on 2TX with UL MIMO and CP-OFDM will perform better than PC2 1TX with DFT-s-OFDM.

Observation 2a:	ACLR becomes the limiting factor only for large allocations, whereas small UL allocations are limited by SEM.

Observation 2b:	Relaxing ACLR will not boost the maximum output power for the satellite links because a UE will be assigned small UL allocations.

Observation 2c:	It is not clear whether relaxations of the existing SEM is possible because it is defined by regulations, not by 3GPP.

Proposal 1a:	In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices and 31dBm 1TX (PC1) for non-handheld devices.

Proposal 1b:	29dBm for the FDD bands and handheld devices can be tackled in the follow-up normative work.

Proposal 2a:	As a baseline approach, existing Tx requirements from the TN specifications are re-used for the high-power NTN UE.

Proposal 2b:	RAN WG4 can study further which requirements, SEM or ACLR, can be relaxed for the best power gains for NTN.

Proposal 3a:	For NTN PC2 power class, existing SEM applies.

Proposal 3b:	For NTN PC2 power class, existing MPR requirements for PC2 in TS 38.101-1 apply.

Proposal 3c:	For NTN PC2 power class, existing ACLR requirement of 31dB for PC2 in TS 38.101-1 apply.

Proposal 3d:	For NTN PC2 power class, A-MPR will be re-evaluated (accounting for the SEM, ACLR and band specific requirements).

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"