3GPP TSG-RAN WG4 Meeting #114	R4-2500302

# Athens, Greece,  17th February – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | draft | rev | <#> | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the 3MHz channel to TR 38.863 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Thales, Echostar, Aalyria Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2024-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When TR 38.863 was completed in Rel-17, the smallest NTN channel bandwidth was 5MHz. With introduction of the 3MHz channel bandwidth for NTN, the corresponding system and RF related parameters should be updated to align this TR with TS 38.101-5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sub-clause 7.2.3, 3MHz channel bandwidth is added to the list of available channel bandwidhts. In sub-clause 7.2.4.2, 3MHz sync raster entries are added. In sub-clause 7.4.2.2.2.2, 3MHz channel bandwidth is added to the table summarising NS flags. In sub-clause 7.4.3.2.2, 3MHz channel is added to the table with REFSENS values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be mismatches between this TR and TS 38.101-5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3, 7.2.4.2, 7.4.2.2.2.2, 7.4.2.2.8, 7.4.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a draft running CR, some values are still TBD because of the ongoing discussions. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.2	Common issues for satellite access node and NTN UE

### 7.2.1	General

This section aims to present the agreed definitions for the newly introduced NTN operating bands. The possible configurations in terms of channel bandwidth, SCS, channel and synchronization raster for NTN operations are introduced as well.

### 7.2.2	Operating bands

The following bands in Table 7.2.2-1 are agreed as exemplary NTN satellite bands in Release 17. Regarding the band numbering for NTN satellite bands, it is agreed to start from the largest band number in FR1 range for NTN satellite bands which fully within FR1 frequency ranges to better differentiate NTN satellite band and TN bands and ensure contiguous band number allocation for NTN satellite bands, the number can be taken in a decreased order with first come, first service.

Table 7.2.2-1: NTN satellite bands in FR1

| NTN satellite band # | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: NTN bands are numbered in descending order from n256. |  |  |  |

### 7.2.3	Channel bandwidth, SCS and spectral utilization

The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in TS 38.104[16] and TS 38.101-1[17] can be reused for NTN satellite system.

The supported channel bandwidths per operating band should be defined based on NTN operator input. The details are given in Table 7.2.3-1.

Table 7.2.3-1: Channel bandwidths and SCS per NTN satellite operating band in FR1

| NTN satellite band # | SCS kHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | Yes | Yes | Yes | Yes | Yes |
| n256 | 30 |  |  | Yes | Yes | Yes |
|  | 60 |  |  | Yes | Yes | Yes |
|  | 15 | Yes | Yes | Yes | Yes | Yes |
| n255 | 30 |  |  | Yes | Yes | Yes |
|  | 60 |  |  | Yes | Yes | Yes |

### 7.2.4	Channel raster and sync raster

#### 7.2.4.1	Channel raster

In order to be compatible with current design of sync raster below 3 GHz, the ARFCN parameters below 3000 MHz will be reused and 100 kHz was chosen as the channel raster for NR NTN satellite bands below 3 GHz.

The applicable ARFCNs per operating and for satellite access node are defined in table 7.2.4.1-1.

Table 7.2.4.1-1: ARFCNs per operating band satellite access node

| NTN satellite band # | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |

#### 7.2.4.2	Sync raster

The applicable SS raster entries per operating band are defined in Table 7.2.4.2-1 and 7.2.4.2-2.

Table 7.2.4.2-1: Applicable SS raster entries per operating band (for above 3MHz channel)

| NTN satellite band # | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |

Table 7.2.4.2-2: Applicable SS raster entries per operating band (for the 3MHz channel)

| NTN satellite band # | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | TBD – <1> – TBD |
| n255 | 15 kHz | Case A | TBD – <1> – TBD |

********** NEXT CHANGED SECTION **********

## 7.4	NTN UE requirements

### 7.4.1	General

In general, NTN UE products are assumed to meet TN UE requirements as specified in TS 38.101-1[17]. The structure and content of the NTN UE specification TS 38.101-5 should be based on TS 38.101-1[17]. The development of TS 38.101-5 should follow the set of conditions that were agreed at the RAN#94e meeting in RP-213618[27].

### 7.4.2	UE Transmission characteristics for satellite access

#### 7.4.2.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0dBi is assumed. Handheld PC3 UE is assumed in Release 17 for satellite access.

#### 7.4.2.2	Conducted transmitter characteristics

##### 7.4.2.2.1	Maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 7.4.2.2.1-1: UE Power Class

| NTN satellite band # | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | +2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

##### 7.4.2.2.2	MPR/AMPR

7.4.2.2.2.1	MPR

UE is allowed to reduce the maximum output power (MPR) due to higher order modulations and transmit bandwidth configurations to meet the general transmission requirements which includes in-band emission, ACLR, out-of-band emission and general spurious emission. As it was agreed those general transmission requirements can be reused from TN, the same requirements specified in Table 6.2.2-1 in TS 38.101-1 [17] can be reused for NTN satellite access UEs.

7.4.2.2.2.2	A-MPR

A-MPR is band specific requirement which is used to meet the additional requirements indicated by network signalling (NS) value.

For n256, based on the regulation conclusion in section 5.2, the A-MPR requirements for n65 could be as starting point. From TS 38.101-1, since the maximum carrier centre frequency of NS_05 and NS_05U is lower than 1960 MHz and the maximum carrier centre frequency of NS_51 is lower than 1980 MHz, it is proposed NS_05, NS_05U and NS_51 don’t need to be considered for n256. The requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 [17] for band n65 can be reused for n256

For n255, based on regulatory requirements, a new NS is required. The following observations were made from the measurement results using an LTE modem and transceiver supporting 1626.5 – 1656.5 MHz:

-	For 5 MHz channel, PA emissions for DFT-s-OFDM are below the spurious emission limits between 1559 and 1606 MHz.

-	For 10 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits for 1 MHz MBW by 5 dB between 1604 and 1606 MHz.

-	For 15 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits for 1 MHz MBW by 9 dB between 1599 and 1606 MHz.

-	For 20 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits by 9 dB between 1584 and 1606 MHz.

-	For 700 Hz MBW, PA emissions for all channel bandwidths are below the spurious emission limits between 1559 and 1606 MHz.

A n255 duplexer with rejection of more than 10 dB will be a sufficient to satisfy the spurious emission limits specified in Table 7.4.2.2.8-1.

Based on this measurement observations, the expected filter attenuation of more than 10 dB below 1610 MHz is going to be adequate to meet the additional emissions requirements between 1559 – 1610 MHz, and therefore, no A-MPR needs to be specified for n255.

In view of the above, the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value shall be specified in Table 7.4.2.2.2.2-1 and the mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 7.4.2.2.2.2-1A.

Table 7.4.2.2.2.2-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.1-1 | 3, 5, 10, 15, 20 | Table 5.3.2-1 in TS 38.101-1 | N/A |
| NS_24 | 6.5.3.3.5 in TS 38.101-5 | n256 | 3, 5, 10, 15, 20 | Table 6.2.3.15-1 in TS 38.101-1 | Clause 6.2.3.5 in TS 38.101-52 |
| NS_57N | 6.5.3.3.1 | n255 | 3, 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in TS 38.101-1 | n2561 |  |  | Table 6.2.3.1-2 in TS 38.101-1 |
| NOTE 1: This NS can be signalled for NTN satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz is not specified, and therefore shall be used as a guard band. |  |  |  |  |  |

Table 7.4.2.2.2.2-1A: Mapping of network signalling label

| NTN satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_57N |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331. |  |  |  |  |  |  |  |  |

For n256, FFS for how to protect n2, n25, n70 and whether A-MPR required.

##### 7.4.2.2.3	Output power dynamics

For Transmit OFF power and Power control, the framework and requirements for existing TN UE can be reused for satellite UE. The transmit OFF power is -50 dBm for 20MHz.

Since handheld UE always transmits the maximum output power for satellite access scenario based on the NR NTN calibration summary in R4-2115628, FFS whether relaxed value needed or not based on existing TN UE requirements

##### 7.4.2.2.4	Frequency error

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the Satellite Access Node.

NOTE:	The requirements is applicable only when PVT ephemeris updated at least once [10] seconds.

##### 7.4.2.2.5	Transmit modulation quality

For the EVM (Error Vector Magnitude), the same requirements specified in TS 38.101-1 [17] sub-clause 6.4.2.1 for QPSK, 16QAM, and 64QAM can be reused for NTN satellite access UEs. NTN satellite access UEs support UL/DL 64QAM for NTN operation as optional with per band.

For the carrier leakage, the same requirements specified in TS 3.101-1 [17] sub-clause 6.4.2.2 can be reused for NTN satellite access UEs.

For the in-band emission, the same requirements specified in TS 3.101-1 [17] sub-clause 6.4.2.3 can be reused for NTN satellite access UEs.

##### 7.4.2.2.6	Spectrum emission mask

For SEM of NTN satellite access UEs, the same requirements of TN UE specified in TS 38.101-1 [17] sub-clause 6.5.2.2 can be reused.

##### 7.4.2.2.7	ACLR

For ACLR, the same ACLR for TN NR UE which is specified in TS 3.101-1 [17] sub-clause 6.5.2.4.1 can be applied for NTN satellite access UEs.

The requirement of ALCR is specified in Table 7.4.2.2.7-1.

Table 7.4.2.2.7-1: NTN satellite access UE ACLR requirement

|  | Power class 3 |
| --- | --- |
| NTN UE ALCR | 30dB |

##### 7.4.2.2.8 	Spurious emissions

The spurious emissions limits specified in TS 38.101-1[17] sub-clause 6.5.3 would also be applicable to NTN satellite access UEs referring to the ERC Recommendation 74-01[28] Annex 3 item 3.1.4 for Mobile Earth Stations (MES) (see Note 1bis) transmitting in the Mobile Satellite Service between 1 GHz and 3 GHz.

Considering FCC rule 47 C.F.R § 25.216 (h) [23], TN UE’s general spurious emission requirements will be inadequate for L-band NTN satellite UE operation in the US. A new NS value and a subclause with Table 7.4.2.2.8-1 needs to be specified for n255 for NTN satellite operations in the US.

Table 7.4.2.2.8-1: Additional requirements for NS_57N

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBW) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f ≤ 1605 | -80 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -80 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -70 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -70 + 24/5 (f-1605) | 1MHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

********** NEXT CHANGED SECTION **********

### 7.4.3	UE Receiver characteristics for satellite access

#### 7.4.3.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

#### 7.4.3.2	Conducted receiver characteristics

##### 7.4.3.2.1	General and diversity characteristics

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for satellite access.

The handheld UE for satellite access is required to be equipped with a minimum of two Rx antenna ports in all NTN satellite operating bands in FR1.

##### 7.4.3.2.2	Reference sensitivity

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For below 6GHz, the REFSENS level can be calculated by the equation below in TR 38.817-01[32]:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

It is noted that the Rx BW is identical to the transmission bandwidth configuration, which is determined by the spectrum utilization. The RB values in the analysis of this contribution are based on the agreed SU for NR.

SNR in Nagoya meeting was agreed as -1dB for NR.

The REFSENS for E-UTRA assumes that the receiver is equipped with two Rx port as a baseline. If 2Rx is considered, the diversity gain is 3dB and the Implementation Margin (IM) uses 2.5dB. In the following calculation, same assumptions are used for NR NTN satellite bands.

For band n255, 9dB noise figure is assumed which is aligned with band n24.

Both dedicate 30MHz duplexer and n65 duplexer are allowed to be used for band n256. Finally, 9dB noise figure is assumed for band n256 as well.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 from TS 38.101-1 [17] (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 from TS 38.101-1 [17])  with parameters specified in Table 7.4.3.2.2-1 and Table 7.4.3.2.2-2.

Table 7.4.3.2.2-1: Two antenna port reference sensitivity QPSK REFSENS

|  | Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # |  | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
|  |  | 15 | TBD | -100.0 | -96.8 | -95.0 | -93.8 |  |
| n255 |  | 30 |  |  | -97.1 | -95.1 | -94.0 | FDD |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |
|  |  | 15 | TBD | -100.0 | -96.8 | -95.0 | -93.8 |  |
| n256 |  | 30 |  |  | -97.1 | -95.1 | -94.0 | FDD |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |

Table 7.4.3.2.2-2: Uplink configuration for reference sensitivity

|  | Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # |  | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
|  |  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n255 |  | 30 |  |  | 24 | 36 | 50 | FDD |
|  |  | 60 |  |  | 10 | 18 | 24 |  |
|  |  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 |  | 30 |  |  | 24 | 36 | 50 | FDD |
|  |  | 60 |  |  | 10 | 18 | 24 |  |