3GPP TSG-RAN WG4 Meeting #114	R4-2500307

Athens, Greece,  17th February – 21st February 2025

Agenda item:	7.26.2.2

Source:	Apple

Title:	On LP-WUR Rx requirements

WI/SI:	NR_LPWUS-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

LP-WUS/WUR shall be introduced to NR in Rel-19. The objectives are covered in the WID [1]. This contribution focuses on the RAN4 core requirements and the way forward [2] from last meeting. The WID lists RAN4 goals as:

# 2  Discussion

Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR Agreements: - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported. - The total value of NF+IM can be considered. - On how to handle the NF values: - Option 1: For NF whether average value can be adopted. - Option 2: two sets NFs for different LP-WUR types with different power consumption target - Option 3: also consider coverage - Capture the table in WF as background information

##### 2.1 On implementation margin for OOK-based LR

When defining REFSENS for a physical implementation a certain margin is required to account for various imperfections such as transmitter noise, base-band limitations, process and temperature variations, multi-receiver combining and other physical limitations. One of the early proposals was to use 2dB [3] which was finally increased to 2.5dB to allow more margins for transmitter noise and baseband impairments [4]. Setting the margin to 2.5dB is also due to the use of two Rx in LTE [5] [6]. It’s considered that the implementation margin compensates for the non-ideal combining of two receivers in a physical implementation. The value has not been changed since and was carried over to NR. From the discussion on MTC it seems imaginable to consider tightening the implementation margin by 0.5dB as wake-up receiver uses single receiver chain. It seems that the portion dedicated to compensating for non-ideal combining might not be required. However, the MTC discussion also had voices which argued that a reduction of implementation margin might lead to higher device cost [7] [8]. RAN4 might discuss whether device cost is a factor for WUR. In summary we think that tightening should not be made without further justification and careful evaluation. With the current information and considering potential device cost impact it is reasonable to consider implementation margin of 2.5dB.

Observation: The REFSENS implementation margin allows physical implementation as it provides room for transmitter noise, base-band limitations, process and temperature variations, multi-receiver combining and other physical limitations. From the discussion on MTC it seems imaginable to consider tightening the implementation margin by 0.5dB as wake-up receiver uses single receiver chain and the portion dedicated to compensating for non-ideal combining might not be required. However, the MTC discussion also had voices which argued that a reduction of implementation margin might lead to higher device cost. RAN4 might discuss whether device cost is a factor for WUR.

Proposal 1: Consider implementation margin of 2.5dB

##### 2.2 On noise figure for WUR types

Noise figure and SNR are critical parameters influencing the RF hardware design and network performance. To achieve low noise figure the LNA is typically one of the first components in the receiver chain. Designing a LNA with low power consumption and low noise is immensely challenging. To achieve high linearity and high gain the LNA requires more stages translating into more power consumption. To effectively reduce the power consumption the gain and noise performance needs to be relaxed.

While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase for LP-WUR. An OFDM type receiver is expected to re-use existing RF architecture from the main receiver. The battery lifetime of the device is enhanced by tweaking the LNA setup for reduced current consumption. This reduces the noise figure performance. Therefore, a delta to the typical 9dB of the main receiver is required. We conducted measurements using an OFDM type WUR receiver and found that a delta of 7dB would be required to satisfy the expected current consumption improvements. This shows that an OFDM type receiver can require noise figure which might be close to OOK type receiver. Last meeting a proposal was made to consider two device categories. One category with low noise figure and one with large noise figure. This approach could be a solution for the described issue and is discussed in section 2.3.

Proposal 2: To achieve high linearity and high gain the LNA requires more stages translating in more power consumption. To effectively reduce the power consumption the noise performance needs to be relaxed. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase considerably for LP-WUR. Measurements indicate that approximately a 7dB increase with respect to the main receiver is required to satisfy the expected reduced power consumption of the LR compared to the MR.

##### 2.3 On low and high NF category and LP-WUR types

In RAN4#113 the contribution [9] provided an interesting take on LP-WUR types. It has been proposed to define the types based on different power consumptions. Each type is featuring its dedicated set of NF and SNR. This is a different approach compared to the one RAN4 is currently discussing where the type is defined by OOK and OFDM-type WUR. Current approach assumes different RF front-end performance and assigns specific NF and SNR. It is assumed that OFDM performs better than OOK type receiver. This can be true but is not guaranteed. The NF of an OFDM type receiver can feature the same NF as an OOK type to preserve more power. The OFDM performance (either performing close to the MR or closer to OOK type) is a design choice mostly dependent on the power saving the designer wants to achieve. Therefore, we would like to propose a generalised approach where types are defined by performance which can be dependent on power consumption, RF performance and other factors. This approach should be more beneficial compared to dividing performance by OFDM and OOK-type.

It is proposed to further discuss the initial proposal from [9] where two LP-WUR types are defined independent on the actual implementation and distinguished by NF performance.

Proposal 3: It is proposed to further discuss the initial proposal from R4-2417885 where two LP-WUR types are defined independent on the actual implementation and distinguished by NF performance.

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure (NF) [8] dB higher than the MR. |
| 2 | The LP-WUR is designed for moderate power consumption, assuming a noise figure (NF) [2] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

##### 2.4 On guard RB for ASCS

Issue 2-3-1: Required number of guard RB for ASCS Agreements: - [0RB] as guard RB for ASCS.

RAN4 discussed whether size of guard RB for ASCS. Analysis showed that more than 1RB does not seem needed and the discussion revolved around the question on defining 0RB or 1RB. The tentative agreement is to use 0RB. At this point we would like to raise the question whether 0RB might be the best choice regarding UE performance and implementation. It has been shown that 1RB provides small improvements for ASCS. However, in [10] it was analysed that 1RB can be considered due to the need of implementation margin and that 1RB would provide benefit for ACS. Other analysis from [11] found that potential filter degradation and degraded phase noise of the LO requires the introduction of guard RBs. If was found that 1 guard RB can be sufficient to provide enough margin for implementation. Considering these aspects, it might be the better choice to define 1RB guard. It is therefore proposed to finalise the agreement with one guard RB.

Proposal 4: When finalising the agreement on guard RB for ASCS it is proposed to use 1RB.

# 4	Conclusions

This contribution provides a discussion based on last meetings way forward on LP-WUR. The following observations and proposals are made:

Observation: The REFSENS implementation margin allows physical implementation as it provides room for transmitter noise, base-band limitations, process and temperature variations, multi-receiver combining and other physical limitations. From the discussion on MTC it seems imaginable to consider tightening the implementation margin by 0.5dB as wake-up receiver uses single receiver chain and the portion dedicated to compensating for non-ideal combining might not be required. However, the MTC discussion also had voices which argued that a reduction of implementation margin might lead to higher device cost. RAN4 might discuss whether device cost is a factor for WUR.

Proposal 1: Consider implementation margin of 2.5dB

Proposal 2: To achieve high linearity and high gain the LNA requires more stages translating in more power consumption. To effectively reduce the power consumption the noise performance needs to be relaxed. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase considerably for LP-WUR. Measurements indicate that approximately a 7dB increase with respect to the main receiver is required to satisfy the expected reduced power consumption of the LR compared to the MR.

Proposal 3: It is proposed to further discuss the initial proposal from R4-2417885 where two LP-WUR types are defined independent on the actual implementation and distinguished by NF performance.

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure (NF) [8] dB higher than the MR. |
| 2 | The LP-WUR is designed for moderate power consumption, assuming a noise figure (NF) [2] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

Proposal 4: When finalising the agreement on guard RB for ASCS it is proposed to use 1RB.

# 4	References

- RP-240801, “Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, 3GPP TSG RAN Meeting #103

- R4-2420528, “WF on UE RF requirements for LP-WUS”, vivo, 3GPP TSG-RAN WG4 Meeting #113

- R4-080696, “TP 36.101: REFSENS and associated requirements”, Ericsson, TSG-RAN Working Group 4 (Radio) meeting #46bis

- R4-080946, “TP TS 36.101: REFSENS, blocking and FRC for Clause 7”, Ericsson, TSG-RAN Working Group 4 (Radio) meeting #47

- R4-103184, “Updated reference sensitivity requirements for Bands 11, 18, 19 and 21”, Ericsson, ST-Ericsson, TSG-RAN Working Group 4 (Radio) meeting #56

- R4-144772, “Low Cost MTC reference sensitivity and TP to Low Cost MTC TR”, Vodafone, 3GPP TSG-RAN WG4 Meeting #72

- R4-145063, “Summary of LC-MTC REFSENS specification discussion”, Nokia Networks, Nokia Corporation, 3GPP TSG-RAN WG4 Meeting #72

- R4-147647, “REFSENS requirements for LC-MTC”, Nokia Networks, Nokia Corporation, 3GPP TSG-RAN WG4 Meeting #73

- R4-2417885, “Further consideration on general aspects of LP-WUS”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #113

- R4-2418217, “Further discussions on LP-WUS REFSENS, ASCS and ACS”, vivo,  Orlando, US, 18th – 22nd November, 2024