3GPP RAN WG4 Meeting #114		R4-2500315

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.1.1.1.1

Source:	Apple

Title:	On Rel-19 HPUE CA DC MSD consideration

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

For HPUE inter-band UL CA/EN-DC, RAN4 has agreed to work on the feasible unified equations, LUT, or simplified guidelines for MSD for HPUE band combinations as normative requirements to replace the existing method using HPUE MSD tables, as captured in the approved “WF on MSD rule” [2] in last RAN4 meeting. Though the initial analysis on single UL MSD in [3] had suggested a few options in the way forward agreement, in our view, using a single PCx value or trajectory for all MSD mechanisms could be oversimplified and may result in either over-relaxing or over-tightening requirement for certain MSD mechanisms and configurations.

In this contribution, we share our views on the HPUE MSD PCx dependency on different single UL MSD mechanisms. For 2UL IMD MSD, we also provide our views on whether IMD orders are sufficient to design LUT/set of equations or both IMD orders and the IMD indexes should be accounted for.

# Discussion

## 2.1	Single UL MSD

Single UL MSD can be due to UL harmonic, Rx harmonic mixing, or cross-band interference. For UL harmonic, the MSD degradation from PC3 to PC2 and PC1.5 would depend on the amount of harmonic power increase in main and diversity Rx paths which also depends on whether the UL is composed by 1Tx (PC2 only) or 2Tx.

Observation 1: For UL harmonic, the MSD degradation from PC3 to PC2 and PC1.5 would depend on the amount of harmonic power increase in main and diversity Rx paths which also depends on whether the UL is composed by 1Tx (PC2 only) or 2Tx.

For PC2 with 1Tx, the power ratio between main signal and 2nd harmonic has been assumed to be the same as PC3 to derive the PCx, meaning that the PC2 PA 2nd harmonic power is 3dB higher than that of PC3 PA. However, for UL harmonic order higher than 2, it has not been confirmed whether the same power ratio for PC3 PA would apply for PC2 PA. Therefore, a unified PCx may not be applicable for all harmonic orders.

Observation 2: For PC2 with 1Tx, the power ratio between main signal and 2nd harmonic has been assumed to be the same as PC3 to derive the PCx.

Observation 3: For UL harmonic order higher than 2, it has not been confirmed whether the same power ratio for PC3 PA would apply for PC2 PA. Therefore, a unified PCx may not be applicable for all harmonic orders.

Proposal 1: RAN4 to confirm whether the same power ratio between the main signal and UL harmonic for PC3 PA would apply for PC2 PA for all harmonic orders.

For Rx harmonic mixing, since the interferer is the UL main signal, interferer power would simply increase by 3 dB and 6 dB for PC2 and PC1.5 respectively. On the other hand, the previous analysis had shown that the Rx harmonic mixing MSD would be dominated by PCB isolation, as a result, the MSD is expected to be the same between 1Tx and 2Tx. And that also means a unified PCx would not be applicable between UL harmonic MSD and Rx harmonic mixing MSD.

Observation 4: For Rx harmonic mixing, the interferer power increases by 3 dB and 6 dB for PC2 and PC1.5 respectively as compared to PC3.

Observation 5: The Rx harmonic mixing MSD would be dominated by PCB isolation, as a result, the MSD is expected to be the same between 1Tx and 2Tx.

Observation 6: A unified PCx would not be applicable between UL harmonic MSD and Rx harmonic mixing MSD.

For cross-band interference, as the interferer is composed by a mixture of intermodulation products with different weighting on different modulation order which would depend on the frequency offset from the main carrier, the evaluation of interferer power usually would rely on simulations or lab measurement. That being said, a unified PCx not only would not be applicable among other MSD mechanisms but also within cross-band interference mechanism itself.

Observation 7: For cross-band interference, the interferer is composed by a mixture of intermodulation products with different weighting on different modulation order which would depend on the frequency offset from the main carrier.

Observation 8: For cross-band interference, a unified PCx would not be applicable within the mechanism itself, nor with other MSD mechanisms.

Based on the above observations, we propose RAN4 to thoroughly evaluate whether a unified PCx would be applicable for all single UL MSD mechanisms.

Proposal 2: RAN4 to thoroughly evaluate whether a unified PCx would be applicable for all single UL MSD mechanisms.

## 2.2	2UL IMD MSD

For 2UL IMD MSD, prior to Rel-19, there has been 3 UL power configurations specified as summarized in Table 2.2-1.

| PC | Aggressor UL Power (dBm) | Release |
| --- | --- | --- |
| PC3 | 20 + 20 | Rel-15 |
| PC2 (2Tx or 3Tx) | 23 + 23 | Rel-18 or earlier |
| PC1.5 (3Tx) | 23 + 27.8 | Rel-18 |
| PC1.5 (2Tx or 3Tx) | 26 + 26 | Rel-19 |

Table 2.2-1 2UL IMD MSD UL power configurations

In Rel-19, we anticipate new PC1.5 configurations with (FDD + TDD) as (PC2 + PC2) and (PC2 + PC1.5) to be introduced with 2Tx or 3Tx. For PC2 FDD band pairing with PC2 TDD band, the FDD band UL may be implemented with 1Tx or 2Tx which could potentially result in different 2UL IMD MSD impact, similar to the UL harmonic MSD with 1Tx or 2Tx. Therefore, it is proposed RAN4 to select an example band combination for 2UL IMD MSD analysis with 1Tx and 2Tx in PC2 FDD band to observe the MSD difference to cover all possible UL configurations before the 2UL IMD MSD simplification studies.

Proposal 3: RAN4 to select an example band combination for 2UL IMD MSD analysis with 1Tx and 2Tx in PC2 FDD band to observe the MSD difference.

On the other hand, in RAN4 2UL IMD MSD for HPUE has been evaluated without taking into account PC2 PA linearity improvement over PC3 PA, meaning that the IMD power was simply scaled up with UL power and IMD order and index as shown in Table 2.2-2.

| PC | UL Power (dBm) | IMD Order | IMD Index | Δ IMD Power (dB) |
| --- | --- | --- | --- | --- |
| PC3 | 20 + 20 | IMD2/3/4/5 | Any | 0 (baseline) |
| PC2 (2Tx or 3Tx) | 23 + 23 | IMD2 | (1,1) | 6 |
|  |  | IMD3 | (2,1), (1,2) | 9 |
|  |  | IMD4 | (3,1), (2,2), (1,3) | 12 |
|  |  | IMD5 | (4,1), (3,2), (2,3), (1,4) | 15 |
| PC1.5 (3Tx) | 23 + 27.8 | IMD2 | (1,1) | 10.8 |
|  |  | IMD3 | (2,1) | 13.8 |
|  |  | IMD3 | (1,2) | 18.6 |
|  |  | IMD4 | (3,1) | 16.8 |
|  |  | IMD4 | (2,2) | 21.6 |
|  |  | IMD4 | (1,3) | 26.4 |
|  |  | IMD5 | (4,1) | 19.8 |
|  |  | IMD5 | (3,2) | 24.6 |
|  |  | IMD5 | (2,3) | 29.4 |
|  |  | IMD5 | (1,4) | 34.2 |
| PC1.5 (2Tx) | 26 + 26 | IMD2 | (1,1) | 12 |
|  |  | IMD3 | (2,1), (1,2) | 18 |
|  |  | IMD4 | (3,1), (2,2), (1,3) | 24 |
|  |  | IMD5 | (4,1), (3,2), (2,3), (1,4) | 30 |
| PC1.5 (3Tx) (2Tx in FDD) | 26 + 26 | IMD2 | (1,1) | TBC |
|  |  | IMD3 | (2,1), (1,2) | TBC |
|  |  | IMD4 | (3,1), (2,2), (1,3) | TBC |
|  |  | IMD5 | (4,1), (3,2), (2,3), (1,4) | TBC |

Table 2.2-2 2UL Δ IMD power relative to PC3 under different IMD order and index

It can be seen that for 2UL with equal power configuration, the IMD power increase only depends on the IMD order but not IMD index. However, for 2UL with unequal power configuration, the IMD power increase would depend on both IMD order and IMD index.

Observation 9: For 2UL with equal power configuration, the IMD power increase only depends on the IMD order but not IMD index. For 2UL with unequal power configuration, the IMD power increase would depend on both IMD order and IMD index.

Considering that the 2UL IMD MSD is rather complicated with unequal power configuration, in the spirit of HPUE MSD simplification, it is proposed RAN4 to only specify 2UL IMD MSD with equal UL power, that is, (20+20) for PC3, (23+23) for PC2, and (26+26) for PC1.5. In the case of (FDD + TDD) with (PC3 + PC1.5) as specified Rel-18, the MSD would be specified with (26+26) dBm while in the conformance test UE is allowed to only reach up to 23 dBm due to the PA limitation.

Proposal 4: RAN4 to only specify 2UL IMD MSD with equal UL power.

# 3	Conclusion

In this contribution, we share our views on the HPUE MSD PCx dependency on different single UL MSD mechanisms. For 2UL IMD MSD, we also provide our views on whether IMD orders are sufficient to design LUT/set of equations or both IMD orders and the IMD indexes should be accounted for.

Observation 1: For UL harmonic, the MSD degradation from PC3 to PC2 and PC1.5 would depend on the amount of harmonic power increase in main and diversity Rx paths which also depends on whether the UL is composed by 1Tx (PC2 only) or 2Tx.

Observation 2: For PC2 with 1Tx, the power ratio between main signal and 2nd harmonic has been assumed to be the same as PC3 to derive the PCx.

Observation 3: For UL harmonic order higher than 2, it has not been confirmed whether the same power ratio for PC3 PA would apply for PC2 PA. Therefore, a unified PCx may not be applicable for all harmonic orders.

Proposal 1: RAN4 to confirm whether the same power ratio between the main signal and UL harmonic for PC3 PA would apply for PC2 PA for all harmonic orders.

Observation 4: For Rx harmonic mixing, the interferer power increases by 3 dB and 6 dB for PC2 and PC1.5 respectively as compared to PC3.

Observation 5: The Rx harmonic mixing MSD would be dominated by PCB isolation, as a result, the MSD is expected to be the same between 1Tx and 2Tx.

Observation 6: A unified PCx would not be applicable between UL harmonic MSD and Rx harmonic mixing MSD.

Observation 7: For cross-band interference, the interferer is composed by a mixture of intermodulation products with different weighting on different modulation order which would depend on the frequency offset from the main carrier.

Observation 8: For cross-band interference, a unified PCx would not be applicable within the mechanism itself, nor with other MSD mechanisms.

Proposal 2: RAN4 to thoroughly evaluate whether a unified PCx would be applicable for all single UL MSD mechanisms.

Proposal 3: RAN4 to select an example band combination for 2UL IMD MSD analysis with 1Tx and 2Tx in PC2 FDD band to observe the MSD difference.

Observation 9: For 2UL with equal power configuration, the IMD power increase only depends on the IMD order but not IMD index. For 2UL with unequal power configuration, the IMD power increase would depend on both IMD order and IMD index.

Proposal 4: RAN4 to only specify 2UL IMD MSD with equal UL power.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2420380 “WF on MSD rule”, Skyworks et al, 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024

R4-2419699 “1UL HPUE NRCA MSD Simplifications for Rel-19”, Skyworks Solutions, Inc., 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024