3GPP RAN WG4 Meeting #114		R4-2500319

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.29.2.1

Source:	Apple

Title:	On NTN RedCap UE HD-FDD REFSENS

WI/SI:	NR_NTN_Ph3-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Considering that the support of RedCap devices operating in FR1 band NR-NTN networks can offer enhanced service capabilities (wideband/broadband) compared to IoT-NTN while ensuring low-complexity devices and the global coverage would clearly benefit RedCap devices, an objective on the “support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands” is included in the Rel-19 new WID on “Non-Terrestrial Networks (NTN) for NR Phase 3” led by RAN2 [1]. In the past few RAN4 meetings, although most of the UE RF requirements for NTN (e)RedCap UE have been agreed and captured in the corresponding WFs approved in each respective RAN4 meeting [2-6], RAN4 still could not reach consensus on the 2Rx HD-FDD REFSENS requirements for n255 and n254. In this contribution, we propose NTN RedCap UE REFSENS to follow the previous agreement for NR RedCap UE on the 2Rx HD-FDD 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS [7].

# Discussion

In the past few RAN4 meetings, although most of the UE RF requirements for NTN (e)RedCap UE have been agreed, RAN4 still could not reach consensus on the 2Rx HD-FDD REFSENS requirements for n255 and n254. Some companies were proposing 5MHz HD-FDD REFSENS tighter than -100dBm, such as -100.2dBm. In our view, the lower bound of the 5MHz HD-FDD REFSENS shall not be less than the lowest 5MHz REFSENS for TDD bands at -100dBm, and that was also the principle previously agreed for NR RedCap UE on 2Rx HD-FDD 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS, as summarized in Table 2-1. Our reasoning is that for FDD band with 5MHz REFSENS already specified at -100dBm as with the lowest 5MHz REFSENS for TDD band, the Tx self-interference should be negligible, and the duplexer insertion loss should already be assumed to be no worse than a single bandpass filter plus a T/R switch as for TDD band or HD-FDD front-end implementation. As a result, no further REFSENS improvement is expected from FD-FDD to HD-FDD.

Observation: For FDD band with 5MHz REFSENS already specified at -100dBm as with the lowest 5MHz REFSENS for TDD band, the Tx self-interference should be negligible, and the duplexer insertion loss should already be assumed to be no worse than a single bandpass filter plus a T/R switch as for TDD band or HD-FDD front-end implementation. As a result, no further REFSENS improvement is expected from FD-FDD to HD-FDD.

| 2Rx FD-FDD 5MHz REFSENS | 2Rx HD-FDD 5MHz REFSENS Tightening |
| --- | --- |
| ≤ -100 dBm | 0 dB |
| > -100 dBm and ≤ -99 dBm | 0.5 dB |
| > -99 dBm | 0.8 dB |

Table 2-1 2Rx HD-FDD 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS

For HD-FDD, since there is no UL self-interference to DL, the REFSENS for channel BW wider than 5MHz can be linearly scaled over 5MHz and calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n = 1 for 15kHz SCS, n = 2 for 30kHz SCS, and n = 4 for 60kHz SCS. Based on the above REFSENS tightening and the scaling over channel BW, the 2Rx HD-FDD REFSENS can be derived and specified as in Table 2-2.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n256 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Table 2-2 Proposed NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements

On the other hand, UL RB restriction would not be needed for HD-FDD REFSENS as there is no UL self-interference to DL. Therefore, the UL configuration for HD-FDD REFSENS shall be specified with full RB allocation which has already been agreed in the approved WF of RAN4 #111 meeting [3]. Table 2-3 summarizes the uplink configurations for NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements.

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | 25 | 50 | 75 | 100 | HD-FDD |
| n256 | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | 100 | HD-FDD |
| n255 | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 |  | HD-FDD |
| n254 | 30 |  | 24 | 36 |  |  |
|  | 60 |  | 10 | 18 |  |  |

Table 2-3 Uplink configurations for NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements

Proposal: NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements and UL configurations are specified as in Table 2-2 and Table 2-3 respectively.

# 3	Conclusion

In this contribution, we propose NTN RedCap UE REFSENS to follow the previous agreement for NR RedCap UE on the 2Rx HD-FDD 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS.

Proposal: NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements and UL configurations are specified as in Table 2-2 and Table 2-3 respectively.

# 4	References

RP-234078 “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Huawei, 3GPP TSG-RAN Meeting #102, Edinburgh, Scotland, December 11th – 15th, 2023

R4-2406610 “WF on NR_NTN_Ph3_UERF”, Qualcomm, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024

R4-2410574 “WF on Rel-19 phase 3 NR-NTN UE RF requirements”, Qualcomm Inc., 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

R4-2413527 “Way Forward for [112][311] NR_NTN_Ph3_UE_RF”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

R4-2416586 “Way Forward for [112bis][310] NR_NTN_Ph3_UE_RF”, Qualcomm, 3GPP TSG RAN WG4 Meeting #112bis, Hefei, China, October 14th – 18th, 2024

R4-2419897 “Way Forward for [113][310] NR_NTN_Ph3_UE_RF”, Qualcomm, 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024

R4-2203692 “RedCap UE HD-FDD REFSENS requirements”, Apple, 3GPP TSG RAN WG4 Meeting #102-e, Online, February 21st – March 3rd, 2022