3GPP TSG-RAN WG4 Meeting #114                      R4-2500329

Athens, GR, 17th - 21nd February 2025

Source:	China Telecom

Title:	Discussion on BS demodulation performance requirements for MMSE-IRC: Simulation Results

Agenda Item:	7.18.3

Document for:	discussion

Introduction

In RAN4#113 meeting, the WF on [112bis][319] NR_demod_Ph5_Part1_General_BS is approved in [1].

This tdoc provides the initial simulation results on BS demodulation performance requirements for MMSE-IRC.

Simulation results

Table 1. Simulation results for PUSCH 1Tx2Rx CP-OFDM

| SCS | BW | Mapping Type | MCS for target cell | Propagation condition for target cell | INR | MMSE (SNR) | MMSE-IRC(SNR) | gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz | 5MHz | A | 2 | TDLB100-400 | HomNet set1:(5.35, N/A) | 2.7 | -0.38 | 3.08 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  |  | HomNet set1:(7.28, -2.52) | 4.1 | 1.38 | 2.72 |
|  |  | B |  |  |  |  |  |  |
|  |  | A | 16 | TDLC300-100 | HomNet set1:(5.35, N/A) | 13.67 | 11.01 | 2.66 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 14.18 | 11.68 | 2.5 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLC300-100 | HomNet set1:(7.28, -2.52) | 15.89 | 13.32 | 2.57 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 15.86 | 13.32 | 2.54 |
|  |  | B |  |  |  |  |  |  |
| 15KHz | 5MHz | A | 2 | TDLB100-400 | HetNet set2:(9.82, N/A) | 6.6 | 1.2 | 5.4 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  | 8.13 | 2.66 | 5.47 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 | HetNet set2: (12.38, -0.97) | 9.25 | 4.1 | 5.15 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  | 9.92 | 4.67 | 5.25 |
|  |  | B |  |  |  |  |  |  |
|  |  | A | 16 | TDLC300-100 | HetNet set2:(9.82, N/A) | 17.49 | 12.45 | 5.04 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  | 17.64 | 12.58 | 5.06 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLC300-100 | HetNet set2: (12.38, -0.97) | 20.24 | 15.19 | 5.05 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  | 20.78 | 15.64 | 5.14 |
|  |  | B |  |  |  |  |  |  |
| SCS | BW | Mapping Type | MCS for target cell | Propagation condition for target cell |  | MMSE (SNR) | MMSE-IRC(SNR) | gain (SNR) |
| 30KHz | 10MHz | A | 2 | TDLB100-400 | HomNet set1:(5.35, N/A) | 2.71 | -0.64 | 3.35 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  |  | HomNet set1:(7.28, -2.52) | 4.7 | 1.63 | 3.07 |
|  |  | B |  |  |  |  |  |  |
|  |  | A | 16 | TDLC300-100 | HomNet set1:(5.35, N/A) | 14.52 | 11.48 | 3.04 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 14.45 | 10.89 | 3.56 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLC300-100 | HomNet set1:(7.28, -2.52) | 15.85 | 13.53 | 2.32 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 15.42 | 13.38 | 2.04 |
|  |  | B |  |  |  |  |  |  |
| 30KHz | 10MHz | A | 2 | TDLB100-400 | HetNet set2:(9.82, N/A) | 6.44 | 1.2 | 5.24 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  | 7.55 | 2 | 5.55 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 | HetNet set2: (12.38, -0.97) |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | A | 16 | TDLC300-100 | HetNet set2:(9.82, N/A) | 18.35 | 13.2 | 5.15 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  | 17.9 | 13.24 | 4.66 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLC300-100 | HetNet set2: (12.38, -0.97) |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  |  |  |  |
|  |  | B |  |  |  |  |  |  |

Table 2. Simulation results for PUSCH 1Tx4Rx CP-OFDM

| SCS | BW | Mapping Type | MCS for target cell | Propagation condition for target cell | INR | MMSE (SNR) | MMSE-IRC(SNR) | gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 5MHz | A | 16 | TDLC300-100 | HomNet set1: (7.28, -2.52) | 12.32 | 7.95 | 4.37 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 12.26 | 7.7 | 4.56 |
|  |  | B |  |  |  |  |  |  |
| 15kHz | 5MHz | A | 16 | TDLC300-100 | HetNet set2: (12.38, -0.97) | 17.08 | 9.22 | 7.86 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  | 15.8 | 8.75 | 7.05 |
|  |  | B |  |  |  |  |  |  |
| SCS | BW | Mapping Type | MCS for target cell | Propagation condition for target cell | INR | MMSE (SNR) | MMSE-IRC(SNR) | gain (SNR) |
| 30kHz | 10MHz | A | 16 | TDLC300-100 | HomNet set1: (7.28, -2.52) | 12.34 | 8.49 | 3.85 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 11.76 | 8.14 | 3.62 |
|  |  | B |  |  |  |  |  |  |
| 30kHz | 10MHz | A | 16 | TDLC300-100 | HetNet set2: (12.38, -0.97) |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  |  |  |  |
|  |  | B |  |  |  |  |  |  |

Table 3. Simulation results for PUSCH 1Tx8Rx CP-OFDM

| SCS | BW | Mapping Type | MCS for target cell | Propagation condition for target cell | INR | MMSE (SNR) | MMSE-IRC(SNR) | gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 5MHz | A | 16 | TDLC300-100 | HomNet set1: (7.28, -2.52) | 9.67 | 5.75 | 3.92 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 9.5 | 5.86 | 3.64 |
|  |  | B |  |  |  |  |  |  |
|  |  | A | 20 | TDLA30-10 |  | 12.2 | 8.42 | 3.78 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 13.2 | 9.88 | 3.32 |
|  |  | B |  |  |  |  |  |  |
| 15kHz | 5MHz | A | 16 | TDLC300-100 | HetNet set2: (12.38, -0.97) | 14.83 | 6.34 | 8.49 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  | 13.51 | 5.6 | 7.91 |
|  |  | B |  |  |  |  |  |  |
|  |  | A | 20 |  |  | 16.3 | 9 | 7.3 |
|  |  | B |  |  |  |  |  |  |
| SCS | BW | Mapping Type | MCS for target cell | Propagation condition for target cell | INR | MMSE (SNR) | MMSE-IRC(SNR) | gain (SNR) |
| 30kHz | 10MHz | A | 16 | TDLC300-100 | HomNet set1: (7.28, -2.52) | 9.86 | 6.33 | 3.53 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 9.53 | 5.72 | 3.81 |
|  |  | B |  |  |  |  |  |  |
|  |  | A | 20 | TDLA30-10 |  | 11.58 | 8.4 | 3.18 |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLB100-400 |  | 12.72 | 8.91 | 3.81 |
|  |  | B |  |  |  |  |  |  |
| 30kHz | 10MHz | A | 16 | TDLC300-100 | HetNet set2: (12.38, -0.97) |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | A |  | TDLA30-10 |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | A | 20 |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |

Reference

R4-2419780, Way Forward for [113][321] NR_demod_Ph5_Part1_General_BS, China Telecom, MediaTek, RAN4#113, Nov 2024.