3GPP TSG-RAN4 Meeting #114	R4-2500354

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 6083 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 36.101 Correction of CA_40-42 R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15_2DL1UL |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CA_40-42 has wrong note in Table 6.2.5-2 and Table 7.3.1-1A as dual UL is specified in Table 5.6A.1-2. Note change note 4 note 7. MSD TP is invalid as simultaneous Rx/Tx is not supported. MSD TP is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note number is chaged and MSD TP is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.101 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************************** Start of changes ****************************************

Table 6.2.5-2: ΔTIB,c (two bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3, CA_1-1-3, CA_1-1-3-3, CA_1-3-3 | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1-5 | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1-7, CA_1-1-7, CA_1-7-7, CA_1-1-7-7 | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1-8 | 1 | 0.3 |
|  | 8 | 0.3 |
| CA_1-11 | 1 | 0.3 |
|  | 11 | 0.3 |
| CA_1-18 | 1 | 0.3 |
|  | 18 | 0.3 |
| CA_1-19 | 1 | 0.3 |
|  | 19 | 0.3 |
| CA_1-20, CA_1-1-20 | 1 | 0.3 |
|  | 20 | 0.3 |
| CA_1-21 | 1 | 0.3 |
|  | 21 | 0.3 |
| CA_1-26 | 1 | 0.3 |
|  | 26 | 0.3 |
| CA_1-28, CA_1-1-28 | 1 | 0.3 |
|  | 28 | 0.6 |
| CA_1-32 | 1 | 0.5 |
|  | 32 | N/A |
| CA_1-38, CA_1-1-38 | 1 | 0.5 |
|  | 38 | 0.5 |
| CA_1-40 CA_1-40-40 | 1 | 0.5 |
|  | 40 | 0.5 |
| CA_1-418 CA_1-41-41 | 1 | 0.5 |
|  | 41 | 0.5 |
| CA_1-42, CA_1-42-42 | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1-43 | 1 | 0.3 |
|  | 43 | 0.8 |
| CA_1-46 | 1 | 0 |
| CA_2-4, CA_2-2-4, CA_2-4-4, CA_2-2-4-4 | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2-5, CA_2-2-5 | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2-7, CA_2-2-7, CA_2-7-7, CA_2-2-7-7 | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2-8 | 2 | 0.3 |
|  | 8 | 0.3 |
| CA_2-12, CA_2-2-12, CA_2-12-12, CA_2-2-12-12 | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2-13, CA_2-2-13 | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2-14, CA_2-2-14 | 2 | 0.3 |
|  | 14 | 0.3 |
| CA_2-17 | 2 | 0.3 |
|  | 17 | 0.8 |
| CA_2-26 | 2 | 0.3 |
|  | 26 | 0.3 |
| CA_2-28 | 2 | 0.3 |
|  | 28 | 0.3 |
| CA_2-29, CA_2-2-29 | 2 | 0.3 |
| CA_2-30, CA_2-2-30 | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2-38 | 2 | 0.5 |
|  | 38 | 0.5 |
| CA_2-46, CA_2-2-46, CA_2-46-46 | 2 | 0 |
| CA_2-48, CA_2-48-48 | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2-49 | 2 | 0.6 |
| CA_2-66, CA_2-2-66, CA_2-66-66, CA_2-2-66-66, CA_2-66-66-66 | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2-71, CA_2-2-71 | 2 | 0.3 |
|  | 71 | 0.3 |
| CA_3-5, CA_3-3-5 | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_3-7, CA_3-3-7, CA_3-7-7, CA_3-3-7-7 | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3-8, CA_3-3-8 | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_3-11 | 3 | 0.8 |
|  | 11 | 0.9 |
| CA_3-18 | 3 | 0.3 |
|  | 18 | 0.3 |
| CA_3-19, CA_3-3-19 | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_3-20, CA_3-3-20 | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_3-21, CA_3-3-21 | 3 | 0.8 |
|  | 21 | 0.9 |
| CA_3-26 | 3 | 0.3 |
|  | 26 | 0.3 |
| CA_3-27 | 3 | 0.3 |
|  | 27 | 0.3 |
| CA_3-28 | 3 | 0.3 |
|  | 28 | 0.3 |
| CA_3-28-32 | 3 | 0.5 |
|  | 28 | 0.7 |
| CA_3-31 | 3 | 0.3 |
|  | 31 | 0.6 |
| CA_3-32 | 3 | 0.5 |
| CA_3-38 CA_3-38 | 3 | 0,5 |
|  | 38 | 0,5 |
| CA_3-40, CA_3-40-40 | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3-41, CA_3-3-41, CA_3-41-41 | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3-42, CA_3-3-42, CA_3-42-42 | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3-43 | 3 | 0.3 |
|  | 43 | 0.8 |
| CA_3-46, CA_3-3-46 | 3 | 0 |
| CA_3-67 | 3 | 0.3 |
| CA_3-69 | 3 | 0.5 |
| CA_4-5, CA_4-4-5 | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4-7, CA_4-4-7, CA_4-7-7 | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4-12, CA_4-4-12, CA_4-12-12, CA_4-4-12-12 | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4-13, CA_4-4-13 | 4 | 0.3 |
|  | 13 | 0.3 |
| CA_4-17 | 4 | 0.3 |
|  | 17 | 0.8 |
| CA_4-27 | 4 | 0.3 |
|  | 27 | 0.3 |
| CA_4-28 | 4 | 0.3 |
|  | 28 | 0.6 |
| CA_4-29, CA_4-4-29 | 4 | 0.3 |
| CA_4-30, CA_4-4-30 | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_4-46, CA_4-46-46 | 4 | 0 |
| CA_4-48 | 4 | 0.3 |
|  | 48 | 0.8 |
| CA_4-71, CA_4-4-71 | 4 | 0.3 |
|  | 71 | 0.3 |
| CA_5-7, CA_5-7-7 | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5-12, CA_5-12-12 | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5-13 | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_5-17 | 5 | 0.8 |
|  | 17 | 0.4 |
| CA_5-25 | 5 | 0.3 |
|  | 25 | 0.3 |
| CA_5-28 | 5 | 0.5 |
|  | 28 | 0.5 |
| CA_5-29 | 5 | 0.5 |
| CA_5-30 | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_5-38 | 5 | 0.3 |
|  | 38 | 0.3 |
| CA_5-40, CA_5-5-40, CA_5-40-40 | 5 | 0.3 |
|  | 40 | 0.3 |
| CA_5-41 | 5 | 0.3 |
|  | 41 | 0.3 |
| CA_5-46 | 5 | 0 |
| CA_5-48 | 5 | 0.3 |
|  | 48 | 0.3 |
| CA_5-66, CA_5-5-66, CA_5-66-66, CA_5-5-66-66 | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_7-8, CA_7-7-8 | 7 | 0.3 |
|  | 8 | 0.6 |
| CA_7-12 | 7 | 0.3 |
|  | 12 | 0.3 |
| CA_7-13 | 7 | 0.3 |
|  | 13 | 0.3 |
| CA_7-20, CA_7-7-20 | 7 | 0.3 |
|  | 20 | 0.3 |
| CA_7-22 | 7 | 0.5 |
|  | 22 | 0.8 |
| CA_7-25 | 7 | 0.5 |
|  | 25 | 0.5 |
| CA_7-26, CA_7-7-26 | 7 | 0.3 |
|  | 26 | 0.3 |
| CA_7-28, CA_7-7-28 | 7 | 0.3 |
|  | 28 | 0.3 |
| CA_7-29 CA_7-7-29 | 7 | 0.3 |
| CA_7-30 | 7 | 0.5 |
|  | 30 | 0.5 |
| CA_7-32 | 7 | 0.7 |
| CA_7-40 CA_7-40-40 | 7 | 0.5 |
|  | 40 | [0.6] |
| CA_7-42, CA_7-42-42 | 7 | 0.5 |
|  | 42 | 0.8 |
| CA_7-46, CA_7-7-46 | 7 | 0 |
| CA_7-66, CA_7-7-66, CA_7-66-66, CA_7-7-66-66 | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_8-11 | 8 | 0.3 |
|  | 11 | 0.4 |
| CA_8-20 | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_8-27 | 8 | 0.8 |
|  | 27 | 0.8 |
| CA_8-2814 | 8 | 0.6 |
|  | 28 | 0.5 |
| CA_8-32 | 8 | 0.3 |
| CA_8-38 | 8 | 0.3 |
|  | 38 | 0.3 |
| CA_8-39 | 8 | 0,3 |
|  | 39 | 0,3 |
| CA_8-40 | 8 | 0.3 |
|  | 40 | 0.3 |
| CA_8-41, CA_8-41-41 | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8-42 | 8 | 0.6 |
|  | 42 | 0.8 |
| CA_8-46 | 8 | 0.6 |
|  | 46 | 0 |
| CA_8-48 | 8 | 0.6 |
|  | 48 | 0.8 |
| CA_11-18 | 11 | 0.3 |
|  | 18 | 0.3 |
| CA_11-26 | 11 | 0.3 |
|  | 26 | 0.3 |
| CA_11-28 | 11 | 0.4 |
|  | 28 | 0.6 |
| CA_11-41 | 11 | 0.3 |
|  | 41 | 0.3 |
| CA_11-42 | 11 | 0.4 |
|  | 42 | 0.8 |
| CA_11-46 | 11 | 0 |
| CA_12-25 | 12 | 0.3 |
|  | 25 | 0.3 |
| CA_12-30 | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_12-46 | 12 | 0 |
|  | 46 | 0 |
| CA_12-48 | 12 | 0.3 |
|  | 48 | 0.3 |
| CA_12-66, CA_12-66-66 | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_12-71 | 12 | 1 |
|  | 71 | 1 |
| CA_13-46, CA_13-46-46 | 13 | 0 |
| CA_13-48, CA_13-48-48 | 13 | 0.3 |
|  | 48 | 0.3 |
| CA_13-66, CA_13-66-66 | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_14-30 | 14 | 0.3 |
|  | 30 | 0.3 |
| CA_14-66, CA_14-66-66, CA_14-66-66-66 | 14 | 0.3 |
|  | 66 | 0.3 |
| CA_18-289 | 18 | 0.5 |
|  | 28 | 0.5 |
| CA_18-41 | 18 | 0.3 |
|  | 41 | 0.3 |
| CA_19-21 | 19 | 0.3 |
|  | 21 | 0.4 |
| CA_19-289 | 19 | 0.5 |
|  | 28 | 0.5 |
| CA_18-42 | 18 | 0.3 |
|  | 42 | 0.8 |
| CA_19-42 | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_19-46 | 19 | 0 |
| CA_20-28 | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_20-31 | 20 | 0.5 |
|  | 31 | 0.5 |
| CA_20-32 | 20 | 0.3 |
| CA_20-38 | 20 | 0.3 |
|  | 38 | 0.3 |
| CA_20-40, CA_20-40-40 | 20 | 0.3 |
|  | 40 | 0.3 |
| CA_20-41 | 20 | 0.3 |
|  | 41 | 0.3 |
| CA_20-42, CA_20-42-42 | 20 | 0.6 |
|  | 42 | 0.8 |
| CA_20-43 | 20 | 0.3 |
|  | 43 | 0.8 |
| CA_20-67 | 20 | 0.5 |
| CA_20-75 | 20 | 0.3 |
| CA_20-76 | 20 | 0.3 |
| CA_21-28 | 21 | 0.4 |
|  | 28 | 0.3 |
| CA_21-42 | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_21-46 | 21 | 0 |
| CA_23-29 | 23 | 0.3 |
| CA_25-26, CA_25-25-26 | 25 | 0.3 |
|  | 26 | 0.3 |
| CA_25-41, CA_25-25-41 | 25 | 0.5 |
|  | 41 | 0.410 |
|  |  | 0.911 |
| CA_25-46 | 25 | 0 |
|  | 46 | 0 |
| CA_25-66 | 25 | 0.5 |
|  | 66 | 0.5 |
| CA_26-38 | 26 | 0.3 |
|  | 38 | 0.3 |
| CA_26-41 | 26 | 0.3 |
|  | 41 | 0.3 |
| CA_26-46 | 26 | 0 |
| CA_26-48, CA_26-48-48 | 26 | 0.3 |
|  | 48 | 0.3 |
| A_26-66 | 26 | 0.3 |
|  | 66 | 0.3 |
| CA_28-32 | 28 | 0.3 |
| CA_28-38 | 28 | 0.3 |
|  | 38 | 0.3 |
| CA_28-40 CA_28-40-40 | 28 | 0.3 |
|  | 40 | 0.3 |
| CA_28-41 | 28 | 0.3 |
|  | 41 | 0.3 |
| CA_28-42, CA_28-42-42 | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_28-46 | 28 | 0 |
| CA_28-66 | 28 | 0.6 |
|  | 66 | 0.3 |
| CA_29-30 | 30 | 0.3 |
| CA_29-66, CA_29-66-66 | 66 | 0.3 |
| CA_29-70 | 70 | 0.3 |
| CA_30-48 | 30 | 04 |
|  | 48 | 0.54 |
| CA_30-66, CA_30-66-66 | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_32-38 | 38 | 0.7 |
| CA_32-42 | 42 | 0.8 |
| CA_32-43 | 43 | 0.8 |
| CA_34-39 | 34 | 01 |
|  | 39 | 01 |
| CA_34-41 | 34 | 01 |
|  | 41 | 01 |
| CA_38-40, CA_38-40-40 | 38 | 04 |
|  | 40 | 04 |
| CA_38-66 | 38 | 0.5 |
|  | 66 | 0.5 |
| CA_39-40 | 39 | 04 |
|  | 40 | 04 |
| CA_39-41 | 39 | 04 |
|  | 41 | 04 |
| CA_39-41 | 39 | 0.57 |
|  | 41 | 0.57 |
| CA_39-42 | 39 | 04 |
|  | 42 | 0.54 |
| CA_39-46 | 39 | 0 |
| CA_40-41 | 40 | 0.54 |
|  | 41 | 0.54 |
| CA_40-42 | 40 | 07 |
|  | 42 | 0.57 |
| CA_40-43 | 40 | 04 |
|  | 43 | 0.54 |
| CA_40-46 | 40 | 0 |
| CA_41-42, CA_41-42-42 | 41 | 04 |
|  | 42 | 0.54 |
| CA_41-42, CA_41-42-42 | 41 | 0.37 |
|  | 42 | 0.87 |
| CA_41-46 | 41 | 0 |
| CA_41-48 | 41 | 04 |
|  | 48 | 0.54 |
| CA_42-43 | 42 | 04 |
|  | 43 | 04 |
| CA_42-46 | 42 | [0.5] |
| CA_46-48, CA_46-48-48 | 48 | 0.8 |
| CA_46-53 | 53 | 0 |
| CA_46-66, CA_46-46-66, CA_46-66-66 | 66 | 0 |
| CA_46-70 | 70 | 0 |
| CA_46-71 | 71 | 0 |
| CA_48-53 | 48 | 0.54 |
|  | 53 | 04 |
| CA_48-66, CA_48-48-66, CA_48-66-66, CA_48-48-66-66 | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_48-71, CA_48-48-71 | 48 | 0.3 |
|  | 71 | 0.3 |
| CA_66-70, CA_66-66-70 | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_66-71, CA_66-66-71 | 66 | 0.3 |
|  | 71 | 0.3 |
| CA_70-71 | 70 | 0.3 |
|  | 71 | 0.6 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances above, truncated to one decimal place for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 2DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 12: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c) NOTE 13: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔTIB,c in Band 42, 43, or 48 is the max(Band 42 ΔTIB,c , Band 43 ΔTIB,c, Band 48 ΔTIB,c). NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |

************************************ No Changes *************************************

Table 7.3.1-1A: ΔRIB,c (two bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3, CA_1-1-3, CA_1-1-3-3, CA_1-3-3 | 1 | 0 |
|  | 3 | 0 |
| CA_1-5, CA_1-1-5 | 1 | 0 |
|  | 5 | 0 |
| CA_1-7, CA_1-1-7, CA_1-7-7 , CA_1-1-7-7 | 1 | 0 |
|  | 7 | 0 |
| CA_1-8 | 1 | 0 |
|  | 8 | 0 |
| CA_1-11 | 1 | 0 |
|  | 11 | 0 |
| CA_1-18 | 1 | 0 |
|  | 18 | 0 |
| CA_1-19 | 1 | 0 |
|  | 19 | 0 |
| CA_1-20, CA_1-1-20 | 1 | 0 |
|  | 20 | 0 |
| CA_1-21 | 1 | 0 |
|  | 21 | 0 |
| CA_1-26 | 1 | 0 |
|  | 26 | 0 |
| CA_1-28, CA_1-1-28 | 1 | 0 |
|  | 28 | 0.2 |
| CA_1-32 | 1 | 0 |
|  | 32 | 0 |
| CA_1-38, CA_1-1-38 | 1 | 0 |
|  | 38 | 0 |
| CA_1-40 CA_1-40-40 | 1 | 0 |
|  | 40 | 0 |
| CA_1-418, CA_1-41-41 | 1 | 0 |
|  | 41 | 0 |
| CA_1-42, CA_1-42-42 | 1 | 0 |
|  | 42 | 0.5 |
| CA_1-43 | 1 | 0 |
|  | 43 | 0.5 |
| CA_1-46 | 1 | 0 |
| CA_2-4, CA_2-2-4, CA_2-4-4, CA_2-2-4-4 | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2-5, CA_2-2-5 | 2 | 0 |
|  | 5 | 0 |
| CA_2-7, CA_2-2-7, CA_2-7-7, CA_2-2-7-7 | 2 | 0 |
|  | 7 | 0 |
| CA_2-8 | 2 | 0 |
|  | 8 | 0 |
| CA_2-12, CA_2-2-12, CA_2-12-12, CA_2-2-12-12 | 2 | 0 |
|  | 12 | 0 |
| CA_2-13, CA_2-2-13 | 2 | 0 |
|  | 13 | 0 |
| CA_2-14, CA_2-2-14 | 2 | 0 |
|  | 14 | 0 |
| CA_2-17 | 2 | 0 |
|  | 17 | 0.5 |
| CA_2-26 | 2 | 0 |
|  | 26 | 0 |
| CA_2-28 | 2 | 0 |
|  | 28 | 0 |
| CA_2-29, CA_2-2-29 | 2 | 0 |
| CA_2-30, CA_2-2-30 | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_2-38 | 2 | 0 |
|  | 38 | 0 |
| CA_2-46, CA_2-2-46 | 2 | 0 |
| CA_2-48, CA_2-48-48 | 2 | 0.2 |
|  | 48 | 0.5 |
| CA_2-49 | 2 | 0.2 |
| CA_2-66, CA_2-2-66, CA_2-66-66, CA_2-2-66-66, CA_2-66-66-66 | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2-71, CA_2-2-71 | 2 | 0 |
|  | 71 | 0 |
| CA_3-5, CA_3-3-5 | 3 | 0 |
|  | 5 | 0 |
| CA_3-7, CA_3-3-7, CA_3-7-7, CA_3-3-7-7 | 3 | 0 |
|  | 7 | 0 |
| CA_3-8, CA_3-3-8 | 3 | 0 |
|  | 8 | 0 |
| CA_3-11 | 3 | 0.3 |
|  | 11 | 0.5 |
| CA_3-18 | 3 | 0 |
|  | 18 | 0 |
| CA_3-19, CA_3-3-19 | 3 | 0 |
|  | 19 | 0 |
| CA_3-20, CA_3-3-20 | 3 | 0 |
|  | 20 | 0 |
| CA_3-21, CA_3-3-21 | 3 | 0.3 |
|  | 21 | 0.5 |
| CA_3-26 | 3 | 0 |
|  | 26 | 0 |
| CA_3-27 | 3 | 0 |
|  | 27 | 0 |
| CA_3-28 | 3 | 0 |
|  | 28 | 0 |
| CA_3-31 | 3 | 0 |
|  | 31 | 0.2 |
| CA_3-32 | 3 | 0 |
|  | 32 | 0 |
| CA_3-38 CA_3-3-38 | 3 | 0 |
|  | 38 | 0 |
| CA_3-40, CA_3-40-40 | 3 | 0 |
|  | 40 | 0 |
| CA_3-41, CA_3-3-41, CA_3-41-41 | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3-42, CA_3-3-42, CA_3-42-42 | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_3-43 | 3 | 0 |
|  | 43 | 0.5 |
| CA_3-46, CA_3-3-46 | 3 | 0 |
| CA_3-67 | 3 | 0 |
|  | 67 | 0 |
| CA_4-5, CA_4-4-5 | 4 | 0 |
|  | 5 | 0 |
| CA_4-7, CA_4-4-7, CA_4-7-7 | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4-12, CA_4-4-12, CA_4-12-12, CA_4-4-12-12 | 4 | 0 |
|  | 12 | 0.5 |
| CA_4-13, CA_4-4-13 | 4 | 0 |
|  | 13 | 0 |
| CA_4-17 | 4 | 0 |
|  | 17 | 0.5 |
| CA_4-27 | 4 | 0 |
|  | 27 | 0 |
| CA_4-28 | 4 | 0 |
|  | 28 | 0.2 |
| CA_4-29, CA_4-4-29 | 4 | 0 |
| CA_4-30, CA_4-4-30 | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_4-46 | 4 | 0 |
| CA_4-48 | 4 | 0 |
|  | 48 | 0.5 |
| CA_4-71, CA_4-4-71 | 4 | 0 |
|  | 71 | 0 |
| CA_5-7, CA_5-7-7 | 5 | 0 |
|  | 7 | 0 |
| CA_5-12, CA_5-12-12 | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5-13 | 5 | 0 |
|  | 13 | 0 |
| CA_5-17 | 5 | 0.5 |
|  | 17 | 0.3 |
| CA_5-25 | 5 | 0 |
|  | 25 | 0 |
| CA_5-28 | 5 | 0 |
|  | 28 | 0 |
| CA_5-29 | 5 | 0 |
| CA_5-30 | 5 | 0 |
|  | 30 | 0 |
| CA_5-38 | 5 | 0 |
|  | 38 | 0 |
| CA_5-40, CA_5-5-40, CA_5-40-40 | 5 | 0 |
|  | 40 | 0 |
| CA_5-41 | 5 | 0 |
|  | 41 | 0 |
| CA_5-48 | 5 | 0 |
|  | 48 | 0 |
| CA_5-66, CA_5-5-66, CA_5-66-66, CA_5-5-66-66 | 5 | 0 |
|  | 66 | 0 |
| CA_7-8, CA_7-7-8 | 7 | 0 |
|  | 8 | 0.2 |
| CA_7-12 | 7 | 0 |
|  | 12 | 0 |
| CA_7-13 | 7 | 0 |
|  | 13 | 0 |
| CA_7-20, CA_7-7-20 | 7 | 0 |
|  | 20 | 0 |
| CA_7-22 | 7 | 0 |
|  | 22 | 0.5 |
| CA_7-25 | 7 | 0 |
|  | 25 | 0 |
| CA_7-26, CA_7-7-26 | 7 | 0 |
|  | 26 | 0 |
| CA_7-28, CA_7-7-28 | 7 | 0 |
|  | 28 | 0 |
| CA_7-29, CA_7-7-29 | 7 | 0 |
| CA_7-30 | 7 | 0.5 |
|  | 30 | 0.5 |
| CA_7-32 | 7 | 0 |
|  | 32 | 0 |
| CA_7-40 CA_7-40-40 | 7 | 0 |
|  | 40 | 0.5 |
| CA_7-42, CA_7-42-42 | 7 | 0 |
|  | 42 | 0.5 |
| CA_7-46, CA_7-7-46 | 7 | 0 |
| CA_7-66, CA_7-7-66, CA_7-66-66, CA_7-7-66-66 | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_8-11 | 8 | 0 |
|  | 11 | 0 |
| CA_8-20 | 8 | 0 |
|  | 20 | 0 |
| CA_8-27 | 8 | 0.3 |
|  | 27 | 0.3 |
| CA_8-2813 | 8 | 0.2 |
|  | 28 | 0.1 |
| CA_8-32 | 8 | 0 |
|  | 32 | 0 |
| CA_8-38 | 8 | 0 |
|  | 38 | 0 |
| CA_8-39 | 8 | 0 |
|  | 39 | 0 |
| CA_8-40 | 8 | 0 |
|  | 40 | 0 |
| CA_8-41,CA_8-41-41 | 8 | 0 |
|  | 41 | 0 |
| CA_8-42 | 8 | 0.2 |
|  | 42 | 0.5 |
| CA_8-46 | 8 | 0 |
| CA_8-48 | 8 | 0.2 |
|  | 48 | 0.5 |
| CA_11-18 | 11 | 0 |
|  | 18 | 0 |
| CA_11-26 | 11 | 0 |
|  | 26 | 0 |
| CA_11-28 | 11 | 0 |
|  | 28 | 0.2 |
| CA_11-41 | 11 | 0 |
|  | 41 | 0 |
| CA_11-42 | 11 | 0 |
|  | 42 | 0.5 |
| CA_11-46 | 11 | 0 |
| CA_12-25 | 12 | 0 |
|  | 25 | 0 |
| CA_12-30 | 12 | 0 |
|  | 30 | 0 |
| CA_12-46 | 12 | 0 |
|  | 46 | 0 |
| CA_12-48 | 12 | 0 |
|  | 48 | 0 |
| CA_12-66, CA_12-66-66 | 12 | 0.5 |
|  | 66 | 0 |
| CA_12-71 | 12 | 0.8 |
|  | 71 | 0.8 |
| CA_13-46, CA_13-46-46 | 13 | 0 |
| CA_13-48, CA_13-48-48 | 13 | 0 |
|  | 48 | 0 |
| CA_13-66, CA_13-66-66 | 13 | 0 |
|  | 66 | 0 |
| CA_14-30 | 14 | 0 |
|  | 30 | 0 |
| CA_14-66, CA_14-66-66, CA_14-66-66-66 | 14 | 0 |
|  | 66 | 0 |
| CA_18-289 | 18 | 0 |
|  | 28 | 0 |
| CA_18-41 | 18 | 0 |
|  | 41 | 0 |
| CA_18-42 | 18 | 0 |
|  | 42 | 0.5 |
| CA_19-21 | 19 | 0 |
|  | 21 | 0 |
| CA_19-289 | 19 | 0 |
|  | 28 | 0 |
| CA_19-42 | 19 | 0 |
|  | 42 | 0.5 |
| CA_19-46 | 19 | 0 |
| CA_20-28 | 20 | 0 |
|  | 28 | 0 |
| CA_20-31 | 20 | 0 |
|  | 31 | 0 |
| CA_20-32 | 20 | 0 |
| CA_20-38 | 20 | 0 |
|  | 38 | 0 |
| CA_20-40, CA_20-40-40 | 20 | 0 |
|  | 40 | 0 |
| CA_20-41 | 20 | 0 |
|  | 41 | 0 |
| CA_20-42, CA_20-42-42 | 20 | 0 |
|  | 42 | 0.5 |
| CA_20-43 | 20 | 0 |
|  | 43 | 0.5 |
| CA_20-67 | 20 | 0 |
| CA_20-75 | 20 | 0 |
| CA_20-76 | 20 | 0 |
| CA_21-28 | 21 | 0 |
|  | 28 | 0 |
| CA_21-42 | 21 | 0 |
|  | 42 | 0.5 |
| CA_21-46 | 21 | 0 |
| CA_23-29 | 23 | 0 |
| CA_25-26, CA_25-25-26 | 25 | 0 |
|  | 26 | 0 |
| CA_25-41, CA_25-25-41 | 25 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_25-46 | 25 | 0 |
|  | 46 | 0 |
| CA_25-66 | 25 | 0.3 |
|  | 66 | 0.3 |
| CA_26-38 | 26 | 0 |
|  | 38 | 0 |
| CA_26-41 | 26 | 0 |
|  | 41 | 0 |
| CA_26-46 | 26 | 0 |
| CA_26-48, CA_26-48-48 | 26 | 0 |
|  | 48 | 0 |
| CA_26-66 | 26 | 0 |
|  | 66 | 0 |
| CA_28-32 | 28 | 0 |
| CA_28-38 | 28 | 0 |
|  | 38 | 0 |
| CA_28-40 CA_28-40-40 | 28 | 0 |
|  | 40 | 0 |
| CA_28-41 | 28 | 0 |
|  | 41 | 0 |
| CA_28-42, CA_28-42-42 | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_28-46 | 28 | 0 |
| CA_28-66 | 28 | 0.2 |
|  | 66 | 0 |
| CA_29-30 | 30 | 0 |
| CA_29-66, CA_29-66-66 | 66 | 0 |
| CA_29-70 | 70 | 0 |
| CA_30-48 | 30 | 04 |
|  | 48 | 0.54 |
| CA_30-66, CA_30-66-66 | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_32-42 | 42 | 0.5 |
| CA_32-43 | 43 | 0.5 |
| CA_34-39 | 34 | 0.21 |
|  | 39 | 0.21 |
| CA_34-41 | 34 | 0.21 |
|  | 41 | 0.21 |
| CA_38-40, CA_38-40-40 | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_38-66 | 38 | 0.5 |
|  | 66 | 0.5 |
| CA_39-40 | 39 | 0.34 |
|  | 40 | 0.34 |
| CA_39-41 | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39-41 | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39-42 | 39 | 04 |
|  | 42 | 0.54 |
| CA_39-46 | 39 | 0 |
| CA_40-41 | 40 | 04 |
|  | 41 | 04 |
| CA_40-42 | 40 | 0.47 |
|  | 42 | 0.57 |
| CA_40-43 | 40 | 0.44 |
|  | 43 | 0.54 |
| CA_40-46 | 40 | 0 |
| CA_41-42, CA_41-42-42 | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41-42, CA_41-42-42 | 41 | 07 |
|  | 42 | 0.57 |
| CA_41-46 | 41 | 0 |
| CA_41-48 | 41 | 04 |
|  | 48 | 0.54 |
| CA_42-43 | 42 | 04 |
|  | 43 | 04 |
| CA_42-46 | 42 | [0] |
| CA_46-48, CA_46-48-48 | 48 | 0.5 |
| CA_46-53 | 53 | 0 |
| CA_46-66, CA_46-66-66 | 66 | 0 |
| CA_46-70 | 70 | 0 |
| CA_46-71 | 71 | 0 |
| CA_48-53 | 48 | 0.54 |
|  | 53 | 04 |
| CA_48-66, CA_48-48-66, CA_48-66-66, CA_48-48-66-66 | 48 | 0.5 |
|  | 66 | 0.2 |
| CA_48-71, CA_48-48-71 | 48 | 0 |
|  | 71 | 0 |
| CA_66-70, CA_66-66-70 | 66 | 0 |
|  | 70 | 0 |
| CA_66-71, CA_66-66-71 | 66 | 0 |
|  | 71 | 0 |
| CA_70-71 | 70 | 0 |
|  | 71 | 0 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances in Table 7.3.1-1A, truncated to one decimal place that would apply for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum 2DL tolerance in Table 7.3.1-1A that would apply for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 12: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔRIB,c in Band 42, 43, or 48 is the max(Band 42 ΔRIB,c , Band 43 ΔRIB,c, Band 48 ΔRIB,c). NOTE 13: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |

************************************ No Changes *************************************

Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | 3 | 1760 | 5 | 25 | 1855 | N/A |  | N/A |
| CA_1A-8A | 1 | 1965 | 5 | 25 | 2155 | 6 | FDD | IMD4 |
|  | 8 | 887.5 | 5 | 25 | 932.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1860 | 20 | 502 | 1940 | 5 | FDD | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | FDD | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 |  | IMD5 |
| CA_2A-46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | FDD | IMD34 |
|  | 46 | 5720 | 20 | 100 | 5720 | N/A |  | N/A |
| CA_2A-48A | 2 | 1852.5 | 5 | 25 | 1932.5 | [12] | FDD | IMD4 |
|  | 48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_2A-49A | 2 | 1852.5 | 5 | 25 | 1932.5 | [12] | FDD | IMD4 |
|  | 49 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_2A-66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A |  | N/A |
| CA_2A-66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 |  | IMD5 |
| CA_3A-5A | 3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | 5 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-5A | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 24 |  | IMD24 |
| CA_3A-7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 |  | IMD4 |
| CA_3A-8A | 3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 |  | IMD44 |
| CA_3A-8A | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A |  | N/A |
| CA_3A-18A | 3 | 1721 | 5 | 25 | 1816 | 4 | FDD | IMD4 |
|  | 18 | 823 | 5 | 25 | 868 | N/A |  | N/A |
| CA_3A-19A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | 19 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-19A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 19 | 838 | 5 | 25 | 883 | 27 |  | IMD24 |
| CA_3A-20A | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 |
|  | 20 | 840 | 5 | 25 | 799 | N/A |  | N/A |
| CA_3A-20A | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | 20 | 847 | 5 | 25 | 806 | 9 |  | IMD4 |
| CA_3A-26A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | 26 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-26A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 26 | 838 | 5 | 25 | 883 | 26 |  | IMD24 |
| CA_3A-38A | 3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | 38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| CA_3A-41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | TDD | N/A |
| CA_3A-42A | 3 | 1740 | 5 | 25 | 1835 | 29.8 | FDD | IMD24 |
|  | 42 | 3575 | 5 | 25 | 3575 | N/A | TDD | N/A |
| CA_3A-42A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | 42 | 3435 | 5 | 25 | 3435 | N/A | TDD | N/A |
| CA_4A-5A | 4 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 26 |  | IMD24 |
| CA_4A-7A | 4 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | 7 | 2535 | 5 | 25 | 2655 | 15 |  | IMD4 |
| CA_5A-7A | 5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | 7 | 2547 | 10 | 50 | 2667 | N/A |  | N/A |
| CA_5A-66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |
| CA_7A-20A | 7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 12 |  | IMD34 |
| CA_7A-40A CA_7A-40C CA_7A-40D CA_7A-40A-40A | 7 | 2510 | 5 | 25 | 2630 | 23 | FDD | IMD3 |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
| CA_7A-26A | 7 | 2556 | 5 | 25 | 2676 | N/A | FDD-FDD | N/A |
|  | 26 | 837 | 5 | 25 | 882 | 16.0 |  | IMD3 |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD-FDD | IMD5 |
|  | 26 | 816.5 | 5 | 25 | 861.5 | N/A |  | N/A |
| CA_8A-20A | 20 | 849.5 | 5 | 25 | 808.5 | 25 | FDD | IMD34 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A |  | N/A |
|  | 20 | 847.5 | 5 | 25 | 806.5 | N/A |  | N/A |
|  | 8 | 892.5 | 5 | 25 | 937.5 | 25 |  | IMD34 |
| CA_8A-41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | 41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_19A-42A3 | 19 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_21A-28A | 21 | 1450.4 | 5 | 25 | 1498.4 | [2.5] | FDD | IMD5 |
|  | 28 | 735.5 | 5 | 25 | 790.5 | N/A | TDD | N/A |
| CA_21A-42A3 | 21 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_28A-42A | 28 | 705.5 | 5 | 25 | 760.5 | [5.5] | FDD | IMD5 |
|  | 42 | 3582.5 | 5 | 25 | 3582.5 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void NOTE 6: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |  |  |  |

**************************************** End of changes ****************************************