3GPP TSG-RAN WG4 Meeting #114	R4-2500375

Athens, Greece, 17-21 Feb 2025

Agenda item:			7.5.2.2

Source:	Nokia

Title:	BS signalling and Initial condition

Document for:	Approval

# 1		Introduction

In this contribution we discuss some open issues in [1].

# 2		Discussion

### 2.1	New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated)>

Way forward:

- For NR-CA, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

- For EN-DC, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

In previous meeting it was our understanding that RAN4 agreed to introduce separate BS signalling to command UE from Type 4 to Type 1 4L and another signalling to command UE fromType 4 to Type 1 larger than 4L per CC. But as English is hard language the WF is not very clear on this.

When looking at R18 network signaling nonCollocatedTypeNR-CA, it is only present for UE configured with maxMIMO-Layers <= 2. There is no ambiguity of maxMIMOLayers when network indicates to switch to Type 1. But for R19, the BS signaling is sent to the UE configured with maxMIMOlayers <=4. If single signaling as R18 is adopted, it is unclear if the UE is operating Type 1 4L or Type 1 larger than 4L per CC when switching to Type 1. Therefore, the BS signaling shall be able to differentiate Type 1 4L from Type 1 larger than 4L.

Observation 1: If single signaling as R18 is adopted, it is unclear if the UE is operating Type 1 4L or Type 1 larger than 4L per CC when switching to Type 1.

There was also proposal to use existing maxMIMO-Layers together with single BS signaling to determine if Type 1 4L or Type 1 larger than 4L per CC is applied. In our understanding, maxMIMO-Layers is to indicate the max number of layers for PDSCH/PUSCH which has been implemented by network for MIMO operation. But the switching from Type 4 to Type 1 is another network decision considering network deployment. Reusing existing parameter would impact the legacy UE and network behaviour hence a separate new BS signaling is preferred for differentiating the MIMO layers when switching to Type 1.

Observation 2: Reusing existing parameter maxMIMO-Layers would impact the legacy UE and network behaviour.

Proposal 1: Separate BS signalling to command UE from Type 4 to Type 1 4L and another signalling to command UE from Type 1 larger than 4L per CC is needed.

### 2.2  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b>

Way forward:

  - When maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

NOTE: This is specified Rel-18 behavior. No need to further discuss.

  - When maxMIMO-layers ＞4

- FFS in the next meeting.

  - When maxMIMO-layers = 4

- FFS in the next meeting.

NOTE: maxMIMO-layers means the existing BS signaling.

For R18 BS signaling nonCollocatedTypeNR-CA as below, we only consider the case when maxMIMO-layer is less than or equal to 2. When UE switches to Type 1, it operates with Type 1 4L while maxMIMO-layers remains as 2. To align with the principle, in R19, the BS signaling is indicated for a UE configured with maxMIMO-layer less than or equal to 4. If a Type 4 UE is configured with maxMIMO-layer larger than 4, UE shall assume collocation and operate  Type 1 6/8L/CC as a default.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

Proposal 2: The R19 BS signaling is indicated for a UE configured with maxMIMO-layer less than or equal to 4.

Proposal 3: If a Type 4 UE is configured with maxMIMO-layer larger than 4, UE shall assume collocation and operate in Type 1 8L as a default.

For Type 4 UEs, if maxMIMO-layers is configured as 4, we think the UE shall assume non-collocated deployment and operate in Type 4 (4L/CC) in the initial connection.  If UE receives network configuration of maxMIMO-Layer = 2, it switches to Type 2 (2L/CC) according to RAN4#112 agreements. If UE receives new R19 BS signalings, it will switch to corresponding Type 1 i.e. Type 1 4L, Type 1 6L or Type 1 8L based on the separate BS signalings.

Proposal 4: If a Type 4 UE is configured with maxMIMO-layers = 4, UE shall assume non-collocated deployment and operate in Type 4 (4L/CC) in the initial connection.

# 3   Conclusion

In this contribution we expressed our views.

Observation 1: If single signaling as R18 is adopted, it is unclear if the UE is operating Type 1 4L or Type 1 larger than 4L per CC when switching to Type 1.

Observation 2: Reusing existing parameter maxMIMO-Layers would impact the legacy UE and network behaviour.

Proposal 1: Separate BS signalling to command UE from Type 4 to Type 1 4L and another signalling to command UE from Type 1 larger than 4L per CC is needed.

Proposal 2: The R19 BS signaling is indicated for a UE configured with maxMIMO-layer less than or equal to 4.

Proposal 3: If a Type 4 UE is configured with maxMIMO-layer larger than 4, UE shall assume collocation and operate in Type 1 8L as a default.

Proposal 4: If a Type 4 UE is configured with maxMIMO-layers = 4, UE shall assume non-collocated deployment and operate in Type 4 (4L/CC) in the initial connection.

# 4   References

[1] R4-2420399, WF on non-collocated intra-band EN-DC and NR-CA, RAN4#113