# 3GPP TSG-RAN WG4 Meeting # 114	R4-2500377

Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	More on Delta Ppowerclass and Delta TRxSRS for 4Tx

Agenda Item:	5.7.1

Document for:	Approval

# Introduction

This contribution is an update of [1] to discuss issues raised in [2] for ΔPPowerClass and ΔTRxSRS for 4Tx.

# Discussion

According to offline discussions in RAN4#113, the last issue is that how to avoid the situation that the UE with 23 dBm x 4 or (23 dBm + 26 dBm) x 2 to perform antenna virtualization (AV) to attain higher power in case unnecessarily smaller ΔPPowerClass like 3 dB for 1TyR is defined, and how to allow UE to implement any one of the assumed PA configurations such as 23 dBm x 4, (23 dBm + 26 dBm) x 2, or 26 dBm x 4 PA in case only larger ΔPPowerClass like 6 dB for 1TyR is defined.

For more specific, if the requirements allow UE to use 26 dBm x 4 PA configuration, ΔPPowerClass should be 3 dB for 1TyR, but if the UE actually implements 23 dBm x 4, then, at least one company expects that the UE may use AV to achieve 26 dBm (29 dBm – 3 dB), though we don’t expect that the UE dare to use AV given that it just makes the original purpose lost. However, at least just specifying ΔPPowerClass = 3 dB for 1TyR makes 23 dBm x 4 PA configuration precluded if AV is not used, since the UE may not be able to meet the configured transmitted power requirement with sufficient relaxation for the lower limit.

Another company, however, argued that just specifying ΔPPowerClass = 6 dB instead of 3 dB for 1TyR makes 26 dBm x 4 PA configuration precluded. Of course, the output power per port during SRS antenna switching is capped by 23 dBm per port for 1TyR, but this does not prevent UE from implementing 26 dBm x 4, since this UE can more easily meet the requirements than UE with 23 dBm x 4. In addition, it would be likely that UE with 4Tx has at least more than or equal to 4Rx so that the loss may be large in some ports. It means that all the power across ports would be lower than 26 dBm to make all the powers from each of the ports equal or close. This is because even if the UE compensates for the lost power due to additional larger IL compared to the IL for the port with the least IL, the UE may not be able to completely compensate for it so that the powers from port(s) with less ILs may be intentionally reduced down to the lowest power per port with the largest IL. For instance, if the port with the largest IL has 5 dB larger IL compared to the port with the least IL and a UE can compensate for 2 dB power for the port with the largest IL, then, the powers across ports may be 23 dBm each (26 dBm – 5 dB + 2 dB). Lastly, given that UE’s power does not reach 29 dBm with 1TyR, it may be less likely that network configures the UE with 1TyR, but rather the network may configure the UE with 2TyR or 4TyR to shorten the SRS antenna switching time. For instance, if 23 dBm x 4 configuration is implemented, 4TyR would be chosen, since the total power is still 29 dBm and the sounding time becomes 1/4 compared to 1TyR.

Observation 1: Specifying ΔPPowerClass = 3 dB alone precludes UE from implementing PA configuration of 23 dBm x 4, given that the UE will not use Antenna Virtualization (AV).

Observation 2: Specifying ΔPPowerClass = 6 dB alone does not preclude UE from implementing PA configuration of 26 dBm x 4 or (23 dBm + 26 dBm) x 2, although the output power is capped by 23 dBm. The UE with 26 dBm x 4 would be able to meet the requirements more easily than UE with 23 dBm x 4.

Observation 3: Capping the power by 23 dBm, i.e., ΔPPowerClass = 6 dB, for 1TyR may not be a big problem given that

- it’s likely that a UE with 4Tx implements at least 4Rx and some of the ILs towards Rx ports may be large so that the UE may not transmit 26 dBm at the first SRS transmission from the port with the least IL to make the power close to the power from the port with the largest IL.

- Since 29 dBm is not expected from any UE with 1TyR, network would not configure UE with 4Tx with 1TyR, but rather 2TyR or 4TyR would be used to shorten sounding time while the power per port for 1TyR are the same or similar to those for 2TyR or 4TyR.

With all the observations, we think that it is reasonable to use ΔPPowerClass = 6 dB. The actual texts that we propose are the same as those in [1, 3].

Proposal 1: Allow inconsistency between PC2 without txDiversity-r16 and PC1.5 without txDiversity4Tx-r18, i.e., keep the current PC2 UE without txDiversity-r16, while don’t introduce the similar mechanism into the requirements for PC1.5 without txDiversity4Tx-r18.

Proposal 2: Introduce the following texts only for PC1.5 4Tx ΔPPowerClass, i.e., do not touch PC2 TxD ΔPPowerClass part.

Proposal 3: Do not introduce new changes into the existing ΔTRxSRS part.

# Summary

We obtained following observations and proposals. A companion CR based on this contribution is also submitted [3].

Observation 1: Specifying ΔPPowerClass = 3 dB alone precludes UE from implementing PA configuration of 23 dBm x 4, given that the UE will not use Antenna Virtualization (AV).

Observation 2: Specifying ΔPPowerClass = 6 dB alone does not preclude UE from implementing PA configuration of 26 dBm x 4 or (23 dBm + 26 dBm) x 2, although the output power is capped by 23 dBm. The UE with 26 dBm x 4 would be able to meet the requirements more easily than UE with 23 dBm x 4.

Observation 3: Capping the power by 23 dBm, i.e., ΔPPowerClass = 6 dB, for 1TyR may not be a big problem given that

- it’s likely that a UE with 4Tx implements at least 4Rx and some of the ILs towards Rx ports may be large so that the UE may not transmit 26 dBm at the first SRS transmission from the port with the least IL to make the power close to the power from the port with the largest IL.

- Since 29 dBm is not expected from any UE with 1TyR, network would not configure UE with 4Tx with 1TyR, but rather 2TyR or 4TyR would be used to shorten sounding time while the power per port for 1TyR are the same or similar to those for 2TyR or 4TyR.

Proposal 1: Allow inconsistency between PC2 without txDiversity-r16 and PC1.5 without txDiversity4Tx-r18, i.e., keep the current PC2 UE without txDiversity-r16, while don’t introduce the similar mechanism into the requirements for PC1.5 without txDiversity4Tx-r18.

Proposal 2: Introduce the following texts only for PC1.5 4Tx ΔPPowerClass, i.e., do not touch PC2 TxD ΔPPowerClass part.

Proposal 3: Do not introduce new changes into the existing ΔTRxSRS part.

# References

[1] R4-2417517, More on Delta Ppowerclass and Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#113

[2] R4-2400341, Delta PpowerClsss for 4Tx for SRS antenna switching, Nokia, RAN4#110

[3] R4-2500378, DeltaP_PowerClass correction for SRS AS for 4Tx, Huawei, HiSilicon, RAN4#114