3GPP TSG-RAN WG4 Meeting #114										    R4-2500390

Athens, Greece, 17th - 21th February 2025

Agenda Item:	7.29.4.1

Source:	MediaTek Inc.

Title:	Discussion on NTN (e)Redcap RRM requirements

WID/SID:        NR_NTN_Ph3-Core

Document for:	Discussion

# 1 Introduction

This paper presents our perspective on RRM requirement, focusing on the unresolved issues identified in RAN4#113 WF [1].

# 2 General consideration

MDT requirements in RRC_IDLE and RRC_INACTIVE state

It has been noted that no MDT requirements have been established for R17/R18 Redcap UE. However, MDT requirements are already defined for non-Redcap R17 NTN UEs. We recognize that logged MDT reports are advantageous for network deployment and planning purposes, and they could be effectively utilized by non-Redcap R17 NTN UEs.

Proposal 1: Not to define MDT requirements for NTN Redcap UEs.

2 General consideration

# 3 1RX

The TN 1Rx can be leveraged for NTN Redcap 1RX, regarding the remaining issues in WF [1], the following is proposed:

Proposal 2: For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows:

- L3 Measurement delay

  - PSS/SSS detection delay is extended by 2-3 samples, based on whether it is intra-f/inter-f and whether it is with MG or not.

  - SSB index identification (PBCH decoding) delay is extended by 2 samples.

- Measurement accuracy relaxation

  - SS-RSRP, SS-RSRQ, SS-SINR, relax relative and absolute accuracy by 1dB

  - L1-RSRP, relax relative and absolute accuracy by 3dB.

# 4 HD-FDD

In general, same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline. As listed in WF [1], at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 3: Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline, and the following requirements will be affected:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

# Conclusion

Proposal 1: Not to define MDT requirements for NTN Redcap UEs.

Proposal 2: For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows:

- L3 Measurement delay

  - PSS/SSS detection delay is extended by 2-3 samples, based on whether it is intra-f/inter-f and whether it is with MG or not.

  - SSB index identification (PBCH decoding) delay is extended by 2 samples.

- Measurement accuracy relaxation

  - SS-RSRP, SS-RSRQ, SS-SINR, relax relative and absolute accuracy by 1dB

  - L1-RSRP, relax relative and absolute accuracy by 3dB.

Proposal 3: Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline, and the following requirements will be affected:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

# References

- R4-2420104, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT, Nov. 2024.

- R4-2420105, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm, Nov. 2024.