3GPP TSG-RAN WG4 Meeting #114	R4-2500404

Athens, Greece, 17th - 21st February 2025

Title: 	SRS imbalance solution considerations

Agenda Item:	7.1.1.3.4

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN4 meeting WF of 6Rx UE requirements was approved. This contribution discusses about of SRS imbalance addressing solutions. Following issues were listed as WF [1].

## Sub-topic 3-1:	General considerations for SRS IL imbalance issue

Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way Forward: RAN4 to further discuss the following options.

-	Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

-	Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1.

Issue 3-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1.

## Sub-topic 3-2:	SRS IL imbalance issue solutions

Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: RAN4 to further discuss the candidate solution framework below and present proposals concerning the details. Companies are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed..

Candidate solution framework:

-	UE performs self-compensation of SRS IL up to its configured maximum output power.

-	FFS on whether condition would introduce any additional RAN4 performance requirement. No RAN1 specification impact.

-	UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g.

-	Per SRS resource power headroom reporting

-	Configured maximum output power per SRS resource reporting

-	SRS insertion loss value reporting (per SRS resource or per UE)

-	Other

-	ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting.

-	Others

-	Details of SRS IL self-compensation and assistance framework are FFS

# Discussion

There have been extensive discussions regarding the imbalance of SRS related to UE hardware implementation, antenna interface, and its impact on network performance, which commenced with Release 18 in both RAN4 and RAN1, as referenced in [2]. To date, no new solutions have been proposed to address the imbalance issue. The methods proposed thus far exhibit drawbacks, either in their ability to resolve the issue, added signalling overhead or changes to RAN1 and RAN2 specifications. Therefore, we reiterate our previous position on the issue by slight adjustments.

SRS imbalance issues become more pronounced as the number of antennas increases. To address this issue, solutions have been proposed since Release 18, focusing on UE self-compensation of SRS imbalance, signalling the imbalance to the network, or both. The former approach is viable only when the power amplifier (PA) has sufficient maximum linear power headroom on the main antenna while providing SRS resources to other antennas. If the value of SRS insertion loss (IL) is high, compensation may be partial; however, this remains a feasible, UE-based solution to address the imbalance issue. Even though the compensation is partial, any amount of compensation would improve the signal-to-noise ratio (SNR) in the gNodeB.

The UE is maximum output power (MOP) limited in cell edge scenarios; however, this is not likely a relevant scenario for MIMO downlink reception. Conversely, in cell edge scenarios, there are alternative methods to SRS-based sounding for estimating channel state information (CSI), which have been discussed in other papers.

Figure 1 represents a simplified solution of an SRS antenna switching (AS) solution for a single band. Only a 1T6R configuration is considered here, as the methodology described can be applied to any SRS AS solutions. After the power amplifier (PA), there is a switch arrangement for allocating SRS resources to the required antenna. The same arrangement can be used for transmit antenna switching (TAS) to optimize over-the-air (OTA) performance, which might be compromised by user interaction with the transmit antenna. The dashed line between the switches and antenna ports represents a reference plane in which transmission (TX) and reception (RX) are calibrated in the factory. At the same time, it provides a reference plane for channel sounding.

Figure 1 Switching arrangement for SRS resource routing

The idea of channel sounding is to provide gNodeB information about radio channel condition by utilizing UL/DL reciprocity and specific reference signal (SRS resource) transmitted to each of the DL antennas for the gNodeB to acquire DL CSI. UE side antennas are to be included into this estimation.

SRS imbalance occurs due to different losses from PA to each antenna. Different power level of SRS symbols, due to UE implementation, arriving at the gNodeB provide erroneous indication of channel conditions which are considered to have non-negligible impact on the DL throughput.

To date the ways to address the SRS imbalance can be grouped into three groups as stated in [3]

- Per SRS resource power headroom reporting

- Configured maximum output power per SRS resource reporting

- SRS insertion loss value reporting (per SRS resource or per UE)

Other options are not precluded.

There are 10 individual proposals listed in [3] that are falling into these categories. Below are some considerations for these proposals

## SRS resource power headroom reporting

Proposals 1,2,3 in [3] suggest utilizing PH reporting to indicate SRS imbalance to the gNodeB. Reference [4] proposed deriving SRS imbalance dynamically from PCMAX per SRS resource and PH report per SRS resource using type 3 kind of report with modification to add new MAC-CE and to cover PUCSH transmission. In [5] there was a proposal to define accurate, per resource power headroom reporting. Both proposals imply changes to power control equations and/or signalling PH report.

Following there is some considerations for PH reporting. Currently type 3 PH report is defined in TS 38.213 as;

and PSRS,b,f,c(i,qs,l) in TS 38.213,

where,

-	PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i

-	PO_SRS,b,f,c(qs) is provided by p0 for active UL BWP b of carrier f of serving cell c and SRS resource set qs provided by SRS-ResourceSet and SRS-ResourceSetId

-	MSRS,b,f,c(i) is a SRS bandwidth expressed in number of resource blocks for SRS transmission occasion i on active UL BWP b of carrier f of serving cell c and μ is a SCS configuration defined in [4, TS 38.211]

-	αSRS,b,f,c(qs) is provided by alpha for active UL BWP b of carrier f of serving cell c and SRS resource set qs

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using RS resource index qd as described in clause 7.1.1 for the active DL BWP of serving cell c and SRS resource set qs [6, TS 38.214]. The RS resource index qd is provided by pathlossReferenceRS associated with the SRS resource set qs and is either an ssb-Index providing a SS/PBCH block index or a csi-RS-Index providing a CSI-RS resource index. If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a MAC CE [11, TS 38.321] can provide by SRS-PathlossReferenceRS-Id a corresponding RS resource index qd for aperiodic or semi-persistent SRS resource set qs.

Further according to 38.213: A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L.

PCMAX,f,c  is defined in TS 38.101-1 as,

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c

PCMAX_L,f,c=MIN{PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass)– MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c+∆TC,c + ∆TRxSRS,

P-MPRc)}

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

Following observations can be drawn;

Observation 1: PO_SRS,b,f,c(qs) and PLb,f,c(qd) are defined for a resource set. Current power control equations do not allow per SRS resource power control or PH reporting.

Regarding the first equation even though UE can fairly accurately know the output power at it’s antenna terminal for every transmit occasion PH reporting toward network is carried out implicitly using several parameters.

Observation 2: PH reporting is dependent on two loosely defined parameters PCMAX,f,c and PLb,f,c(qd) The former is very much UE specific with worst case implementation margins included in various parameters. Also tolerance in TS 38.101-1 Table 6.2.4-1 adds more ambiguity to PCMAX,f,c. PL term derived from DL RS is not necessarily associated with an antenna to be sounded.

Observation 3: ∆TRxSRS used in PCMAX,f,c equation is a worst case estimate of possible SRS IL that may happen in an UE and not associated to a specific antenna.

Observation 4: Previous observations indicate great uncertainty on the SRS IL estimation by PH reporting.

## Configured maximum output power per SRS resource reporting

Proposal 5 in WF [3] and in [5] suggest to modify existing equation in TS 38.101-1 for PCMAX,f,c as below,

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) –

MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

In other words replacing ∆TRxSRS by UE specific value ∆TRxSRS,UE. which would be relatively easy to define by a UE vendor.

Observation 5: Reporting ∆TRxSRS,UE would improve the SNR for SRS in gNodeB side but as it is a value used for multiple antennas and yet it would not solve problem on SRS imbalance in high end of the output power range between antennas.

## SRS insertion loss value reporting (per SRS resource or per UE)

Other proposals in WF are related to SRS IL reporting to gNodeB. SRS IL reporting can be divided into two different methods: static and dynamic reporting.

SRS IL reporting refers to the reporting of insertion loss associated to each DL antenna transmitting the SRS resource. This association itself is static and does not need to change according to UL antenna allocation or RX chain allocation. This is illustrated in table 1 for SRS-TxPortSwitch capability 1t6r. UE must report IL association for each DL antenna for each SRS-TxPortSwitch capability, and antenna allocation it is using for DL. If static SRS IL reporting is used UE compensation for SRS IL shall not be used. However, UE may compensate frequency response for the SRS transmission so that one value per antenna represents SRS insertion loss accurately.

Observation 6: SRS IL association per antenna is static and is not dependent on transmitter antenna swapping or RX chain allocation.

Observation 7: SRS IL reporting shall be based per antenna.

Observation 8: SRS IL reporting shall include report for DL antenna allocation.

Observation 9: UE may compensate SRS IL frequency response of a band for static SRS IL reporting.

| ## Scenario | ## SRS IL (1t6r) |  |  |  |  |  | ## Assigned UL Antenna |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## Ant1 | ## Ant2 | ## Ant3 | ## Ant4 | ## Ant5 | ## Ant6 |  |
| ## 1 (6+6 DL) | ## 0 | ## aa | ## bb | ## cc | ## dd | ## ee | ## Ant1 |
| ## 2 (6+6 DL) | ## 0 | ## aa | ## bb | ## cc | ## dd | ## ee | ## Ant4 |
| ## 3 (4+4 DL) | ## 0 |  | ## bb |  | ## dd | ## ee | ## Ant6 |

Table 1 SRS IL and UL antenna association

RAN1 specification is written in a way that it assumes SRS IL compensation when UL power level ≤  PCMAX_L,f,c. Dynamic SRS IL compensation relates to operation SRS imbalance per antenna may not be constant as a function of output power as the UE sets PCMAX within low and high limits. Each antenna posesses its own maximum power capability and compensation beyond this level is infeasible without unwanted out of and in band emissions. Not utilizing UE capabilities for SRS IL imbalance compensation fully will lead to compromised SNR at the gNodeB. If dynamic power reporting utilizing the UE compensation for SRS IL can be considered UE must dynamically report residual IL imbalance above a certain power threshold for each antenna to the gNodeB.

Observation 10: dynamic reporting assumes that UE is compensating the SRS IL based on its capabilities.

Observation 11: For dynamic reporting UE shall report gNodeB output power dependent SRS imbalance per antenna.

# Conclusion

In this contribution, we have made number of observations. Summarizing all the observations we can make following conclusions.

- Depending on the solution modifications to signalling are required to cover following per resource PH reporting, per resource Pcmax reporting, type3 PH reporting to cover PUSCH reporting, static SRS IL reporting or dynamic SRS IL imbalance per antenna and antenna allocation.

- PH reporting and Pcmax reporting both provide inaccurate SRS IL/SRS IL imbalance estimation.

- Static/dynamic SRS IL reporting provide more accurate SRS IL estimation but may require extensive signalling overhead.

Therefore, we reiterate our proposal from previous meetings again.

Proposal 1: No changes to RAN4, RAN2 or RAN1 specifications. SRS IL compensation shall be up to UE implementation.

# 4	References

[1] R4-2420376, WF on 6Rx UE requirements, RAN4#113

[2] R4-2404935, Views on 6Rx for handheld and FWA UE, RAN4#110bis, Samsung

[3] R4-2417095, WF on 6Rx UE requirements, RAN4#112bis

[4] R4-2419380, On the SRS IL imbalance reporting details, RAN4#113, Ericsson

[4] R4- 2415266, Further study on SRS IL imbalance, RAN4#112bis, China Telecom

[5] R4-2415686, Views on SRS insertion loss compensation and reporting enhancements, RAN4#112bis, intel Corporation