3GPP TSG RAN WG4 Meeting #114	R4-2500411

Athens, Greece, 17th – 21st February 2025

Source:		Rohde & Schwarz

Title:	Discussion on NGSO channel model parameters

Agenda Item:	7.10.4

Document for:	Approval

# Introduction

During the last RAN4 meeting the discussion on a realistic doppler/delay profile for NGSO satellites made significant progress by agreeing a on procedure to derive the doppler/delay profile [1]. However, some details like requirements on the accuracy of the profile and the update rate require further clarifications.

# Discussion

## Accuracy requirements

During the last RAN4 meeting, a way forward [1] has been agreed where the basic principles on how to derive the doppler/delay profile based on the Keplerian model have been agreed. As part of the procedure the following steps 2 & 3 were captured:

- Step 2: Generate the estimated satellite position/velocity vector based on Keplerian model at the time T1 with the initial ephemeris information provided in step 1 - Step 2.1: Calculate the Mean Anomaly value - Step 2.2: Derive the Eccentric Anomaly at time T1 by solving Kepler’s equation - Step 2.3: Derive the True Anomaly at time T1 - Step 2.4: Covert the orbital elements in ECI format into the satellite position/velocity state vector in ECEF format with considering the earth rotation - Note: The detail numerical methods to solve the Kepler’s equation in step 2.2, including number of iterations, accuracy of requirement, can be up to implementation, for example - Numerical method - Newton-Raphson method - Number of iterations - Option 1: 4 - Accuracy requirement E-E0>ϵ - Option 1: ϵ=10-8 - Option 2: ϵ=10-6

- Step 3: Obtain the propagation delay and Doppler shift trajectory based on the interpolation operation at the anytime between T0 and time T1. - Step 3.1: Obtain the satellite position/velocity at time T1 - Step 3.2: Obtain the propagation delay and Doppler shift trajectory between time T0 and time T1, (T0 <= T1) - Note: The update granularity of propagation delay and Doppler shift should be [1] slot - Note: The details interpolation method can be up to implementation, with meets some accuracy requirement, for example - Numerical method - Option 1: cubic spline interpolation - Option 2: linear interpolation - Option 3: calculate propagation delay and Doppler shift at any time instant - Other options are not precluded - Accuracy requirement - Option 1: - Max¦delay-error¦ ~ 3e-13 s - Max¦delay-rate error¦ ~ 3.28e-12 - Other options not precluded

From a test system implementation background especially the accuracy requirements (marked in yellow) on the doppler/delay profile are of high importance. We thus analysed the accuracy requirements with a focus on test system feasibility and came to the following conclusions and proposals:

For the accuracy requirement on E-E0>ϵ in step 2 of the procecures, we propose to adapt Option 1: ϵ=10-8, since this can be seen as feasible from a TE implementation point of view and provides a sufficiently good accuracy. Lower accuracies may also be technically feasible,

Proposal 1: RAN4 agrees on an accuracy of  ϵ=10-8 for the E-E0>ϵ requirement.

With respect to the requirements on the maximum delay errors discussed in step 3 of the procedure, we have a different view compared to the proposed accuracy values in the way forward. When looking at the Orbit resolution for the satellite trajectory, this is 1.3m. Typically the acceptable error required to sufficiently measure/emulate the scenario is one magnitude lower. Therefor we propose the following instead of the values in Option 1:

  - Max|delay-error| ~ 5e-10 s

  - Max|delay-rate error| ~ 1e-10 s/s

Proposal 2: RAN4 agrees on an accuracy for Max|delay-error| of ~ 5e-10 s and Max|delay-rate error| of ~ 1e-10 s/s.

## Update granularity of the doppler/delay profile

Another important point for the profile emulated by the test equipment is the rate with which the profile is updated, so that the DUT always sees the expected doppler and delay. As part of the way forward the general assumption has been that the doppler/delay shall be updated on a per slot basis, but further feedback on the feasibility of this assumption was requested.

Issue 1-1-3-3: Assumption of updated granularity for propagation delay and Doppler shift calculation - WF - Option 1 (WF in the last meeting) - The Doppler shift and propagation delay should be calculated and updated based on agreed TE emulated channel model per slot - TE vendors are encouraged to provide the feedback about the implementation feasibility to update the Doppler shift and propagation delay per slot

Based on our internal analysis, we can confirm that it is feasible for the test equipment to update the doppler/delay profile on a per slot basis.

Observation 1:  An update of the doppler/delay profile per slot is feasible from TE implementation point of view.

## UE propagator model

Issue 1-2-1 UE assumption on propagator model for satellite motion for RAN4 simulation purpose - WF - Option 1: Consider Keplerian model for UE assumption on the propagator model - Option 2: - Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span. - Option 3: It is up to UE implementation what method is used to estimate the doppler shift and propagation delay based on the signalled ephemeris information

In the evolution of the GNSS system the accuracy of the orbit propagation has been continuously improved over the decades, because it is clearly specified how the UE receivers propagate the satellite position based on the provided Kepler elements. This optimization would have not been possible if the propagation on the UE side would have been left up to implementation. Henceforth, it would be beneficial to standardize a common propagation model for all NTN UEs.

Proposal 3: A common propagation model for all NTN UEs should be standardized, to allow the optimization of the NTN performance.

# Summary

In this contribution we shared our views on open issues for the doppler and delay profiles for NGSO satellites. We have the following observation and proposals:

Observation 1:  An update of the doppler/delay profile per slot is feasible from TE implementation point of view.

Proposal 1: RAN4 agrees on an accuracy of  ϵ=10-8 for the E-E0>ϵ requirement.

Proposal 2: RAN4 agrees on an accuracy for Max|delay-error| of ~ 5e-10 s and Max|delay-rate error| of ~ 1e-10 s/s to 1e-11 s/s.

Proposal 3: A common propagation model for all NTN UEs should be standardized, to allow the optimization of the NTN performance.

# References

[1]	R4-2419786; “Way Forward for [113][322] NTN_testing_NGSO_channel_model”; Samsung; RAN4#113 November 2024