3GPP TSG-RAN WG4 Meeting #114	R4-2500427

Athens, GR, 17th – 21st February, 2025

Source:	Nokia

Title:	Discussion on Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	7.16.2

Document for:	Discussion

Introduction

The study item on Spatial Channel Model (SCM) for demodulation performance requirements (FS_NR_demod_SCM) progressed well in RAN4#113, resulting in an agreed WF [1].

This contribution addresses the open issues captured in the last WF [1], in particular the alignment and comparison setups (including AAV configuration), requested alignment intermediate steps, and discusses initial alignment/comparison results.	
Furthermore, we highlight some oversights in earlier configuration assumptions and

We want to further highlight our companion simulation results contribution [2].	
Please also refer to it for an up-to-date TP-style description of the 827 CDL channel model, and delivery of the first alignment spreadsheet(s).

The structure of this contribution follows the last WF [1] to advance open issues and adds new topics, where found necessary.	
We do not extend our fundamental observations regarding spatial channel models (e.g., TDL, TDL nonLow, SCM, CDL, AAV) and use cases in this contribution. We instead refer to our earlier contributions, especially [3], for this purpose:

| Tdoc | Title | Meeting |
| --- | --- | --- |
| R4-2402277 | Discussion on 8Rx general demodulation aspects and spatial channel models | RAN4#110 |
| R4-2411300 | Discussion on Spatial Channel Model for Demodulation Performance Requirements | RAN4#112 |
| R4-2415282 | Discussion on Spatial Channel Model for Demodulation Performance Requirements | RAN4#112-bis |
| R4-2418043 | Discussion on Spatial Channel Model for Demodulation Performance Requirements | RAN4#113 |

Common for all Spatial Channel Modelling Methodologies

Test cases for SU-MIMO

The WF from RAN4#113 [1] sets the current of SU-MIMO test cases as follows:

Cases for SU-MIMO Agreement: Single-User PDSCH: 8Rx: 8 Layer, MCS 13 on both codewords (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting (FFS Whether it is already covered in PMI) 4Rx: 4 Layer, MCS 13 (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer Single-User PMI 4Rx: 4 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random) 4Rx: 4 Layer (eType II) – Full Throughput Curves (PMI Follow) 4Rx: 2 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random) 4Rx: 2 Layer (eType II) – Full Throughput Curves (PMI Follow) Interested companies can use initially 8 CSI-RS Ports, more ports can be used to identify and show relevant impacts. Note: Several companies would prefer to focus on PDSCH until the modelling is stable Way Forward (from RAN4#113): Random PMI as a baseline, FFS Fixed. Interested companies invited to choose based on most common PMI in PMI histogram. #### Duplex Mode, CBW and SCS Agreement: Carrier Frequency: 3.5 GHz Duplex Mode: TDD CBW: 40 MHz SCS: 30 kHz #### Receiver Type Agreement: IRC Baseline for SU-MIMO; Interested Companies to use IRC and E-IRC for MU-MIMO and LMMSE for SU-MIMO

For better readability, we will treat SU PDSCH and SU PMI in separate subsections.

SU PDSCH setups

Fixed precoding

In our earlier contributions, a non-random precoder choice was found to be useful to capture the impact of typical MIMO effects (e.g., quality difference between MIMO layers, midterm stable spatial preferences) on the receiver implementation performance (e.g., regularization strategies, spatial distinction via covariance matrix estimation or other means).	
Any non-random precoding strategy combined with a spatial channel model can serve this purpose, but for requirement and test simplicity it may be desirable to have a fixed precoding choice.

Concerning choosing fixed precoding indices during alignment and comparison, we maintain our previous argument (see also section 3 of [3]) that it is feasible to make a fixed PMI choice based on the most frequently chosen “PMI_follow” under any given channel model with midterm stable large-scale parameters. I.e., using the most common PMI from a histogram of signaled/chosen follow PMIs.	
With this approach PDSCH performance should be comparable, without disclosing or mandating a specific PMI that might not fit the expectations of the receiver algorithm.

In our simulation results contribution [2], we disclose the chosen PMI for AAV 2 only, but we will update the simulation collection summary during the meeting to also include other AAV choices.

A non-random precoder choice was found to be useful to capture the impact of typical MIMO effects.

RAN4 to adopt fixed precoding test cases for SU PDSCH SCM evaluation.

RAN4 to choose the most common PMI from a histogram of signaled/chosen follow PMIs as fixed precoder in SU PDSCH SCM evaluations.

Note it is our understanding that “follow PMI” in PDSCH configurations is instantaneous, i.e., it means to apply the PMI as the receiver side would have chosen, without reporting delay.

If follow PMI is used in the context of PDSCH test cases, then it is instantaneous “follow PMI”, i.e., it means to apply the PMI as the receiver side would have chosen, without reporting delay.

Please find hereunder the PMI_follow histogram for CDL 827 + AAV 2&3, that we used to derive the fixed precoder in our results collection contributions:

| 827 CDLC UMa – AAV 2 SNR = 25dB TP CW0 = 61% TP CW1 = 91% | 827 CDLC UMa – AAV 3 SNR = 22dB TP CW0 = 97% TP CW1 = 92% |
| --- | --- |

Figure 1: 827 CDLC UMa – AAV 2&3 follow PMI histogram.

Receiver Type

For SU cases, the receiver structure does not overly matter for the SCM comparisons. However, we want to clarify that there are generally three (non-advanced) baseline receivers to be considered in demodulation performance requirements:

TR 38.833 IRC (+regularization), TR 38.878 E-IRC (+regularization), and product linear receivers (LMMSE).

For PDSCH requirements, it is customary to use the product linear receiver implementation, while for CSI/PMI requirements the IRC/e-IRC reference are often used to remove the receiver performance “variable” from the CSI derivation and feedback functionality implementation.	
However, often the CSI feedback in a product is optimized to match the receiver implementation. Hence the latter is usually not insisted on, unless there are alignment issues.

For PDSCH requirements, the product linear receiver implementations are often used (while for CSI/PMI requirements the IRC/e-IRC reference are frequently used).

For alignment and comparison of SCM models, it is useful to remove the receiver implementation variable, hence the agreement to have IRC as baseline for SU-MIMO.

SU PDSCH observations

With stable test cases having been agreed in the last meeting, we expect the first results and observations to be brought to RAN4#114. We have disclosed our results for all CDL based SCM + AAV combinations in [2], and we make the following initial statements.

Note that we understand the alignment of simulation results to be indicated by there being at least one cluster of curves of [3] companies that are within [3dB] of each other. The presence of more than one cluster of results would point towards systematic differences in either simulation setup or receiver implementation between the companies in differing clusters; this would not directly indicate misalignment on SCM implementation.	
Initial alignment can be evaluated from the simulation result collection summary to be carried out at RAN4#114. Similarity, reasons for misalignment may be explored from the same collection

Initial alignment of SCM implementation for each test case with random precoders (i.e., alignment cases) shall be considered at RAN4#114 based on the simulation result collection summary.

From our initial results we observe the following:

8Rx, 8 Layer, 2CW, 8 CSI-RS Ports (4,1), 10 vs. 100 Hz Doppler

CDL variants

- As expected from typical deployment.

  - Both CWs perform differently, even for random precoding (differing precoder pool for both CWs).

- The AAV options’ performances differ.	
AAV choice has a pronounced impact on the channel diversity/condition/rank.

- Requirements setting seems possible at practical SNR levels for all AAVs, however 1Y may need very low MCS configuration.

- The Doppler values have very limited impact (higher Doppler resulting in marginally higher SNR requirements), which is in line with expectations of DM-RS based demodulation of sufficient density and FO within FOE/FOC capabilities.

Legacy TDL

- From earlier contributions (e.g.,  [3]) we report:

  - TDL low: Equal performance of both CWs, i.e., not in line with typical MIMO expectations.

  - TDL nonLow: SNR requirements at infeasible levels. Equal performance for random precoding, i.e., not in line with typical MIMO expectations.

Modified TDL

- None tested.

4Rx, 4 Layer, 1 CW, 4 CSI-RS Ports (2,1), 10 vs. 100 Hz Doppler

CDL variants

- As expected from typical deployment.

  - Substantial gain from selecting precoder that is aligned with mid term stable spatial properties of the channel vs. random precoding.

- The AAV options’ performances differ.	
AAV 1Y is worst, AAV 1X mid, AAV 2 is best. Up to 6dB difference.	
AAV choice has a pronounced impact on the channel diversity/condition/rank.

- Requirements setting seems possible at practical SNR levels for all AAVs, however 1Y may need very low MCS configuration.

- The Doppler values have limited impact (higher Doppler resulting in marginally higher SNR requirements), which is in line with expectations of DM-RS based demodulation of sufficient density and FO within FOE/FOC capabilities.

- In [3] we also made the following observations

  - Additionally tested fixed PMI (on top of follow and random).

  - Additional gain of follow over fixed. Especially at low SNR the curves don’t overlap.

Legacy TDL

- From earlier contributions (e.g.,  [3]) we report:

  - TDL low: Very small performance gap between random and follow/fixed.

  - TDL nonLow: Infeasible SNR requirements.

Modified TDL

- None tested.

SU PMI setups

Reference receiver

In PMI setups there is a whole set of additional variables introduced (when compared to PDSCH). To remove the impact of the demodulator it is common to agree on a baseline receiver implementation.

For CSI/PMI requirements the IRC/e-IRC reference receivers are frequently used, TR 38.833 IRC (+regularization) and TR 38.878 E-IRC (+regularization) (while for PDSCH requirements, the product linear receiver implementations are often used).

Thus, we propose

For SU PMI setups, use IRC (+regularization) as reference receiver architecture.

Main expectations of PMI over PDSCH

There are two main additions in SU PMI over SU PDSCH:

- Precoding application delay

  - In follow PMI in SU PMI test cases, there is an application delay of the PMI of 6.5ms.

  - In deployment we expect that for slowly changing channels (e.g., 10Hz Doppler) the PMI selection strategy should include small scale fading (SSF) considerations, while for quickly changing channels (e.g., 100Hz Doppler) the better PMI selection strategy may be to focus on midterm stable directions of the channel.

  - It will be instructive to see the impact of those strategies, and whether these implementation differences show up in all channel model candidates.

- eType II precoders

  - In deployment we expect that for rank 1&2 cases, the performance of type I CBs would be rather close to eType II (or even above).

  - For rank >2, the spatial basis constraints of type I are expected to cause issues, with eType II having more freedom/resolution, but may or may not be able to exploit it (e.g., due to complexity constraints in the implementation).

Missing configuration alignment

Additionally, it seems that our group did (unsurprisingly) overlook a number of configurations that are required for simulation alignment. At least the following settings were additionally used by Nokia:

- eType II and Type I are both compared in their wide band configurations (i.e., N_3=1 for eType II).

- eType II configuration

  - paramCombination-r16=5 (L=4, pν=1/4, β=3/4)

- CSI-RS configuration

  - 8Tx: row = 6

  - 4Tx: row = 4, if 4 Tx is used.

  - Power boosting: As per Table C.3.1-1 in TS 38.101-4 v18.0, i.e., same (sum) EPRE per RE carrying CSI-RS or data.

    - Note that this configuration may cause different per OFDM symbol transmission powers, if a CSI-RS only slot/symbol is used. It will, however, assure equal per OFDM symbol transmission powers, if CSI-RS is puncturing or “rate-matching” data.

Translated into proposals:

SU PMI test cases to use eType II CB paramCombination-r16=5 (L=4, pν=1/4, β=3/4).

SU PMI test cases to compare eType II and Type I performance with wide band configurations, or at least not WB vs. SB.

SU PMI test cases to use CSI-RS configuration row = 6 for 8Tx and row = 4 for 4Tx (if 4Tx is used), as well as, choosing beta_{CSI-RS} in accordance with Table C.3.1-1 in TS 38.101-4 v18.0, i.e., same EPRE for data and CSI-RS.

eType II random

As discussed during the NR_eMIMO WI, there are several ways to construct a random eTypeII precoder. However, we note that, unlike Type I SP codebook, for which there is a limited set of possible precoder matrices, for eType II codebook the number of possible precoders reconstructed from the PMI is much larger, hence it is impractical to choose the precoder from a complete list of possible precoders.

Two practical methods to randomly chose the precoder from the eType II codebook are

- Simplest approach: A random PMI can be reconstructed by a random selection of the indicators i1 and i2 used in the formula of Table 5.2.2.2.5-5 of TS 38.214 (v16.2.0 ff).

- Amplitude 1 beams only:

  - Select the main beam index (i18,l) from 2L beams randomly, l is the layer index.

  - Fix the main beam with following amplitude (38.214 Table 5.2.2.2.5-2, Table 5.2.2.2.5-3 in spec) and phase coefficient indicators. Amplitude=1, Phase =0.

  - Select the remaining NZ coefficients from the table of amplitude and phase coefficient indicators over the (Beam x FD basis) grid until reaching K0 per layer.

  - Note: The number of chosen coefficients per layer should be chosen randomly (number of ones in i17 bitmap).

  - Loop by the layer and repeat the step i-iii. Stop if 2K0 is reached across all layers.

  - Precoder is constructed according to 38.214 Table 5.2.2.2.5-5.

In our opinion, (a) is preferable from a test coverage perspective as it also covers “unusual” indications.

If required, there are feasible methods to chose random precoders for eTypeII CB, most notably using a random selection of the indicators i1 and i2 used in the formula of Table 5.2.2.2.5-5 of TS 38.214.

SU PMI observations

With stable test cases having been agreed in the last meeting, we expect the first results and observations to be brought to RAN4#114. We have disclosed our results for all CDL based SCM + AAV combinations in [2], and we make the following initial statements.

Note that we understand the alignment of simulation results to be indicated by there being at least one cluster of curves of [3] companies that are within [3dB] of each other. The presence of more than one cluster of results would point towards systematic differences in either simulation setup or receiver implementation between the companies in differing clusters; this would not directly indicate misalignment on SCM implementation.	
Initial alignment can be evaluated from the simulation result collection summary to be carried out at RAN4#114. Similarity, reasons for misalignment may be explored from the same collection

From our initial results we observe the following:

4Rx: 4 Layer (type I – follow/random vs. eType II follow) 10 vs. 100 Hz Doppler

CDL variants

- In all test cases T1f > eT2f > T1r.	
This may be explained by our CSI selection schemes, where T1f is full exhaustive search (W1 and W2) at great complexity, while eT2f only does exhaustive search for W1 and derives W2 by correlation with EVs.	
Additionally, the choice of WB only and relatively few CSI-RS ports may impact the results.

Legacy TDL

- From earlier contributions (e.g.,  [3]) we report:

  - TDL low: Very small performance gap between random and follow/fixed.

  - TDL nonLow: Infeasible SNR requirements.

Modified TDL

- None tested.

4Rx: 2 Layer (type I – follow/random vs. eType II follow) 10 vs. 100 Hz Doppler

As for 4 layers, but additionally:

CDL variants

- Bigger gaps in 4 layers than 2 layers, as well as larger Doppler impact.

Test cases for MU-MIMO

The WF from RAN4#113 [1] sets the current of MU-MIMO test cases as follows:

#### Cases for MU-MIMO Agreement: Interested companies to bring to RAN4#114 views on the following cases: PDSCH 2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE) Interested companies are encouraged to assess 2+2 layers with IRC and E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

For better readability, we will treat MU PDSCH and MU PMI in separate subsections.

MU PDSCH Setup and observations

In the last meeting we agreed to have MU PDSCH test and alignment cases, with the distinction between target and co-scheduled user being made via precoding at the transmitter.	
The setup seems complete in our view and initial alignment can be evaluated from the simulation result collection summary to be carried out at RAN4#114. Similarity, reasons for misalignment may be explored from the same collection.

From our initial results we observe the following:

4Rx4Tx: 2+2 Layers (with IRC and eIRC, type I orthogonal and random precoding) 10 Hz Doppler

CDL variants

- Follows deployment expectations.

  - Both IRC and eIRC perform similarly.

  - Attributed to signal and interference received signals coming from different spatial supports, with interference indirectly mitigated by focusing on the signal support (already effective with SU IRC).

- Earlier results (e.g.,  [3]) also show a non-exaggerated gap between linear and non-linear receivers.

Legacy TDL

- From earlier contributions (e.g., Figure 20 in [3]) we report:

  - TDL low: Large performance gap between IRC and eIRC, attributed to signal and interference being received on randomized spatial supports.

  - TDL nonLow: Large performance gap between IRC and eIRC, attributed to signal and interference being received on same spatial supports. Additionally, high SNR requirements.

Modified TDL

- None tested.

In Rel-17, there was a drawn-out argument on whether R-ML was required for significant performance improvement in MU scenarios, or whether linear methods perform sufficiently well. Due to a large performance gap between R-ML and (e)IRC in legacy channel models, it was ultimately decided to have the more costly R-ML performance as the minimum performance. We don’t make this observation in the same magnitude using SCM candidates.	
In order to align our earlier observations with non-linear receivers with other contributors, we would like to add it to the MU PDSCH test case.

Include the R-ML (reduced maximum likelihood) receiver in MU PDSCH test cases.

MU PMI

Given the workload in this study, it may be prudent to focus on SU PDSCH/PMI and MU PDSCH until June 2025.

Common test case assumptions

During the last meeting, the group often evoked the distinction between rapidly and slowly changing channel. Usually this was abbreviated as the 3kph speed/10Hz Doppler and 30kph speed/100Hz Doppler use cases.	
This common distinction does not seem to have been captured in the WF or have been agreed. Hence, we would like to rectify this.

RAN4 shall consider test cases both in low speed (3kph/10Hz Doppler) and mid speed (30kph/100Hz Doppler) conditions.

Metrics for comparison

The question of metrics for comparison of test cases under different channel models has been discussed many times during the meeting. Until now we did not formalize (or need to formalize) such metrics, as SCM candidates were not stable enough to predict their behaviours.	
The intent from the beginning was to verify all channel model in MIMO test cases, whether either typical MIMO effects observed in deployment or expected from MIMO systems are well reflected in the simulation output.

As contributors are expected to deliver alignment results in this meeting, we will be able to verify if channel model candidate implementations inhibit common trends that can be attributed to expected MIMO effects. We have seen such trends in our results, but alignment with other companies can give confidence that those are not artifacts of our implementation, but rather properties of the channel models.

#### Metrics for comparison Contributions welcome at RAN4#114.

In the prior “observation” sections we have already listed several trends that we attribute to expected typical MIMO effects and can be taken as a basis for comparison metrics; assuming said trends are replicated by other contributors.

To be more specific, expect to see at least most of the following typical MIMO effects in a demodulation channel model:

- SU PDSCH 2CW

  - SDM receiver performance differences per CW.

- SU PDSCH 1CW

  - Gains of fixed/follow precoder over random precoder.

  - SDM receiver performance (through covariance estimation) impacted by channel evolution rates.

- MU PDSCH

  - Similar performance of linear receivers with or without inter-user IRC.

  - Linear receivers perform worse, but not unreasonably so, than non-linear receivers.

- SU PMI

  - CSI report selection performance and implementation impacted by channel evolution rates.

  - Relative CB PMI selection performance difference impacted by number of layers.

- MU PMI

  - CSI report selection performance and implementation impacted by interference presence (through receiver SDM bias choice).

- All (also non-MIMO)

  - Typical SNR operating points.

Additional expectations for demodulation channel models outside of typical MIMO effects are:

- Performance trends shall be explainable from the channel model construction.

- Alignment with channel measurements, e.g., did-term stable principal directions and angle diversity (subjective).

- Alignment with performance in deployment (subjective).

- Alignment with RAN1 feature development assumptions (subjective).

- Usefulness of model RAN4, e.g., re-use by AIML work, etc. (subjective).

Hence, we propose:

Metrics for comparison to be based on expected typical MIMO effects (e.g. those listed above), explainability of performance trends, alignment with channel measurement, alignment with trends in deployment, and reuse potential in RAN4.

CDL Based Methodologies

Long-term SNR definition

The question of SNR definition in CDL has been popping up every so often. Generally, we see no need to change the classics RAN4 demod receive side pre-EQ SNR definition, but we will elaborate in the following.

Long-term receive side pre-EQ SNR definition

In the last meeting [1] it was agreed to

Agreement: Define normalised receive signal power as the following: The sum of signal power is normalised across all Rx RF chains using a single normalisation factor. The normalisation factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain].

In our understanding, this is a longer formulation of the classical RAN4 pre-EQ SNR definition, that averages over each receive branch, each RE, and the full simulation duration.

Receive SNR is defined as pre-EQ SNR, that averages over each receive branch, each RE, and the full simulation duration. It takes AAVs at both TX and RX side into account, as well as, all interactions of AAVs and propagation environment spatial properties.

However, we want to define that “long-term average” is to be taken as the full simulation duration, or at least [100] slots. In particular, normalizing the channel model or receive power to “1”, or another defined value, at each slot is not in line with the standard RAN4 SNR definition, as it for example overexaggerates the performance impact of deep faded slots. This is especially visible in PMI profiles.

Per slot channel model or receive power normalization is overexaggerating impact of deep faded (or bad precoder choice) slots.

Define “long-term average” in receive signal power definition to be the full simulation duration, or at least [100] slots.

Channel model normalization

It is generally useful for receive side SNR control in link level simulators, that the channel model does not add or remove energy from the transmitted signal. Alternatively, it needs to be known how much the energy/power changes, so it can be considered when adding noise to the receiver input and achieve the target receive SNR.

We usually think of power in the time domain (e.g., the power of a OFDM symbol or rather the average of its samples), but as the time <-> frequency domain transformation is linear, it is equally possible to think about power in the frequency domain. In this case, one needs to consider appropriate scaling, when converting power between symbols and bins.

There are several factors that impact the channel model power/normalization, a few of which we want to highlight in the following.

Note that we use the terms channel model and channel interchangeably here, albeit that we try to use channel model to highlight that the AAV is part of the channel (model).

The CDL channel model under discussion is comprised of three parts: “Tx AAV”, “Propagation environment/clusters”, and “Rx AAV”. The channel model normalization applies to the combination of these blocks.

Channel model normalization/power impact from AAV

In a SCM the chosen AAV has impact on the receive power, and thus the channel normalization for receive side SNR definitions.	
This is easy to visualize, when approximating the AAV as a beamformer and the SCM as clusters with powers and dedicated DoAs/DoDs (directions of arrival/departure).

If we chose a beam that points directly towards a strong cluster, a lot of energy will “arrive” at the receiver; this is proportional to the cluster power. If we chose a beam that points directly towards a weak (or no) cluster, then very little energy will “arrive” at the receiver.	
I.e., in one case the adds more energy to the transmission than in the other case.

A SCM’s modification of the transmission energy/power depends on the interaction of the spatial properties of both the SCM and the AAV. Long term normalization factors depend on the chosen AAV.

Channel model normalization/power impact from specification implementation

It may be obvious, but deserves repeating, that the channel model power depends on the exact implementation of a common specification. Every single factor included in the formulations impacts the input/output levels relation, and thus, power.	
This includes also mistakes in the specification!	
I.e., if an implementation of the SCM fixes and error of the SCM specification, then the normalization factors are hard to compare.

To compare normalization factors, the SCM specification needs to be implemented exactly as is; including mistakes. Alternatively, factors need to be back-calculated to undo fixes.

As an example, see the 38.827 beamforming weight definition, as per specification:

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe.

In our implementation we considered fixing the obvious beam weight normalization error as follows:

α1,1…αMe,Ne=12MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe.

This is an implementation change that can be back-calculated as

Ratio1Y=1MeNe212MeNe2=1821 (2*8)2=14

Which is supported by numerical evidence.

We also note that the channel model normalization factor/power gain for 827 CDL should not be “1”, as the 827 tables are normalized to 0dB on the strongest cluster. No effort was made to normalize the channel model to unit power (Frobenius norm = 1).

The 827 CDL channel model Frobenius norm is not “1”, if implemented as per specification.

Channel model normalization/power intermediate results

Following all the background presented above, these are our long term (full simulation time) average channel model gains per receive antenna, and for each receive antenna, under the 8x8/4x4 CDLC 363ns channel models:

|  | 4x4 CDLC 363ns |  |  |  | 8x8 CDLC 363ns |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AAV | 1X | 1Y | 2 | 3 | 1X | 1Y | 2 | 3 |
| Avg RX branch | 1.13 | 2.31 | 0.50 | 1.50 | 1.56 | 2.04 | 0.52 | 2.55 |
| Each RX branch | 1.23 1.11 1.14 1.04 | 2.45 2.28 2.31 2.20 | 0.54 0.50 0.48 0.46 | 1.62 1.51 1.44 1.41 | 1.62 1.86 1.63 1.33 1.50 1.62 1.55 1.40 | 2.08 2.30 2.08 1.72 2.00 2.15 2.09 1.92 | 0.52 0.58 0.54 0.45 0.51 0.55 0.52 0.47 | 2.54 2.80 2.62 2.28 2.48 2.66 2.60 2.41 |
| Assumptions: - 1/MeNe AAV weight normalization for all AAVs. - Average over the whole simulation duration (10’000 slots). - Average over 100 random starting seeds (all seeds converge to same value eventually) |  |  |  |  |  |  |  |  |

Table 1: Long term average channel model gains

We don’t see this intermediate step as particularly useful, as the normalization depends on such minute implementation details and changes between AAVs and their configurations.	
It is more productive to focus on measuring the long-term pre-EQ SNR in each simulation, to confirm that it matches expectations; and to adapt if not.

Do not focus on alignment of measured normalization factors for CDL channel with different AAV options but leave it up to simulation implementation to keep track of the long-term pre-EQ power measurements and adapt.

Re-evaluation of 38.901 CDL

In the last meeting, it came up during the discussion whether TR 38.901 CDL may really be unfit for creating demodulation performance requirements.	
It was our understanding from digging into the history of TR 38.827, that the contributing companies in 2019 concluded that depending on the starting seed, 901 would lead to different performance outcomes, and AAV needs to be specified to achieve alignment in absolute performance terms. However, we are open to have a trial run of 38.901 in parallel to the 753 CDL work, to confirm this again.

We have some results for this meeting (see Figure 2) for CDLC following “exact implementation” 38.827 and 38.901 using TR 38.901 tables for CDLC UMa. I.e., CDL 901 does not set the DoAs to 90 degs and does not apply a randomness reduction framework.	
We note that we did not run into convergence issues of the channel model normalization power. Meaning that different seeds did converge to a single long term channel power per channel model+AAV combination.

These initial results are not in line with what the authors of 38.827 discussed, but at this point we can’t exclude issues with our setup.	
Thus, we only make the following tentative observation at this point, and we will continue to study and try to align with other contributors.

In preliminary evaluations for 38.901 CDL with AAV options, the long term per RE channel power (average Frobenius norm) does seem to converge to the same value for different starting seeds. We would like to further study this and align with other contributors, before we consider proposing 38.901 CDL to be possible to align on absolute performance terms.

| 10Hz | 100Hz |
| --- | --- |
| 10Hz Doppler, orthogonal |  |

Figure 2: CDLC Uma 365ns performance differences between TR 38.827 and TR 38.901 implementation
(using both channel model parameters and derivation procedures from respective specifications).

Polarization and coordinate system interactions in 901/827 based CDL

It has been brought up at the start for the FS_NR_demod_SCM study, when we were initially evaluating the formulations in 38.827, but it merits repeating that even with omnidirectional AE field patterns, the polarized radiation patterns in the channel coefficient derivation cannot be set to “1”. See for example in TS 38.827:

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,me,ne,θθn,m,ZOD,φn,m,AODFtx,s,me,ne,φθn,m,ZOD,φn,m,AOD

αs,me,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,me,neλ0expj2πrrx,n,mT.vλ0t,	(7.2-9)

The 901/827 CDL models have angle specific inter-polarization power imbalance included, which goes above the XPD (“κ”) factor.	
Physically this effect comes from reflection and transmission being differently angular dependent for both polarizations. Meaning that depending on cluster DoD (defined in GCS, but LCS matters here) unequal amounts of power are shifted between || and ∟ polarizations. (We refrain from putting a copy right protected image from our favourite RF textbook here.)

This is, however, captured in the GCS/LCS transformation and field derivation sections of TS 38.901. Note that the \psi is the same in both sections:

| From 38.901 section 7.1.3: | From 38.901 section 7.3.2: |
| --- | --- |

Capturing this as an observation:

Even with omnidirectional AE field patterns, the CDL formulations capture inter-polarization power imbalance that depend on cluster angles. Implementation of field patterns as “1” is not capturing this effect and the whole procedure as captured in TS 38.901 is necessary.

TR 38.753 CDL Model

In the last meeting, the formalization of the 38.753 CDL model candidate was opened.	
The definition of such the complete model involves a number of decisions to be taken, as is nicely visualized in the last WF [1]. A large number of agreements have already been made:

Figure 3: 38.753 CDL model candidate components from WF [1].

Generally, we are open to any model derivation that is demonstrated to be aligned along a cluster of contributors. We are also open to re-consider the randomness reduction framework based on our results from section 3.3, assuming the results are found to be alignable with other contributors.

Referring yet again back to section 3.3, we don’t see a significant advantage in undoing the 38.827 ZoA modification. The channel condition and performance of the resulting channel model are comparable. As such we shall prioritize the use of an unmodified TR 38.837 based CDL channel model, as it is already used for demodulation (OTA) requirements and implemented in commercial test equipment.

The channel condition and performance of CDL models with and without 38.827 ZoA modifications are comparable.

Assuming the 38.827 model derivation are found to be alignable this meeting, agree these parameters as the TR 38.753 CDL Model.

Comparison of methodologies

In our understanding, this topic is the application of the “Metrics for comparison” issue from section 2.4.

Apply the metrics for comparison between channel model candidates from the test cases, to compare the CDL and TDL methodologies among each other.

To carry out the full comparison, we will need to look at the results collection summary from RAN4#114, as we could not implement all options. Nor do we know whether our implementation is aligned with other contributors.

Preliminary observations were given in sections 2.1.2, 2.1.4 and 2.2.1, with the preliminary outcome:

CDLC UMa 365 10/100Hz as specified in TR 38.827 satisfies almost all metrics for comparison that we propose, while legacy TDL fails on most metrics. The 827 CDL channel model is found to be a viable outcome of the demod SCM study.

Simulation Assumptions Summary

In this section we aim to succinctly summarize the impact of the proposals made in this contribution on the simulation alignment configuration:

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |  | MU-MIMO PDSCH (FR1) |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  | TDD |  |  |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |  |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |  |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |  | IRC, E-IRC |
| Number of Tx Ports |  | 8,4 | 8 |  | 4 |
| Number of Rx Ports/Antennas |  | 8,4 | 4 |  | 4 |
| Number of layers |  | 8,4 | 4,2 |  | 2+2 |
| PMI |  | Random, FFS Fixed Fixed based on follow PMI histogram max count. | Type I: Follow, Random eType II: Follow |  | UE1: Random UE2: Random and not equal to UE1, Orthogonal |
| Waveform |  | CP-OFDM with normal CP |  |  |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |  |  |
| MCS |  | 13 (64 QAM table) |  |  |  |
| PDSCH configuration | Mapping type | Type A |  |  |  |
|  | k0 | 0 |  |  |  |
|  | Starting symbol (S) | 2 |  |  |  |
|  | Length (L) | 12 |  |  |  |
|  | PDSCH aggregation factor | 1 |  |  |  |
|  | Resource allocation type | Type 0 |  |  |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |  |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |  |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |  |  |
|  | Number of additional DMRS | 1 |  |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |  |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |  | type-I-SP |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) paramCombination-r16=5 (L=4, pν=1/4, β=3/4) N_3=1 (“WB”) |  | (N1,N2,O1,O2) = (2,1,4,1) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD WB |  | For Random precoding: for every PRB Bundle (size=2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer | 8 CSI-RS Ports (4,1) |  | TBD |
|  | Density (ρ) | 1 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |  |  |
|  | Row | 4 CSI-RS Ports: row = 4 8 CSI-RS Ports: row = 6 |  |  |  |
|  | Power offset | βCSI-RS in accordance with Table C.3.1-1 in TS 38.101-4 |  |  |  |
| CQI/RI/PMI delay |  | N/A | 6.5 ms | N/A |  |
| Number of HARQ Processes |  | 8 |  |  |  |
| Maximum HARQ transmissions |  | 4 |  |  |  |
| Channel Models |  | The purpose of the study item. Low speed (3kph/10Hz Doppler) and mid speed (30kph/100Hz Doppler) conditions. |  |  |  |
| Testing Metric |  | Full Throughput Curves |  |  |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |  |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |  |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |  |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |  |  |
|  | BS Antenna Polarisation | (+45, -45) |  |  |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |  |  |
|  | UE Antenna Polarisation | (0, 90) |  |  |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |  |  |
|  | Antenna Panel Placement | YZ Plane |  |  |  |

The format of results delivery (i.e., full throughput curves) was only captured in the informative part of [1], but we would propose to transform this into an official agreement:

The simulation results for SU PDSCH/PMI and MU PDSCH/PMI are expected to be delivered as full throughput curves.

Conclusion

In this contribution the TR 38.827 based CDL spatial channel model is found to be a reasonable demod (i.e., baseband branches based) channel model “black box”, that can be aligned between implementations of different contributors. It also exhibits MIMO effects and properties that we expect and observe in deployment, it is spatially consistent, predicable, and explainable. It can be directly applied to SU use cases and is extendable to spatially consistent MU setups (both single and multi-cell).	
The first alignment and comparison results have been provided in this contribution, to be compared to other contributors’ results during RAN4#114.

The following observations and proposals were made:

=== Common for all Spatial Channel Modelling Methodologies

== Test cases for SU-MIMO

= SU PDSCH setups

Fixed precoding

- A non-random precoder choice was found to be useful to capture the impact of typical MIMO effects.

- RAN4 to adopt fixed precoding test cases for SU PDSCH SCM evaluation.

RAN4 to choose the most common PMI from a histogram of signaled/chosen follow PMIs as fixed precoder in SU PDSCH SCM evaluations.

If follow PMI is used in the context of PDSCH test cases, then it is instantaneous “follow PMI”, i.e., it means to apply the PMI as the receiver side would have chosen, without reporting delay.

Receiver Type

For PDSCH requirements, the product linear receiver implementations are often used (while for CSI/PMI requirements the IRC/e-IRC reference are frequently used).

= SU PDSCH observations

Initial alignment of SCM implementation for each test case with random precoders (i.e., alignment cases) shall be considered at RAN4#114 based on the simulation result collection summary.

= SU PMI setups

Reference receiver

For CSI/PMI requirements the IRC/e-IRC reference receivers are frequently used, TR 38.833 IRC (+regularization) and TR 38.878 E-IRC (+regularization) (while for PDSCH requirements, the product linear receiver implementations are often used).

For SU PMI setups, use IRC (+regularization) as reference receiver architecture.

SU PMI test cases to use eType II CB paramCombination-r16=5 (L=4, pν=1/4, β=3/4).

SU PMI test cases to compare eType II and Type I performance with wide band configurations, or at least not WB vs. SB.

SU PMI test cases to use CSI-RS configuration row = 6 for 8Tx and row = 4 for 4Tx (if 4Tx is used), as well as, choosing beta_{CSI-RS} in accordance with Table C.3.1-1 in TS 38.101-4 v18.0, i.e., same EPRE for data and CSI-RS.

eType II random

If required, there are feasible methods to chose random precoders for eTypeII CB, most notably using a random selection of the indicators i1 and i2 used in the formula of Table 5.2.2.2.5-5 of TS 38.214.

= SU PMI observations

None

== Test cases for MU-MIMO

= MU PDSCH Setup and observations

Include the R-ML (reduced maximum likelihood) receiver in MU PDSCH test cases.

= MU PMI

None

== Common test case assumptions

RAN4 shall consider test cases both in low speed (3kph/10Hz Doppler) and mid speed (30kph/100Hz Doppler) conditions.

== Metrics for comparison

Metrics for comparison to be based on expected typical MIMO effects (e.g. those listed above), explainability of performance trends, alignment with channel measurement, alignment with trends in deployment, and reuse potential in RAN4.

=== CDL Based Methodologies

==Long-term SNR definition

Long-term receive side pre-EQ SNR definition

Receive SNR is defined as pre-EQ SNR, that averages over each receive branch, each RE, and the full simulation duration. It takes AAVs at both TX and RX side into account, as well as, all interactions of AAVs and propagation environment spatial properties.

Per slot channel model or receive power normalization is overexaggerating impact of deep faded (or bad precoder choice) slots.

Define “long-term average” in receive signal power definition to be the full simulation duration, or at least [100] slots.

== Channel model normalization

The CDL channel model under discussion is comprised of three parts: “Tx AAV”, “Propagation environment/clusters”, and “Rx AAV”. The channel model normalization applies to the combination of these blocks.

Channel model normalization/power impact from AAV

A SCM’s modification of the transmission energy/power depends on the interaction of the spatial properties of both the SCM and the AAV. Long term normalization factors depend on the chosen AAV.

Channel model normalization/power impact from specification implementation

To compare normalization factors, the SCM specification needs to be implemented exactly as is; including mistakes. Alternatively, factors need to be back-calculated to undo fixes.

The 827 CDL channel model Frobenius norm is not “1”, if implemented as per specification.

Channel model normalization/power intermediate results

Do not focus on alignment of measured normalization factors for CDL channel with different AAV options but leave it up to simulation implementation to keep track of the long-term pre-EQ power measurements and adapt.

== Re-evaluation of 38.901 CDL

In preliminary evaluations for 38.901 CDL with AAV options, the long term per RE channel power (average Frobenius norm) does seem to converge to the same value for different starting seeds. We would like to further study this and align with other contributors, before we consider proposing 38.901 CDL to be possible to align on absolute performance terms.

== Polarization and coordinate system interactions in 901/827 based CDL

Even with omnidirectional AE field patterns, the CDL formulations capture inter-polarization power imbalance that depend on cluster angles. Implementation of field patterns as “1” is not capturing this effect and the whole procedure as captured in TS 38.901 is necessary.

== TR 38.753 CDL Model

The channel condition and performance of CDL models with and without 38.827 ZoA modifications are comparable.

Assuming the 38.827 model derivation are found to be alignable this meeting, agree these parameters as the TR 38.753 CDL Model.

=== Comparison of methodologies

Apply the metrics for comparison between channel model candidates from the test cases, to compare the CDL and TDL methodologies among each other.

CDLC UMa 365 10/100Hz as specified in TR 38.827 satisfies almost all metrics for comparison that we propose, while legacy TDL fails on most metrics. The 827 CDL channel model is found to be a viable outcome of the demod SCM study.

=== Simulation Assumptions Summary

The simulation results for SU PDSCH/PMI and MU PDSCH/PMI are expected to be delivered as full throughput curves.

- References

- R4-2419782, “Way Forward for [113][320] NR_SCM”, Moderator (Nokia), Nokia, RAN4#113.

- R4-2500428, “Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#113.

- R4-2418043, “Discussion on Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#113.