3GPP TSG-RAN WG4 Meeting #114                              						         R4-2500437

# Athens, Greece, 17th – 21th Feburary, 2025

Source: 	Apple

Title: 	On signalling support for type 4 UE

Agenda Item:	7.5.2.2

Document for:	Discussion

# Introduction

Type 4 UE capability and new BS signaling have been discussed in the past RAN4 meetings and the latest agreements were captured in the WF [1]. The following issues remain open for further study.

- <Issue 2-2-2:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated)>

- <Issue 2-2-3:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b>

This contribution will give further analysis on these open issues and presents our views.

# Discussion

The following has been concluded for BS signalling and UE behaviour during initial access.

### <Issue 2-2-2:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated)>

Way forward:

- For NR-CA, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

- For EN-DC, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

### <Issue 2-2-3:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b>

Way forward:

  - When maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

NOTE: This is specified Rel-18 behavior. No need to further discuss.

  - When maxMIMO-layers ＞4

- FFS in the next meeting.

  - When maxMIMO-layers = 4

- FFS in the next meeting.

NOTE: maxMIMO-layers means the existing BS signaling.

At first, we think there seems some confusion in the agreement for issue 2-2-2. Since type 4 UE is FWA UE, switching only happen one time when the UE establish CA operation. While it seems that the agreement is suggesting that type 4 will be the default UE behaviour, e.g. it only focuses on type 4 to type 1. In our view, whether we need a default UE behaviour for type 4 UE largely depends on the way of signalling definition.

In Rel-18, a default UE behaviour in non-collocated condition is assumed for type 2 and introduced BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for type 2 UE for the case of maxMIMO-Layers with value less than or equal to 2. If maxMIMO-layers ＞2, it is collocated. The description for nonCollocatedTypeNR-CA in TS 38.331 is reproduced as below.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

nonCollocatedTypeMRDC

This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

When it comes to Rel-19, we have 3 cases to address depending on maxMIMO-layer >4, <4 or =4. As indicated in the agreement, the case of  maxMIMO-layer<4 has been concluded in the last meeting. The remaining case to be further addressed is when maxMIMO-layers ＞4 and when maxMIMO-layers =4.

When maxMIMO-layers ＞4, similar consideration can be borrowed from Rel-18. In this case, no new BS signaling is needed, and the UE is type 1.

When maxMIMO-layers =4, the signalling for type 4 UE needs to well address the following 2 cases additionally. In this regard, Rel-18 signalling design (absent - present way) cannot well meet the purpose. And the suggested way [3] of “absent, present with value=0, present with value=1” seems a good direction to go. Following this way, there seems no need to define a default UE behavior since every status is clearly commanded by network signaling.

- Rel-19 type 4 capable UE behavior operate under Rel-18 network

As indicated in [3], assuming a Rel-19 UE indicates both Type-4 and Type-2, then Rel-18 NW only understand Type-2 and use maxMIMO-layers=4 to control UE work under collocated if following above design, but Rel-19 UE would see "new Rel-19 NW signaling =absent + maxMIMO-layers=4", then this UE misthink Non-collocated Type-4. Discrepancy between UE and gNB happens.

- Only type 2 capable Rel-19 UE behavior under Rel-19 network.

Since type 2 capability is reported, network just handle such UE by following Rel-18 design. No issue in our view.

Proposal 1: No BS signalling is needed for maxMIMO-layers ＞4. The UE is type 1.

Proposal 2: New Rel-19 BS signalling is needed for the case of maxMIMO-layer=4 and it can be defined in the way of “absent, present with value=0, present with value=1”

# Summary

This contribution presented our consideration on the open issues for type 4 UE. The following observations and proposals are concluded.

Proposal 1: No BS signalling is needed for maxMIMO-layers ＞4. The UE is type 1.

Proposal 2: New Rel-19 BS signalling is needed for the case of maxMIMO-layer=4 and it can be defined in the way of “absent, present with value=0, present with value=1”

# References

[1] R4-2420399, WF on non-collocated intra-band EN-DC and NR-CA, KDDI, RAN4#113

[2] R4-2418499, Discussion on UE Capability and BS Signaling for non-collocated Type 4a4b, KDDI, RAN4#113

[3] R4-2418303, Discussion on UE and NW behavior for non-collocated deployment, Samsung, RAN4#113