3GPP TSG-RAN WG4 Meeting #114	 														R4-2500442

Athens, Greece, 17th – 21st February 2025

Agenda item:			7.27.2.2

Source:	Apple

Title:	Further discussion on CSI-RS based L1-RSRP measurement

Document for:	Discussion

# Introduction

During the past meetings, there have been extensive discussions on CSI-RS based L1 measurement for NR mobility enhancements WI [1-3]. WF in [3] captured all the conclusions and open issues for CSI-RS based L1 measurement in RAN4#113 meeting.

This contribution will give further analysis on the open issues and present our proposals.

# Discussion

## Measurable or detectable

The following is concluded from [3]

Issue 3-1-3: Whether to use detectable or measurable?

<Candidate solutions>

- Option 1: The difference of the definition between detectable and measurable need to be further clarified. (CATT)

- Option 2: Use detectable. (Apple, ZTE)

- Option 3: Use measurable. (Xiaomi, OPPO, Ericsson, Nokia)

<Recommended WF>

- Continue discussion. RAN4 can first try to align understanding on what “detectable” and “measurable” mean.

This issue comes from the discussion on known cell condition for CSI-RS based L1 measurement. In the last meeting we proposed to use detectable. After further thinking the word “detectable” seems used for SSB because there are clear identification requirements for SSB, and it is possible to perform cell search and detect SSB without prior knowledge. With this understanding detectable seems to be used for unknow cell identification. While measurable is used for cell signal measurement in connected mode or after a cell is detected. Also, after further check the requirement applicability for CSI-RS based L3 measurement, it also seems “measurable” is more suitable here to follow similar logic as L3 CSI-RS based measurement. E.g. The CSI-RS from the target cell configured for L1 measurement should remain measurable.

Proposal 1: it is proposed to use “measurable” for known cell condition for CSI-RS based L1 measurement.

## Side condition for CSI-RS based L1 measurement

The following remains open for further discussion from [3]

<Candidate solutions>

- Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (MTK)

- Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Ericsson, Nokia, Saumsung)

<Recommended WF>

- Continue discussion.

It is suggested to reuse the existing side condition for CSI-RS based L1-RSRP measurement for serving cell as the starting point for CSI-RS based L1-RSRP measurement for neighbour cell.

#### Proposal 2: it is proposed to use detectable and legacy side condition of SINR=-3dB.

## Measurement procedure

The following is concluded from [3].

- Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

  - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

There has been extensive discussion on whether to keep SSB based L1 measurement (step 2) as the condition for CSI-RS based L1 measurement in FR2. The UE needs to perform Rx beam sweeping on CSI-RS resource for neighbour cells without prior SSB based L1 measurement as the basis, which will make the measurement delay for neighbour cell quite long (N times scaling). In this regard, we think it make sense to keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2. On the other hand, there is a request to make this step as optional to allow more capable UE to skip this step and save the measurement time, e.g. a UE with Channel based beam training for L1 measurement may not need step 2. In this case, specification support shall be considered to optimize the measurement procedure for such UE with capability reporting.

Proposal 3: Whether step 2 can be skipped or not can be based on UE capability reporting.

## Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement

<Agreement>

- A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and

    - It is applied for SCS = 60KHz

  - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

Regarding the [], we think it’s ok to remove it. Since within active BWP, no matter how many CSI-RS resources are configured the SCS and CP will all be the same. It is reasonable to use the current wording for the first 2 bullets.

Regarding the 3rd bullet on centre frequency, one problem for intra-frequency/inter-frequency definition of CSI-RS based L1 measurement is how to determine the reference centre since multiple CSI-RS resources may be configured for both serving cell and neighbour cell. We think the same logic as CA/DC can be reused here, e.g. a UE could have multiple intra-frequency neighbour cells as long as the centre of the CSI-RS resource to be measured is the same as any of the CSI-RS resources within active BWP.

Proposal 4: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

  - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

## Measurement on CSI-RS with repetition ON

<Candidate solutions>

- Option 1: Considering the potential large overhead, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition ON in FR2-1. (MTK)

- Option 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. (Huawei, CATT)

  - Option 2a: Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1. (vivo)

There seems a preference to skip the case with repletion ON. While there seems no clear conclusion in RAN1 whether CSI-RS with repletion ON can be supported for L1 measurement for mobility. We would prefer to wait for RAN1 agreement on this at this stage.

Proposal 5: wait for RAN1 agreement on whether to support L1 measurement on CSI-RS with repletion on.

## Measurement on CSI-RS with repetition OFF

<Candidate solutions>

- Option 1: From RRM requirements perspective, for CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1. (vivo)

  - Option 1a: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. Considering that there is no gain compared to SSB based L1 measurement, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition OFF in FR2-1. (MTK)

  - Option 1b: In FR2, RAN4 will define the requirements for the following case, (CATT)

    - CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

- Option 2: Define first requirements for the case where the repetition is ‘OFF’ (Nokia)

For L1 measurement with repetition OFF, beam training on CSI-RS resources can be avoided and the L1-RSRP measurement is only performed after UE has done SSB based L1-RSRP measurement or UE reported the capability that it is capable of skipping SSB based L1-RSRP measurement. We think RAN4 can prioritize this scenario.

Proposal 6: RAN4 to prioritize L1 measurement on CSI-RS with repetition OFF.

## CSI-RS resource type

The following options are open for further study.

<Candidate solutions>

- Option 1: In addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells. (CATT)

- Option 2: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS. (Apple, vivo)

- Option 3: Start from periodic CSI RS measurements and wait for wait until RAN1 agrees on aperiodic and semi-persistent CSI-RS support. (Nokia)

- Option 4: RAN4 define CSI-RS based L1 measurement for periodic and semi-persistent. (Samsung)

- Option 5: RAN4 should define CSI-RS based L1 measurement requirements for periodic, aperiodic and semi-persistent CSI-RS (ZTE)

It is noted that Option 2 and Option 3 are actually the same. Besides periodic CSI-RS, there seems also interests to cover aperiodic and semi-persistent CSI-RS when specifying requirements. We can focus on periodic CSI-RS and further check aperiodic and semi-persistent CSI-RS when RAN1 conclude them.

Proposal 7: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision.

## Supported measurement type

<Agreement>

- CSI-RS based L1 measurement without gap is supported.

- FFS on CSI-RS based L1 measurement with gap

The following was concluded for further discussion.

For CSI-RS based L1 measurement without GAP, it includes the case of intra-frequency measurement with target cell CSI-RS to be measured fully confined within the active BWP, and inter-frequency measurement with target cell CSI-RS to be measurement fully confined within the active BWP. We would like to further check whether to consider intra-frequency measurement with the target cell CSI-RS to be measured partly confined within the active BWP, for which GAP would be needed.

Proposal 8: RAN4 to further check whether to consider intra-frequency measurement with the target cell CSI-RS to be measured partly confined within the active BWP, for which GAP would be needed.

<Candidate solutions>

- Option 1: In Rel-19 LTM, at least the type 1 MG should be supported for CSI-RS based L1 measurement. (CATT)

- Option 2: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer. (CATT)

- Option 3: For CSI-RS based L1 with gap measurement, if RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements. (CATT)

- Option 4: RAN4 to further discuss type of MG for the CSI-RS based L1-RSRP measurements. (Ericsson)

Measurement gap related

RAN4 to further consider whether to consider the case that GAP is needed at first in issue 2-8. If needed, we think option 1 is reasonable, e.g. in Rel-19, LTM, at least the type 1 MG should be supported for CSI-RS based L1 measurement.

Proposal 9: decide measurement GAP after RAN4 conclude measurement type. If GAP is needed, then consider type 1 GAP as minimum. Other GAP types are FFS.

## CSI-RS based L1 measurement w.r.t active BWP

<Candidate solutions>

- Option 1: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM. (CATT)

- Option 2: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK, Xiaomi, OPPO)

- Option 3: Support CSI-RS based L1-RSRP measurement within active BWP and outside active BWP, and same principle can be used as start point, e.g., defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement. (ZTE)

- Option 4: If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s). (vivo)

- Option 5: From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

  - Option 5a: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

  - Option 5b: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

  - Option 5c: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

First of all, regarding option 5 on the clarification of CSI-RS of candidate cell outside active BWP, we think option 5a is more reasonable but would like to change the wording a bit to make it accurate enough. We think the CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP.

Regarding whether to consider CSI-RS measurement outside active BWP, this can be discussed under measurement type in issue 2.8.

Proposal 10: The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP.

## Restriction on CSI-RS resource configuration

<Candidate solutions>

- Option 1: It is proposed to introduce a CSI-RS based L1 measurement timing configuration. (Apple)

- Option 2: RAN4 requirements apply only when all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type. (Huawei, Qualcomm)

- Option 3: Only the CSI-RS resources with the same center frequency, SCS and CP type can be measured within one measurement gap occasion. (Huawei)

- Option 4: The CSI-RSs of the target cells are considered as the same frequency layer if the SSBs of these target cells are of the same SSB frequency, SCS and CP type. (vivo)

CSI-RS resource allocation can be very flexible, which may largely increase the complexity in measurement requirement development. In Rel-16 CSI-RS based L3 measurement, for simplicity the requirement is developed based on some specific assumptions on the resource configuration as below.

-    All CSI-RS resources on one inter-frequency layer are configured within a window of up to 5ms, and

-    The periodicity of the configured CSI-RS resources is 10ms, 20ms or 40ms, and

-    CSI-RS resources for measurements and the associated SSB for cell identification are configured within measurement gap.

Since both measurement GAP and frequency layer definition is related to the CSI-RS resource configuration, it is proposed RAN4 conclude the resource configuration issue at first. E.g. whether to follow the same approach of L3 measurement by restricting the CSI-RS configuration in the spec or introduce some measurement timing configuration (e.g. L1-CMTC) for CSI-RS based L1 measurement. If the latter approach, L1-CMTC is to be configured by network via RRC signaling and should include at least the following information:

  - Frequency location (e.g. center frequency + bandwidth)

  - Time location (e.g. time offset + periodicity)

  - CSI-RS resource ID

The benefit of introducing CSI-RS measurement timing configuration can be several folds: on one hand it can avoid hard limitation for resource configuration; on the other hand, the UE only need to perform measurement on the resources indicated by the timing configuration, which can also reduce scheduling restriction to avoid throughput degradation since the more symbols UE needs to measure the more scheduling restrictions apply. Thirdly it can help to simplify the definition of frequency layer. In the last meeting, there were some discussions on how to define frequency layer for CSI-RS based L1 measurement. Due to the flexibility of CSI-RS configuration, the frequency layer definition needs to consider periodicity factor in addition to those included in the intra/inter-frequency definition. If we address the periodicity factor in the L1-CMTC, the definition of frequency layer can be simple by only considering SCS, CP type, centre frequency and BW.

Proposal 11: It is proposed to introduce a CSI-RS based L1 measurement timing configuration.

## Scenarios to be considered w.r.t. RTD

RAN4 concluded the following for further study.

- RTD is the received time difference among CSI-RS and SSB from different cells.

  - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP.

  - Alt 2: only define requirements for RTD<CP.

    - NW shall only enable this when RTD<CP is guaranteed.

We think RTD<CP should be taken as the baseline. It is noted that R18 SSB based L1 RSRP measurement on neighbor cell already addressed the case when RTD is larger than CP by UE capability. UE capabiltiy should also be considerd if RAN4 agrees to consider the case where RTD is larger than CP between serving cell and neighbour cell on the same carrier since it will put a lot complexity for UE implementaiton in terms of inreased number of FFTs. Both otpion 1 and option 2 are ok for us. For otpion 2, the group need to introduce RTD reporting to assit network make the judgement on when RTD<CP is satisfied.

Proposal 12: Either option 1 or option 2 is ok. RTD level reporting might be needed if option 2 is adopted.

<Candidate solutions>

- Option 1: Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain. (Ericsson)

- Option 2: Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity) (Nokia)

- Option 3: If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”(MTK)

- Option 4: From RRM requirement perspective, at least if RRM requirements for CSI-RSs outside active BWP are to be considered, RAN4 needs to clarify the frequency layer of the CSI-RS based L1 measurement. (vivo)

  - The CSI-RSs of the target cells are considered as the same frequency layer if the SSBs of these target cells are of the same SSB frequency, SCS and CP type.

  - No RRM requirements are defined, if CSI-RSs on the same frequency layer are configured with the different SCS or CP type.

  - Note: No restriction on the center frequency and/or BW of the CSI-RS.

Definition of frequency layer

First of all, we think it’s ok to focus on the same SCS and CP types. E.g. do not define RRM requirements for the case where target CSI-RS to be measured are with different SCS and CP type. But we still need to discuss the case when centre frequency is different and the case when centre frequency is the same but not fully confined within active BWP. We propose to focus on supported intra-frequency/inter-frequency definition and measurement type to be supported before we start this discussion.

Proposal 13: Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition and measurement type are concluded.

## Whether to consider CSI-RS based L1-SINR measurement

<Candidate solutions>

- Option 1: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. (CATT)

- Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1. (Apple, Qualcomm, ZTE)

- Option 3: RAN4 focus on CSI-RS based L1-RSRP measurement. (Samsung)

Proposal 14: it is proposed RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1.

## CSI-RS accuracy requirements

<Candidate solutions>

- Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (ZTE, Apple)

Proposal 15: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

## LS to RAN1

LS to RAN1 on intra-frequency/inter-frequency definition is needed if proposal 5 is agreed.

Proposal 16: it is proposed to send a LS to RAN1 as below:

# Overall description

RAN4 discussed intra-frequency and inter-frequency definition for CSI-RS based L1 measurement and concluded that a measurement is defined as an intra-frequency CSI-RS based L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

-	the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

-	It is applied for SCS = 60KHz

-	the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

A measurement is defined as an inter-frequency CSI-RS based L1 measurement if the intra-frequency condition is not met.

# Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to consider the RAN4 decision on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

# Conclusion

This contribution proposed our consideration on open issue for CSI-RS based L1 measurement. The following proposals are concluded.

Proposal 1: it is proposed to use “measurable” for known cell condition for CSI-RS based L1 measurement.

#### Proposal 2: it is proposed to use detectable and legacy side condition of SINR=-3dB.

Proposal 3: Whether step 2 can be skipped or not can be based on UE capability reporting.

Proposal 4: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

  - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

Proposal 5: wait for RAN1 agreement on whether to support L1 measurement on CSI-RS with repletion on.

Proposal 6: RAN4 to prioritize L1 measurement on CSI-RS with repetition OFF.

Proposal 7: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision.

Proposal 8: RAN4 to further check whether to consider intra-frequency measurement with the target cell CSI-RS to be measured partly confined within the active BWP, for which GAP would be needed.

Proposal 9: decide measurement GAP after RAN4 conclude measurement type. If GAP is needed, then consider type 1 GAP as minimum. Other GAP types are FFS.

Proposal 10: The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP.

Proposal 11: It is proposed to introduce a CSI-RS based L1 measurement timing configuration.

Proposal 12: Either option 1 or option 2 is ok. RTD level reporting might be needed if option 2 is adopted.

Proposal 13: Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition and measurement type are concluded.

Proposal 14: it is proposed RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1.

Proposal 15: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

Proposal 16: it is proposed to send a LS to RAN1 as below:

# Overall description

RAN4 discussed intra-frequency and inter-frequency definition for CSI-RS based L1 measurement and concluded that a measurement is defined as an intra-frequency CSI-RS based L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

-	the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

-	It is applied for SCS = 60KHz

-	the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

A measurement is defined as an inter-frequency CSI-RS based L1 measurement if the intra-frequency condition is not met.

# Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to consider the RAN4 decision on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

# Reference

[1] RP-234036, New WID: NR mobility enhancements Phase 4, Apple, RAN#102

[2] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#112bis

[3] R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, RAN4#113