3GPP TSG-RAN WG4 Meeting #114                               						     R4-2500443

# Athens, Greece, 17th – 21st Feburary, 2025

Source: 	Apple

Title: 	On ATG UE RF requirement applicability

Agenda Item:	5.6.1

Document for:	Discussion

# Introduction

ATG UE RF requirement applicability at antenna connectors were discussed in the last meeting. A WF was agreed in [1] with the following aspects open for further study.

- Tx requirements definition for two types of ATG UE antenna

- Occupied bandwidth for ATG

- Frequency error for ATG

- EVM requirements

- Rx requirements definition for two types of ATG UE antenna

This contribution will discuss the open issue and give our proposal.

# Discussion

## Open issue for Tx requirement

The followings need to be further checked in February meeting,

- Option 1: The requirements for Frequency error, Error Vector Magnitude, Carrier leakage and EVM equalizer spectrum flatness are defined per layer.

- Option 2: All transmit signal quality requirements should be specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antennas(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array.

Though they are listed as 2 options, it seems they are not contradicting with each other. Option 1 is talking about how we require them when there are more than one layers and option 2 gives the physical reference point where the requirement should be measured. We would prefer to give a combined definition for the mentioned requirement of frequency error, error vector magnitude, carrier leakage and EVM equalizer spectrum flatness. E.g. these requirements are defined per layer at the antenna connector (s) with one or multiple omni-directional antenna (s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array.

Proposal 1: the requirements for frequency error, error vector magnitude, carrier leakage and EVM equalizer spectrum flatness shall be required per layer at the antenna connector (s) with one or multiple omni-directional antenna (s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array

## Open issue for Rx requirement

The following needs to be further discussed in February meeting.

- Rx antenna port requirements of the ATG UE need to be specified at the antenna connector(s) or TAB connectors.

- The power should be fed to each connector during the test

We presented CR in 2024 to update the REFSENS requirements while it was argued that the implementation for ATG is very special and CR is not approved. But we still think it is general enough to require that the requirement should be applicable at the antenna connector.

Proposal 2: Rx requirements of ATG UE need to be specified at the antenna connector (s) or TAB connector (s).

## ATG UE antenna definition

- ATG UE with one or multiple passive antenna(s) and ATG UE with the antenna array.

The following is concluded for further discussion.

For ATG UE antenna definition with non-array antenna, we think it appropriate to follow the same approach as handheld UE. E.g. no need to add the word “passive”.

Proposal 3: ATG UE antenna definition is “ATG UE with one or multiple antenna(s) and ATG UE with the antenna array.”

# Summary

This contribution presented our consideration on open issue for Rel-18 ATG UE RF requirement. The following proposals are presented for discussion.

Proposal 1: the requirements for frequency error, error vector magnitude, carrier leakage and EVM equalizer spectrum flatness shall be required per layer at the antenna connector (s) with one or multiple omni-directional antenna (s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array

Proposal 2: Rx requirements of ATG UE need to be specified at the antenna connector (s) or TAB connector (s).

Proposal 3: ATG UE antenna definition is “ATG UE with one or multiple antenna(s) and ATG UE with the antenna array.”

# References

[1]	R4-2420338, WF on NR ATG UE RF, CMCC, RAN4#113