3GPP TSG-RAN WG4 Meeting #114	 R4-2500456

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.25.2.1

Source: 	Apple

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion

# Introduction

RAN4 requirements for OD-SSB was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are quite many open issues as captured in [1].

In this contribution, we will discuss all open issues listed in RAN4 WF [1]. After discussion, corresponding proposals are provided.

# Discussion

### Sub-topic 1-1: General On-demand SSB(OD-SSB) requirements

Issue 1-1-2: L1 measurement before SCell activation

- Proposals

  - Option 1: L1 measurements are not supported for deactivated SCell.

  - Option 2: Wait RAN1’s further clarification on the applicable scenario.

Before SCell activation the SCell is in deactivated mode. Currently L1 measurement on deactivated SCell is not supported, except for LTM. We don’t think L1 measurement for LTM based on OD-SSB is in the scope. During SCell activation, sometimes L1 measurement is needed (e.g. in FR2 unknown case). In legacy it is based on always on SSB. In NES scenario it is feasible to be based on OD-SSB. Similarly, L1 measurement based on OD-SSB after SCell activation is also feasible, RAN4 requirements are defined based on TSSB. We only need to clarify that TSSB refers to OD-SSB periodicity in this case.

Proposal 1: L1 measurement based on OD-SSB before SCell activation is not considered.

### Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 1)

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement

Agreement:

For the case of CSSF = 1:

- OD-SSB periodicity regardless of measCycleSCell and DRX cycle.

  - FFS whether a lower bound will be specified.

FFS for the other cases of CSSF.

Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement

- Option 1: The same CSSF as legacy

- Option 2:

  - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements.

    - RRM requirements on PSCC and other SCCs are not applicable during the time-window.

  - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed.

Other option is not precluded.

The minimum sampling internal is the OD-SSB periodicity. However, if the OD-SSB periodicity is too short, it would significantly increase UE power consumption. On the other hand, if the measurement results in scheduling restriction, keeping measurement in every OD-SSB occasion would result in contiguous data interruption. One solution is to follow existing non-DRX RRM measurement requirement, i.e. adding a minimum sampling interval like 40ms.

Proposal 2: a lower bound is necessary in measurement requirement in fast mode.

Regarding other cases of CSSF, it is about impact on other carriers. There are two different cases. Case one is that there are more than one carriers with active OD-SSB to measure.  Case two is there is no any other carrier configured with active OD-SSB but there are other legacy carriers, e.g. with always-on SSB.

For case one, if it has to be included in the scope, CSSF is necessary to scale the corresponding measurement requirements since UE has only up to two searchers. One of them is assigned to PCell and the other one needs to be shared among carriers configured with active OD-SSB.

For case two, CSSF is also necessary since we don’t expect any change in searcher assumption in this work item. We can accept that carrier with active OD-SSB shall be prioritized, provided that T2 (in the following issue 1-2-3) is introduced. In other words, UE can suspend the measurement on legacy carriers considering that network may activate the SCell with OD-SSB soon. Another way to prioritize OD-SSB carrier is to assign more measurement opportunities, e.g. by applying different scaling factors between carriers with OD-SSB and legacy carriers.

Proposal 3: RAN4 shall not change searcher assumption in this work item.

Proposal 4: for the case where there are more than one carriers configured with active OD-SSB:

- Option 1: not supported in this release

- Option 2: introduce CSSF in corresponding measurement requirement

Proposal 5: for the case where there is at least one regular carrier (e.g. with legacy SSB) besides the carrier configured with active OD-SSB:

- Option 1: introduce CSSF in corresponding measurement requirement

- Option 2: prioritize measurement on OD-SSB carrier and allow extension on measurement on other carriers, provided that T2 in issue 1-2-3 is agreed.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation?

Agreement:

- After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective.

- The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)?

Since UE needs to perform fast measurement in phase 1, it is reasonable to introduce T2 to avoid significant UE power consumption and negative impact on other carriers (assuming UE needs to prioritize measurement on carrier with OD-SSB). Regarding length of T2, one solution is to use the maximum period in known cell definition, e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1. Reason is as long as the cell is considered as known, UE can skip or relax RRM measurement.

Proposal 6: length of T2 is same as that defined in known cell definition, e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1.

Another issue we would like to discuss is about interruption. For case 1, before OD-SSB is activated there is nothing to measure. There is no point for UE to keep the RF chain on. Upon OD-SSB activation, UE needs to perform fast measurement on OD-SSB immediately. UE would switch on the RF chain upon OD-SSB activation. If network doesn’t activate the SCell during the fast window. UE may switch off the RF chain after the fast window for power saving. Therefore, UE shall be allowed to cause two interruptions before and after the fast measurement window. After the fast window, interruption is same as legacy.

Proposal 7: upon OD-SSB activation, UE shall be allowed to cause two interruptions before and after the fast measurement window. After the fast window, interruption is same as legacy.

### Sub-topic 1-3: OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

- Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 1-1: The measurement periodicity is OD-SSB periodicity

  - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB

- Option 2: UE always follow legacy requirements based on measCycleSCell

- Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

We continue supporting option 2 and 3. Before activation of OD-SSB, UE measures the SCell based on always-on SSB following measCycleSCell. The measurement period depends on several aspects, e.g. DRX cycle, number of carriers to measure, length of measCycleSCell, Kp and so on. Upon OD-SSB activation, if the SCell is already known, it is unnecessary for UE to perform fast measurement since the SCell can already be activated in a fast mode (fulfill known definition). But if the cell is somehow unknown upon OD-SSB activation, we can accept to perform fast measurement similarly with case 1 as a compromised solution.

Proposal 8: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2:

- Option 1: RAN4 requirement is defined based on legacy measCycleSCell (preferred)

- Option 2:

  - When OD-SSB is activated, UE determines whether to perform fast measurement or not

    - If the serving cell is known (based on RRM on always-on SSB), UE doesn’t need to perform fast measurement. Existing measurement requirements based on measCycleSCell can be applied.

    - If the serving cell is unknown, UE performs measurement in a fast mode within a time window. After the time window, UE falls back to slow mode.

      - Design of fast mode, slow mode and time window can be same as that discussed in case 1.

The last thing we would like to discuss is how to facilitate SCell activation in case 1. Note that typically in case 1 target cell is unknown to UE since no always-on SSB before OD-SSB activation. The whole SCell activation would be quite long for unknown target cell since UE needs to start from scratch. Note that in R18 SSB-less scenario, there is a reference cell from which UE can borrow some information like AGC and timing so that the activation latency could be reduced:

referenceCell Indicates the reference cell, i.e. the cell which provides the timing reference and AGC source for the inter-band SSB-less SCell. If the reference cell is an SCell or PSCell, it should be an activated SCell or activated PSCell. If this field is absent a "default cell" is the reference cell as defined in TS 38.133 [14].

However, referenceCell configuration only applies for inter-band SSB-less SCell. We think it is beneficial to extend the concept to cover SCell with OD-SSB operation.

Proposal 9: extend referenceCell to cover SCell with OD-SSB operation.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for OD-SSB. After discussion, the following conclusions are provided:

Proposal 1: L1 measurement based on OD-SSB before SCell activation is not considered.

Proposal 2: a lower bound is necessary in measurement requirement in fast mode.

Proposal 3: RAN4 shall not change searcher assumption in this work item.

Proposal 4: for the case where there are more than one carriers configured with active OD-SSB:

- Option 1: not supported in this release

- Option 2: introduce CSSF in corresponding measurement requirement

Proposal 5: for the case where there is at least one regular carrier (e.g. with legacy SSB) besides the carrier configured with active OD-SSB:

- Option 1: introduce CSSF in corresponding measurement requirement

- Option 2: prioritize measurement on OD-SSB carrier and allow extension on measurement on other carriers, provided that T2 in issue 1-2-3 is agreed.

Proposal 6: length of T2 is same as that defined in known cell definition, e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1.

Proposal 7: upon OD-SSB activation, UE shall be allowed to cause two interruptions before and after the fast measurement window. After the fast window, interruption is same as legacy.

Proposal 8: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2:

- Option 1: RAN4 requirement is defined based on legacy measCycleSCell (preferred)

- Option 2:

  - When OD-SSB is activated, UE determines whether to perform fast measurement or not

    - If the serving cell is known (based on RRM on always-on SSB), UE doesn’t need to perform fast measurement. Existing measurement requirements based on measCycleSCell can be applied.

    - If the serving cell is unknown, UE performs measurement in a fast mode within a time window. After the time window, UE falls back to slow mode.

      - Design of fast mode, slow mode and time window can be same as that discussed in case 1.

Proposal 9: extend referenceCell to cover SCell with OD-SSB operation.

# Reference

- R4-2420098, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

- R1-2407565, LS on timeline for On-demand SSB operation on SCell, Apple