3GPP TSG-RAN WG4 Meeting #114							         			R4-2500461

Athens, GR, 17th – 21st February 2025

Title: 	Additional test points for CA_n40A-n41C and CA_n41C-n79A with n41C UL

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.3.3	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 2 DL with up to 2UL (NR_CADC_R19_2BDL_xBUL)	[NR_CADC_SUL_R19-Core]

Document for:	Approval

# 1	Background

In R4#113, we proposed additional MSD test points for CA_n40A-n41C and CA_n41C-n79A, with n41C UL in [1], and captured input from other companies combined with other agreements, in a way forward [2]. In this contribution, we made minor adjustments to our proposals, in view of the latest agreements.

# Discussion

## 2.1 Way Forward Input

WF [2] provides a number of agreements that describes progress on the definition of a second test point for the two band combinations with n41C UL:

Agreement for CA_n40A-n41C DL with CA_n41C UL:

- For the first and second test point align to the highest band n40 5MHz victim channel at 2397.5MHz.

  - For the first test point average of 41.3dB and 42.5dB = 41.7dB is used

  - For the second test point, proponent of the 50MHz case could provide a scaled 5MHz MSD for averaging of values.

- For the CA_n41C carrier frequencies use the CA channel separation with Fc_low = 2565MHz and Fc_high = 2644.8 and rework 1RB+1RB positions to hit 2397.5MHz 5MHz victim channel.

- For the second test point use 270RB3 and 162RB0 as the allocation which correspond to fully allocated contiguous RB allocation

- The Table format and two test point place in the TS is discussed in topic 3.

Agreement for CA_n41C-n79A DL with CA_n41C UL:

- For the first test point align to the highest band n79 10MHz victim channel at 4995MHz.

  - For the first test point average of 12.4dB and 8.4dB = 10.4dB is used

- For the second test point align to the highest band n79 10MHz victim channel at 4995MHz or 40MHz victim channel at 4980MHz.

- For the second test point, proponent of the 40MHz or 10MHz case could provide a scaled MSD for averaging of values.

- For the CA_n41C carrier frequencies use the CA channel separation with Fc_low = 2565MHz and Fc_high = 2644.8 and rework 1RB+1RB positions to hit the 4995MHz 10MHz victim channel.

- For the second test point use 270RB3 and 162RB0 as the allocation which correspond to fully allocated contiguous RB allocation

- The Table format and two test point place in the TS is discussed in topic 3.

Agreement on capturing test points in the TS:

- Applicable to 1UL band case with contiguous intra-band ULCA unless otherwise stated.

- Companies are encouraged to propose how to capture the two test points in the same table so that they are not confused as two separate issues.

- The 1RB+1RB is the mandatory baseline test point.

- For H2 and H3 related interference, the victim channel is chosen the closest to the related harmonic frequency.

- The second optional test point uses contiguous fully allocated intra-band contiguous ULCA case.

- FFS if a new table is needed but if so, the legacy 1UL 2CC test points should be moved to that table for both contiguous and non-contiguous intra-band UL CA

Before we adjust our test point definitions according to the two first agreements, we believe it is important to first address the last section on how and where the test points are captured in the specification.

## 2.2 How to capture the additional test point

Most of the agreements in way forward [2] were already included in our proposal in [1], where our preference is to continue capturing both MSD test points as IMDs, within the IMD MSD tables.

For this we re-iterate our proposal, while only adding the parts agreed in [2].

Proposal on mandatory and optional test points for 1 band 2CC contiguous ULCA MSD:

- Applicable to the 1UL band case with contiguous intra-band ULCA unless otherwise stated.

- The 1RB+1RB is the mandatory baseline test point.

- For H2 and H3 related interference, the victim channel is defined as the closest to the related harmonic frequency.

- The second optional test point uses the contiguous, fully allocated, intra-band contiguous ULCA case.

- The two test points are part of the same MSD table, with the 1RB+1RB test point first. To eliminate rework, these are located in the 2UL IMD MSD tables, as this is the current position for the mandatory 1RB+1RB test point.

- The two test points use the same configuration (UL/DL frequencies, UL/DL CBW), except the UL CCs RB allocations:

  - The FULs are defined as the closest to the victim band, and the contiguous CC BWs are such that the maximum aggregated BW that is obtained still fits within the band.

  - The victim band FDL is defined as the closest to the considered IMD product frequency range, using the lowest DL CBW.

  - The 1RB+1RB RBstart is selected to best center the target IMD in the victim DL channel. This test point is mandatory.

  - The Full+Full RBstart of the lowest UL channel is selected such that the Full+Full allocation is contiguous with RBstart for the upper channel defined as 0. This test point is optional.

- Both test point interference levels are calculated based on meeting SEM and ACLR (MPR is allowed).

- These rules will be captured in the PRD.

## 2.3 CA_n40A-n41C DL with CA_n41C UL

Our input from [1] in last meeting, was already very close to satisfying the agreements in the way forward [2] only requiring some minor adjustments with the MSD value modified to the agreed average. The second test point MSD may be subject to averaging with results from other companies.

Proposal for CA_n40A-n41C DL with CA_n41C UL MSD test point to add to the IMD related MSD table.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD314 |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=52) | 2644.8 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | [34.4] | TDD | IMD314,X |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

## 2.4 CA_n41C-n79A DL with CA_n41C UL

Our input from [1] in last meeting, was already very close to satisfying the agreements in the way forward [2] only requiring some minor adjustments with the MSD value modified to the agreed average. The second test point MSD may be subject to averaging with results from other companies.

Proposal for CA_n41C-n79A DL with CA_n41C UL MSD test point to add to the IMD related MSD table.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=111) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=161) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD414 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | [5.3] | TDD | IMD414,X |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we quickly revisited our proposals from [1] based on last meeting agreements in [2].

Proposal on mandatory and optional test points for 1 band 2CC contiguous ULCA MSD:

- Applicable to the 1UL band case with contiguous intra-band ULCA unless otherwise stated.

- The 1RB+1RB is the mandatory baseline test point.

- For H2 and H3 related interference, the victim channel is defined as the closest to the related harmonic frequency.

- The second optional test point uses the contiguous, fully allocated, intra-band contiguous ULCA case.

- The two test points are part of the same MSD table, with the 1RB+1RB test point first. To eliminate rework, these are located in the 2UL IMD MSD tables, as this is the current position for the mandatory 1RB+1RB test point.

- The two test points use the same configuration (UL/DL frequencies, UL/DL CBW), except the UL CCs RB allocations:

  - The FULs are defined as the closest to the victim band, and the contiguous CC BWs are such that the maximum aggregated BW that is obtained still fits within the band.

  - The victim band FDL is defined as the closest to the considered IMD product frequency range, using the lowest DL CBW.

  - The 1RB+1RB RBstart is selected to best center the target IMD in the victim DL channel. This test point is mandatory.

  - The Full+Full RBstart of the lowest UL channel is selected such that the Full+Full allocation is contiguous with RBstart for the upper channel defined as 0. This test point is optional.

- Both test point interference levels are calculated based on meeting SEM and ACLR (MPR is allowed).

- These rules will be captured in the PRD.

Proposal for CA_n40A-n41C DL with CA_n41C UL MSD test point to add to the IMD related MSD table.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD314 |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=52) | 2644.8 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | [34.4] | TDD | IMD314,X |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

Proposal for CA_n41C-n79A DL with CA_n41C UL MSD test point to add to the IMD related MSD table.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=111) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=161) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD414 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | [5.3] | TDD | IMD414,X |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

# References

[1] R4-2417792 MSD test points for CA_n40A-n41C and CA_n41C-n79A with n41C UL, Skyworks Solutions Inc., R4#113

[2] R4-2420471 WF on UL CA_n41C MSD test point and its capture in the TS, Skyworks, Huawei, Qualcomm, R4#113