3GPP TSG-RAN WG4 Meeting #114	R4-2500467

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SBFD operation

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made [1]:

### Sub-topic 1-2: RRM impacts of SBFD operation #### Issue 1-2-1: Requirements for SSB based measurement - Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. #### Issue 1-2-2: Requirements for CSI-RS based L1 measurement - Agreements (from online session) - For CSI-RS colliding with UL transmission - For aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission. - For periodic and semi-persistent CSI-RS resource, RAN4 will send RAN1 LS for clarification if needed. - For CSI-RS puncturing in frequency domain in SBFD symbols, - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs. - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs. - FFS for the DUD configuration. - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions. - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold. - FFS the measurement behavior for DUD configuration. - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement. - For CSI-RS distributed over SBFD and non-SBFD symbols, - Ask RAN1 clarification on whether related RAN1 agreement applies for L1 measurement (L1-RSRP/SINR measurement and RLM/BFD/CBD measurement). Further discuss the wording when drafting the LS. #### Issue 1-2-3: Requirements for CSI-RS based L3 measurement - Agreements (from online session) - Companies to check the existing requirement, and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed. #### Issue 1-2-4: Requirements for scheduling restriction - Agreements (from Wed AH session) - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction. - Further discuss the impact to inter-frequency SSB measurement without gap case.

Additionally, the following topics have been captured in the topic summary [2]:

#### Issue 1-2-5: Requirements for RACH requirements - Proposals - Proposal 1 (LGE): - RAN4 need to update the correct behavior for the RACH occasion switching operation. And other RRM impact by RACH occasion switching operation also need to check. - Proposal 2 (vivo, E///, MTK): - No impact on RACH requirements due to SBFD operation. - Recommended WF - Discuss whether and how RACH occasion switching operation would impact RACH requirements. #### Issue 1-2-6: Requirements for UL PC and or spatial relation update - Proposals - Proposal 1 (E///): - RAN4 confirm there is no impact on requirements. - Proposal 2 (MTK): - RAN4 RRM should close this issue of UL PC and spatial relation update without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals. - Recommended WF - As baseline, there is no impact on requirements for UL power control and or spatial relation update due to SBFD operation. - This can be revisited if impacts are identified in the future based on further RAN1 agreements. #### Issue 1-2-7: Requirements for MG and BWP switch - Proposals - Proposal 1 (vivo, HW): - Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification, e.g. in MG and BWP switch requirements. - Proposal 2 (MTK): - There is no need to update the term UL slot for MG and BWP switch. - Recommended WF - Discuss whether to update the wording in the spec to include transmission in SBFD slots in addition to the UL slots, wherever applicable (e.g. in MG and BWP switch requirements). #### Issue 1-2-8: Requirements for UL resource muting - Proposals - Proposal 1 (LGE, CTC, vivo, E///): - RAN4 to confirm there is no RRM impact due to UL resource muting. - Proposal 2 (MTK): - RAN4 RRM should close this issue of UL resource muting without agreement on whether there is impact or not. If companies identify there is impact in the future, then companies can bring further proposals. - Recommended WF - As baseline, there is no RRM impact due to UL resource muting. - This can be revisited if impacts are identified in the future based on further RAN1 agreements. #### Issue 1-2-9: Requirements for generic SBFD operation - Proposals - Proposal 1 (MTK): - RAN4 RRM should close this issue of generic SBFD operation with no further agreement. - Recommended WF - The issue is covered by Issue 1-2-4. Close this issue without agreement.

Discussion

Requirements for CSI-RS based L1 measurement

In the last RAN4 meeting, an LS was sent to RAN1 about CSI-RS based measurements distributed over SBFD and non-SBFD symbols, and collision between CSI-RS measurement and dynamic UL transmissions [R4-2420165]. While RAN4 waits for the reply from RAN1 on these two aspects, the following other issues related to CSI-RS could be discussed in this RAN4#114 meeting as captured in the agreements from RAN4#113:

#### Issue 1-2-2: Requirements for CSI-RS based L1 measurement - Agreements (from online session) - For CSI-RS puncturing in frequency domain in SBFD symbols, - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs. - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs. - FFS for the DUD configuration. - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions. - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold. - FFS the measurement behavior for DUD configuration. - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement. #### Issue 1-2-3: Requirements for CSI-RS based L3 measurement - Agreements (from online session) - Companies to check the existing requirement, and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed.

CSI-RS puncturing in frequency domain in SBFD symbols

RAN4 has agreed that existing accuracy requirements for CSI-RS based measurements can be reused for DU operation in case the number of consecutive CSI-RS PRBs is no less than the existing threshold.

For DUD operation, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, there is FFS whether existing accuracy requirements and related additional side conditions for CSI-RS based measurements can apply. One of the main concerns is that splitting the RBs in the two subbands for DUD configurationcould potentially lead to worse accuracy performance than having all 48 RBs contained in one of the subbands. This includes two cases:

  - the number of consecutive CSI-RS PRBs in one DL subband is no less than the existing threshold, while the number of consecutive CSI-RS PRBs in the other DL subband is less than the existing threshold, e.g. 48PRB + 8 PRBs

  - the number of consecutive CSI-RS PRBs in either DL subband is less than the existing threshold, e.g. 40PRB + 8 PRB

Moreover, it was also raised that worse accuracy could be obtained when measuring the 40+8 RBs across the two subbands, as compared to measuring only 40 RBs on one of the subbands. This could be a consequence of additional noise captured by the UE DL receiver in case the UL subband is included as part of the measurement bandwidth.

To address these concerns, we think that RAN4 needs to specify additional conditions for which existing CSI-RS accuracy requirements should apply in case of SBFD DUD operation, for instance:

- In the simplest case, existing accuracy requirements shall apply if all the CSI-RS RBs are contained in one of the sub bands and the number of consecutive CSI-RS RB is no less than the existing threshold.

- Second, existing requirement should also apply if the CSI-RS RBs are split across the two DL subbands, and the number of consecutive RBs in at least one of the DL subbands exceeds the current threshold.

  - In other words, distributing the PRBs as 48PRB + 8 PRBs should not bring worse performance than the case with 48PRB + 0 PRBs.

- If the CSI-RS RBs are split across the two DL subbands and neither of the two DL subbands contain the minimum required number of consecutive CSI-RS RBs, RAN4 may agree on a new threshold or condition on the minimum number of CSI-RS RBs that shall be present in each DL subband. For instance, it can be specified that the total number of CSI-RBs (across both DL subbands) shall be no less than 48, while the number of contiguous CSI-RS RBs in each DL subband should be no less than 20.

On the accuracy requirements for CSI-RS based L1 measurements, RAN4 should specify one or more conditions for which existing CSI-RS accuracy requirements should apply in case of SBFD DUD operation. Consider one or more of the following options:

  - Existing accuracy requirements shall apply if all the CSI-RS RBs are contained in one of the DL sub bands and the number of consecutive CSI-RS RB is no less than the existing threshold.

  - Existing requirement should also apply if the CSI-RS RBs are split across the two DL subbands, and the number of consecutive RBs in at least one of the DL subbands exceeds the current threshold.

  - If the CSI-RS RBs are split across the two DL subbands and neither of the two DL subbands contain the minimum required number of consecutive CSI-RS RBs, a new threshold can be introduced on the minimum number of CSI-RS RBs that shall be present in each DL subband.

- In case the new specified conditions are not fulfilled, a compensation factor can be defined to relax existing accuracy requirements.

In the previous RAN4 meeting, the applicability of the CSI-RS based L1 measurement requirements has been discussed and captured in the topic summary [2].. There were divergent views regarding whether the CSI-RS for L1-RSRP/ L1-SINR and the CSI-RS for RLM/BFD/CBD should be restricted to one symbol type with occasions on both symbol types, so an LS was sent to RAN1 about this topic[4].

The RAN4 measurement requirements for CSI-RS L1-RSRP and L1-SINR, and for CSI-RS based RLM/BFD/CBD are based on the periodicity of the CSI-RS resources. If these measurements are restricted to one symbol type, the measurement requirements need to be updated accordingly.

In RAN1 #118bis it was agreed:

Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Our view is that the same principal from the agreement above shall apply at least for L1-RSRP and L1-SINR as these are one type of CSI report.

The same principle, that only CSI-RS transmission occasions within one valid symbol type are used for CSI derivation, shall apply to at least CSI-RS based L1-RSRP and L1-SINR as these are one type of CSI report.

Currently, the L1-RSRP and L1-SINR measurement period is derived based on the periodicity of CSI-RS resource. If only the CSI-RS transmission occasions that overlap with a given symbol type are used to measure the L1-RSRP or the L1-SINR, the measurement period needs to be discussed.

RAN4 to discuss the CSI-RS based L1-RSRP and L1-SINR measurement period depending on the valid symbol type (SBFD or non-SBFD symbols)

We consider that RLM/BFD/CBD are different from L1-RSRP and L1-SINR measurements, so we wait for RAN1 guidance on that aspect.

Requirements for CSI-RS based L3 measurement

The problem of CSI-RS being punctured in SBFD symbols is also relevant for CSI-RS L3 measurements. For instance, we expect that CSI-RS that is transmitted from a neighbour cell for the purposes of inter-cell mobility would need to be punctured in case the CSI-RS resources are located in a SBFD symbol (of the neighbour cell) and overlap with the UL sub band (of the neighbour cell). This was discussed by RAN2 in RAN2#128 where the following agreement was reached:

- RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

In order to avoid blocking RAN2’s progress, we think it is important that this issue is discussed in RAN4#114. The UE is already aware of the CSI-RS configuration of the neighbour cells from the MO configuration. We see that there are different possibilities regarding on how the UE derives the CSI-RS valid resources from the neighbour cells:

- The UE has information about the neighbour cell SBFD configuration

- The UE assumes that the neighbour cell valid CSI-RS resources are the same as the serving cell.  There are three possible cases:

  - The neighbour cell SBFD configuration is the same as the serving cell.

  - The neighbour cell SBFD configuration is different from the serving cell,

  - The neighbour cell is not configured with SBFD.

In our view, in order to measure CSI-RS from neighbor cell, the UE would need to be aware of the SBFD configuration from the neighbour cell, e.g. at least whether the neighbouring cell operates in SBFD mode and the potential UL-DL RB split in SBFD symbols. Without being aware of the neighbour cell SBFD configuration, in case the SBFD configuration is different among the neighbour cells, as in cases 2b and 2c above, the UE might puncture incorrectly the CSI-RS for L3 measurements, impacting the measurement performance.

Observation 2: Without being aware of the SBFD configuration of the neighbor cell, the UE cannot correctly derive the valid neighbor cell CSI-RS resources, which might impact the measurement performance.

To avoid blocking the progress of other working groups on this issue, we think RAN4 should discuss requirements for CSI-RS based L3 measurement under the assumption that the UE is aware or provided information of the SBFD configuration of neighbouring cells. RAN4 may inform RAN2 on this assumption which may then trigger RAN2 to discuss the signalling details.

For L3 CSI-RS mobility measurements, RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighbouring cells.

Requirements for scheduling restriction

- Agreements (from Wed AH session) - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction. - Further discuss the impact to inter-frequency SSB measurement without gap case.

DL/UL transition

During the RAN4#113 meeting, it has been agreed to check RAN1 agreements and potential impact to RAN4 scheduling restrictions due to DL/UL transition.

In RAN1 #116, there was an agreement regarding the collisions between receptions in DL subband, and transmissions in UL subband captured below:

Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

According to the scheduling availability requirements in TS 38.133, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured and on K data symbols before and after each consecutive SSB symbols to be measured within SMTC window duration. In both FR1 and FR2-1, K equals to 1 or 2 (60 kHz SCS).

The Tx/Rx transition time is specified in TS 38.211 as summarized below. RAN1 has not had any discussions regarding transition time in the current Rel-19 work item, so we assume that these existing requirements should also apply for SBFD operation.

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3. A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3. … Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx Transition time; FR1; FR2. NTx-Rx; 25600; 13792. NRx-Tx; 25600; 13792.

Tc is approximately 5.09 ns, so the transition time in FR1 is approximately 13 µs, and 7 µs in FR2. The duration of 1 OFDM symbol for different subcarrier spacings is given in the table below. Note that in all cases, the duration of an K OFDM symbos are longer than the transition time.

| Subcarrier spacing [kHz] | OFDM symbol duration [µs] |
| --- | --- |
| 15 | 66.73 |
| 30 | 33.37 |
| 60 | 16.68 |
| 120 | 8.35 |

In FR1 and FR2-1, the Tx-Rx transition time according to TS 38.211 is shorter than the duration of an OFDM symbol, which is the margin around SSB symbols in which the UE is not expected to transmit in UL.

The existing RAN4 scheduling restriction requirements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions.

Inter-frequency SSB measurement without gap

In the last RAN4 meeting, it has been discussed if new scheduling restrictions are needed for the case in which the SBFD UE is capable of performing inter-frequency measurements without gaps and is configured to do so. The proposal discussed in RAN4 #113 was to define DL scheduling restrictions for the case in which the neighbor cell SSBs are at least partially overlapping with the UL subband of the serving cell as in the example in the Figure below:

Figure 1 – Neighbour Cell SSBs partially overlapping with Serving Cell UL subband.

Some of the conditions for an inter-frequency measurement without gap are copied below from TS 38.133:

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE. - For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.9.3. - Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15].

Additionally, it is important to discuss the impact of some relevant RAN1 agreements and working assumptions:

Working Assumption (RAN1#118b) For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

The working assumption is that for SSBs symbols configured with SBFD subbands, the SSB is assumed to be within the DL subband. For the SBFD aware UE, only DL receptions within DL usable PRBs are allowed. Our understanding is that the working assumption applies to Serving cells configured with SBFD. The scenario we are discussing is the one in which the neighbor cell either does not support SBFD or has a different SBFD configuration from the serving cell.

Our understanding of the RAN1 working assumption is that the serving cell SSB block is assumed to be within DL subband. The expected behavior for SBFD aware UEs that “only DL receptions within usable PRBs are allowed” only applies to signals/ channels from the serving cell.

In meeting RAN#117, it has been agreed that:

Agreement (RAN1#117) Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

SSB measurements are prioritized over configured UL transmission and dynamically scheduled UL transmission as per agreement in RAN1#117.

If a UE is configured to measure an SSB, let’s say the neighbor cell SSB, we assume that the downlink reception is prioritized over configured UL transmission as per RAN1 agreement copied above. So, while configured to perform inter-frequency measurements without gaps, there should be UL scheduling restrictions. In FR1, According to TS 38.133, there are already UL scheduling restrictions specified in clause 9.3.9.3.1. There are also DL scheduling restrictions specified if the subcarrier spacing from the SSB and the data are different. In FR2, there are already DL and UL scheduling restrictions in place.

Therefore, for the scenario in which the neighbor cell SSB overlaps with the UL subband, if the UE is capable of performing inter-frequency measurements without gaps, and the neighbor cell SSB is overlapping with the serving cell UL subband, the UE should still be able to receive DL signals from the serving cell within the DL subband, so there is no need to define new DL scheduling restrictions.

There is no need to define new DL scheduling restrictions for inter-frequency measurements without measurement gaps when the neighbour cell SSB or CSI-RS is overlapping with the UL subband, if all the conditions for supporting inter-frequency measurements without measurement gaps are met.

RACH requirements

In the last meeting, it has been proposed to update the RACH behavior due to SBFD operation [2]:

- Proposals - Proposal 1 (LGE): - RAN4 need to update the correct behavior for the RACH occasion switching operation. And other RRM impact by RACH occasion switching operation also need to check. - Proposal 2 (vivo, E///, MTK): - No impact on RACH requirements due to SBFD operation.

In TS 38.133, the correct behaviour when not receiving Random Access Response is:

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [7].

Currently, the correct UE behavior is to update the PRACH transmission power if the Random Access Response is not received. In the RAN2 #127bis[3], RAN2 has agreed that:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

According to the agreement above, the UE is allowed to switch to legacy RACH occasions. Therefore there will be two different behaviours depending on whether the configured number of times of RACH attempts in SBFD RACH occasions has been reached or not. Therefore, our view is that RAN4 migth need to update the correct behaviour when not receiving Random Access Response after a certain number of attempts in SBFD symbols, depending on how the agreement above is captured in RAN2 specification.  However, there is still one open aspect about the case when the UE select legacy ROs first. This issue has not been finalized yet in RAN2, so we propose to wait for the agreement in RAN2 before RAN4 works on updating the RACH requirements.

RAN4 to wait for RAN2 conclusions before updating the correct behavior when the UE does not receive the random access response in SBFD symbols.

Conclusion

The following Observations and Proposals were made:

Proposal 1: On the accuracy requirements for CSI-RS based L1 measurements, RAN4 should specify one or more conditions for which existing CSI-RS accuracy requirements should apply in case of SBFD DUD operation. Consider one or more of the following options:

a.	Existing accuracy requirements shall apply if all the CSI-RS RBs are contained in one of the DL sub bands and the number of consecutive CSI-RS RB is no less than the existing threshold.

c.	If the CSI-RS RBs are split across the two DL subbands and neither of the two DL subbands contain the minimum required number of consecutive CSI-RS RBs, a new threshold can be introduced on the minimum number of CSI-RS RBs that shall be present in each DL subband.

In case the new specified conditions are not fulfilled, a compensation factor can be defined to relax existing accuracy requirements.

Proposal 2: The same principle, that only CSI-RS transmission occasions within one valid symbol type are used for CSI derivation, shall apply to at least CSI-RS based L1-RSRP and L1-SINR as these are one type of CSI report.

Observation 1: Currently, the L1-RSRP and L1-SINR measurement period is derived based on the periodicity of CSI-RS resource. If only the CSI-RS transmission occasions that overlap with a given symbol type are used to measure the L1-RSRP or the L1-SINR, the measurement period needs to be discussed.

Proposal 3: RAN4 to discuss the CSI-RS based L1-RSRP and L1-SINR measurement period depending on the valid symbol type (SBFD or non-SBFD symbols)

Proposal 4: For L3 CSI-RS mobility measurements, RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighbouring cells.

Observation 2: In FR1 and FR2-1, the Tx-Rx transition time according to TS 38.211 is shorter than the duration of an OFDM symbol, which is the margin around SSB symbols in which the UE is not expected to transmit in UL.

Proposal 5: The existing RAN4 scheduling restriction requirements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions.

Observation 3: Our understanding of the RAN1 working assumption is that the serving cell SSB block is assumed to be within DL subband. The expected behavior for SBFD aware UEs that “only DL receptions within usable PRBs are allowed” only applies to signals/ channels from the serving cell.

Observation 4: SSB measurements are prioritized over configured UL transmission and dynamically scheduled UL transmission as per agreement in RAN1#117.

Proposal 6: There is no need to define new DL scheduling restrictions for inter-frequency measurements without measurement gaps when the neighbour cell SSB or CSI-RS is overlapping with the UL subband, if all the conditions for supporting inter-frequency measurements without measurement gaps are met.

Proposal 7: RAN4 to wait for RAN2 conclusions before updating the correct behavior when the UE does not receive the random access response in SBFD symbols.

- References

- R4-2420097 – WF on RRM requirements for NR_Duplex_evo, Huawei, HiSilicon, RAN4 #113

- R4-2418281 – Topic Summary for [113][223]NR_duplex_evo, Huawei, HiSilicon, RAN4 #113

- R2-2409212 – Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD, RAN2 #127bis

- R4-2420165 – LS on CSI-RS measurement with SBFD operation, RAN4 RAN4#113