3GPP TSG-RAN4 Meeting #114	R4-2500477

# Athens, Greece, 17th - 21st February, 2025

Agenda item:		7.10.4

Source:				Keysight Technologies UK Ltd

Title:					On updated granularity for propagation delay and Doppler shift

Document for:		Approval

# Introduction

As part of Rel-19, RAN4 working group has been working on how to specify the TE_emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT NTN in the FR1-NTN bands and the corresponding LTE bands.

During RAN4#113, the way forward described in [1] was approved including the following statement:

This contribution provides company views on the feedback required about the implementation feasibility to update the Doppler shift and propagation delay per slot.

# Discussion

Assuming a NGSO visibility window of 900 s (15 minutes), having a granularity of one slot (0.5ms) for the satellite orbit Doppler shift and propagation delay to emulate the channel model implies 1.8e6 entries in a file where each line contains not only the Doppler shitft and propagation delay but also the satellite ephemeris format either in position vector or orbital parameters format.

Observation 1: The size of the file to be handled by the test equipment (TE) for the appropriate signalling and the channel emulation of a dynamic NGSO satellite is too big when the granularity required in the orbit information is one slot, which might be challenging to the test system.

For a test equipment vendor, it would be much simpler and efficient to rely on any interpolation method able to get the satellite orbital information from data stored with a much lower sample rate (compared to 0.5ms). However, it is understood that the accuracy of such interpolation method might have an impact on the UE performance characterized with such interpolated channel and hence a simulation assumption/TE requirement on interpolation accuracy shall be defined when quantifying the NGSO demodulation requirements for dynamic conditions so all test equipment can provide equivalent results.

Additionally, there is no need that interpolated values matches with slots boundaries, as the error in the ephemeris is negligible.

Proposal 1: RAN4 not to rely on a slot granularity for the definition of the channel model representing the NGSO propagation model.

Proposal 2: RAN4 to define a simulation assumption/TE requirement on interpolation accuracy when quantifying the the NGSO demodulation requirements for dynamic conditions so all test equipment can provide equivalent results.

# Conclusion

This contribution provides company views on the feedback required about the implementation feasibility to update the Doppler shift and propagation delay per slot.

The following observations and conclusions were made in this contribution:

Observation 1: The size of the file to be handled by the test equipment (TE) for the appropriate signalling and the channel emulation of a dynamic NGSO satellite is too big when the granularity required in the orbit information is one slot, which might be challenging to the test system.

Proposal 1: RAN4 not to rely on a slot granularity for the definition of the channel model representing the NGSO propagation model.

Proposal 2: RAN4 to define a simulation assumption/TE requirement on interpolation accuracy when quantifying the the NGSO demodulation requirements for dynamic conditions so all test equipment can provide equivalent results.

# References

- R4-2419786, Way Forward for [113][322] NTN_testing_NGSO_channel_model, Samsung, 3GPP TSG-RAN WG4 Meeting #113, November 2024