3GPP TSG-RAN4 Meeting #114	        R4-2500496 Athens, Greece, 17th February – 21st March 2025

Source: 	Qualcomm Inc.

Title: 	7 MHz Channel Bandwidth for Bands n26 and n5

Agenda Item:	7.9.2

Document for:	Discussion

WI Code:	NR_FR1_7MHz_BW-Core

# Introduction

RAN has agreed a work item for FR1 NR support to include 7 MHz channel bandwidth for bands n26 and n5 [1]. Part of the work item objectives for RAN4 is to specify system parameters. In this contribution system parameters for the dedicated spectrum are discussed and proposals for the spectrum utilization are made.

# Discussion

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4]

  - Introduce the channel bandwidth 7MHz

    - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements for UL and DL

- Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4]

  - BS and UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed:

    - MPR (relative bandwidth criteria)

    - Additional Maximum Power Reduction (A-MPR)

- Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4]

NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

The WI objectives which are relevant to RAN4 RF work are copied below.

From the WI objectives it can be observed that while the RAN4 requirements are intended to cover 7 MHz spectrum allocation in bands n26 and n5, the support is requested for 15 kHz SCS only.

Observation 1: Only one SCS is in WI scope of addition of a new 7 MHz channel bandwidth.

Proposal 1: Introduction of new 7 MHz channel bandwidth in bands n26 and n5 with SCS = 15 kHz.

## Spectrum Utilization

Maximum transmission bandwidth configuration for 7 MHz channel bandwidth is analyzed based on number of resource blocks (RBs) and the corresponding guard bands. There are two potential candidates, 36 RBs and 37 RBs which would be increasing spectrum utilization substantially. Minimum guard bands for 36 and 37 RBs would be 252.5 kHz and 162.5 kHz, respectively. Minimum guard band in kHz as well as percentage of channel bandwidth is illustrated in Figure 1.

Figure 1: Minimum Guard Band Trend with 15 kHz SCS over Channel Bandwidth

It can be seen that 37 RBs maximum transmission bandwidth configuration would be significantly more stringent than what is defined for other channel bandwidth. Using 37 RBs would set more stringent implementation requirements both for Tx, to meet emission requirement, and for Rx, to be able to tolerate blockers close to wanted signal.

Observation 2: 37 RBs spectrum utilization for 7 MHz channel bandwidth would set stricter requirements both for Tx and Rx compared to existing channel bandwidths.

For analyzing the impact on Tx performance, we simulated the impact on achievable output power. A power class 3 (PC3) transmitter was calibrated to just meet ACLR with DFT-s-OFDM waveform with Start_RB = 0 and L_CRB = 100 and MPR = 1 dB. IQ-image and carrier leakage of 28 dBc was used. Using these assumptions, we studied how much power backoff is needed to meet SEM and ACLR. SEM definition used is as shown in Table 1 and Fig.2. Whereas, ACLR is measured for both candidates 36, 37 RBs as shown in Fig. 3. Spurious emission mask definition is provided in Table 2.

Table 1: SEM used in Spectrum Utilization Study

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 7.0 MHz | Measurement bandwidth |
| ± 0-1 | -13 | 1% of Channel BW |
| ± 1 – 5 | -10 | 1 MHz |
| ± 5 – 7 | -13 |  |
| ± 7 – 12 | -25 |  |

Figure 2: Spectrum Emission Mask used for 7 MHz Channel Bandwidth

Figure 3: NR ACLR for 7 MHz Channel Bandwidth

Table 2: Spurious Emission Mask used for 7 MHz Channel Bandwidth

| Channel Bandwidth (MHz) | OOB boundary FOOB (MHz) |
| --- | --- |
| 7 | BWChannel + 5 |

The results were generated for 1 RB at the edge of the channel as well as fully allocated channel. Both DFT-s-OFDM and CP-OFDM waveforms were used. The results are summarized in Figs. 4 to 7. The black lines in the Figure illustrate the allowed MPR based on current inner/outer/edge RB allocation rules for NR PC3.

Figure 2: DFT-s-OFDM QPSK, Fully Allocated Channel

Figure 3: DFT-s-OFDM QPSK, 1 RB at the Edge of Channel

Figure 4: CP-OFDM QPSK, Fully Allocated Channel

Figure 5: CP-OFDM QPSK, 1 RB at the Edge of Channel

ACLR performance for fully allocated channel is quite similar for both waveform types irrespective of RBs. However, it can be seen from the plots above that significantly more power backoff is needed especially for edge RB allocation with 37 RBs to meet SEM. Therefore, it is strongly recommended to select 36 RBs as maximum transmission bandwidth configuration for 7 MHz channel bandwidth.

Observation 3: Significantly more power backoff is needed with 37 RBs spectrum allocation.

Proposal 2: Include 36 RBs for maximum transmission bandwidth configuration for 7 MHz channel bandwidth.

# Conclusions

In this contribution introduction of new channel bandwidth 7 MHz with 15 kHz SCS was discussed. Following observations and proposals were made:

Observation 1: Only one SCS is in WI scope for the addition of a new 7 MHz channel bandwidth.

Proposal 1: Introduction of new 7 MHz channel bandwidth in bands n26 and n5 for SCS = 15 kHz.

Observation 2: 37 RBs spectrum utilization for 7 MHz channel bandwidth would set stricter requirements both for Tx and Rx compared to existing channel bandwidths.

Observation 3: Significantly more power backoff is needed with 37 RBs spectrum allocation.

Proposal 2: Include 36 RBs for maximum transmission bandwidth configuration for 7 MHz channel bandwidth.

# References

[1]	RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5