3GPP TSG-RAN WG4 Meeting #114	R4-2500503

# Athens, Greece, 17th - 21th, Feb 2025

Title:	Discussion on UE RF requirements for LP-WUR

Source:	Samsung

Agenda item:	7.26.2.2

Document for:	Discussion

1.	Introduction

In previous RN4 meetings, the LP-WUS UE RF performance metric is confirmed, the baseline architecture was agreed for FR1 and initial consensus was also achieved for FR2. Based on the agreed performance metric and baseline architecture, we share our views on remaining open issues related with REFSENS and other receiver requirements for LP-WUR.

2. 	Discussion

2.1 Noise Figure

In last meeting there was extensive discussion on whether to define two sets of requirements for potential two sets of UE types, either OOK & OFDM, or moderate power consumption & ultra-low power consumption, and the agreement was as following [1]

Issue 2-2-1: General views on REFSENS for LP-WUS Agreements: - Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values.

The dominant factor of REFSENS is Noise Figure. The outcome of TR 38.869 records the noise figure values as following table. It can be seen that obvious higher NF value is assumed for LR than that of MR

Table 7.1.1a-3: Relative power consumption and noise figure for OOK-1/2/4 with homodyne/zero-IF architecture with baseband envelope detection

| Source reference | [7A-1] | [7A-2] | [7A-3] | [7A-4] | [7A-5] | [7A-6] | [7A-7] | [7A-8] | [7A-9] | [7A-10] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.09 for single-branch, 0.01 or 0.02 for each additional branch | 0.5 | 0.1~1 | 0.1 | 0.05~ 0.5 | 0.5~1 | 0.1~0.5 | 4 | ~1 | 0.1~0.5 |
| Noise figure (dB) | 15 | 10~15 | [10-16] | 15 | 12 | 15 | 12~15 | 15 | ~15 | 12 |

LNA is the key component for Noise Figure according to the cascaded equation for Noise Figure. If LNA is assumed for both OOK based receiver and OFDM based receiver, there is large possibility that one set of requirements is applicable. However, above table indicates that the absence of LNA is one potential implementation to enable ultra-low power consumption for LP-WUR.

Observation 1:	If LNA is assumed for both OOK based receiver and OFDM based receiver, there is large possibility that one set of requirements is applicable.

Observation 2:	TR outcome indicates that the absence of LNA is one potential implementation to enable ultra-low power consumption for LP-WUR.

It is true an OOK based LP-WUR can be equipped with LNA, but RAN4 minimum requirement should not preclude the implementation of ultra-low power consumption for LP-WUR without LNA. To enable the possible implementation of ultra-low power consumption for LP-WUR, it is proposed to assume LNA absence for OOK-based receiver or ultra-low power consumption LR.

Proposal 1:	RAN4 should discuss the LNA assumption for the two types of receivers. The absence of LNA should be assumed at least for OOK-based receiver or ultra-low power consumption LR

2.2 blocking and spurious response

In last meeting the impacted requirements have been discussed and the agreement is as following [1]

Issue 1-1-4: views on which requirements should be updated in core spec Agreements: - The part should be updated for LP-WUS/WUR in spec. - The additional requirements are not precluded, if needed. Sections or requirements; Needed or not?. Operation bands; FFS. UE channel bandwidth; FFS. Channel raster; FFS. REFSENS; YES. Maximum input level; YES. ACS; YES. ASCS; YES. In-band blocking; YES. Out-of-band blocking; YES. Narrow band blocking; YES. Spurious response; FFS. Wide band Intermodulation; FFS. Spurious emissions; FFS. Annex A Measurement channels; YES.

It is common understanding that out-of-band blocking and spurious response are two correlated requirements. Given out-of-band blocking is already agreed to be specified for LP-WUR, it is also necessary to specify spurious response altogether.

Proposal 2:	Specify both out-of-band blocking and spurious response requirements for FR1 LP-WUR.

Note that for FR2, there is only in-band blocking even for MR. The same principle can be used for LR.

Proposal 3:	No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR.

2.3 FR2 architecture assumption

The baseline architecture for FR2 OFDM based LP-WUR was discussed and agreed as following [1]

Issue 2-2-6: Baseline OFDM-based architecture for FR2 LP-WUR Agreements: - FR2 LPWUR adopts zero-IF topology baseline assumption. FFS details - Multiple frequency conversion in RF domain is not precluded - For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features: - It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized - FFS on whether and how to combine two Rx chains - Antenna gain is that of a single element - Coverage target should be considered

Though it is assumed that the FR2 LP-WUR comprises 2 Rx chains, it does not mean the 2 Rx chains would work simultaneously as 2 Rx chains will consume a lot of power. The 1T1R assumption for FR1 LP-WUR should also be applied for FR2, while the equipment of 2 Rx chains is to select the best polarization from the antenna element.

Proposal 4:	FR2 LP-WUR comprises 2 Rx chains but 1T1R assumption is still applicable. The equipment of 2 Rx chains is to select the best polarization from the antenna element.

3. 	Conclusion

Observation 1:	If LNA is assumed for both OOK based receiver and OFDM based receiver, there is large possibility that one set of requirements is applicable.

Observation 2:	TR outcome indicates that the absence of LNA is one potential implementation to enable ultra-low power consumption for LP-WUR.

Proposal 1:	RAN4 should discuss the LNA assumption for the two types of receivers. The absence of LNA should be assumed at least for OOK-based receiver or ultra-low power consumption LR

Proposal 2:	Specify both out-of-band blocking and spurious response requirements for FR1 LP-WUR.

Proposal 3:	No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR.

Proposal 4:	FR2 LP-WUR comprises 2 Rx chains but 1T1R assumption is still applicable. The equipment of 2 Rx chains is to select the best polarization from the antenna element.

4. 	References

- R4-2420528    “WF on UE RF requirements for LP-WUS”, vivo

- TR 38.869