3GPP TSG-RAN WG4 Meeting # 114											                 R4-2500542

Athens, Greece, February 17 – 21, 2025

Agenda item:			7.28.3

Source:	Moderator (Nokia)

Title:	Topic summary for [113][228] NR_XR_Ph3

Document for:	Information

# Introduction

This summary includes the proposals and issues for discussion related to Rel-19 enhancements for extended reality for NR Phase 3, under thread [114][228] NR_XR_Ph3.

This summary is based on the contributions submitted to the agenda items:

- 7.28  XR for NR Phase 3

  - 7.28.1  General aspects

  - 7.28.2  RRM core requirements

  - 7.28.3  Moderator summary and conclusions

The issues for discussion are organized in topics which are as follows:

- Topic 1 Scenarios for measurement skipping

- Topic 2 Measurements with gaps

- Topic 3 Measurements without gaps

- Topic 4 XR skipping requirements impact

- Topic 5 Time offset for measurement skipping processing

- Topic 6 Need/feasibility of UE assistance information

- Topic 7 Additional signalling aspects

The following issues are proposed to be prioritized for discussion

- Issue 1-1: Deployment scenarios

- Issue 1-3: L3 Measurements without gaps with scheduling restrictions

- Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Issue 1-5: L1/L2 triggered mobility measurements and procedures

- Issue 4-3: Formulas for measurement delay extension

- Issue 6-1: General on UAI

- Issue 4-1: How to capture impact of measurement skipping

- Issue 7-1: Semi-static schemes

- Issue 7-2: Skipping indication on frequency domain

- Issue 3-1: Interruptions with measurements without gaps

- Issue 3-2: Applicability of SMTC for measurements without gaps

- Issue 5-3: Time offset requirement for skipping of measurements without gaps

The latest work item description can be found at:

- RP-243318, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

Previous way forwards for information

- R4-2414027, Workplan for Rel-19 XR phase 3, Nokia

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia

- R4-2416864, WF on RRM requirements for XR_Ph3 , Nokia

- R4-2420103, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, To: RAN1, cc: RAN2, Source: Qualcomm

# Topic #1: RRM requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2501611 | Ericsson | Draft LS response on UE assistance information 7.28.1 Moderator: Contents of the LS draft are considered in multiple issues. |
| R4-2502241 | Nokia | General aspects for XR enhancements 7.28.1 Deployments scenarios for XR enhancements - RAN4 to not apply measurement skipping for FR2-2 measurements. RAN4 to consider for XR enhancements only NR-DC scenario with MCG and SCG in different FRs using per-FR gaps. CA scheduling restrictions RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. |
| R4-2500216 | InterDigital France R&D, SAS | NR_XR_Ph3 RRM core requirements analysis 7.28.2 Proposal 1: If NR-DC gap skipping is to be supported, semi-static indication/UE assistance combined with DCI dynamic indications solutions are required. Proposal 2: Support Option 2a: Measurement skipping does not apply for measurements without gaps. Proposal 3: Support Option 2a: Measurement skipping does not apply for L1 measurements and procedures. Proposal 4: Support Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM. Proposal 5: Support Type 1 as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS. Proposal 6: Support Option 2a: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information. Proposal 7: Support Option 2a: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. Proposal 8: If UAI mechanism is agreed, the rules for UAI gap skipping shall be left for UE implementation. Proposal 9: Support Option 1: Follow RAN1 working assumption and consider DCI only. Proposal 10: Support Proposal 1 from the above listed options: Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. |
| R4-2500595 | NTT DOCOMO, INC. | Discussions on RRM core requirements for XR 7.28.2 Proposal 1: At least for 100MHz CBW band, support skipping of measurements for L3 measurements without gaps Proposal 2: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc Proposal 3: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap |
| R4-2500827 | vivo | Further discussion on RRM requirements for XR 7.28.2 Proposal 1: Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective. Proposal 2: Measurement skipping does not apply for L3 measurements without gaps. Proposal 3: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction. Proposal 4: Only L1 measurements for LTM within gaps are supported for gap skipping. Proposal 5: Both of the two cases, i.e., there is impact to measurements and no impact to measurements due to gap skipping, are supported. Proposal 6: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements. Proposal 7: When there is impact to measurements due to gap skipping, RRM measurement requirements are defined by using NR-U like approach with carefully deciding the max number of gaps skipped during the measurement period. Proposal 8: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. Proposal 9: RAN4 to study possible restrictions on dynamic indication by DCI, e.g., the gaps to be skipped during a time period is indicated by RRC in advance, to achieve good trade-off between data deliver delay and mobility performance. Proposal 10: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information. Proposal 11: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement. Proposal 12: UE to indicate whether it can meet L3 measurements requirements when unused gaps reported by UE is used for data scheduling. Proposal 13: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured. Proposal 14: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information. Proposal 15: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Additional information about gaps/SMTCs not used for measurement is still needed. Proposal 16: RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported. Proposal 17: RAN4 LS to RAN1/2 the agreements on introduction of RRC-based indication, and additionally if restriction skipping for L3 measurement outside gap and/or L1 measurement is not supported from RAN4 perspective. Proposal 18: RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication. |
| R4-2501223 | Huawei, HiSilicon | Discussion on RRM requirements for Rel-19 XR 7.28.2 Proposal 1: Deprioritize FR2-2 and NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements Proposal 2: RAN4 to only consider Tx/Rx in occasions of L3 measurement. Proposal 3: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction. Proposal 4: 3 ms optional capability only apply when UE is only configured with NR measurement. Proposal 5: RAN4 to consider following approach to move forward: - RAN4 to introduce semi-static indication to skip measurement gap. - RAN4 discuss whether UE is allowed to only support semi-static indication. Proposal 6: To enable different UE implementations, RAN4 to introduce UAI with different measurement requirements, and it is up to UE to report one of / or both UAI: - UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply. - UAI about the numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly. Proposal 7: For the case where there is no measurement impact, the conditions are as follows: - UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and - The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI. For case when the measurement delay is extended. Proposal 8: When the measurement delay is extended, consider NR-U like approach to extend the delays by the number of MG occasions until enough valid MG occasions are available. And RAN4 to define side conditions when too much MG occasions are cancelled (e.g. no requirements). Proposal 9: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. |
| R4-2500913 | ZTE Corporation, Sanechips | Discussion on RRM aspects of R19 XR 7.28.2 Proposal 1: For NR-DC, apply aligned principle as EN/NE-DC, i.e. not support DCI-based measurement gap skipping for NR-DC. Proposal 2: To confirm the agreement around the SRVCC measurement achieved in last meeting: the measurement gap skipping is not supported as long as the measurement for SRVCC is configured for the UE. Proposal 3: For L3 measurement without gap, to move forward, the following two solutions could be considered: - Solution 1: Consider the condition of low mobility criterion. The measurement skipping indication is applicable when UE is in low mobility status. - Solution 2: Measurement gap skipping indication is applicable to the measurement without gap as well as the measurement with gap. - Not to introduce additional restriction skipping indication. Proposal 4: For L1 measurement, not support measurement skipping. Proposal 5: For L1 measurement for mobility, apply same decision as normal L1 measurement. Proposal 6: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. Proposal 7: Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered. Proposal 8: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. Proposal 9: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. Proposal 10: No need to define additional condition for the optional capability of 3 ms. Proposal 11: No matter whether allow the UAI mechanism, it is up to NW to decide whether and which gap/restriction can be skipped. Proposal 12: UAI is beneficial provided the semi-static measurement gap skipping indication is allowed. |
| R4-2500333 | China Telecom | RRM requirements on XR enhancements 7.28.2 Proposal 1: To keep RAN4 workload reasonable, measurement skipping does not apply for L3 measurements without gaps with scheduling restrictions. Proposal 2: Measurement skipping does not apply for L1 measurements and procedures. Proposal 3: For L1 measurement for mobility, apply same decision as normal L1 measurement. Proposal 4: On how to capture impact of measurement skipping: - If UAI is to be introduced and applied, the measurement delay should not be extended. - If UAI is not to be introduced in RAN4 or is not applied, the cell identification and measurement delay is extended by number of occasions which are cancelled due to XR traffic. Proposal 5: On the principle for measurement delay calculation, we prefer option 2 to follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. |
| R4-2501612 | Ericsson | On RRM requirements for XR 7.28.2 Proposal 1: FR2-2 is not considered in this WI. Proposal 2: MGs associated with PRS (positioning) measurements shall not be cancelled, e.g., because positioning can be used for emergency and also because these measurements are handled by the core network node (LMF). Proposal 3: MUSIM MGs shall not be cancelled. Proposal 4: Network controlled pre-configured MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs. Proposal 5: Concurrent MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs. Proposal 6: FFS the impact of the following issues for UL XR operation and their relation to the cancelled MGs: - Issue 1: Ensuring that the UE has the correct UL timing prior to cancelling MGs, - Issue 2: Guard period for DL-to-UL switching, - Issue 3: Impact of RF switching from DL to UL and back. Proposal 7: RAN4 will not specify any UE requirements dependent on XR UAI (if such is to be defined). Proposal 8: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133. Proposal 9: Inform RAN1 about potential ambiguity in patterns or number of measurement occasions indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle. Proposal 10: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because: - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided. - UE is not expected to monitor control channel beyond DRX on (and changing this assumption for XR may not be justified), - UE can be configured with more than one DRX configurations, - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations. Proposal 11: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation. Proposal 12: RAN4 to choose between the following enhancement methods to the legacy RRM measurement period requirements to account for the UE XR operation: - pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor), - dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs. Proposal 13: RAN4 to discuss MG configurations relevant for MG cancellation, e.g., - minimum MGL, e.g., MGL≥3 ms, - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays. Proposal 14: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined. |
| R4-2502085 | MediaTek inc. | Discussion on RRM requirements for XR 7.28.2 Proposal 1: RAN4 should de-prioritize FR2-2 and NR-DC. Proposal 2: RAN4 should deprioritize the scheduling restriction impact on XR traffic for both L1 and L3 measurements. Proposal 3: Only L1 measurements for LTM within gaps are supported for gap skipping. Proposal 4: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type. Proposal 5: RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic. Proposal 6: RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types. Proposal 7: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic. Proposal 8: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. Proposal 9: RAN4 shall introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window. Proposal 10: RAN4 shall discuss and define the max number of occasions cancelled during a measurement period. Proposal 11: RAN4 shall try to down-select the potential solution using UAI. Proposal 12: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period. |
| R4-2500460 | Apple | Discussion of RRM impact of XR for NR Phase 3 7.28.2 Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario. Proposal 2: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap. Introduce an optional UE capability to support skipping of measurements without gaps. Proposal 3: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. Introduce an optional UE capability to support skipping of RLM and BFD. Proposal 4: priorities of each gap type: Cap types; First prio; Second prio; Don’t include. Type 1; √. Type 2; √. NCSG; √. MUSIM; √. Positioning; √. Rel 18 concurrent gap; √. PreConfigured; √. Proposal 5: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. Proposal 6: max number of occasions cancelled during a measurement period shall be defined. Proposal 7: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation. |
| R4-2501086 | CMCC | Discussion on XR (eXtended Reality) for NR Phase 3 7.28.2 Proposal 1: it is proposed that measurement skipping apply for measurements without gaps. Proposal 2: it is proposed that measurement skipping apply for L1 measurements and procedures. Proposal 3: it is proposed to - Apply legacy measurement delay for the following condition: - the skipped measurement occasion(s) indicated by network is the unused measurement occasion - Otherwise, extend measurement delay Proposal 4: for time offset, there is no need to have condition for the optional capability of 3ms. Proposal 5: it is proposed to introduce UE assistance information related to measurement occasions, e.g. the number or location of needed measurement occasions for measurement (or the number or location of measurement occasions which can be skipped). Proposal 6: for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes. |
| R4-2502150 | Nokia | Discussion on XR ph3 RRM requirements 7.28.2 Proposal 1: Measurement skipping does apply for measurements without gaps for the following scenarios: a. Intra-frequency without gap for SSB within active BWP in FR2-1 b. For UE supporting NeedForInterruptions-r18 Proposal 2: For the requirements of measurements without gaps, reuse agreement on time offset requirement for measurement gap skipping Proposal 3: If interruptions are needed for measurements without gaps, skipping indication cancel the interruptions as well as scheduling restrictions. Proposal 4: DCI skipping command applies for an entire SMTC. Proposal 5: Measurement skipping does not apply for L1 measurements and L1 procedures. Proposal 6: Measurement skipping does not apply for L1 measurements and LTM procedures. Proposal 7: Discuss first the requirements without UAI consideration. Proposal 8: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as a. Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), where Nskip is the number of skipped MG occasions in the measurement period. b. Use this formula as example and follow similar approach for the other cases. Proposal 9: RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority. Proposal 10: High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is a. Nskip ≤8 × (CSSF – Nhigh) for inter-frequency measurements and Nskip ≤ 5 × (CSSF – Nhigh) for intra-frequency measurements b. where Nhigh is the number of high priority layers. Proposal 11: No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration. Proposal 12: RAN4 to consider mobility impacts due to measurement skipping and to start with the handover procedure. Proposal 13: RAN4 to discuss how to mitigate the impact of measurement delay extension in mobility events. Proposal 14: No condition is needed for the optional capability of 3 ms. Proposal 15: Do not define UAI related to measurement skipping in Rel 19. Proposal 16: Follow RAN1 working assumption and consider DCI scheme only. |
| R4-2502229 | Qualcomm Incorporated | RRM requirements for XR enhancements 7.28.2 Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Proposal 2: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI atleast X ms before the MG occasion, where - X=5ms as baseline and X=3ms as optional UE capability. - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Proposal 3: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L1 measurements. Proposal 4: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L3 measurements without gaps. Proposal 5: For DCI based dynamic indication, RAN4 to extend the L3 delays by the number of MGs deactivated during the period. |

## Open issues summary

### Topic 1 Scenarios for measurement skipping

#### Issue 1-1: Deployment scenarios

- Background information: In RAN4 #112 bis the following agreement regarding deployment scenarios was reached R4-2416864:

< Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation

- Background information: In RAN4 #113 the following agreement regarding deployment scenarios was reached R4-2420103:

< Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID <Way forward>: - Further discussion needed for - FR2-2 - NR-DC

- Proposals related to frequency range

  - Option 1 (Huawei, Mediatek): Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

  - Option 2 (Nokia, Ericsson): FR2-2 is not considered in this WI.

- Proposals related to dual connectivity

  - Options that don’t allow NR-DC

    - Option 1 (Huawei, Mediatek): Deprioritize NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

    - Option 2 (ZTE): Not support for DCI-based measurement gap skipping for NR-DC.

  - Options supporting NR-DC

    - Option 3 (Nokia): RAN4 to consider for XR enhancements only NR-DC scenario with MCG and SCG in different FRs using per-FR gaps.

    - Option 4 (Interdigital): If NR-DC gap skipping is to be supported, semi-static indication/UE assistance combined with DCI dynamic indications solutions are required.

  - Ask other working groups

    - Option 3 (vivo): Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective.

- Recommended WF

  - Proposals on FR2-2

    - Try to agree on Option 1 or 2 frequency range

  - Proposals on NR-DC

    - Please discuss the proposals

#### Issue 1-2: Types of measurements to be considered

- Background information: In RAN4 #113 the following agreement regarding deployment scenarios was reached R4-2420103:

## Issue 1-2: Types of measurements to be considered <Agreement>: - The following measurement types are considered for XR measurement skipping - Intra-frequency - Inter-frequency - LTE-inter- RAT (NR-EUTRAN FDD/TDD) - The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS.

- Proposals regarding which types of measurements are considered for XR enhancements

  - Proposal 1 (ZTE): To confirm the agreement around the SRVCC measurement achieved in last meeting: the measurement gap skipping is not supported as long as the measurement for SRVCC is configured for the UE.

- Recommended WF

  - ZTE concern is on clarification of SRVCC not supported. Discussion needed

#### Issue 1-3: L3 Measurements without gaps with scheduling restrictions

- Proposals from companies

  - Proposal 1 (CMCC): measurement skipping apply for measurements without gaps.

  - Proposal 2 (NTT DOCOMO): At least for 100MHz CBW band, support skipping of measurements for L3 measurements without gaps.

  - Proposal 3 (ZTE): Consider the condition of low mobility criterion. The measurement skipping indication is applicable when UE is in low mobility status.

  - Proposal 4 (ZTE): Measurement gap skipping indication is applicable to the measurement without gap as well as the measurement with gap. Not to introduce additional restriction skipping indication.

  - Proposal 5 (Apple): Support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap. Introduce an optional UE capability to support skipping of measurements without gaps.

  - Proposal 6 (Nokia): Measurement skipping does apply for measurements without gaps for the following scenarios:

    - Intra-frequency without gap for SSB within active BWP in FR2-1

    - For UE supporting NeedForInterruptions-r18

  - Proposal 7 (Interdigital, vivo, China Telecom, Qualcomm): Measurement skipping does not apply for measurements without gaps

  - Proposal 8 (Huawei, Mediatek): Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

- Merged proposals from moderator

  - Option 1: measurement skipping apply for measurements without gaps, considering the following sub-options:

    - Option 1a: no limitation on scenarios for skipping of measurements without gaps

    - Option 1b: Skipping of measurements without gaps depend on UE capability.

    - Option 1c: Apply for 100 MHz BW

    - Option 1d: consider low mobility criteria.

    - Option 1e: Skipping applies for the following scenarios

      - Intra-frequency without gap for SSB within active BWP in FR2-1

      - For UE supporting NeedForInterruptions-r18

  - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported

    - Option 2a: Not applicable

    - Option 2b: Deprioritize discussion.

- Recommended WF

  - Please consider the options.

#### Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Proposals form companies

  - Option 1: Support skipping of L1 measurements and procedures.

    - Option 1a (CMCC): measurement skipping apply for L1 measurements and procedures.

    - Option 1b (Apple): it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. Introduce an optional UE capability to support skipping of RLM and BFD.

    - Option 1c (NTT DOCOMO): At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

  - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures.

    - Option 2a (Interdigital, ZTE, China Telecom, Huawei?, Qualcomm, Nokia): Measurement skipping does not apply for L1 measurements and procedures

    - Option 2b (vivo): RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

    - Option 2c (Mediatek): RAN4 should deprioritize the scheduling restriction impact on XR traffic for both L1 and L3 measurements.

- Merged proposals form the moderator

  - Option 1: Support skipping of L1 measurements and procedures.

    - Option 1a: apply for all L1 measurements and procedures.

    - Option 1b: apply scheduling restriction due to RLM and BFD.

    - Option 1c: Apply at least for 100MHz CBW band

  - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures.

    - Option 2a: not apply

    - Option 2b: deprioritize the discussion.

- Recommended WF

  - Discuss based on moderator merged options

#### Issue 1-5: L1/L2 triggered mobility measurements and procedures

- Proposals from companies

  - Proposal 1 (NTT DOCOMO): At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap

  - Proposal 2 (Interdigital, Huawei, Nokia): measurement skipping do not apply for L1 measurements and procedures for LTM

  - Proposal 3 (vivo, Mediatek): Only L1 measurements for LTM within gaps are supported for gap skipping.

  - Proposal 4 (ZTE, China Telecom): For L1 measurement for mobility, apply same decision as normal L1 measurement.

- Merged proposals form the moderator

  - Option 1: Support skipping of L1 measurements and procedures for LTM.

    - Option 1a: Apply at least for 100MHz CBW band

    - Option 1b: Apply only for measurement within gaps

  - Option 2: Not to support skipping of L1 measurements and procedures for LTM.

  - Option 3: reuse decision as for normal L1 measurements.

- Recommended WF

  - Discuss based on the merged proposals

### Topic 2 Measurements with gaps

#### Issue 2-1: General approach for defining requirements for measurements with gaps

- Proposals

  - Proposal 1 (Mediatek): RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

  - Proposal 2 (Mediatek): RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types.

- Recommended WF

  - Please check for proposal 1 if the current agreement is enough.

  - Discussion on Proposal 2 needed.

#### Issue 2-2: Types of measurement gaps

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Company proposals on the priorities for RAN4 to work are summarized using based on the following table

| Gap types | First priority | Second priority | Shall not be cancelled |
| --- | --- | --- | --- |
| Type 1 | Moderator: Type 1 is omitted since it is already agreed |  |  |
| Type 2 | Apple | Ericsson |  |
| NCSG | Apple | Ericsson |  |
| MUSIM |  | Apple | Ericsson |
| Positioning |  |  | Ericsson, Apple |
| Rel 18 concurrent gap | Apple |  |  |
| PreConfigured | Apple | Ericsson |  |

- Recommended WF

  - Recommended to further progress on other issues before coming back to gap types.

### Topic 3 Measurements without gaps

#### Issue 3-1: Interruptions with measurements without gaps

- Proposals

  - Proposal 1 (Nokia): If interruptions are needed for measurements without gaps, skipping indication cancel the interruptions as well as scheduling restrictions.

- Recommended WF

  - Please discuss the interruption requirements for measurements without gaps

#### Issue 3-2: Applicability of SMTC for measurements without gaps

- Proposals

  - Proposal 1 (Nokia): DCI skipping command applies for an entire SMTC.

- Recommended WF

  - Please discuss the proposal.

### Topic 4 XR skipping requirements impact

#### Issue 4-1: How to capture impact of measurement skipping

- Previous WF

<Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay <Way forward>: Proposals related to measurement delay requirements: - Option 1: Proposals without distinction of XR and legacy requirements - Option 1a: As minimum requirements, cell detection, index reading, and measurement period is extended. - Option 1b: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. - Option 1c: extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network. - Option 1d: RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR. - Option 2: Proposals with distinction of conditions where measurement impact is not needed - Option 2a: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information. - Option 2b: - Apply legacy measurement delay and accuracy requirements for the following condition: - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

- Proposals from the companies:

  - Proposal 1 (Interdigital):

    - For the cases where measurement impact is needed

      - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

        - FFS how to extend.

    - For the cases where measurement impact is not needed

      - FFS the feasible conditions and necessary UE assistance information.

  - Proposal 2 (China Telecom):

    - If UAI is to be introduced and applied, the measurement delay should not be extended.

    - If UAI is not to be introduced in RAN4 or is not applied, the cell identification and measurement delay is extended by number of occasions which are cancelled due to XR traffic.

  - Proposal 3 (CMCC): Apply legacy measurement delay for the following condition:

    - the skipped measurement occasion(s) indicated by network is the unused measurement occasion

    - Otherwise, extend measurement delay

  - Proposal 4 (Nokia): Discuss first the requirements without UAI consideration.

  - Proposal 5 (Nokia): RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

    - High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

      - Nskip ≤8 × (CSSF – Nhigh) for inter-frequency measurements and Nskip ≤ 5 × (CSSF – Nhigh) for intra-frequency measurements

      - where Nhigh is the number of high priority layers..

  - Proposal 6 (Ericsson): To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation.

  - Proposal 7 (Ericsson): RAN4 will not specify any UE requirements dependent on XR UAI (if such is to be defined).

  - Proposal 8 (Huawei): For the case where there is no measurement impact, the conditions are as follows:

    - UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and

    - The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI.

  - Proposal 9 (vivo): Both of the two cases, i.e., there is impact to measurements and no impact to measurements due to gap skipping, are supported.

  - Proposal 10 (vivo): When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

- Merged proposals from moderator

  - Option 1: Discuss first the requirements without UAI consideration.

Option 2: Apply legacy measurement delay for the following condition

    - If the measurement skipping does not exceed the limit/positions indicated by UAI report

  - Option 3: High priority layers don’t extend measurement delay if skipping ratio does not exceed a limit.

- Recommended WF

  - Discuss based on the merged options

#### Issue 4-2: Principles for measurement delay calculation

- Proposals

  - Proposals based on scaling factor

    - Option 1a (Mediatek): Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

      - Introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

    - Option 1b (Ericsson): pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor).

  - Proposals based on NR-U requirements

    - Option 2 (Interdigital, vivo, ZTE, China Telecom): Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

    - Option 3 (Huawei): When the measurement delay is extended, consider NR-U like approach to extend the delays by the number of MG occasions until enough valid MG occasions are available. And RAN4 to define side conditions when too much MG occasions are cancelled (e.g. no requirements).

    - Option 4 (Ericsson): dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

    - Option 5 (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

    - Option 6 (Qualcomm): For DCI based dynamic indication, RAN4 to extend the L3 delays by the number of MGs deactivated during the period.

- Recommended WF

  - Since there are many proposals that might have similar meaning, we suggest focus on the discussion based on Issue 4-3

#### Issue 4-3: Formulas for measurement delay extension

Moderator: In order to give more clarity on the proposals of Issue 2, it is proposed to discuss based on the formulas.

- Proposals

  - Options inspired by NR-U requirements

    - Option 1 (Nokia): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

      - Max(200 ms × CSSFinter, Ceil( 8  * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

      - where Nskip is the number of  skipped MG occasions in the measurement period.

      - Use this formula as example and follow similar approach for the other cases.

    - Option 2 (Moderator): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

      - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

      - where Nskip is the number of  skipped MG occasions in the measurement period.

      - Use this formula as example and follow similar approach for the other cases.

  - Options based on scaling factor like in concurrent gaps

    - Option 3 (Moderator): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

      - Max(200 ms, Ceil( 8  * Kgap * Kskip) × Max(MGRP , SMTC period)) × CSSFinter,

      - where Kskip = Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

      - Use this formula as example and follow similar approach for the other cases

- Recommended WF

  - Please discuss based on Option 1, 2, and 3

    - Option 2 is the moderator’s interpretation of proposals 2 to 6 in Issue 4-2.

    - Option 3 is the moderator’s interpretation of Proposal 1a in Issue 4-2

#### Issue 4-4: Conditions for measurement skipping

- Proposals

  - Option 1: conditions are needed

    - Option 1a (vivo, Mediatek, Apple): For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified.

    - Option 1b (Ericsson): The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133

    - Option 1c (Ericsson): Maximum acceptable time separation between two measurement occasions available for RRM measurements

  - Option 2 (ZTE, Nokia): No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

- Recommended WF

  - Discussion on the conditions for skipping is needed.

#### Issue 4-5: UL XR impact

- Proposals

  - Proposal 1 (Ericsson): FFS the impact of the following issues for UL XR operation and their relation to the cancelled MGs:

    - Issue 1: Ensuring that the UE has the correct UL timing prior to cancelling MGs,

    - Issue 2: Guard period for DL-to-UL switching,

    - Issue 3: Impact of RF switching from DL to UL and back.

- Recommended WF:

  - Discussion needed

#### Issue 4-6: DRX

- Proposal

  - Proposal 1 (Ericsson): DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

    - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

    - UE is not expected to monitor control channel beyond DRX on (and changing this assumption for XR may not be justified),

    - UE can be configured with more than one DRX configurations,

    - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

  - Recommended WF

    - Discuss the proposal

#### Issue 4-7: Measurement gap configurations relevant to skipping

- Options

  - Option 1 (Ericsson): discuss MG configurations relevant for MG cancellation, e.g.,

    - minimum MGL (e.g., MGL≥3 ms, due to low gain with MG skipping),

    - maximum MGL (e.g., MGL≤6 ms),

    - maximum MGRP (e.g., MGRP≤320 ms to avoid too long delays), etc.

- Recommended WF

  - Please discuss if there is need for limiting which gap configurations can be skipped.

### Topic 5 Time offset for measurement skipping processing

#### Issue 5-1: Time offset requirement for measurement gap skipping

- Background information: Agreement from RAN4 #113 (R4-2420103)

## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed

- Proposals

  - Option 1 (Huawei): 3 ms optional capability only apply when UE is only configured with NR measurement.

  - Option 2 (ZTE, CMCC, Nokia): No need to define additional condition for the optional capability of 3 ms

- Recommended WF

  - Discussion needed

#### Issue 5-2: Time offset requirement for cancelling skip indication

- Background information: RAN1 agreement

A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component.

- Proposals

  - Option 1 (Qualcomm): The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI atleast X ms before the MG occasion, where

    - X=5ms as baseline and X=3ms as optional UE capability.

    - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

- Recommended WF

  - Please consider if Option 1 can be agreed.

#### Issue 5-3: Time offset requirement for skipping of measurements without gaps

- Proposals

  - Option 1 (Nokia): For the requirements of measurements without gaps, reuse agreement on time offset requirement for measurement gap skipping

- Recommended WF

  - Pending on decision whether measurements without gaps are to be considered for measurement skipping.

### Topic 6 Need/feasibility of UE assistance information

Sub-topic description

RAN1 has sent an LS (R1-2405736) to RAN4, requesting RAN4 to decide whether to introduce any UE assistance information related to measurement occasions. This sub-topic includes issues relating to the need and feasibility of such UE assistance information.

Open issues and candidate options before meeting:

#### Issue 6-1: General on UAI

- Proposals form the companies

  - Proposals on general principles

    - Proposal 1 (Mediatek): RAN4 shall try to down-select the potential solution using UAI.

    - Proposal 2 (Ericsson): RAN4 will not specify any UE requirements dependent on XR UAI (if such is to be defined).

    - Proposal 3 (Ericsson draft LS): RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI, in case the latter is to be supported.

    - Proposal 4(ZTE): UAI is beneficial provided the semi-static measurement gap skipping indication is allowed.

  - Proposals related to measurement occasions

    - Proposal 1 (vivo): patterns of gaps occasions not used for measurement within a time period.

    - Proposal 2 (vivo): UE to indicate whether it can meet L3 measurements requirements when unused gaps reported by UE is used for data scheduling.

    - Proposal 3 (vivo): study how UE assistance information/UE capability is reported when DRX is configured.

    - Proposal 4 (Huawei): number of occasions which can be skipped when legacy measurement requirements can apply.

    - Proposal 5 (Huawei): numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly

    - Proposal 6 (Apple): UE indicates NW a status whether it can tolerate measurement cancellation.

    - Proposal 7 (CMCC): number of occasions that are needed for measurement

    - Proposal 8 (CMCC): position of occasions that are needed for measurement

    - Proposal 9 (CMCC): number of occasions that can be skipped

    - Proposal 10 (CMCC): position of occasions that can be skipped

    - Proposal 11(Nokia): Do not define UAI related to measurement occasions.

    - Proposal 12 (vivo): The information about gaps not used for measurement are reported as UE capability.

  - Proposals related to channel conditions

    - Proposal 1 (vivo) Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Additional information about gaps/SMTCs not used for measurement is still needed.

- Proposals merged by the moderator

  - Proposals on general principles

    - Proposal 1: RAN4 will not specify any UE requirements dependent on XR UAI (if such is to be defined).

    - Proposal 2: RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI, in case the latter is to be supported.

  - Proposals related to measurement occasions

    - Option 1: Do not define UAI related to measurement occasions.

    - Option 2: The UE indicates a pattern of unused gaps

    - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

    - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability.

  - Proposals related to channel conditions

    - Option 1: Provide information on channel conditions

    - Option 2: do not provide information on channel conditions

- Recommended WF

  - Discussion based on the options merged by the moderator.

#### Issue 6-2: Rules for UAI gap skipping

- Proposals

  - Proposal 1 (Interdigital): the rules for UAI gap skipping shall be left for UE implementation.

  - Proposal 2 (ZTE): It is up to NW to decide whether and which gap/restriction can be skipped.

- Recommended WF

  - Discuss this issue based on the outcome of 5-1.

#### Issue 6-3: UAI dependency on DRX

- Proposals

  - Proposal 1 (Ericsson): Inform RAN1 about potential ambiguity in patterns or number of measurement occasions indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle.

- Recommended WF

  - Discuss this issue based on the outcome of 5-1.

### Topic 7 Additional signalling aspects

#### Issue 7-1: Semi-static schemes

- Proposals form companies

  - Proposal 1 (Interdigital, Nokia): Follow RAN1 working assumption and consider DCI only.

  - Proposal 2 (vivo): RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported.

    - RAN4 LS to RAN1/2 the agreements on introduction of RRC-based indication, and additionally if restriction skipping for L3 measurement outside gap and/or L1 measurement is not supported from RAN4 perspective.

  - Proposal 3 (Huawei): RAN4 to consider following approach to move forward:

    - RAN4 to introduce semi-static indication to skip measurement gap.

    - RAN4 discuss whether UE is allowed to only support semi-static indication.

  - Proposal 4 (Mediatek): RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

  - Proposal 5 (Apple): In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

  - Proposal 6 (CMCC): for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes.

  - Proposal 7 (Qualcomm): In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well.

  - Proposal 8 (vivo): RAN4 to study possible restrictions on dynamic indication by DCI, e.g., the gaps to be skipped during a time period is indicated by RRC in advance, to achieve good trade-off between data deliver delay and mobility performance.

- Recommended WF

  - Further discussion based on recommended options

    - Option 1: Do not consider semi-static solutions

    - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution.

      - Option 2a: RRC pattern-based

      - Option 2b: MAC CE bitmap for skipping indication

      - Option 2c: RRC configuration patterns with MAC CE activation

    - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

#### Issue 7-2: Skipping indication on frequency domain

- Proposals

  - Option 1 (Interdigital, ZTE): For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

  - Proposal 2 (Nokia):

    - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

    - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

  - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

  - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

- Recommended WF

  - This issue has not been discussed yet in RAN4. Companies views is needed.

#### Issue 7-3: Network indication

- Proposals

  - Proposal 1 (vivo):

    - NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement.

- Recommended WF

  - Please discuss the proposal.

#### Issue 7-4: Partial measurement skipping

- Proposals

  - Proposal 1 (ZTE): Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered.

- Recommended WF

  - Please discuss the proposal.

#### Issue 7-5: Measurement delay mitigation

- Proposals

  - Proposal 1 (Nokia): RAN4 to consider mobility impacts due to measurement skipping and to start with the handover procedure.

  - Proposal 2 (Nokia): RAN4 to discuss how to mitigate the impact of measurement delay extension in mobility events.

- Recommended WF

  - Please discuss the proposal.