3GPP TSG-RAN WG4 Meeting #114	R4-2500545

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.30.4

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [114][231] IoT_NTN_Ph3

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for RRM requirements for R19 IoT (Internet of Things) NTN (non-terrestrial network) phase 3, including the following topics covered

- Topic#1: RRM core requirements (AI 7.30.3)

Recommended issues for online discussion:

- Issue 1-1-1

# Topic #1: RRM core requirements (AI 7.30.3)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500854 | CMCC | Proposal 1: Suspend the discussion of UL timing requirement until further relevant agreement from RAN2 is available. |
| R4-2501484 | Nokia | - Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 1-1: CB-msg3 EDT enhancement

#### Issue 1-1-1: Procedural requirements for CB-MSG3 transmission

Background:

- Some RAN2 agreement in RAN2#128 excerpted as below:

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

Proposals:

- Proposal 1 (Nokia): Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133.

Moderator’s Note:

- In R4-2501484 (Nokia), the following are suggested for 6.6A:

  - UE behavior when contention resolution timer expires: whether the UE will select a new preamble depends on the new behaviour to be defined in RAN2. Another problem is that the transmission of a new PRACH after a failed contention based is now dependent upon the expiration of the backoff timer. However, the backoff timer is never initiated as the UE does not monitor for Msg2.

  - Updates on the Correct behaviour when receiving a message over Temporary C-RNTI

  - Updates, if needed, on the PRACH transmission power when occurring as fallback from CB-MSG3 transmission.

Recommended WF:

- Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133.

  - FFS the content to include.

#### Issue 1-1-2: Impact on RRM requirements for CB-MSG3 transmission

Background:

- Agreed LS R4-2416918 in RAN4#112bis

  - For Note 2, RAN4 has already agreed that UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN respectively in [R4-2414114].

Proposals:

- Proposal 1 (CMCC): Suspend the discussion of UL timing requirement until further relevant agreement from RAN2 is available.

Moderator’s Note:

- In R4-2500854 (CMCC), the potential impact would be the RSRP threshold that determines whether CB-Msg3 can be used

  - RAN2 introduced a RSRP threshold that determines whether CB-Msg3 can be used. Then how UE derive the RSRP may need further discussion, for example, UE uses Srxlev to compare with RSRP threshold, or UE use the RSRP value derived by new delay requirement to compare with RSRP threshold. We can wait further clarification from RAN2 to see whether RAN4 need to define some new delay requirements.

Recommended WF:

- Suspend the discussion until any updates on RRM requirement are identified upon RAN2 relevant agreements for CB-MSG3 transmission.