# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500554

# Athens, Greece, 17th-21st Feb, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on testability of LP-WUR

# Agenda Item:   7.26.2.4

# Document for:  Approval

- Introduction

In last RAN4 meeting, RAN4 discussed testability for UE RF requirements and how to perform testing. The agreement is listed as follows. RAN4 agreed that only one RRC state needs to be tested, but which RRC state to choose hasn’t been decided. And whether the MR should be waken up during LR testing is still under discussion[1].

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing Agreements: - The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested. Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance Agreements: - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point: - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance. - FFS if the MR should only be woken up when data transfer the system simulator is needed. - Other options are not precluded.

- Discussion

According to the WF, RAN4 is going to determine at least a general framework, including test mode, for the test of LP-WUR. As we discussed in our previous paper [2], the test in IDLE mode requires MR to stay in a cycle of waken-up and sleeping, which may cause long test duration. While the test in CONNECTED mode is relatively easy and some legacy test mechanisms can be reused. Thus, as agreed in last meeting that only one RRC state is going to be tested, we prefer to test the LP-WUR in CONNECTED mode. The detailed test implementation can be left to RAN5.

Moreover, some companies proposed a new test mode that not to wake up the MR during LP-WUR test but only wake up the MR when data transfer (for example test data collection) is needed. This method avoids unnecessary MR activities during LP-WUR test and can save test time effectively. However, it requires a special mechanism to prevent the MR from waking up during the LP-WUR test and to wake up the MR in time when test procedure is finished, or the MR should be designed to wake up periodically to collect test data. Besides, transferring test data from LP-WUR to MR is needed. Another question is, whether this new test mode can be regarded as a special case of RRC IDLE state.

Therefore, if this new test mode is going to be used, the implementation issues should be considered. If this new test mode is possible in terms of implementation, we can also agree with it. Otherwise, we prefer to test the LP-WUR in CONNECTED mode.

Proposal 1: If the new test mode that does not need to wake up MR is possible in terms of implementation, we also agree with it. Otherwise, we prefer to test the LP-WUR in CONNECTED mode.

Proposal 2: The detailed test implementation can be left to RAN5.

- Conclusion

Proposal 1: If the new test mode that does not need to wake up MR is possible in terms of implementation, we also agree with it. Otherwise, we prefer to test the LP-WUR in CONNECTED mode.

Proposal 2: The detailed test implementation can be left to RAN5.

- Reference

- R4-2420394 WF on UE RF requirements for LP-WUS, vivo, RAN4#113, Orlando, USA, Nov 2024.

- R4-2418830 Discussion on testability of LP-WUR, ZTE, RAN4#113, Orlando, USA, Nov 2024.