# 3GPP TSG-RAN WG4 Meeting #114	R4-2500564

# Athens, GR, 17th – 21st February, 2025

Agenda item:			7.13.5

Source:	Moderator (Huawei)

Title:	Topic summary for [114][303] NR_BS_RF_Part2_CLTA

Document for:	Information

# Introduction

Focus of this thread is co-location test antenna (CLTA) topic of the NR_BS_RF_req_evo work item. CLTA-related objectives were extracted from the WID in RP-243270 for reference:

- Core:

  - Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition.

- Perf:

  - CLTA: Investigate whether the existing test approach can be improved, and if feasible, improve test methods for BS/IAB OTA co-location requirements and tests for AAS-based test specifications for FR1.

# Topic #1: CLTA

## Companies’ contributions summary

The following CLTA-related contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2501323 | Nokia | Observation 1: Increasing the averaging to 200 samples does not bring significant benefit. Observation 2: It would be beneficial to have predefined and tunable spurs to verify test antennas performance. Observation 3: Doubling the measurement distance to 20cm doesn’t guarantee lower results. Observation 4: Beam steering is a technology of paramount importance for AAS. Observation 5: Radio transmission and reception specifications together with conducted and radiated conformance testing specifications for BS, IAB and repeaters could be impacted. Observation 6: TX spurious emissions, RX blocking, TX IMD and TDD OFF power requirements could be impacted. Observation 7: Conducted requirement limits might change depending on the coupling level. Observation 8: OTA requirement limits would not change. Observation 9: If results indicate a larger coupling loss for higher frequencies, then the distance between CLTA and DUT could be increased to reflect higher coupling loss. Needs to be studied further. Observation 10: Beam-steering is applied mostly on the horizontal plane and side-by-side is the worst case in regarding beam steering. Proposal 1: Beam steering need to be considered as an important parameter in the simulation setup. Proposal 2: Prioritize side-by-side deployment in the evaluation of the BS-to-BS isolation. |
| R4-2501875 | ZTE Corporation, Sanechips | Observation 1: if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB with10cm separation distance. Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc. Proposal 1: propose to consider the following specification for BS-BS isolation analysis: TS 38.104, TS 38.141-1/2, TS 38.174, TS 38.176-1/2, TS 38.106, TS 38.115-1/2 Proposal 2: propose to consider the following specific requirement for BS-BS isolation analysis. - Transmitter OFF power requirement - Co-location spurious emission requirement/Protection of the BS receiver of own or different BS - Tx IMD requirement - Co-located OOBB requirement Proposal 3: propose to further consider the co-location reference antenna or removal of certain requirement after the worst coupling loss is confirmed. Observation 3: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level. Observation 4: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it. Observation 5: Based on the initial simulation results for FR1 high bands, coupling loss is expected to be larger than 50dBc regardless from array to array or array to antenna port. |
| R4-2502160 | Ericsson | Observation 1: The A-to-A coupling loss at band n104 is significantly higher than the legacy 30 dB value. Observation 2: The A-to-A coupling loss is significantly higher in band n104 compared to the 3.5 GHz case. Observation 3: The A-to-SA coupling loss is significantly higher in band n104 compared to the 3.5 GHz case. Observation 4: A-to-A and A-to-SA coupling losses are different. Observation 5: The non-uniform coupling loss with respect to subarray position in the Array-to-subarray will impact requirements. Proposal 1: Co-location of two systems at different bands should be studied. Proposal 2: Co-location of systems with different array size should be studied, e.g. Urban Macro and Urban micro. Proposal 3: Study the impact on requirements of non-uniform coupling loss with respect to subarray position in the array-to-subarray case. |
| R4-2502135 | Huawei | Proposal 1: Remove TX IMD CLTA test for frequency bands above 4.9 GHz. |

## Open issues summary

The following sub-topics were identified based on the formal Proposals, as capture in contributions listed in clase 1.1.

### Sub-topic 1-1: BS-to-BS isolation analysis

Discussion on BS-to-BS isolation analyses were provided in multiple contributions.

- Proposals (non-exclusive)

  - Proposal 1:  Prioritize side-by-side deployment in the evaluation of the BS-to-BS isolation. (R4-2501323, Nokia)

  - Proposal 2:  Co-location of two systems at different bands should be studied. (R4-2502160, Ericsson)

  - Proposal 3:  Co-location of systems with different array size should be studied, e.g. Urban Macro and Urban micro. (R4-2502160, Ericsson)

  - Proposal 4:  Study the impact on requirements of non-uniform coupling loss with respect to subarray position in the array-to-subarray case. (R4-2502160, Ericsson)

  - Proposal 5: Propose to further consider the co-location reference antenna or removal of certain requirement after the worst coupling loss is confirmed. (R4-2501875, ZTE)

- Recommended WF

  - BS-to-BS isolation: Agree to use isolation measurement and simulation data collected below as baseline (no beamforming case):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 6.7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | ZTE: <-54 | ZTE: <-52 | ZTE: <-56 |  |
| Isolation in horizontal domain (dB) | ZTE: <-31 | ZTE: <-34 Huawei: -46 Ericsson: A-to-A: -55 (±20 for beamforming); A-to-SA: -55 (±15 for beamforming) | ZTE: <-46 | Ericsson: A-to-A: -75 (±25 for beamforming); A-to-SA: -70 (±20 for beamforming) |

  - Proposals 2, 3, 4: further discussion needed as proposals are generic and may require per-requirement verification.

  - Proposal 5 requires further clarification as it is not formulated clear enough.

### Sub-topic 1-2: Beam stering consideration for co-location scenario

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1: Beam steering need to be considered as an important parameter in the simulation setup. (R4-2501323, Nokia)

- Recommended WF

  - Online discussion on clarifications in Proposal 1.

### Sub-topic 1-3: Impacted specifications

- Proposals

  - Proposal 1: Propose to consider the following specification for BS-BS isolation analysis (R4-2501875, ZTE).

    - TS 38.104

    - TS 38.141-1/2

    - TS 38.174

    - TS 38.176-1/2

    - TS 38.106

    - TS 38.115-1/2

- Recommended WF

  - Impacted specifications: Consider Proposal 1 as baseline.

### Sub-topic 1-4: Impacted requirements

- Proposals

  - Proposal 1: Propose to consider the following specific requirement for BS-BS isolation analysis. (R4-2501875, ZTE)

    - Transmitter OFF power requirement

    - Co-location spurious emission requirement/Protection of the BS receiver of own or different BS

    - Tx IMD requirement

    - Co-located OOBB requirement

- Recommended WF

  - Impacted requirements: Consider Proposal 1 as baseline.

### Sub-topic 1-5: TX IMD co-location

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1:  Remove TX IMD CLTA test for frequency bands above 4.9 GHz. (R4-2502135, Huawei)

- Recommended WF

  - Online discussion: Check whether Proposal 1 would be agreeable as baseline, subject to the conclusion on sub-topic 1-1.