3GPP TSG-RAN WG4 Meeting #114	R4-2500603

Athens, Greece, 17th – 21st February 2025

# Source: 	ZTE Corporation

# Title: 	Discussion on power domain enhancements for single carrier

# Agenda item:	7.1.1.2.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on power domain enhancements for single carrier.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

  - 2.1 Approaches to enable MPR reduction for both scenario 1 and scenario2

- Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation Agreement in main session: - Extended CBW based approach - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE - Both scenario 1 and 2 should be covered - The network should ensure the co-existence be met when signaling the extended CBW. - Issue 1-1-2: Approaches of calculating the new inner RB region - Options - Option 1: The NW signals the extended number of RBs and RB shift towards the UE configured CBW or RRC signalling to indicate the extended RB number at each side of the UE CBW for the UE to calculate the new inner, outer, and edge regions - The extended number of RBs may not necessarily be the same as those in the BS CBWs - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - Option 2: Based on the fixed extension ratio, it is up to the NW to judge the gap width on both sides of the UE and grant the permission to UE using the reduced MPR for the new inner region - The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - Option 3: For a UE supporting [capability] and configured with a UE channel bandwidth within a carrier with a bandwidth greater than the transmission bandwidth configuration of this UE channel bandwidth, NRB = N´RB is the said carrier bandwidth as indicated by carrierBandwidth in [SIB1] and the condition for an inner RB allocation is modified accordingly - The intersection of the Inner region relative to the NW CBWs and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE Examples of new inner region just for illustration purpose: Example 1: single CC scenario; Example 2: multi-CC scenario. - WF - FFS description of deriving the RB allocation region inside the UE CBW which could use inner RB MPR with information of the extended UE CBW - Candidate wordings, other options are not precluded - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - The intersection of the Inner MPR region of the extended UE CBW with the original MPR region based on the UE CBW - For enhanced MPR requirements, at least in the case where BW is extended by 0.5 * UE CBW on one or both sides of UE CBW, an RB allocation (LCRB) within the UE CBW is considered as an extended inner RB allocation if an RB allocation-extCBW (of extended CBW) fully covering the same frequency location is also determined as an inner RB allocation (according to existing 38.101-1 MPR formula with corresponding NRB of the extended CBW applied). - Further improve the wording (better with illustrated figures) to derive a generic solution - How UE get the extended RB number as well as the RB shift is one possible way, other methods are not precluded - FFS how to reflect the new inner region in the spec - Issue 1-1-3: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: The extended RBs on one side should be equal to NRB/2, where NRB is the maximum transmission bandwidth configuration of the original UE CBW - Option 2: The extended RB number for each side can be different and the value could vary from zero to the one specified in Table 5.3.2-1 in TS 38.101-1, but it is no need to exceed the one corresponding to 50MHz, which is the half of the maximum UE CBW 100MHz. - Option 3: Width of Extended UE channel bandwidth can be up to BS channel bandwidth but no more or it can be narrower than BS CBW - Other options are not precluded - WF - Extension on each side of UE CBW could be different - FFS on extended RB numbers/channel bandwidth, which are up to X on each side of UE CBW, depends on the adjacent spectrum by operator in all possible scenarios - FFS whether extended RB number/channel bandwidth should be limited to max 50MHz at each side, which means the extended CBW is relevant to the configured UE CBW - Fixed extended RB number/channel bandwidth to X is not precluded. - Extension is not used for the specific UE UL transmission but can be used by other UEs UL transmissions. It is generally understood that going beyond X on each side does not further reduce the MPR. - FFS on X value

The above is agreement and way forward reached in last meeting [2], we agree to use the extended CBW based approach to cover both scenario 1 and 2. The motivation of the extended CBW based approach is converting full RB allocation in UE configured CBW to inner RB allocation with an extended UE CBW. Based on the definition of inner RB allocation in TS 38.101-1, the extended CBW should be the twice of UE configured CBW to convert full RB allocation in UE configured CBW to inner RB allocation with an extended UE CBW. In actual deployment, it cannot always achieve this goal. But we can maximize the utilization of BS CBW to use the extended CBW based approach. We give the following details of this method.

Single carrier operation from NW perspective:

- If the minimum offset between UE CBW and BS CBW is larger than or equal to 1/2 UE CBW, the extended UE CBW can be the twice of UE CBW, which can convert full RB allocation in UE CBW to new inner RB allocation. If not, the extended UE CBW can be replaced by the BS CBW, which can convert partial outer RB allocation in UE CBW to inner RB allocation with an extended UE CBW.

- Figure 2.1-1: the extended UE CBW based approach for single carrier operation

- For DL-only intra-band contiguous CA case:

- When there is no adjacent in-band/out-of-band co-existence issue, and when the minimum offset between UE CBW and aggregated BS CBW is greater than 1/2 UE CBW, the extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation. If not, the extended UE CBW can be replaced by the aggregated BS CBW, which can convert partial outer RB allocation in UE CBW to inner RB allocation with an extended UE CBW.

- Figure 2.1-2: the extended UE CBW based approach for DL-only intra-band contiguous CA case

Proposal 1:

- For single carrier operation from NW perspective, if the minimum offset between UE CBW and BS CBW is larger than or equal to 1/2 UE CBW, the extended UE CBW can be the twice of UE CBW, which can convert full RB allocation in UE CBW to new inner RB allocation. If not, the extended UE CBW can be replaced by the BS CBW, which can convert partial outer RB allocation in UE CBW to inner RB allocation with an extended UE CBW.

- For DL-only intra-band contiguous CA case, when there is no adjacent in-band/out-of-band co-existence issue, and when the minimum offset between UE CBW and aggregated BS CBW is greater than 1/2 UE CBW, the extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation. If not, the extended UE CBW can be replaced by the aggregated BS CBW, which can convert partial outer RB allocation in UE CBW to inner RB allocation with an extended UE CBW.

For the new inner RB allocation in UE CBW, we can define it based on the extended UE CBW. From the following picture, we can observe that new inner RB allocation (yellow within the red line) in UE CBW is the intersection of inner RB allocation in the extended UE CBW and UE RB allocation, in which UE RB allocation corresponds to the configured UE CBW.

Figure 2.1-3: New inner RB allocation in UE CBW based on the extended UE CBW

For the extended UE CBW, its inner RB allocation can be expressed by the following equation:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB,extended/2)

where ceil(x) is the smallest integer greater than or equal to x, NRB,extended is the maximum transmission bandwidth configuration of the extended channel bandwidth, RBStart,Low, RBStart, RBStart,High is relative to the extended UE CBW.

For the new inner RB allocation in UE CBW, RBStart should meet:

- RBshift  ≤  RBStart  ≤  NRB + RBshift

- where RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended UE CBW.

- In addition, the new inner RB allocation should be under the hypotenuse of the right triangle of UE RB allocation, which can be expressed by the following equation:

- RBStart − RBshift + LCRB ≤ NRB

- where NRB is the maximum transmission bandwidth configuration of the configured UE channel bandwidth.

- The above can be deduced to

- RBStart ≤ NRB + RBshift − LCRB

- Based on the above analysis, the new inner RB allocation in UE CBW can be defined based on the extended UE CBW as follows:

NRB,extended is the maximum transmission bandwidth configuration of the extended channel bandwidth.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

where ceil(x) is the smallest integer greater than or equal to x.

- Proposal 2: The new inner RB allocation in UE CBW can be defined based on the extended UE CBW as follows:

NRB,extended and NRB are the maximum transmission bandwidth configuration of the extended channel bandwidth and configured UE channel bandwidth respectively.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

- where RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended UE CBW and ceil(x) is the smallest integer greater than or equal to x.

  - 2.2 Applicable requirements

In-band emissions

- Issue 1-2-1: Where to use extended IBE in the larger BS CBW - Proposals - Proposal 1: IBE should be used between edges of UE CBW and extended UE CBW. (vivo, ZTE, Huawei, Qualcomm, China Telecom, Skyworks) - Alt 1: extension of the IBE is NOT used in the guard band of UE CBW. - Alt 2: extension of the IBE is also used in the guard band of UE CBW. - Alt 3: extension of the IBE applies in the guard band of the UE CBW but not in the guard band of the extended UE CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS the application of IBE in guard band of UE CBW and/or extended UE CBW - Issue 1-2-2: How to derive the extended IBE - Proposals - Proposal 1: (vivo) - For IBE of both original CBW and extended CBW, NRB should be based on the original UE CBW, ΔRB should also be limited within the original UE CBW. (vivo, ZTE, Huawei) - The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW. (vivo, ZTE, Huawei) - For UE CBW with full RB allocation, the IBE of the extended part could be simplified to: IBE_general = max⁡(PRB − 30 dB, max⁡{−25, 20∗log10EVM −3 +5LCRB, −57dBm + 10log10(SCS/15kHz)− PRB}) - Proposal 2: (ZTE) - Use the IBE limit of -25 dB between edges of UE CBW and extended UE CBW for full RB allocation scenario - Proposal 3: (Huawei) - For the full RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs is set to +1(-1). The equation for IBE derivation is: max−25, 10log10EVM−3, −57dBm+10log10SCS15kHz−PRB - Proposal 4: (China Telecom) - For full RB allocation, if the extended UE CBW exists, the IBE should be applied. - Proposal 5: (Qualcomm) - The IBE between the edge of the UE BW and the edge of the extended UE BW should be an extension of the IBE for the signal within the UE BW. - WF - FFS the method to derive the extended IBE, which should cover both cases for partial RB allocation and full RB allocation in the UE CBW

In last meeting, we achieved an agreement that IBE should be used between edges of UE CBW and extended UE CBW. For the application of IBE in guard band of UE CBW, we can treat it together with the argument about the general limit equation of IBE is based on the UE CBW or extended UE CBW.

According to the following equation of general requirement for in-band emissions, the limit is related to the UE channel bandwidth, allocated RB and the starting frequency offset between the allocated RB and the measured non-allocated RB. Based on the reason for limiting in-band emission, the interference from UE that is received with high power at the gNB may drastically reduce the SNR for another UE that is received with weaker signal strength, and the situation improves significantly if some sort of transmit power control in the uplink is used. Since the extended UE CBW is only used to apply ACLR, SEM and SE instead of transmitting data, the network deploys UE based on the real UE CBW rather than extended UE CBW, which means that it may exist the case another UE is deployed within the extended UE CBW.

In last meeting, a proposal was raised that the IBE based on the extended UE CBW, where NRB in the equation is the extended UE CBW, and apply the extension of the IBE in the guard band of the UE CBW. If doing so, IBE will be relaxed. In order to control UE to UE interference, it’s better not to relax the IBE. So we propose to extend the IBE limit in the outermost RB of UE CBW to the range between edges of UE CBW and extended UE CBW for UE CBW with non-full RB allocation case and use the following equation between edges of UE CBW and extended UE CBW for full RB allocation case. In addition, we think IBE should not be applied in the guard band of UE CBW and the extended UE CBW.

Proposal 3: IBE should be used between edges of UE CBW and extended UE CBW

    - The extension of the IBE is not applied in guard band of UE CBW and the extended UE CBW.

Proposal 4:

    - Extend the IBE limit in the outermost RB of UE CBW to the range between edges of UE CBW and extended UE CBW for non-full RB allocation scenario

    - Use the IBE limit of the following equation between edges of UE CBW and extended UE CBW for full RB allocation scenario

ACLR, SEM, SE

- Issue 1-2-3: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW. (Nokia, Qualcomm, vivo, ZTE, LGE, OPPO, Huawei, China Telecom) - Alt 1: The OOBE requirements are based on UE CBW - Alt 2: The OOBE requirements are based on extended UE CBW Alt 1. Alt 2. - WF - ACLR and SEM should be applicable from the edge of extended UE CBW. - FFS the following alternatives - Alt 1: The OOBE requirements are based on UE CBW - Alt 2: The OOBE requirements are based on extended UE CBW - Issue 1-2-4: Boundary to apply SE - Proposals - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. (vivo, LGE) - Proposal 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz. (Nokia) - Proposal 3: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW (Qualcomm, OPPO) - WF - FFS with consideration of Issue 1-2-3

Compare with Alt 1 and Alt 2, Alt 2 means some relaxation on the level of SEM limit more than the application range of SEM, which is excluded in before discussion. From the simulation in [3], it can be observed that applying MPR of inner RB allocation to all RB allocations in UE CBW can still meet OOBE requirement when the OOBE requirements are based on UE CBW. Hence, we propose that ACLR and SEM should be applicable from the edge of extended UE CBW and the OOBE requirements are based on UE CBW. For boundary to apply SE, OOB boundary FOOB should be the extended UE CBW + 5MHz.

Proposal 5: ACLR and SEM should be applicable from the edge of extended UE CBW

    - The OOBE requirements are based on UE CBW

    - OOB boundary FOOB to apply SE should be the extended UE CBW + 5MHz.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1:

- For single carrier operation from NW perspective, if the minimum offset between UE CBW and BS CBW is larger than or equal to 1/2 UE CBW, the extended UE CBW can be the twice of UE CBW, which can convert full RB allocation in UE CBW to new inner RB allocation. If not, the extended UE CBW can be replaced by the BS CBW, which can convert partial outer RB allocation in UE CBW to inner RB allocation with an extended UE CBW.

- For DL-only intra-band contiguous CA case, when there is no adjacent in-band/out-of-band co-existence issue, and when the minimum offset between UE CBW and aggregated BS CBW is greater than 1/2 UE CBW, the extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation. If not, the extended UE CBW can be replaced by the aggregated BS CBW, which can convert partial outer RB allocation in UE CBW to inner RB allocation with an extended UE CBW.

- Proposal 2: The new inner RB allocation in UE CBW can be defined based on the extended UE CBW as follows:

NRB,extended and NRB are the maximum transmission bandwidth configuration of the extended channel bandwidth and configured UE channel bandwidth respectively.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

- where RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended UE CBW and ceil(x) is the smallest integer greater than or equal to x.

Proposal 3: IBE should be used between edges of UE CBW and extended UE CBW

    - The extension of the IBE is not applied in guard band of UE CBW and the extended UE CBW.

Proposal 4:

    - Extend the IBE limit in the outermost RB of UE CBW to the range between edges of UE CBW and extended UE CBW for non-full RB allocation scenario

    - Use the IBE limit of the following equation between edges of UE CBW and extended UE CBW for full RB allocation scenario

Proposal 5: ACLR and SEM should be applicable from the edge of extended UE CBW

    - The OOBE requirements are based on UE CBW

    - OOB boundary FOOB to apply SE should be the extended UE CBW + 5MHz.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

- R4-2420527, WF on Power domain enhancement, Huawei, HiSilicon

- R4-2419400, Discussion on power domain enhancement for NR single carrier, MediaTek Inc.