3GPP TSG-RAN WG4 Meeting #114	R4-2500604

Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to 38.101-1 MPR applicability for FR1 intra-band contiguous UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture the agreement for MPR applicability for FR1 intra-band contiguous UL CA reached in last few meetings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specify the MPR applicability for intra-band contiguous CA with single CC activated. - Specify applicable emission requirements for PC2/PC3 intra-band contiguous CA with single CC activated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPR for FR1 intra-band contiguous UL CA can not be reduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.1, 6.5A.2.2, 6.5A.2.4, 6.5A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

6.2A.2	UE maximum output power reduction for CA

6.2A.2.1	UE maximum output power reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.2A.1.1-1 with contiguous RB allocation is specified in Table 6.2A.2.1-1 for UE power class 3 CA bandwidth classes B and C. The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth classe C when the signalling is indicated for dualPA-Architecture IE. The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1b for power class 2 CA bandwidth classes B and C with TxD supported.

For intra-band contiguous CA with single CC activated, the MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. The MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated. The MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent.

In case the modulation format or waveform type is different on different component carriers then the requirement is set by rules applied to the waveform type (DFT-s-OFDM or CP-OFDM) and modulation order used in the configuration with the largest MPR..

Unless otherwise specified, pi/2 BPSK in following MPR tables refers to both variants of pi/2 BPSK referenced in clause 6.2.2 Table 6.2.2-1.

Table 6.2A.2.1-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Table 6.2A.2.1-1a: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | QPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | 16QAM | 2.5 | 4.01 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.51 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.5 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.0 | 5.01 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5.01 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. |  |  |  |  |  |

Table 6.2A.2.1-1b: Contiguous RB allocation for Power Class 2 with dual Tx2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | QPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.5 | 5.01 | 3.5 | 8 |
|  | 64QAM | 4.0 | 5.51 | 6 | 8 |
|  | 256QAM | 6.5 | 7.0 | 8 | 8.5 |
| CP-OFDM | QPSK | 3.0 | 5.51 | 4.0 | 8.5 |
|  | 16QAM | 3.5 | 5.51 | 4.0 | 8.5 |
|  | 64QAM | 4.0 | 5.51 | 5.5 | 8.5 |
|  | 256QAM | 7.0 | 7.0 | 7.5 | 8.5 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. NOTE 2: UE indicating TxD supported |  |  |  |  |  |

For CA bandwidth class B and bandwidth class C with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2^µ1 + LCRB2 ∙ 2^µ2,

NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2.

If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2,

if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table Table 6.2A.1.1-1 with non-contiguous RB allocation is specified in Table 6.2A.2.1-2 for UE power class 3 CA bandwidth classes B and C. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.1-3 for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth classe C when the signalling is indicated for dualPA-Architecture IE. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.1-4 for power class 2 CA bandwidth classes B and C with TxD supported.

Table 6.2A.2.1-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 31 | 6.5 | 13 | 31 | 7.5 | 13.5 |
|  | QPSK | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 16QAM | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 64QAM | 5 | 6.5 |  | 5 | 7.5 |  |
|  | 256QAM | 6.5 | 7 |  | 6.5 | 7.5 |  |
| CP-OFDM | QPSK | 3.51 | 7 | 14 | 3.51 | 8 | 14.5 |
|  | 16QAM | 3.51 | 7 |  | 3.51 | 8 |  |
|  | 64QAM | 5 | 7 |  | 5 | 8 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 8 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.1-4: non-contiguous RB allocation for Power Class 2 with dual Tx4

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 41 | 7.5 | 14 | 41 | 8.5 | 14.5 |
|  | QPSK | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 16QAM | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 64QAM | 6 | 7.5 |  | 6 | 8.5 |  |
|  | 256QAM | 7.5 | 8 |  | 7.5 | 8.5 |  |
| CP-OFDM | QPSK | 4.51 | 8 | 15 | 4.51 | 9 | 15.5 |
|  | 16QAM | 4.51 | 8 |  | 4.51 | 9 |  |
|  | 64QAM | 6 | 8 |  | 6 | 9 |  |
|  | 256QAM | 8.5 | 8.5 |  | 8.5 | 9 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

For CA bandwidth classes B and C with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0, when both uplink CCs are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2,

RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between the upper edge of the Transmission Bandwidth Configuration NRB1 of CC1 and the lower edge of the Transmisson Bandwidth Configuration NRB2 of CC2.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation when it is not satisfying inner allocation conditions and when the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_CA , BWgap and BWGB,low are as defined for the Inner region.

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

# << Next change >>

6.5A.2	Out of band emission for CA

6.5A.2.1	General

This clause contains requirements for out of band emissions for UE configured of carrier aggregation.

6.5A.2.2	Spectrum emission mask

6.5A.2.2.1	Spectrum emission mask for intra-band contiguous CA

For intra-band contiguous carrier aggregation the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the aggregated channel bandwidth. For intra-band contiguous carrier aggregation, the power of any UE emission shall not exceed the levels specified in Table 6.5A.2.2.1-1 for the specified channel bandwidth. For intra-band contiguous CA with single CC activated, the spectrum emission mask applies based on the aggregated channel bandwidth independent of dualPA-Architecture IE indication.

For power class 2 intra-band contiguous carrier aggregation, the spectrum emission mask is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

Table 6.5A.2.2.1-1: General NR CA spectrum emission mask

| ΔfOOB (MHz) | Spectrum emission limit(dBm) | MBW(MHz) |
| --- | --- | --- |
| ± 0 - 1 | -13 | Min(0.01*BWchannel_CA, 0.4) |
| ± 1 - 5 | -10 | 1MHz |
| ± 5 – BWchannel_CA | -13 | 1MHz |
| ±BWchannel_CA- BWchannel_CA+5 | -25 | 1MHz |

# << Next change >>

6.5A.2.4	Adjacent channel leakage ratio

6.5A.2.4.1	NR ACLR

6.5A.2.4.1.1	NR ACLR for intra-band contiguous CA

For intra-band contiguous carrier aggregation the carrier aggregation the Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent aggregated channel bandwidth at nominal channel spacing. The assigned aggregated channel bandwidth power and adjacent aggregated channel bandwidth power are measured with rectangular filters with measurement bandwidths specified in Table 6.5A.2.4.1.1-1 for power class 3 and 6.5A.2.4.1.1-2 for power class 2. If the measured adjacent channel power is greater than –50dBm then the NRACLR shall be higher than the value specified in Table 6.5A.2.4.1.1-1 for power class 3 and 6.5A.2.4.1.1-2 for power class 2. For intra-band contiguous CA with single CC activated, the ACLR applies based on the aggregated channel bandwidth independent of dualPA-Architecture IE indication.

Table 6.5A.2.4.1.1-1: General requirements for intra-band contiguous CA ACLR power class 3

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 30 dB |
| CA Measurement bandwidth (NOTE 1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |
| Difference between ACLR MBW center and Fc,low | MBWshift= (MBWACLR_CA-MBWACLR,low)/2 |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1, respectively. |  |

Table 6.5A.2.4.1.1-2: General requirements for intra-band contiguous CA ACLR power class 2

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 31 dB |
| CA Measurement bandwidth (NOTE 1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |
| Difference between ACLR MBW center and Fc,low | MBWshift= (MBWACLR_CA-MBWACLR,low)/2 |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1, respectively. |  |

# << Next change >>

6.5A.3	Spurious emission for CA

6.5A.3.1	General spurious emissions

For inter-band carrier aggregation with uplink assigned to two NR bands, the spurious emission requirement Table 6.5.3.1-2 apply for the frequency ranges that are more than FOOB as defined in Table 6.5.3.1-1 away from edges of the assigned channel bandwidth on a component carrier. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier then it does not apply.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands the requirements in Table 6.5.3.1-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5.3.1-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA spurious emission requirement.

For intra-band contiguous carrier aggregation the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB as specified in Table 6.5A.3.1-1 the spurious emission requirements in Table 6.5.3.1-2 are applicable. For power class 2 intra-band contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE. For intra-band contiguous CA with single CC activated, FOOB is defined based on the aggregated channel bandwidth independent of dualPA-Architecture IE indication.

Table 6.5A.3.1-1: Boundary between out of band and spurious emission domain for intra-band contiguous carrier aggregation

| Aggregated Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel_CA | BWChannel_CA + 5 |

For intra-band non-contiguous carrier aggregation transmission the spurious emission requirement is defined as a composite spurious emission requirement. Composite spurious emission requirement applies to frequency ranges that are more than FOOB away from the edges of each carrier in the gap and out of the gap. Composite spurious emission requirement is defined as follows

a)	Composite spurious emission requirement is a combination of individual sub-block spurious emission requirements

b)	In case the sub-block consist of one component carrier the sub-lock spurious emission requirement and FOOB are defined in subclause 6.5.3.1

c)	If for some frequency an individual sub-block spurious emission requirement overlaps with the general spectrum emission mask or the sub-block bandwidth of another sub-block then it does not apply

For intra-band non-contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per operating band), the spurious emission requirement is defined as follows. For the NR band supporting one component carrier the requirements in Table 6.5.3.1-2 apply for frequency ranges that are more than FOOB (MHz) from the edges of assigned channel bandwidth as defined in Table 6.5.3.1-1. For the NR band supporting two contiguous component carriers the requirements in Table 6.5.3.1-2 apply for frequency ranges that are more than FOOB (MHz) from the edges of assigned aggregated channel bandwidth as defined in Table 6.5A.3.1-1. If for some frequency a spurious emission requirement of a single component carrier or two contiguous component carriers overlap with the spurious emission requirement or channel bandwidth of another component carrier or two contiguously aggregated carriers then it does not apply.

# << End change >>