3GPP TSG-RAN WG4 Meeting #114	R4-2500608

Athens, Greece, 17th – 21st February 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters for NR NTN Ku band

# Agenda item:	7.11.3

# Document for:	Approval

# 1 Introduction

In RAN#106 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from system parameters perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

# 2 Discussion

  - 2.1 Band definition

- Sub-topic 1-1: Defining bands by regulatory authority Agree the following band definition Table X: Band definitions for Ku band Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. nX(Note 1); 14000 MHz - 14500 MHz; 10700 MHz - 12750 MHz; FDD. nY(Note 2); 14000 MHz - 14500 MHz; 10700 MHz - 12700 MHz; FDD. NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25..

- The above is the agreement we reached in last meeting. Furthermore, according to the objective of the revised WID, it states that define two sets of bands, one for FR1 numerology and the other for FR2 numerology. We propose to define the following bands in specification, which is shown in Table 2.1-1 and Table 2.1-2.

Table 2.1-1: NTN satellite bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n249](Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| [n248](Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

Table 2.1-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n509](Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| [n508](Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

Proposal 1: It is proposed to specify the following band definitions for Ku band, which is shown in Table 2.1-1 and Table 2.1-2.

  - 2.2 Channel bandwidth per operating band

- Based on the objective in the revised WID, we propose to define the following channel bandwidths for Ku band for two sets of bands.

Table 2.2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| [n249] | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| [n248] | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |

Table 2.2-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| [n509] | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| [n508] | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |

Proposal 2: It is proposed to define the following channel bandwidths for Ku band for two sets of bands.

- Due to the maximum supporting channel bandwidth for FR1-NTN is 30MHz with no corresponding RF requirements, if Ku band supports 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing in FR1-NTN, it needs to specify maximum transmission bandwidth configuration, minimum guard band and the corresponding RF requirements for these channel bandwidths in FR1-NTN. And we propose to define the same maximum transmission bandwidth configuration and minimum guard band as in TS 38.101-1 for these channel bandwidths in FR1-NTN.

- Observation 1: If Ku band supports 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing in FR1-NTN, it needs to specify maximum transmission bandwidth configuration, minimum guard band and the corresponding RF requirements for these channel bandwidths in FR1-NTN.

- Proposal 3: It is proposed to define the same maximum transmission bandwidth configuration and minimum guard band as in TS 38.101-1 for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz SCS in FR1-NTN.

  - 2.3 Channel raster and sync raster

- Based on the above, we propose to define the channel raster entries and SS raster entries for Ku band as follows.

Table 2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n249] | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| [n248] | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1246666 |

Table 2.3-2: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n509] | 120 | 1333334 – <2> – 1366666 | 1113336 – <8> – 1250000 |
| [n508] | 120 | 1333334 – <2> – 1366666 | 1113336 – <8> – 1246664 |

Table 2.3-3: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n249] | 30 kHz | Case C | 12850 – <1> – 14266 |
| [n248] | 30 kHz | Case C | 12850 – <1> – 14232 |

Table 2.3-4: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n509] | 120 kHz | Case D | 12864 – <12> – 14256 |
|  | 240 kHz | Case E | 12888 – <24> – 14232 |
| [n508] | 120 kHz | Case D | 12864 – <12> – 14220 |
|  | 240 kHz | Case E | 12888 – <24> – 14208 |

- Proposal 4: It is proposed to define the channel raster entries and SS raster entries for Ku band as follows.

  - 2.4 TX-RX frequency separation

- For Ku band in FR1-NTN, TX-RX frequency separation needs to be defined. So we propose to specify the default TX-RX frequency separation for Ku band in FR1-NTN.

Table 2.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n249] | -2525 MHz |
| [n248] | -2550 MHz |

- Proposal 5: It is proposed to specify the default TX-RX frequency separation for Ku band in FR1-NTN as follows.

# 3 Conclusion

In this contribution, we shared some views on system parameters for NTN Ku band and the proposal is made as follows:

Proposal 1: It is proposed to specify the following band definitions for Ku band, which is shown in Table 2.1-1 and Table 2.1-2.

Table 2.1-1: NTN satellite bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n249](Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| [n248](Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

Table 2.1-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n509](Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| [n508](Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

Proposal 2: It is proposed to define the following channel bandwidths for Ku band for two sets of bands.

Table 2.2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| [n249] | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| [n248] | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |

Table 2.2-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| [n509] | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| [n508] | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |

- Observation 1: If Ku band supports 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing in FR1-NTN, it needs to specify maximum transmission bandwidth configuration, minimum guard band and the corresponding RF requirements for these channel bandwidths in FR1-NTN.

- Proposal 3: It is proposed to define the same maximum transmission bandwidth configuration and minimum guard band as in TS 38.101-1 for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz SCS in FR1-NTN.

- Proposal 4: It is proposed to define the channel raster entries and SS raster entries for Ku band as follows.

Table 2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n249] | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| [n248] | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1246666 |

Table 2.3-2: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n509] | 120 | 1333334 – <2> – 1366666 | 1113336 – <8> – 1250000 |
| [n508] | 120 | 1333334 – <2> – 1366666 | 1113336 – <8> – 1246664 |

Table 2.3-3: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n249] | 30 kHz | Case C | 12850 – <1> – 14266 |
| [n248] | 30 kHz | Case C | 12850 – <1> – 14232 |

Table 2.3-4: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n509] | 120 kHz | Case D | 12864 – <12> – 14256 |
|  | 240 kHz | Case E | 12888 – <24> – 14232 |
| [n508] | 120 kHz | Case D | 12864 – <12> – 14220 |
|  | 240 kHz | Case E | 12888 – <24> – 14208 |

- Proposal 5: It is proposed to specify the default TX-RX frequency separation for Ku band in FR1-NTN as follows.

Table 2.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n249] | -2525 MHz |
| [n248] | -2550 MHz |

# 4 Reference

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)