3GPP TSG-RAN WG4 Meeting #114	R4-2500609

Athens, Greece, 17th – 21st February 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN Ku band

# Agenda item:	7.11.4

# Document for:	Approval

# 1 Introduction

In RAN#106 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from system parameters perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

# 2 Discussion

  - 2.1 NTN VSAT types

According to the objective of the revised WID, it states that

- Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements

- Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band.

- In current specification 38.101-5, NTN VSAT types only define Fixed VSAT with NGSO and GSO, Mobile VSAT with GSO. There is no NTN VSAT type defined for Mobile VSAT with NGSO. So we need to clarify whether to introduce a new NTN VSAT type for Mobile VSAT with NGSO.

- Proposal 1: Need to clarify whether to introduce a new NTN VSAT type for Mobile VSAT with NGSO.

  - 2.2 Maximum EIRP requirement

- Issue 1-2-2: Maximum EIRP and TRP requirement - The following aspects will be consider if setting the maximum EIRP & TRP of the NTN Ku band in 38.101-5 - An absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios, while allowing separating consideration for non-GSO scenarios - Upper EIRP limits in the regulation - Enable flexibility for different application and scenarios - Issue 1-2-3: Maximum EIRP requirements and the minimum antenna size for the CEPT Ku band - For the CEPT Ku band, the following requirements from ECC regulation will be considered, FFS on how to reflect to the specifications and the VSAT types. - NTN VSAT operating with GSO 14.0 – 14.25 GHz Max EIRP: 60 dBW (from ECC Decision(17)04) - NTN VSAT operating with GSO 14.25 – 14.5 GHz Max EIRP: 50 dBW (from ECC Decision(03)04) - NTN VSAT operating with GSO 14.0 – 14.25 GHz Max EIRP: 60 dBW (from ECC Decision(06)03) - Mobile VSAT / Land ESIM 14.0 – 14.5 GHz Max EIRP: 54.5 dBW (from ECC Decision(18)04) - Mobile VSAT / Maritime ESIM 14.0 – 14.5 GHz minimum antenna size: 0.6 m (from ECC Decision (05)10)

In last meeting, some ECC regulations involved in the maximum EIRP and the minimum antenna size were raised. Since the revised WID specifies that specify RF requirements for mobile VSAT served by NGSO and GSO and exclude NTN VSAT types 1 to 3, we only need to consider ECC regulation related to mobile VSAT with NGSO and GSO when defining RF requirements for Ku band. And the following ECC regulation will be considered when specifying the maximum EIRP requirements for Ku band.

| ECC Decision | NTN VSAT type | Requirement |
| --- | --- | --- |
| ECC Decision(18)04 | Mobile VSAT with GSO | Max EIRP: 54.5 dBW |
| ECC Decision(18)05 | Mobile VSAT with NGSO | Max EIRP: 54.5 dBW |
| ECC Decision(19)04 | Mobile VSAT with GSO and NGSO | Max EIRP: 50 dBW |

- Proposal 2: It is proposed to only consider ECC regulation related to mobile VSAT with NGSO and GSO when specifying the maximum EIRP requirements for Ku band.

  - 2.3 Off-axis EIRP emission density limit within the operating band

- Issue 1-2-4: Off-axis EIRP requirements for the CEPT Ku band - For the UL band 14.0 - 14.5 GHz (E-to-S) concerning requirements from ECC regulation and ETSI standards: - Specify the off-axis EIRP density limit based on EN 301 428, EN 302 340 and EN 302 977 as a starting point, the ECC decision can be further checked whether additional updated is needed.

The above is way forward in last meeting, and we need to take ECC regulation and ETSI standards into consideration when specifying off-axis EIRP emission density limit. In last meeting, we already checked ETSI standards, and ECC decision needs to be further considered.

In addition, in last RAN meeting, a revised WID was approved which adds mobile VSAT with NGSO to the objective and demands that only specify RF requirements for mobile VSAT with GSO and NGSO. So we only need to consider how to specify RF requirements for mobile VSAT with GSO and NGSO for Ku band.

EN 301 428, EN 302 340 and EN 302 977 all apply to mobile VSAT, and only EN 302 977 clarifies that the regulation is for mobile VSAT with GSO.

EN 301 428 (14.0-14.5 GHz)

EN 302 340, EN 302 977 (14.0-14.5 GHz)

Observation 1: EN 301 428, EN 302 340 and EN 302 977 all apply to mobile VSAT, and only EN 302 977 clarifies that the regulation is for mobile VSAT with GSO.

Based on the above analysis, we further check the ECC Decision, and find the following requirement for mobile VSAT with GSO in ECC Decision(18)04. It can be observed that for the maximum EIRP of FSS earth station, the FCC EIRP mask is used. In other words, the maximum EIRP of FSS earth station is same as off-axis EIRP density limits for co-polarized transmissions in the plane tangent to the GSO arc for the conventional Ku-band defined in FCC Title 47 Chapter I Subchapter B Part 25.

ECC Decision(18)04

FCC 25.218 (14.0-14.5 GHz)

Observation 2: ECC Decision(18)04 specifies off-axis EIRP requirement for mobile VSAT with GSO, which is same as the requirement in FCC regulation.

  - 2.4 On-axis spurious requirement and off-axis spurious requirement

- Issue 1-2-5: On-axis spurious and off-axis spurious requirement for the CEPT Ku band - For the UL band 14.0 - 14.5 GHz (E-to-S) concerning requirements from ECC regulation and ETSI standards: - Specify the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981 as the starting point - Specify the same requirement as Ka band for the general and off-axis spurious limits in “Emissions disabled” state as starting point. - FFS on off-axis spurious limits in “Carrier-on” and “Carrier-off” state

In last meeting, we checked on-axis spurious and off-axis spurious requirement in EN 303 980, EN 303 981 and EN 302 977 for Ku band. Based on the following picture cited in [3], it can be found that EN 303 980 and EN 303 981 apply for mobile VSAT with NGSO, and EN 302 977 is applied for mobile VSAT with GSO.

On-axis spurious requirement in “Carrier-on” state

EN 302 977

EN 303 980, EN 303 981

For on-axis spurious limits in “Carrier-on” state, EN 302 977 gives different requirements for outside and inside with dBW in any 100 kHz for Ku band in UL 14.0–14.5 GHz. However, EN 303 980 and EN 303 981 define the same requirement for both outside and inside as 44-K dBm/MHz.

Observation 3: For on-axis spurious limits in “Carrier-on” state, EN 302 977 gives different requirements for outside and inside with dBW in any 100 kHz for Ku band in UL 14.0–14.5 GHz.

Proposal 3: It is proposed to use on-axis spurious limits in “Carrier-on” state in EN 302 977 as a starting point for mobile VSAT with GSO for Ku band.

Proposal 4: It is proposed to specify the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981 for mobile VSAT with NGSO for Ku band.

Off-axis spurious requirement

For radiated field strength limits at 10m from the NTN VSAT, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is same as Ka band in TS 38.101-5.

For off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz are also same as Ka band in TS 38.101-5.

Observation 4: For the general and off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band are same for mobile VSAT with GSO and NGSO for Ku band.

For off-axis spurious limits in “Carrier-on” and “Carrier-off” state, EN 303 980, EN 303 981 and EN 302 977 define a little different requirements for Ku band in UL 14.0–14.5 GHz.

EN 303 980, EN 303 981

EN 302 977

Proposal 5: It is proposed to use off-axis spurious limits in “Carrier-on” and “Carrier-off” state in EN 302 977 as a starting point for mobile VSAT with GSO for Ku band.

Proposal 6: It is proposed to use off-axis spurious limits in “Carrier-on” and “Carrier-off” state specified in EN 303 980, EN 303 981 as a starting point for mobile VSAT with NGSO for Ku band.

  - 2.5 Receiver antenna off-axis performance

For receiver antenna off-axis performance, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify different requirements for different VSATs. EN 303 980 and EN 303 981 apply for mobile VSAT with NGSO, while EN 302 977 and EN 302 340 are applied for mobile VSAT with GSO. Receiver antenna off-axis performance in EN 303 977 and EN 302 340 is same as the requirement for Ka band in TS 38.101-5.

EN 303 977 and EN 302 340

Observation 5: For receiver antenna off-axis performance, EN 303 977 and EN 302 340 have the same requirement as defined in TS 38.101-5 for Ka band.

Proposal 7: It is proposed to use the same requirement as defined in TS 38.101-5 for Ka band for mobile VSAT with GSO operating in Ku band.

For mobile VSAT with NGSO for Ku band, EN 303 981 defines two sets of receiver antenna off-axis performance for two classes of VSAT. The difference between two classes is that class A transmits at elevation angles of 50°or greater, relative to the horizontal plane, and class B transmits at elevation angles of 25°or greater. However, in EN 303 980, it specifies only one set requirement same as the requirement of class A in EN 303 981.

EN 303 980

EN 303 981

Observation 6: EN 303 981 defines two sets of receiver antenna off-axis performance for two classes of mobile VSAT with NGSO for Ku band.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

- Proposal 1: Need to clarify whether to introduce a new NTN VSAT type for Mobile VSAT with NGSO.

- Proposal 2: It is proposed to only consider ECC regulation related to mobile VSAT with NGSO and GSO when specifying the maximum EIRP requirements for Ku band.

Observation 1: EN 301 428, EN 302 340 and EN 302 977 all apply to mobile VSAT, and only EN 302 977 clarifies that the regulation is for mobile VSAT with GSO.

Observation 2: ECC Decision(18)04 specifies off-axis EIRP requirement for mobile VSAT with GSO, which is same as the requirement in FCC regulation.

Observation 3: For on-axis spurious limits in “Carrier-on” state, EN 302 977 gives different requirements for outside and inside with dBW in any 100 kHz for Ku band in UL 14.0–14.5 GHz.

Proposal 3: It is proposed to use on-axis spurious limits in “Carrier-on” state in EN 302 977 as a starting point for mobile VSAT with GSO for Ku band.

Proposal 4: It is proposed to specify the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981 for mobile VSAT with NGSO for Ku band.

Observation 4: For the general and off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band are same for mobile VSAT with GSO and NGSO for Ku band.

Proposal 5: It is proposed to use off-axis spurious limits in “Carrier-on” and “Carrier-off” state in EN 302 977 as a starting point for mobile VSAT with GSO for Ku band.

Proposal 6: It is proposed to use off-axis spurious limits in “Carrier-on” and “Carrier-off” state specified in EN 303 980, EN 303 981 as a starting point for mobile VSAT with NGSO for Ku band.

Observation 5: For receiver antenna off-axis performance, EN 303 977 and EN 302 340 have the same requirement as defined in TS 38.101-5 for Ka band.

Proposal 7: It is proposed to use the same requirement as defined in TS 38.101-5 for Ka band for mobile VSAT with GSO operating in Ku band.

Observation 6: EN 303 981 defines two sets of receiver antenna off-axis performance for two classes of mobile VSAT with NGSO for Ku band.

# 4 Reference

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2419790, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

- R4-2411190, NTN Ku-band – Regulatory aspects and band(s) definition, Ericsson