3GPP TSG-RAN WG4 Meeting #114	R4-2500611

Athens, Greece, 17th – 21st February 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters and UE RF requirements for the new NB-IoT TDD operating NTN band

# Agenda item:	7.31.2

# Document for:	Approval

# 1 Introduction

In RAN#106 meeting, a revised Work Item [1] was approved on introduction of introduction of IoT-NTN TDD mode with the following objective. In this contribution, we want to share some views about the new NB-IoT TDD operating NTN band from system parameters and UE RF requirements perspective.

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

# 2 Discussion

  - 2.1 Channel numbers

### Issue 1-1: Operating Bands <Band number> - RAN4 to consider the following proposals for the requirement definition during the normative work item phase in future RAN4 meetings. - Assign “[249]” E-UTRA operating band number for the new TDD [1616-1626.5] MHz NTN NB-IoT frequency band introduction. E-UTRA operating bands for satellite access E-UTRA Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. 256; 1980 MHz; –; 2010 MHz; 2170 MHz; –; 2200 MHz; FDD. 255; 1626.5 MHz; –; 1660.5 MHz; 1525 MHz; –; 1559 MHz; FDD. 254; 1610 MHz; -; 1626.5 MHz; 2483.5 MHz; -; 2500 MHz; FDD. 2532; 1668 MHz; -; 1675 MHz; 1518 MHz; -; 1525 MHz; FDD. 252; 2000 MHz; -; 2020 MHz; 2180 MHz; -; 2200 MHz; FDD. [249]; [1616] MHz; -; [1626.5] MHz; [1616] MHz; -; [1626.5] MHz; TDD. NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations.. - Use a channel raster of 100 kHz and a Step size of <1> - Both FDL_low and FUL_low equal 1616 MHz. - For the range of both NDL and NUL: - Option 1: Starts from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) or 2^18-2^14-1=245759 (in-between FDD NTN NOffs-DL and FDD NTN NOffs-UL) - Option 2: {198196 - 198300}

- According to the way forward in last meeting, there are two options for the range of both NDL and NUL:

    - Option 1: Starts from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) or 2^18-2^14-1=245759 (in-between FDD NTN NOffs-DL and FDD NTN NOffs-UL)

    - Option 2: {198196 - 198300}

- Based on the analysis in [3], we propose to start from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) for the range of both NDL and NUL and channel numbers for the new TDD NTN NB-IoT operating band can be defined as Table 2.1-1.

Table 2.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [249] | 100 | 1616 | [196607] | [196607-<1>-196711] | 1616 | [196607] | [196607-<1>-196711] |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Proposal 1: It is proposed to start from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) for the range of both NDL and NUL and channel numbers for the new TDD NTN NB-IoT operating band can be defined as Table 2.1-1.

  - 2.2 UE RF requirements

### Issue 1-3: Impact on UE RF requirements Agreement: - The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition. 36.102 Clause: Requirements; Band Agnostic; Re-use for TDD band [249]; Note. 6.2B.1: UE maximum output power; Yes; Yes; Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm). 6.2B.1: UE maximum output power tolerance; No. 6.2B.2: UE maximum output power reduction; Yes; Yes. 6.2B.3: UE additional maximum output power reduction; No; -; Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254.. 6.2.B.4: Configured transmit power; Yes; Yes. 6.3: Output power dynamics; Yes; Yes FFS; Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern.. 6.4: Transmit signal quality; Yes; Yes. 6.5B.3.2: Spectrum emission mask; Yes; Yes. 6.5B.3.4: Adjacent Channel Leakage Ratio; Yes; Yes. 6.5B.4.2: General spurious emissions; Yes; Yes. 6.5B.4.3: Spurious emissions for UE co-existence; No; Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence.. 6.5B.4.4: Additional spurious emissions; No; Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249].. 6.6: Transmit intermodulation for category NB1 and NB2; Yes; Yes. 7.2: Diversity characteristics; Yes; Yes. 7.3: Reference sensitivity power level; No; No; To be studied for IoT-NTN TDD mode.. 7.4: Maximum input level; Yes; Yes. 7.5: Adjacent Channel Selectivity; Yes; Yes. 7.6B.2: In-band blocking; No; Yes; Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253. 7.6B.3: Out-of-band blocking; No; Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253. 7.6B.4: Narrow band blocking; No; Yes. 7.7: Spurious response; Yes; Yes. 7.8: Intermodulation characteristics; Yes; Yes. 7.9: RX spurious emission; No; Yes; Re-use spurious emission requirements from Table 7.9.1-1 of 36.101. A.1 DL reference measurement channels; NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced.. A.2 OFDMA Channel Noise Generator (OCNG); OCNG Patterns for TDD should be introduced.. A.3 Testing related to Satellite Access; No impact.. A.4 UL reference measurement channels; Reference measurement channels for TDD should be introduced..

The above is the agreement reached in last meeting, there are some remaining requirements need to be studied. One is reference sensitivity power level. For TN NB-IoT, it uses the following equation to define REFSENS and defines the same reference sensitivity for UE category NB1 and NB2 for all FDD and TDD bands.

PREFSENS = -174+10*log10(BW) + NF + IM + 1/f Noise +SNR

Based on the above equation, we can observe that IoT-NTN TDD mode has no impact on REFSENS requirement. Thus, the existing REFSENS -108.2dBm can be reused for the new TDD NTN NB-IoT operating band.

Proposal 2: It is proposed to reuse REFSENS -108.2dBm for the new TDD NTN NB-IoT operating band.

# 3 Conclusion

In this contribution, we shared some views on the impact on RF due to the periodic pattern and the observations are made as follows:

Proposal 1: It is proposed to start from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) for the range of both NDL and NUL and channel numbers for the new TDD NTN NB-IoT operating band can be defined as Table 2.1-1.

Table 2.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [249] | 100 | 1616 | [196607] | [196607-<1>-196711] | 1616 | [196607] | [196607-<1>-196711] |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Proposal 2: It is proposed to reuse REFSENS -108.2dBm for the new TDD NTN NB-IoT operating band.

# 4 Reference

- RP-243293, Revised WID on introduction of IoT-NTN TDD mode, Iridium Satellite LLC

- R4-2419795, Way Forward for [113][317] IoT_NTN_TDD, Moderator (Qualcomm Incorporated)

- R4-2419720, Discussion on the First Introduction of an NTN NB-IoT TDD Frequency Band, THALES