3GPP TSG-RAN WG4 Meeting #114                                                          R4-2500634

Athens, GR, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | Rev | - | Current version: | - |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on NTN less than 5MHz (9.2C NR intra-frequency measurements for SAN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-2-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is not any RRM requirements for NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  |  |  |  |
| Summary of change: |  | To define the intra-frequency measurement requirements for NTN less than 5MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2C.5, 9.2C.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of Change #1 ---

9.2C.5	Intra frequency measurements without measurement gaps

9.2C.5.1	Intra frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2C.5.1-1

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2C.5.1-2

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2C.5.2-1

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and

if SMTCs do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

if SMTCs partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

-	NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap is the number of SMTCs that partially overlap with each other.

CSSFintra: it is a carrier specific scaling factor and is determined

according to CSSFoutside_gap,i in clause 9.1.5.1.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where

-	If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP_max is the maximum MGRP across all configured per-UE measurement gap. Otherwise, MGRP_max is the MGRP of configured measurement gap.

-	Starting from the beginning of any SMTC occasion:

-	Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and

-	Navailable_SAN is the number of SMTC occasions within the window W that don’t collide with any non-dropped MG occasion within or outside the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1C.8.3. The collision rule between SMTC occasion and measurement gap occasion is defined in section 9.1C.9.1

Kp = [1] when Navailable_SAN = 0 and measurement gap sharing in clause 9.1.2.1a shall apply.

Kp = 1 when intra-frequency SMTC is fully non overlapping with measurement gaps.

For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

Klayer1_measurement: it is scaling factor for sharing between L3 and L1 measurement, and Klayer1_measurement =1, if GEO satellites are measured on the carrier, or if LEO satellites are measured on the carrier and UE supports parallelMeasurementWithoutRestriction, otherwise

Klayer1_measurement =1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement =1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index

Table 9.2C.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5 x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2C.5.1-2: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2C.5.1-3: Time period for time index detection (FR1) for a target cell with 12 or 15 PRB SSB[TBD]

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( [7] x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x [7] x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil([7] x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The UE is allowed to skip measurements on intra-frequency cells, in the interval between t-serviceStart and the satellite switch completion, when it is performing soft satellite switching with resynchronization. In this case, for the measurement initiated but not completed before the beginning interval, the total time to detection can be longer.

The requirements in clause 9.2C.5.1 and 9.2C.5.2 are not applicable when the overall overhead ratio due to scheduling restriction caused by all configured SMTCs (i.e. scheduling restriction overhead of all SMTCs in one SMTC periodicity), is larger than 75%.

9.2C.5.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 9.2C.5.2-1.

If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra

Table 9.2C.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

--- End of Change #1 ---

--- Start of Change #2 ---

9.2C.6	Intra-frequency measurements with measurement gaps

9.2C.6.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2C.6.2-1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2C.6.2-2.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2C.6.3-1.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. When the UE is configured with concurrent measurement gaps and the two measurement gaps are fully overlapping with MGRP=160ms, Kgap = 2. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3.

Kgap is only applicable for UE supporting parallelMeasurementGap-r17. When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0, or if one SMTC overlaps more than one MGs associated to the frequency layer.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and

if SMTCs within a measurement gap do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

if SMTCs within a measurement gap partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

-	NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap is the number of SMTCs that partially overlap with each other.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

Table 9.2C.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x 5) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

Table 9.2C.6.2-2: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 3 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 3) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 3 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

Table 9.2C.6.2-3: Time period for time index detection (Frequency range FR1) [for a target cell with 12 or 15 PRB SSB][TBD]

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, [7] x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x [7]) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | [7] x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

9.2C.6.3	Intrafrequency Measurement Period

The measurement period for FR1 intrafrequency measurements with gaps is as shown in table 9.2C.6.3-1.

Table 9.2C.6.3-1: Measurement period for intra-frequency measurements with gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5) x Kmulti_SMTC x Kgap x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

The UE is allowed to skip measurements on intra-frequency cells, in the interval between t-serviceStart and the satellite switch completion, when it is performing soft satellite switching with resynchronization. In this case, for the measurement initiated but not completed before the beginning of this interval, the total time to measure can be longer.

--- End of Change #2 ---