3GPP TSG-RAN WG4 Meeting #114                        R4-2500637

# Athens, Greece, 17th – 21nd Feb, 2025

Agenda item:			7.11.2

Source:	Xiaomi

Title:	Discussion the assumptions and parameters for NR NTN Ku Band coexistence study

Document for:	Discussion

Background

The General requirements of the Rel-19 NR NTN Ku band work item had been discussed in RAN4#114. This document captured the Xiaomi’s opinions about the remaining issues of Ku-band NTN coexistence study parameters and assumptions.

NTN Ku band Coexistence study based on ITU regulations

NTN Ku band parameters

Phase-array or flat-panel antenna VSAT of NTN Ku-band should consider the typical values and the maximum values of the parameter. Therefore, we encourage satellite vendors to provide more detailed information regarding this type of VSAT.

Proposal 1: Encourage satellite vendors to provide more information regarding Ku-band flat-panel VSAT.

Proposal 2: Reuse antenna patterns defined in TR38.921 as a starting point.

TN Ku band parameters

It is easy to obverse that the frequency band is close to the new IMT band of 14800-15350MHz which already have sharing study results in RAN4. With this study the TN parameter introduced in the TR38.922 which could serve as a valuable reference of the Ku-band TN system parameters. And consider the work efficiency of RAN4 and the similarity between the two band:

Proposal 3: Propose RAN4 to consider the assumptions and parameters in the TR38.922 of 14800-15350MHz as the starting of the TN Ku-band BS and UE assumptions and parameters.

Proposal 4: Consider 7 dB Noise figure for BS and 8dB Noise figure for UE.

ACLR model and parameter

The adjacent channel model and parameter of Ku-band can refer to clause 5.1.1 in TR 36.942 as baseline, and further discuss the characteristic of this band to decide the ACIR model.

Proposal 5: ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline.

Proposal 6: Consider the same TN and NTN UE number as the previous NTN co-existence studies as below.

|  | TN | NTN |
| --- | --- | --- |
| The number of active UE (DL) | 1 | 1 |
| The number of active UE (UL) | 3 | 10 |

Channel model

In the previous NTN co-existence studies, RAN4 already introduced a group of channel model which is stable to use. The different propagiton models between TN and NTN nodes already been discussed serveral times. It is  Therefore, in this study we can keep those channel model.

Proposal 7: RAN4 can consider the follow propagation model in the following table.

Other co-existence study models

For the rest of the models that should considered during the co-existence study, it is able to reuse the existing models.

Proposal 8: It is proposed to reuse transmission power control model in the clause 6.2.6 of TR38.863.

Proposal 9: It is proposed to reuse Received power model in the clause 6.2.7 of TR38.863.

Proposal 10: It is proposed to reuse Performance metric in the clause 6.2.8 of TR38.863.

Proposal 11: It is proposed to reuse Throughput ~ SNR mapping model in the clause 6.2.9 of TR38.863.

Conclusion

Proposal 1: Encourage satellite vendors to provide more information for Ku-band flat-panel VSAT.

Proposal 2: Reuse antenna patterns defined in TR38.921 as a starting point.

Proposal 3: Propose RAN4 to consider the assumptions and parameters in the TR38.922 of 14800-15350MHz as the starting of the TN Ku-band BS and UE assumptions and parameters.

Proposal 4: Consider 7 dB Noise figure for BS and 8dB Noise figure for UE.

Proposal 5: ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline.

Proposal 6: Consider the same TN and NTN UE number as the previous NTN co-existence studies as below.

|  | TN | NTN |
| --- | --- | --- |
| The number of active UE (DL) | 1 | 1 |
| The number of active UE (UL) | 3 | 10 |

Proposal 7: RAN4 can consider the follow propagtion model in the following table.

Proposal 8: It is proposed to reuse transmission power control model in the clause 6.2.6 of TR38.863.

Proposal 9: It is proposed to reuse Received power model in the clause 6.2.7 of TR38.863.

Proposal 10: It is proposed to reuse Performance metric in the clause 6.2.8 of TR38.863.

Proposal 11: It is proposed to reuse Throughput ~ SNR mapping model in the clause 6.2.9 of TR38.863.