3GPP TSG-RAN WG4 Meeting #114	                                                                                            R4-2500640

Athens, GR, 17 Feb –21 Feb, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT device 1 RF requirements

Agenda item:	7.24.2.2

Document for:	Discussion

- Introduction

In RAN#106, a new WI “Solutions for Ambient IoT (Internet of Things) in NR” [1] is approved. This WI is based on the SI study in RAN WGs. The scope of the WI is limited that only device 1 is studied. This contribution provides our analysis for device 1 RF requirements based on the discussion in SI phase [2].

- Discussion

  - Tx requirements

According to the WF [3] in SI phase, the following requirements will be defined for Device 1,

- backscattering power or power backscattering loss

- Occupied bandwidth

The following requirements need more discussion.

- Modulation quality requirements

- Unwanted emissions or SEM and spurious emissions

2.1.1 Backscattering power

For backscattering power, the backscattering loss needs some discussion. In TR 38.769, the following is assumed for the in the link budget template.

| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | OOK: 6 dB PSK: 0 dB FSK: Y dB It is applicable for device 1 and 2a. Companies to report and justify their assumptions for Y. Companies to report in row 3D if they assume any additional related loss. |
| --- | --- | --- | --- |

In the WID, it’s clarified that D2R backscattering supports only OOK and BPSK modulations. In TR 38.769, backscatter loss is 6dB for OOK modulation, and 0dB for PSK. The understanding of backscatter loss in RAN1 and RAN4 may be different. According to RAN1 discussion, 6 dB comes from the OOK modulation 3 dB power loss and DC removal 3dB loss in reader side. So from device 1 radiated power aspect, our understanding is that only the OOK modulation 3 dB power loss should be considered. To avoid the misunderstanding, maybe BPSK can be used for the output power. For radiated power, use one of OOK or BPSK could be ok.

There’s another issue needs to be discussed that if power loss due to device 1 nonideal impedance match should be considered. Assuming there’s power loss L from hardware, the OOK backscattered radiated power can be calculated as

PR, OOK = PI – L – 3

The BPSK backscattered radiated power can be calculated as

PR, BPSK = PI – L

The backscatter loss L from hardware needs more discussion.

Proposal 1: The backscatter loss L from hardware needs discussion and decision in RAN4.

For the maximum input power to device, it can be calculated according to the assumptions in the co-existence simulation. Reader output power is assumed as 33 dBm and the minimum distance between reader and device is 1m. FSPL for 1m distance of 900MHz is ~31 dB. To have some margin, the maximum input power to device can be assumed approximately as 3 dBm.

Proposal 2: The maximum input CW power to calculate device 1 radiated power can be assumed as 3 dBm.

2.1.2 Occupied bandwidth

For occupied bandwidth, firstly it should be clarified how to understand it. Should it be understood as the BW of 99% transmitted power or the definition in TR 38.769, i.e. the transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R. We discussed this aspect in our contribution [4]. So for the occupied BW, the meaning of the occupied BW should be clarified first to further discuss the requirement.

Proposal 3: How to understand the occupied BW should be clarified first, i.e. if it’s the definition in TR 38.769 or the BW of 99% transmitted power.

If the understanding is the BW of 99% transmitted power, we think following NR approach is ok.

Proposal 4: If OBW is understood as the BW of 99% transmitted power, the OBW can be defined as following,

The occupied bandwidth for all transmission bandwidth configurations shall be less than the channel bandwidth.

2.1.3 Modulation quality

Modulation quality is TBD in SI phase. Modulation quality is one of the valid RF requirements to guarantee the link performance. For AIoT device 1, frequency error is not easy to be defined as a RF requirement because CW is generated outside the device. However, the timing of the chip should be in a relatively accurate level to make the reader received the signal in the correct timing point. For R19 WI, small frequency shift is supported for device 1, so the signal quality for different frequency shift should also be guaranteed. Therefore, we have the following proposal.

Proposal 5: Modulation quality requirement is defined for Device 1.

If the above proposal can be approved, the problem is how to define and test the device 1 modulation quality. EVM is usually a measure of modulation quality and error performance in complex wireless systems. Device 1 backscattering signal is not complex signal although BPSK can be modulated. And there’s no constellation for OOK modulation. So whether EVM is still suitable for device 1 Tx modulation quality requirement needs discussion. Our preliminary understanding is that BER or BLER may be considered for both OOK and BPSK modulation quality requirement. Whether all of the frequency shift positions should be tested also needs discussion, RAN4 need to wait RAN1 conclusion related to frequency shift.

Observation 1: EVM may not be a suitable methodology for AIoT device 1 Tx modulation quality requirement, BER or BLER can be a candidate for further discussion.

Observation 2: How to test the frequency shift performance needs further discussion.

2.1.4 Emission requirement

For the emission requirement, our understanding is that it should be tested to limit the emission. How to name the requirement may not be that important. SEM and ACLR are defined for NR UE for the emission near by the Tx channel, spurious emission is defined for the emission relatively outside the OOB boundary. In ETSI EN 302 208, tag unwanted emission requirement is defined as following for lower band. In the figure, it’s called spectrum mask.

It doesn’t matter much if the requirement is named as unwanted emission or spectrum mask. The frequency range for the requirement should cover the emission range of SEM and spurious emissions.

Proposal 6: Unwanted emission or spectrum mask requirement should be defined. The frequency range for the requirement should cover the emission range of SEM and spurious emissions.

The details of the requirement need more discussion considering both regulation and device RF performance.

  - Rx requirements

According to the WF [3] in SI phase, the following requirements will be defined for Device 1,

- Reference sensitivity

The following requirements need more discussion.

- Maximum input power

- Interference rejection

2.2.1 Reference sensitivity

For reference sensitivity, NR REFSENS approach can’t be used that no throughput in AIoT system. BER or BLER may also not be very suitable. If BER or BLER approach is used, device needs to decode enough number of bits to measure 1% BLER. Success rate is used to test integrator’s REFSENS in [5], it may be a candidate for device 1 REFSENS requirement.

Proposal 7: Success rate can be considered as the criteria for device 1 REFSENS requirement.

2.2.2 Maximum input power

The maximum input power is TBD in SI stage. For device 1, there’re two kinds of input power, i.e. R2D signal and CW signal. For CW signal, it may not be very necessary that the maximum radiated power requirement is assumed that the input CW signal is the maximum input CW signal. For R2D signal, there’s no AGC issue for device 1, we’re not sure if it’s very necessary. Generally, we think device 1 is very low-cost device, so defining only necessary requirements bring benefits to the commercial success. But we don’t have strong opinion.

Proposal 8: If maximum input power requirement is defined, it should be the R2D signal not CW signal.

2.2.3 Interference rejection

Interference rejection requirement is originally proposed to consider the interference from NR system. The R19 WI only includes the D1T1-B scenario, so we think interference rejection requirement is not needed for R19 WI.

Proposal 9: Interference rejection requirement is not needed for the D1T1-B deployment scenario.

- Conclusion

This contribution provides our views for the device 1 RF requirements. We have the following proposals,

For Tx requirements,

Proposal 1: The backscatter loss L from hardware needs discussion and decision in RAN4.

Proposal 2: The maximum input CW power to calculate device 1 radiated power can be assumed as 3 dBm.

Proposal 3: How to understand the occupied BW should be clarified first, i.e. if it’s the definition in TR 38.769 or the BW of 99% transmitted power.

If the understanding is the BW of 99% transmitted power, we think following NR approach is ok.

Proposal 4: If OBW is understood as the BW of 99% transmitted power, the OBW can be defined as following,

The occupied bandwidth for all transmission bandwidth configurations shall be less than the channel bandwidth.

Proposal 5: Modulation quality requirement is defined for Device 1.

Proposal 6: Unwanted emission or spectrum mask requirement should be defined. The frequency range for the requirement should cover the emission range of SEM and spurious emissions.

For Rx requirements,

Proposal 7: Success rate can be considered as the criteria for device 1 REFSENS requirement.

Proposal 8: If maximum input power requirement is defined, it should be the R2D signal not CW signal.

Proposal 9: Interference rejection requirement is not needed for the D1T1-B deployment scenario.

- References

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106

[2] TR 38.769

[3] R4-2420470, “WF on RF requirements for A-IoT”, RAN4#113, Huawei

[4] R4-2500639, “Discussion on AIoT system parameters”, RAN4#114, Xiaomi

[5] ETSI EN 302 208