# wi3GPP TSG-RAN WG4 Meeting #114	R4-2500647

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.1.1.2.1

Source: 	Xiaomi

Title: 	Power domain enhancements for single carrier

Document for:	Discussion

- Introduction

In Rel-19 UE RF WI, one of major objective was to study and specify power enhancement on single carrier with emission requirements relaxtion:

- Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM

In this contributions, views on remaning open issues were provided.

- Discussion

Sub-topic 1-1: Approaches to enable MPR reduction for both scenario 1 and scenario2

Extended CBW approach was adopted for MPR reduction in last RAN4 meeting. From deployment scenario and network scheduling perspective, extended CBW/extended RB numbers on each side of UE CHBW can have wide range with the granularity from zero to half of maximum CHBW in FR1 i.e., 50MHz CHBW.  On the other side, the concept of extended CBW was used to introduce MPR reduction by converting outer RB allocation to inner RB allocation. From RAN4 requirements perspective, we shall consider the actual impact to MPR reduction, evaluation work and test effort to decide confined extend CBW range or fixed values for RAN4 requirement introduction.

Observation 1: Extended CBW approach adopted to serve following purpose

- Specify MPR applicability from outer RB allocation to inter RB allocation

- Specify requirement applicability between IBE and OOBE

Proposal 1: RAN4 shall decide confined fixed value(s) for extended CBW (fixed ratio to UE CHBW) with associated MPR and requirement applicability rules.

Based on evaluated results from several companies in previous RAN4 meetings, half BW extension on each side can enable inner RB MPR for whole region of UE CHBW. In order to convert all outer RB allocation to inner RB allocation on channel edge, the extended BW on one side shall not less than half of UE CBW. Otherwise, there are still inner RB and outer RB allocation under extended CBW, RAN4 needs to spend lots of effort to evaluate different cases to judge the suitable MPR or change the inner RB/outer RB definition with multiple breaking points. This will complicate the specification, and test and make the feature difficult to be implemented and deployed.

The intrinsic reason behind of MPR reduction was to revise the applicable frequency range between OBE (ALCR and SEM) and IBE i.e., from the channel edge of UE CBW to extended CBW with channel edge extension without degrading the system performance under target scenarios (scenario 1-1,1-2 and scenario 2 mentioned in the WF [1]).

In ITU regulation, 250% of actual bandwidth criteria (from transmission bandwidth center) was used to decide the frequency boundary between SEM/ACLR and spurious emission.  In existing RAN4 requirements, OOB boundary FOOB was specified as CBW+5MHz from channel edge.  Shifting the SEM applicable frequency range shall not conflict with ITU regulation which means the upper bound of extended BW on one side shall not exceed CBW-5MHz.

Proposal 2: The extended BW on one side shall be in the range of CBW/2 ~ CBW – 5MHz.

Proposal 3: Fixed extended BW on one side as CBW/2 for initial MPR evaluation work.

Sub-topic 1-2: Applicable requirements

IBE requirements

RAN4 agreed that IBE shall be applied in the range between the edge of extended CBW and edge of UE CBW instead of SEM.  The opening question is whether IBE applied for the guard-band in UE CBW and/or extend CBW.

From UE implementation perspective, we didn’t see the difficulty to meet the IBE within guard-band of UE CBW or extend CBW. The original reason why existing IBE not applied during guard-band was due to no RB is expected to be scheduled on guard-band which was served as transient frequency gap for channel filtering. However, in Rel-15 NR for wider BW operation with asymmetric CBW in network and UE side, it’s not precluded NW can schedule transmission in the guard-band from one carrier to another carrier with partially overlapped CHBWs for different UEs.

Proposal 4: Extension of the IBE is used in the guard band of UE CBW, but not used in the guard-band of extended CBW.

For the applicable of IBE during the range between the edge of extended CBW and edge of UE CBW shall be same value as last unused RB in original UE CHBW with  flat value; and the NRB and LRB shall follow original UE CHBW allocation.

For fully allocation scenario, IBE can be transformed as following:

max-25,20⋅log10EVM-3,-57dBm+10log10SCS/15kHz-PRB

Proposal 5: The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW.

- NRB and LRB shall follow original UE CHBW allocation.

- For fully RB allocation case: IBE following the equation as following: max-25,20⋅log10EVM-3,-57dBm+10log10SCS/15kHz-PRB

Boundary to apply ACLR and SEM

ACLR and SEM should be applicable from the edge of extended UE CBW. Regarding the OOBE requirements whether shall be based on UE CBW or based extended CBW, we proposed to use UE CBW instead of extended CBW otherwise we may fail to meet ITU regulation on boundary between OOBE and spurious emission and/or degrade adjacent system performance with unintended relaxed requirements.  The boundary between SE and OBUE from channel edge of UE CBW shall be not larger than 200% UE CHBW.

Proposal 6: The OOBE requirements are based on UE CBW.

Proposal 7: The boundary of SE and OOBE as UE CHBW +5MHz from the channel edge of extended CBW.

Sub-topic 1-3: Signaling and others

On signalling aspects i.e., how to indicate extended RBs/BW location is strongly depending on the discussion/decision on Sub-topic 1-1 for possible extended BW/RB number on one side and associated MPR applicability.

From deployment perspective, the extended BW needs to be combined with BS channel bandwidth or aggregated bandwidth for multi-carrier depending deployment scenario. However, from UE perspective, UE only needs to know the applicable requirements and MPR values with extended BW, the deployment scenario and BS channel bandwidth or aggregated bandwidth can be transparent to UE. Network shall take the responsibility to guarantee system performance without degradation on adjacent system and take proper scheduling to enable the feature or not.

If RAN4 agreed to fix the extended BW on one side as half/BW of UE CBW for inner RB MPR applicability, then UE only needs to know whether enabling power boosting or not and/or on which sides or both sides as well as network can guarantee the actual BS bandwidth or spectrum holding can be equal or large than the gap specified in RAN4 for extended BW.

Observation 2: Actual BS channel bandwidth/aggregated bandwidth and deployment scenarios can be transparent to UE.

Proposal 8: Postpone the decision and discussion on the detailed signalling on extend CBW location and width which pending on sub-topic 1-1 decision.

3		Conclusion

In this controbution, views on remaining open issues are provided.

Sub-topic 1-1: Approaches to enable MPR reduction for both scenario 1 and scenario

Observation 1: Extended CBW approach was adopted to serve following purpose

- Specify MPR applicability from outer RB allocation and inter RB allocation

- Specify requirement applicability between IBE and OBE

Proposal 1: RAN4 shall decide confined fixed value(s) for extended CBW (fixed ratio to UE CHBW) with associated MPR and requirement applicability rules.

Proposal 2: The extended BW on one side from the channel edge of UE CBW shall be in the range of UE CBW/2 ~ UE CBW – 5MHz.

Proposal 3: Fixed extended BW on one side as CBW/2 for initial MPR evaluation work.

Sub-topic 1-2: Applicable requirements

Proposal 4: Extension of the IBE is used in the guard band of UE CBW, but not used in the guard-band of extended CBW.

Proposal 5: The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW.

- NRB and LRB shall follow original UE CHBW allocation.

- For fully RB allocation case: IBE following the equation as following: max-25,20⋅log10EVM-3,-57dBm+10log10SCS/15kHz-PRB

Proposal 6: The OOBE requirements are based on UE CBW.

Proposal 7: The boundary of SE and OOBE as UE CHBW +5MHz from the channel edge of extended CBW.

Sub-topic 1-3: Signaling and others

Observation 2: Actual BS channel bandwidth/aggregated bandwidth and deployment scenarios can be transparent to UE.

Proposal 8: Postpone the decision and discussion on the detailed signalling on extend CBW location and width which pending on sub-topic 1-1 decision.

Reference

[1] R4-2420527, WF on Power domain enhancement, Huawei