# 3GPP TSG-RAN WG4 Meeting #114	R4-2500648

# Athens, GR, 17th – 21st February, 2025

Agenda Item:			7.10.2.2.1

Source: 	Xiaomi

Title: 						RF requirements for NR-NTN

Document for:	Discussion

- Introduction

Rel-19 NTN agreed to introduce HPUE for NR-NTN operation for uplink coverage improvement.

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, views on remaining open issues were provided.

- Discussion

ACLR requirements

In last RAN4 meeting, initial conclusion reached for co-existence study as following:

Issue 1-8 to Issue 1-10: Required ACIR/ACLR for from co-ex study Agreement: Based on the results collected till RAN4#113, observations as following for the required ACLR from co-existence study evaluation perspective: For NR-NTN PC2: - Less than 22 dB with 1.5km isolation distance assumed (baseline assumption) - [26.5 dB] with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For NR-NTN PC1.5: - Less than 22.9 dB with 1.5km isolation distance assumed (baseline assumption) - [30.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For NR-NTN PC1: - Less than 23.7 dB with 1.5km isolation distance assumed (baseline assumption) - [28.3/32.7] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case

On isolation distance, we prefer to align with previous assumption the deployment scenario and restriction shall be consistency independent to device power classes which belongs to UE capability.

For TN operation, following ACLR is specified based on co-existence:

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

On NTN operation, the critical issue is coverage due to long distance between satellite over the space to terminal. With reasonable ACLR, it will bring benefits to improve uplink coverage with less MPR and or A-MPR.

Based on above consideration, we suggest to specify following ACLR values:

Proposal 1: Specify NR-NTN HPUE ACLR as following:

- PC2: 22 dBc

- PC1.5: 23 dBc

- PC1: 24 dBc

Target device types and device type assumption

In general, we prefer to leverage existing TN network design on FDD bands as much as possible with following thinking:

- Harmonize the common design between TN and NTN operation for echo-system

- Shared RF chain between NTN and TN operation on similar frequency range for handheld device for cost and form factor limitations

- Reduce RAN4 workload on leverage the conclusion from TN to NTN

And in current FR1 FDD TN operation, following status can be summarized:

- FDD PC3: 1Tx and handheld device

- FDD PC2: 1Tx and applicable for handheld device

- FDD PC1.5: not support for FDD bands, TDD bands via 2Tx

- FDD PC1: only for non-handheld device with 1Tx

With above observations, our preference as following:

Proposal 2: On target device type and UE architecture assumption, RAN4 focus below cases in Rel-19

- For PC2, both 1Tx and 2Tx can be considered and 1Tx shall be prioritized.

- For PC1.5, prioritize non-handheld device types with 1Tx

- For PC1, only focus on non-handheld device with 1Tx

- Conclusion

In this contribution, views on remaining open issues were provided.

Proposal 1: Specify NR-NTN HPUE ACLR as following:

- PC2: 22 dBc

- PC1.5: 23 dBc

- PC1: 24 dBc

Proposal 2: On target device type and UE architecture assumption, RAN4 focus below cases in Rel-19

- For PC2, both 1Tx and 2Tx can be considered and 1Tx shall be prioritized.

- For PC1.5, prioritize non-handheld device types with 1Tx

- For PC1, only focus on non-handheld device with 1Tx

Reference

[1] R4-2420403, WF on NR NTN HPUE, Samsung