3GPP TSG-RAN WG4 Meeting # 114                                R4-2500696

Athens, GR, 17th – 21st February, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion impacts on requirements of AI/ML mobility

Agenda Item:	7.21.2

Document for:	Approval

- Introduction

In RAN plenary #105 meeting, a revised SID[1] for Rel-19 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this SID are listed as follows.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this document, we will provide some views on requirements impacts based on RAN2 assumptions of AI/ML for mobility from the RRM perspective.

- Discussion

In previous meeting, we discussed the priorities of different use cases and have reached consensus that RAN4 to start the discussion with RRM measurement prediction use-case [2]. For measurement event prediction use-case and RLF use-case, more progress is needed from RAN2, and RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2.

RAN4#112bis: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

RAN2 has discussed measurement event prediction use-case many times and reached some agreements. To our knowledge, RAN2 focused on discussing simulation assumptions based on measurement event prediction in previous meetings and plans to provide simulation results in this meeting. The progress of RAN2 is replicated as follows.

RAN2#127bis Agreements - Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement). FFS what is KPI. - Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing. Focus on temporal case B. - Companies will prioritize simulations on indirect method. Companies can bring simulations on direct method and should report what method is being used. - Measurement event prediction results are expected in RAN2#129 Agreements on simulation assumptions - The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified. - Companies can pick and report what they are using for filtering options (similar to RRM prediction) - Companies will focus on sub-case 2 for measurement event prediction. Companies can simulate other sub-cases if they wish and report what they are using. - Leave the simulation parameter discussion for email discussion. Pick only one value for A3. - It is up to company implementation how to model UE behavior after A3 event is trigger. Focus on intermediate KPIs for this exercise. System level KPI is FFFs - For measurement event prediction, traffic is not simulated. Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference.

Based on RAN2’s progress, we perfer RAN4 to discuss both indirect measurement event and direct measurement event prediction. Meanwhile, RAN2 has an agreement that the simulation assumption of RRM measurement prediction can be reused unless otherwise specified, so we can apply the analysis results of the simulation parameters for RRM measurement prediction to measurement event prediction.

Proposal 1: RAN4 should discuss the RRM impact on indirect measurement event prediction and direct measurement event prediction.

Proposal 2: Measurement error should be considered into measurement event prediction, which is similar as RRM measurement prediction.

In our understanding, the current focus should continue to be on RRM measurement prediction, and we provide our views on RRM impact on RRM measurement prediction use-case in the follow.

In R18 AI/ML for air, there were several typical use cases incorporated, including CSI feedback enhancement, beam management enhancement and positioning enhancement. Since L3 measurement is based on filtering of L1 measurement, the study of AI/ML for air can be leveraged for mobility purpose. Majority of the study of beam management in AI/ML for air can be reused when we extend L1 beam measurement from serving cell to neighbor cell.

It’s noted that both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. We can observe that there are three cases need to be considered for cell level measurement. For case 1, beam level results are predicted first and then cell level results are derived. For case 2 and case 3, cell level results are predicted directly. For beam level measurement prediction, under the discussion in RAN2, L3 filtered beam level prediction cases are lower priority and there are three cases as well.

=> Cell level measurement prediction model For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results => Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

According to the current assumption of RAN2, evaluation scenarios for RRM measurement prediction can be observed from blew table.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain caseA | 2ndgoal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain caseB | 1stgoal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1stgoal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2ndgoal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1stgoal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1stgoal | Intra-cell |

we noticed that intra cell is mentioned in methodology. It needs to be clarified that intra cell prediction in RAN2 includes two scenarios: serving cell prediction for serving cells and neighbor cell prediction for neighbor cells. Therefore, RAN4 should consider both serving cell and neighbor cell measurment prediction for intra-cell.

Proposal 3: RAN4 should consider both serving cell and neighbor cell measurment prediction for intra-cell prediction. Specifically, serving cell (input) to predict the serving cell (output) and neigbour cell i (input) to predict the neigbour cell i (output).

In the pervious meeting, we have reached agreement that define requirement for L3 cell level RSRP prediction, and further dicussion is needed for L1 beam-level and L3 beam-level. In the current test case for delay and accuracy, the results for A1 and C are the same if the filtering is closed in test case. Meanwhile, some company mentioned that L1-RSRP is for beam management and perfers not to consider point A1 in AI mob. It’s fine for us not to consider point A1.

Proposal 4: Due to L1-RSRP is for beam management, it’s fine for us not to consider point A1 in AI Mob.

As to point E, which is related to beam level measurement prediction, we should discuss whether to support beam level measurement prediction in RAN4 first. According to RAN2’s agreement, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4). Therefore, RAN4 should deprioritize the beam level measurement prediction.

Observation1: Both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. And according to RAN2’s agreements, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4).

Proposal 5：RAN4 to deprioritize studying the impacts on requirements for L3 beam-level prediction.

Proposal 6: If L3 filtered beam level prediction cases are lower priority, the definition of prediction accuracy for point E will correspondingly be given low priority.

For measurement prediction accuracy of cell-level measurement prediction, metrics for measurement prediction accuracy defined in RAN2 can be used as start point. In the last meeting, we have discuss this issue online and reached consensus blew.

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

In this meeting, we should finish the definition on ground truth of L3-RSRP. We have agreement in RAN4#112bis that consider conducted testing as a beseline option for testing in FR1. Based on this, we can use the transmitted power as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test. Meanwhile, for FR2, RAN4 to leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. So, we can reuse the definiation of ground truth on R19 AI BM.

Proposal 7: For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test.

Proposal 8: For FR2, the definiation of ground truth on R19 AI BM can be reused, that is, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP).

Based on the RAN2’s agreement, we should define the prediction accuray of the cell-level measurement prediction, and RSRP difference to the actual measurement should be defined. Meanwhile, measurement reduction rate is also be considered as one KPI and it can be depotitized compared to measurement prediction accuracy.

RAN2#126 => Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases => Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) - Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: - MRRT= skipped measurement time instances / total measurement time instances - Measurement reduction rate in spatial domain (MRRS): - MRRS = skipped beams to be measured/ total beams to be measured

RAN2 has defined measurement reduction for intra-frequency scenario, including MRRT and MRRS. Measurement reduction rate in temporal domain (MRRT) is the ratio between skipped measurement time instances and total measurement time instances, and measurement reduction rate in spatial domain (MRRS) is tha ratio between skipped beams to be measured and total beams to be measured. In our understanding, these two indicators can be used to evaluate the benefits brought by AI models in mobilioty.

Proposal 9: RAN4 should study the RRM impact of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances.

For RRM measurement prediction, we should discuss scenario 2, 3, 4 for cell level measurement prediction. For intra-frequency temporal domain prediction, both caseA and caseB, it can observed that UE will skip some measurement occasions, which means that only subset of measurement occasions are measured so that SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy.

Intra-frequency temporal domain case A: In case A, continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. Figure 3 Example of temporal domain case A Intra-frequency temporal domain case B-To be discussed in Phase 2: In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) after measurement result(s) in previous prediction window is/are skipped. Figure 4: Example of temporal domain case B (Note) Note: Other patterns are not excluded

For inter-frequency inter-cell (frequency domain), not all configured MOs need to be measurement, just need to consider the actually measurement frequency layers when we define measurement delay requirements. Thus, the CSSF assumptions need to be updated.

Observation 2: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements.

Proposal 10: RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility.

As to the measurement error, many companies proposed this issue last meeting and we have initial discussed online. Finially, there is no consensus on how to handle the measurement error.

Issue 2-1-3: Impacts of measurement error on prediction accuracy - Candidate options: - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. Way Forward: Many companies are supporting Option 4. Companies are encouraged to bring proposals on what input RAN4 should provide to RAN2 to consider impact of error on measurements used for prediction on the accuracy of the predicted measurement/event.

In our view, this is entirely a RAN4 issue, and even if LS is sent to RAN2 to consider measurement errors in the simulation, RAN2 still requires a lot of assistance from RAN4. So for this issue, we prefer to do the evaluation in RAN4. In addition, the R19 AI/ML BM is currently discussing the impact of measurement error on the prediction accuracy requirements in RAN4, so it is reasonable to complete them in RAN4.

Similar as AI/ML BM, when defining the prediction accuracy requirements, the impact of measurement error should be considered. The methodology shall be similar too. In the current discussion of AI BM, BB errors under different signal-to-noise ratios are obtained through link level simulation, and then added to system level simulation to obtain testdata with added errors. RF errors are then added to evaluate the impact of measurement errors on system simulation. To avoid redundant discussions, we can directly utilize the simulation results of AI BM.

Proposal 11: The methodology of adding measurement error to AI mob should be similar to AI BM. To avoid redundant discussions, the simulation results of AI BM in RAN4 should be utilized as much as possible.

The preliminary results obtained by adding the BB error generated in the current AI BM to the AI Mob simulation model of RAN2 are summarized as follows.

Table 1-1. Simulation result on FR1

| RRM sub use case | UE Speed (km/h) | [observation window length, prediction window length] | Average L3 cell level RSRP difference (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BB Error=0dB | BB Error=0.3dB | BB Error=1dB |
| RRM sub use case 1 | 30 | [400ms, 200ms] | 0.29 | 0.32 | 0.41 |
|  |  | [400ms, 400ms] | [0.41, 0.43] | [0.65, 0.83] | [0.87,1.31] |
| RRM sub use case 2 | 30 | [400ms, 200ms] | 0.28 | 0.31 | 0.40 |
|  |  | [400ms, 400ms] | [0.39, 0.43] | [0.43, 0.46] | [0.51,0.53] |

Table 1-2. Simulation result on FR2

| RRM sub use case | UE Speed (km/h) | [observation window length, prediction window length] | Average L3 cell level RSRP difference (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BB Error=0dB | BB Error=0.3dB | BB Error=1dB |
| RRM sub use case 1 | 60 | [800ms, 400ms] | 0.70 | 0.77 | 0.87 |
|  |  | [800ms, 800ms] | [0.75, 1.17] | [0.81，1.29] | [0.93，1.46] |
|  |  | [1200ms, 1200ms] | [0.80, 1.24, 1.73] | [0.87，1.35，1.83] | [0.97，1.48，2.00] |
| RRM sub use case 2 | 60 | [800ms, 400ms] | 0.62 | 0.66 | 0.80 |
|  |  | [800ms, 800ms] | [0.70, 1.17] | [0.75，1.20] | [0.85，1.27] |
|  |  | [1200ms, 1200ms] | [0.74, 1.20, 1.79] | [0.80，1.25，1.84] | [0.94，1.36，1.96] |

We can observe that BB errors can affect the accuracy of predictions, and the larger the BB error, the greater the impact on prediction performance. So we can draw a preliminary conclusion that it is necessary to align with the actual scenario, and the impact of measurement errors should be considered in the simulation of RAN2.

Proposal 12: Measurement errors will affect the accuracy of predictions, and the larger the BB error, the greater the impact on prediction performance.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML monility:

Proposal 1: RAN4 should discuss the RRM impact on indirect measurement event prediction and direct measurement event prediction.

Proposal 2: Measurement error should be considered into measurement event prediction, which is similar as RRM measurement prediction.

Proposal 3: RAN4 should consider both serving cell and neighbor cell measurment prediction for intra-cell prediction. Specifically, serving cell (input) to predict the serving cell (output) and neigbour cell i (input) to predict the neigbour cell i (output).

Proposal 4: Due to L1-RSRP is for beam management, it’s fine for us not to consider point A1 in AI Mob.

Observation1: Both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. And according to RAN2’s agreements, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4).

Proposal 5：RAN4 to deprioritize studying the impacts on requirements for L3 beam-level prediction.

Proposal 6: If L3 filtered beam level prediction cases are lower priority, the definition of prediction accuracy for point E will correspondingly be given low priority.

Proposal 7: For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test.

Proposal 8: For FR2, the definiation of ground truth on R19 AI BM can be reused, that is, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP).

Proposal 9: RAN4 should study the RRM impact of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances.

Observation 2: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements.

Proposal 10: RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility.

Proposal 11: The methodology of adding measurement error to AI mob should be similar to AI BM. To avoid redundant discussions, the simulation results of AI BM in RAN4 should be utilized as much as possible.

Proposal 12: Measurement errors will affect the accuracy of predictions, and the larger the BB error, the greater the impact on prediction performance.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR , OPPO (rapporteur)

- R4-2420095, WF on RRM requirements for FS_NR_AIML_Mob, Nokia

- R4-2416855, WF on RRM requirements for FS_NR_AIML_Mob, Nokia