# 3GPP TSG-RAN WG4 Meeting #114	R4-2500732

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Further discussion on HPUE for intra-band contiguous and non-contiguous CA

Agenda Item:	7.1.1.1.2

Document for:	Approval

Introduction

In the last meeting, RAN4 further discussed MPR evaluation methodology and Pcmax,c/Pcmax issue on PC1.5 for intra-band contiguous and non-contiguous and the WF was captured in [1]. In the summary [3], more simulation results and more ways to derive MPR was raised and collected in. However, no conclusion was made. The PSD assumption and MPR applicability was also discussed. In addition, the Pcmax,c and Pcmax for intra-band NC ULCA(with dualPA) were also discussed.

In this contribution, we will continue on the discussion on PC1.5 for intra-band contiguous and non-contiguous CA.

Discussion

The WF for MPR evaluation was captured as follows:

## 1.1 MPR values for intra-band contiguous ULCA (with TxD) Agreement: - Class B would not be urgent. As common understanding, RAN4 will work on Class C in this Rel-19 WID - Companies are encouraged to provide the MPR values for both handheld UE and FWA in next meeting ## 1.2 PSD assumption to derive MPR/A-MPR requirements for intra-band NC ULCA (with dualPA) Agreement: - Equal PSD as starting point - Companies are encouraged to check the MPR difference with 6dB PSD imbalance - Inform RAN5 about the PSD condition for MPR derivation in RAN4 - The MPR difference compared to PC2 is supposed to be less than 3dB for PC1.5 in order to have performance gain over PC2 - Companies to clarify the PA configuration assumptions for MPR evaluation for PC3, PC2 and PC1.5 intra-band NC UL CA, for example, (PC3+PC3) for PC3 ## 1.3 MPR applicability for intra-band contiguous and non-contiguous ULCA Proposal： - No matter how th MPR is evaluated, equal or unbalanced PSD, the MPR specified for the total power covers unequal UL PSD and transmission bandwidths across CCs. (Ericsson) Agreement: - FFS for NC CA in next meeting - Stop the discussion for contiguous CA because equal PSD has already been agreed as assumption to derive MPR requirements

MPR derivation

In last meeting, it was proposed to reuse the existing requirements as starting point, with the assistance of new simulation / measurements. In addition, some measurements were done to evaluate the MPR difference between 1Tx and 2Tx, to provide another reference to derive new requirements. Here we resubmitted these observation and proposal:

Observation 1 (Re-submission): The MPR difference between 1Tx and 2Tx considering full RB allocation is ranging from 0~1dB.

Proposal 1 (Re-submission)： PC 1.5 UL CA MPR can use exiting PC1.5 single carrier and PC2 single carrier and UL CA MPR data (as shown in below equation) as starting point, and measurement/simulation results can be used to assist in deriving MPR requirements.

PC1.5 2Tx CA MPR ~= PC2 2Tx CA MPR + [PC1.5 2Tx SC MPR - PC2 2Tx SC MPR]

MPR applicability

For MPR applicability, it has been confirmed that equal PSD was agreed as assumption to derive MPR for contiguous CA in earlier stages, and still FFS for NC CA.

In our understanding, we support Ericsson’s proposal that:

“No matter how the MPR is evaluated, equal or unbalanced PSD, the MPR specified for the total power covers unequal UL PSD and transmission bandwidths across CCs”

After all, the requirements would be too complicated if further differentiation were made on the different power imbalance condition. In addition, this is also aligned with current condition for contiguous CA. Some reasonable margin could be considered to account for the imbalance effect, but the final requirements should be power balance/imbalance agonistic.

Proposal 2: The MPR requirements for intra-band NC ULCA should be power balance/imbalance agonistic.

Pcmax,c and Pcmax Equations (Intra-band NC ULCA dualPA)

In the WF, there was following agreement:

Agreement:

- PCMAX,C is upper bounded by 26dBm

  - How to reflect it in spec is FFS

- For PCMAX, FFS in next meeting

  - Option 1: No change of current spec

  - Option 2: PPowerClass,CA = min{29,26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm assuming 6dB PSD difference ,

or, =26+10*log(1+min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2)) assuming equal PSD,

can be used as a starting point.

For the Pcmax equation, we tend to use option 1 here, that is no change of current spec.

The equations in option 2 have considered that fact that the total transmitted power would be restricted by the equal PSD assumption. The difference is that whether 6dB imbalance would be considered or not. The assumption is that the total power would not achieve 29dBm with the PSD restriction, thus a new lower limitation might be helpful.

Although the principles looks reasonable, it is felt that this is still a bit awkward to setting up this kind of restrictions. The equal PSD is actually not that guaranteed, and the different CC will have independent power control procedure. In addition, this is only an upper bound, and may only be used for some PHR calculation if really so. Even with some imperfections, the real performance impact may be neglectable.

Proposal 3: For Pcmax equation for Intra-band NC ULCA dualPA, keep the current spec and do not consider the PSD restriction in the equation.

Conclusion

This contribution discusses MPR evaluation framework for PC1.5 intra-band contiguous and non-contiguous CA. The following proposals are made:

Observation 1 (Re-submission): The MPR difference between 1Tx and 2Tx considering full RB allocation is ranging from 0~1dB.

Proposal 1 (Re-submission)： PC 1.5 UL CA MPR can use exiting PC1.5 single carrier and PC2 single carrier and UL CA MPR data (as shown in below equation) as starting point, and measurement/simulation results can be used to assist in deriving MPR requirements.

PC1.5 2Tx CA MPR ~= PC2 2Tx CA MPR + [PC1.5 2Tx SC MPR - PC2 2Tx SC MPR]

Proposal 2: The MPR requirements for intra-band NC ULCA should be power balance/imbalance agonistic.

Proposal 3: For Pcmax equation for Intra-band NC ULCA dualPA, keep the current spec and do not consider the PSD restriction in the equation.

- References

[1] R4-2420329, WF on HPUE RF requirements, Samsung, RAN4#113.

[2] R4-2418198, Discussion on Intra-band contiguous and non-contiguous UL CA with PC1.5, vivo, RAN4#113.

[3] R4-2418139, Topic summary for [113][117] NR_ENDC_RF_Ph4_part2, Moderator (Samsung), RAN4#113