3GPP TSG-RAN WG4 Meeting # 114 	R4-2500745

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on the RF requirement for device 1

Agenda Item:	7.24.2.2

Document for:	Approval

- Introduction

RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

  - Use band n8 as an example band

In [1], the WID for R19 AIoT was approved:

In this contribution, we provide our views on the RF requirements for device1.

- Discussion

  - General

In [2], the OTA based test method is agreed as baseline for device but conductive test is not precluded and such test may be achieved by using fixture. Considering the time is limited in R19, it is better only focus on one set of requirements. Since the OTA test method will be discussed parallel in this release, it will be more efficient to define radiated RF requirement accordingly.

Proposal 1: For device 1, only the radiated based RF requirement is defined for device 1 in Rel-19 WI.

  - Tx Requirement

- Radiated power/Backscatter loss

During SI stage, the MOP of device was discussed. Since the device 1 can only reflect the power from the power source and the power control is not available, the maximum output power seems to be meaningless for such device type. However, a requirement to guarantee the device can radiate sufficient power under a certain power level from CW source should be considered, which is actually to restrict the backscatter loss.

Observation 1: The backscatter loss of device can be implicitly defined in radiated power under a fixed incoming source power level.

Define a certain incoming power from CW source directly will make the test procedure complicated and define a certain distance between CW node and device is much easier way. In [3], the RFID spec define the output power with 20cm distance, as shown in figure I.

Figure I Illustration for device output power test in [3]

Proposal 2: The min peak EIRP of device and the certain distance between device and CW source should be defined together.

- Transmit signal quality

As described in WID, both OOK and BPSK need to be supported for device 1, so the modulation quality needs to be guaranteed. For BPSK, the EVM requirement in LTE is defined as 17.5%, which can be reused for device.

Proposal 3: For BPSK, the EVM requirement of LTE BPSK can be reused for device, i.e., 17.5%.

For OOK, such waveform is not used in traditional LTE and NR system, and it is not clear how to define the modulation quality for OOK since the EVM is usually used for QAM signal.

Observation 2: For OOK, there is no precedent of modulation quality requirement and a new requirement need to be introduced.

From OOK demodulation perspective, the OOK need the power difference between ON chip and OFF chip is clear enough, so one possible way to define modulation quality is the power ratio between ON chip and OFF chip. We can also try to use similar way as EVM in time domain. There is reference point for QAM signal for EVM calculation and for OOK, the amplitude 1 & 0 can be used as ideal signal for normalized transmitted signal. In addition, considering similar discussion exist in LP-WUS, we can also wait the conclusion.

Proposal 4: Further discuss the following requirement options for OOK modulation quality:

Option 1: Power ratio between ON chip and OFF chip.

Option 2: The EVM of OOK is defined as the average amplitude difference between normalized transmitted signal and the ideal 1&0 reference signal.

Option 3: Wait for the conclusion of LP-WUS.

Another possible transmit quality requirement is frequency error. Even though device 1 does not support large frequency shift (corresponding to CFO), small frequency shift (corresponding to SFO) is available as WID described. In RAN1, the SFO is assumed as up to 10% during SI and whether to define this as a RF requirement may need more discussion.

- Unwanted emission

Considering the radiated requirement should be defined for device, one reference is the current FR2 requirement. In FR2, the unwanted emission requirements are defined as TRP-based, including SEM, ACLR, spurious emission. However, TRP-based requirement needs to scan all test grid point which will enlarge the test time and burden on the low-cost device.

Observation 3: In FR2, the unwanted emission requirements are defined as TRP-based and the test time will be enlarged.

In R17, it was discussed that to change the FR2 emission requirement from TRP-based to EIRP-based [4]. Although the proposal is not approved, the measurement results shows that it is possible to define the unwanted emission as EIRP-based by taking the difference between EIRP and TRP into consideration.

Observation 4: It is possible to define the unwanted emission as EIRP-based by counting the difference between TRP and peak EIRP.

Proposal 5: For simplicity, the unwanted emission for device is defined as EIRP-based requirement at the peak direction to alleviate the test burden for low-cost device.

Classes of emission A1A and A1B with fluctuations

When large short-period variations of the received field are present, the specifications given below for single-channel, amplitude-modulated, continuous-wave telegraphy (Class A1A and A1B), represent the desirable performance obtainable from a transmitter with an adequate input filter and sufficiently linear amplifiers following the stage in which keying occurs

The occupied bandwidth is a regulation requirement to limit the out of band emission, which is originally from ITU-R. In ITU-SM 328 [5], the occupied bandwidth is defined for classes of emission A1A and A1B with fluctuations, as shown in below:

Considering device 1 is not implemented with amplifier, the occupied bandwidth may not be still applicable. In addition, the OOK has worse spectral efficiency and it may be quite challenge for device 1 to satisfy the traditional 99% occupied bandwidth.

Observation 5: The occupied bandwidth in ITU-R SM 328 may not be applicable for device 1.

Proposal 6: It is suggested to consider not to define the occupied bandwidth for device 1.

For LTE/NR, the out of band emission include SEM, ACLR, spurious emission which represent different regions. In [3], a composite emission mask is defined as unwanted emission requirement, as shown in Figure II

Figrue II Unwated emssion defined in [3]

Considering the ACLR is not needed for device as discussed in SI, only absolute power limit is used for emission requirement, we prefer only define a composite emission mask as unwanted emission requirement for device for simplicity.

Proposal 7: It is suggested that only a composite emission mask is defined as unwanted emission requirement for device.

  - Rx Requirement

Before dive into the specific requirement discussion, the performance metric for Rx requirement should be decided first. For LTE/NR, 5% throughput loss of reference channel is used which is not suitable for AIoT device. In LP-WUS discussion, the miss detection rate was agreed as metric for Rx requirement, which can be considered used for device.

Observation 6: For LP-WUR, the Rx requirement is defined based on miss detection rate.

Proposal 8: It is suggested that the Rx requirement of device is defined based on miss detection rate.

- REFSENS

During SI stage, the sensitivity for device 1 is assumed as -36 dBm. Unlike cellular network, the usage scenarios of AIoT may be customized. In some scenarios, the request of coverage may be small and the cheaper device can be designed and used, and it is hard to define a common REFSENS for all scenario.

Observation 7: The REFSENS is mainly to ensure the device has sufficient coverage performance and the required coverage is various which depends on the usage scenario.

One possible way to leave more flexibility for device design is to allow the manufacturer to declare its own REFSENS, and we only define the certain performance metric, e.g., 10% MDR.

Proposal 9: It is suggested to consider the device REFSENS is declared by manufacturer and the miss detection rate is less than 10% within a certain period.

- Maximum input power

If the REFSENS is to define the maximum distance between device and reader, the maximum input power is to define the minimum distance since the Rx component may not still work normally when a high-power signal hit the receiver. In SI co-existence study, 1m is assumed minimum distance between reader and device, which can used as starting point for the requirement derivation.

Proposal 10: The maximum input power for device 1 is derived based on 1m minimum distance.

- Conclusion

In this contribution, we share our initial views on AIoT device.

Proposal 1: For device 1, only the radiated based RF requirement is defined for device 1 in Rel-19 WI.

Observation 1: The backscatter loss of device can be implicitly defined in radiated power under a fixed incoming source power level.

Proposal 2: The min peak EIRP of device and the certain distance between device and CW source should be defined together.

Proposal 3: For BPSK, the EVM requirement of LTE BPSK can be reused for device, i.e., 17.5%.

Observation 2: For OOK, there is no precedent of modulation quality requirement and a new requirement need to be introduced.

Proposal 4: Further discuss the following requirement options for OOK modulation quality:

Option 1: Power ratio between ON chip and OFF chip.

Option 2: The EVM of OOK is defined as the average amplitude difference between normalized transmitted signal and the ideal 1&0 reference signal.

Option 3: Wait for the conclusion of LP-WUS.

Observation 3: In FR2, the unwanted emission requirements are defined as TRP-based and the test time will be enlarged.

Observation 4: It is possible to define the unwanted emission as EIRP-based by counting the difference between TRP and peak EIRP.

Proposal 5: For simplicity, the unwanted emission for device is defined as EIRP-based requirement at the peak direction to alleviate the test burden for low-cost device.

Observation 5: The occupied bandwidth in ITU-R SM 328 may not be applicable for device 1.

Proposal 6: It is suggested to consider not to define the occupied bandwidth for device 1.

Proposal 7: It is suggested that only a composite emission mask is defined as unwanted emission requirement for device.

Observation 6: For LP-WUR, the Rx requirement is defined based on miss detection rate.

Proposal 8: It is suggested that the Rx requirement of device is defined based on miss detection rate.

Observation 7: The REFSENS is mainly to ensure the device has sufficient coverage performance and the required coverage is various which depends on the usage scenario.

Proposal 9: It is suggested to consider the device REFSENS is declared by manufacturer and the miss detection rate is less than 10% within a certain period.

Proposal 10: The maximum input power for device 1 is derived based on 1m minimum distance.

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei)

- 3GPP TR 38.769

- ETSI EN 302 208

- R4-2207674 EIRP-based test metric for FR2 SEM verifications, Apple, RAN4#103e

- ITU-R SM.328-11